3GPP TSG-RAN WG2 Meeting #109e	 Tdoc R2-2000442

E-meeting, 24th May – 28th Feb 2019

Agenda Item:	6.2.3.1

Souce:	MediaTek Inc.

Title:	UE Capabilities for Supporting Unlicensed Channel measurements

Document for:		Discussion and decision

# 1 Introduction

Frequency of NR-U cells will be shared by un-licensed networks nodes, (e.g. WLAN nodes). Thus, measurement and report configurations in NR-U need to be enhanced to capture and report channel metrics of un-licensed network nodes and neighbouring NR-U cells. Recent studies in 38.889 [1] have clearly identified this necessity for including channel occupancy in connected mode mobility in NR-U.

For connected mode mobility, the main issue identified for NR operation in unlicensed band is the reduced transmission opportunities for different signalling due to LBT failure. The following modifications to mobility-related procedures have been identified as beneficial to study: - Modifications to mobility-related measurements and/or triggers considering limitations related to high channel occupancy. NR-U needs to consider techniques to handle increased interference levels in the unlicensed channel for mobility-related decisions. - Potential modifications to the measurement reporting quantities, to the measurement reporting triggers and to the condition used by the UE when delaying the time at which it applies a reconfiguration for mobility that are based at least on channel occupancy and RSSI should be studied.

In this paper we observe that UE capabilities in NR need to be enhanced for capturing, analysing and reporting relevant un-licensed channel metrics, like channel occupancy of NR-U cells for efficient connected and/or idle mode operations.

# 2 Discussion

Typically, gNB (network) enquires UE’s capabilities by sending UECapabilityEnquiry message and UE responds by transmitting UECapabilityInformation [2]. In NR-U, gNB will configure measurement objects for un-licensed channels and UE will use these measurement objects to measure channels and report the channel measurement results to gNB. However, the UE needs to inform the gNB if it is capable of performing un-licensed channel measurements by using UECapabilityInformation.

2.1 UE Capability for RSSI and Channel Occupancy in NR

RSSI based channel occupancy has already been included and supported in LTE Rel. 14 [3], under the capability “rssi-AndChannelOccupancyReporting-r13”. This field defines whether the UE supports measurement and reporting for RSSI and channel occupancy and is only applicable if the UE supports downlink LAA operation.

Observation 1: In LTE LAA standards, RSSI and channel occupancy have been included under the UE capability “rssi-AndChannelOccupancyReporting-r13.

Hence, it is imperative that NR-U needs to include a similar UE capability for indicating RSSI and channel occupancy measurements. The measurement metric for RSSI and channel occupancy is now decided in RAN2 [2]. Thus, RAN 2 should consider defining a suitable UE capability similar to rssi-AndChannelOccupancyReporting.

Proposal 1: UE capabilities in NR-U need to be enhanced for indicating the support of RSSI and channel occupancy measurements of un-licensed NR-U cells.

# 3 Conclusions

In this contribution, we point out the necessity of extending UE capabilities in NR-U. We show out that UE can re-use the existing Release 15 capabilities as the baseline and subsequently explore new un-licensed channel capabilities. Our observations and proposals are summarized below:

Observation 1: In LTE LAA standards, RSSI and channel occupancy have been included under the UE capability “rssi-AndChannelOccupancyReporting-r13.

Proposal 1: UE capabilities in NR-U need to be enhanced for indicating the support of RSSI and channel occupancy measurements of un-licensed channels.

# 4 References

- 3GPP TR 38.889 V16.0.0 (2018-12), Technical Specification Group Radio Access Network; Study on NR-based access to unlicensed spectrum (Release 16).

- 3GPP TS 38.306 V15.3.0 (2018-09), Technical Specification, Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio access capabilities (Release 15).

- 3GPP TS 36.306 V15.3.0 (2018-09), Technical Specification, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities (Release 15).

- R2-1903055, “Handover Improvement in NR-U” (MediaTek)