3GPP TSG-RAN WG2 Meeting #109-e	                             R2-2000492

Electronic Meeting, 28th Feb – 6 th Mar, 2020

Source:	vivo

Title:          TP on 38.331 of provisioning of timing information for EN-DC

Agenda Item:	6.7.2.1

Document for:	Discussion and Decision

- Introduction

This contribution is to provide the text proposals for the 38.331 to support the provisioning of timing information for EN-DC, based on our contribution R2-2000491.

- Text Proposal

The following text proposal is based on the latest Running CR approved in Email discussion 108#32.

------------------------------------------------Start of Change-------------------------------------------------------------------

- –	ReferenceTimeInfoNR

The ReferenceTimeInfoNR message is used to provide reference time generated by the SgNB.

Direction: Secondary gNB to Master eNB

ReferenceTimeInfoNR

-- ASN1START

-- TAG-REFERENCE-TIME-INFONR-START

ReferenceTimeInfoNR ::= SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

referenceTimeinfoNR            ReferenceTimeInfoNR-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

ReferenceTimeInfoNR-IEs ::=    SEQUENCE {

ReferenceTimeInfoNR        ReferenceTimeInfo    OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                     OPTIONAL

}

-- TAG-REFERENCE-TIME-INFONR-STOP

-- ASN1STOP

------------------------------------------------End of Change--------------------------------------------------------------------