3GPP TSG-RAN WG2 Meeting #109-e	                             R2-2000493

Electronic Meeting, 28th Feb – 6 th Mar, 2020

Source:	vivo

Title:          TP on 36.331 of provisioning of timing information for EN-DC

Agenda Item:	6.7.2.1

Document for:	Discussion and Decision

- Introduction

This contribution is to provide the text proposals for the 36.331 to support the provisioning of timing information for EN-DC, based on our contribution R2-2000491.

- Text Proposal

The following text proposal is based on TS 36.331 f80

------------------------------------------------Start of Change-------------------------------------------------------------------

- 5.6.1	DL information transfer

- 5.6.1.1	General

Figure 5.6.1.1-1: DL information transfer

The purpose of this procedure is to transfer NAS, (tunnelled) non-3GPP dedicated information or time reference information from E-UTRAN to a UE in RRC_CONNECTED.

- 5.6.1.2	Initiation

E-UTRAN initiates the DL information transfer procedure whenever there is a need to transfer NAS, non-3GPP dedicated information or time reference information. E-UTRAN initiates the DL information transfer procedure by sending the DLInformationTransfer message.

- 5.6.1.3	Reception of the DLInformationTransfer by the UE

Upon receiving DLInformationTransfer message, the UE shall:

1>	if the UE is a NB-IoT UE; or

1>	if the dedicatedInfoType is present and set to dedicatedInfoNAS:

2>	forward the dedicatedInfoNAS to the NAS upper layers.

1>	if the dedicatedInfoType is present and set to dedicatedInfoCDMA2000-1XRTT or to dedicatedInfoCDMA2000-HRPD:

2>	forward the dedicatedInfoCDMA2000 to the CDMA2000 upper layers;

1>	if timeReferenceInfo is included:

2>	calculate the time reference of PCell based on the included time, timeInfoType and referenceSFN in timeReferenceInfo;

2>	calculate the inaccuracy of the time reference of PCell based on the uncertainty and other implementation-related inaccuracies, if uncertainty is included in timeReferenceInfo;

2>	inform upper layers of the time reference and of PCell, if uncertainty is included in timeReferenceInfo, of the inaccuracy of the time reference of PCell.

1>	if timeReferenceInfoNR is included:

2>	calculate the time reference of PSCell based on the included time, timeInfoType and referenceSFN in timeReferenceInfoNR;

2>	calculate the inaccuracy of the time reference of PSCell based on the uncertainty and other implementation-related inaccuracies, if uncertainty is included in timeReferenceInfoNR;

2>	inform upper layers of the time reference of PSCell and, if uncertainty is included in timeReferenceInfoNR, of the inaccuracy of the time reference of PSCell.

---------------------------------------------------Next Change-------------------------------------------------------------------

- –	DLInformationTransfer

The DLInformationTransfer message is used for the downlink transfer of NAS, non-3GPP dedicated information or time reference information.

NOTE:	The UE may use the time reference information provided in the timeReferenceInfo IE for numerous purposes, possibly involving upper layers e.g. to synchronise the UE clock.

Signalling radio bearer: SRB2 or SRB1. If only timeReferenceInfo is included in the message, SRB1 is used. Otherwise, SRB1 is used only if SRB2 not established yet, and if SRB2 is suspended, E-UTRAN does not send this message until SRB2 is resumed.

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

DLInformationTransfer message

-- ASN1START

DLInformationTransfer ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

dlInformationTransfer-r8			DLInformationTransfer-r8-IEs,

dlInformationTransfer-r15			DLInformationTransfer-r15-IEs,

spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

DLInformationTransfer-r8-IEs ::=	SEQUENCE {

dedicatedInfoType					CHOICE {

dedicatedInfoNAS					DedicatedInfoNAS,

dedicatedInfoCDMA2000-1XRTT			DedicatedInfoCDMA2000,

dedicatedInfoCDMA2000-HRPD			DedicatedInfoCDMA2000

},

nonCriticalExtension				DLInformationTransfer-v8a0-IEs		OPTIONAL

}

DLInformationTransfer-v8a0-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

DLInformationTransfer-r15-IEs ::=	SEQUENCE {

dedicatedInfoType-r15				CHOICE {

dedicatedInfoNAS-r15				DedicatedInfoNAS,

dedicatedInfoCDMA2000-1XRTT-r15		DedicatedInfoCDMA2000,

dedicatedInfoCDMA2000-HRPD-r15		DedicatedInfoCDMA2000

}																		OPTIONAL,	-- Need ON

timeReferenceInfo-r15				TimeReferenceInfo-r15				OPTIONAL,	-- Need ON

timeReferenceInfoNR-r16			OCTET STRING           		  	    OPTIONAL,

nonCriticalExtension				DLInformationTransfer-v8a0-IEs		OPTIONAL

}

-- ASN1STOP

timeReferenceInfoNR-r16 field descriptions

timeReferenceInfoNR-r16 Includes the ReferenceTimeInfoNR message as defined in TS 38.331 [82]. In this version of the specification, the field is only used to transfer the reference time of NR PSCell.

------------------------------------------------End of Change--------------------------------------------------------------------