3GPP TSG-RAN WG2 Meeting #109 electronic	R2-2000518

24th Feb – 6th Mar 2020

Source: 	Huawei, HiSilicon

Title: 		Remaining issues for routing

Agenda Item:	6.1.3

Document for:		Discussion and Decision

- Introduction

In RAN2 #107 meeting, the following agreement about BAP layer in IAB network is achieved [1]:

The BAP address of the IAB node is used to differentiate traffic to be delivered to upper layers from traffic to be delivered to egress RLC layer (FFS for the Donor node).

Based on discussion in previous RAN2 meetings, the UL and DL routing across wireless links rely on the routing function in BAP layer, which is configured with one or multiple routing entries. Each routing entry contains a BAP routing ID and a corresponding next hop node, and the BAP routing ID is composed by a BAP address and a BAP path ID. The BAP address in each DL packets indicates the access IAB node.  But according to the above agreements, the processing of UL packets in the IAB donor is still FFS. In this contribution, we are going to analyse more details about routing for wireless backhaul links in an IAB network.

- Discussion

Issue 1: Whether to configure the BAP address to the IAB donor DU?

In RAN2 #108 meeting, we have the following agreement about BAP layer in IAB network [2]:

For both UL and DL, The BAP header for Data PDU has a length of 3B, which hold 1 D/C bit, 3 R bits, 10 bits for BAP address, and 10bits for BAP path ID (this overrides earlier agreement).

For the DL transmission, each IAB node is configured with a BAP address, and the IAB node will use this BAP address to check whether it is the destination of the DL packets in wireless backhaul transmission.  Obviously, the UL packet also carries a destination BAP address in the BAP header. And the length of BAP address in UL packet is agreed to be same as in the DL packet. About the UL processing in the BAP entity of IAB donor DU, there are two different options, which are listed as follows.

Option 1: IAB donor DU always deliver the payload of any received BAP data PDU to upper layer.

Option 2: the IAB donor DU check the BAP address in received BAP data PDU and deliver those ones which contain same BAP address as the BAP address for the IAB donor DU.

As shown in the endorsed stage 2 running CR of TS 38.300[3], the BAP entity for UL transmission terminates at the IAB donor DU. And we believe that no matter which node the BAP entity locates at, the data transmission modelling of BAP layer should be consistent. From such perspective, option 2 is more reasonable, since the IAB donor DU will use same data transmission model as the DL processing in each IAB node.

Otherwise, if option 1 is agreed, the BAP layer in the IAB donor DU performs a different data transfer modelling, i.e. skipping the BAP address checking step. In addition, in some abnormal case, if IAB donor DU receives some packets which not been forwarded correctly by previous links, option 1 will allow the IAB donor DU still forwarding these packet to upper layers (i.e. the IP layer), and the packets will still be routed in the IP domain of IAB donor DU. If the packets cannot be routed successfully in the IP domain, it will be discarded finally. In such case, it is better to discard these packets by the IAB donor DU before deliver to upper layer to avoid causing useless data forwarding in the IP domain.

Thus, option 2 is suggested to be preferred by RAN2, and it is straightforward that each IAB donor DU should be configured with a BAP address also.

Proposal 1: RAN2 agrees on option 2 for the UL packet processing in IAB donor DU:

- Option 1: IAB donor DU always deliver the payload of any received BAP data PDU to upper layer, without checking the BAP address .

- Option 2: the IAB donor DU checks the BAP address in received BAP data PDU and deliver those ones which contains same BAP address as its configured BAP address.

Proposal 2: The IAB donor DU is configurable with its BAP address by the IAB donor CU, before receives any UL BAP data PDUs.

Issue 2: The UL BAP address indicates IAB donor CU or IAB donor DU?

The agreements about the definition of BAP address is “Each BAP address defines a unique destination (unique for IAB network of one Donor, either an IAB access node, or the IAB donor)” [4]. For upstream destination, it has not been clarified whether the BAP address should identify an IAB donor CU or the IAB donor DU.

There are 3 options for this issue:

Option 1. The BAP address in UL packets indicates the IAB donor DU

Option 2. The BAP address in UL packets indicates the IAB donor CU

Option 3. Vague meaning, i.e. no need to clarify which node the BAP address in UL packets indicates.

At first, option 3 is not a good choice, since anyway the BAP address is used to identify some nodes, and it is too strange to not clarifying what this address means.

Among the rest two options, we think using the BAP address to identify the IAB donor DU is reasonable since the BAP layer based routing will be performed across the wireless BH links, IP based routing will be used for the intra donor routing. Otherwise, if the BAP address is used to identify the IAB donor CU, each IAB donor DU will be responsible to forward packets with different BAP address carried in upstream packets, and multiple IAB donor DUs will share same BAP address only if these DUs connects to a same IAB donor CU-UP. In such case, it is meaningless for the IAB donor DU to check whether the BAP address of upstream packet is same as the one configured to it or not. To be honest, this is workable, but will defy the BAP layer receiving procedure of IAB node, and will cause split receiving operation for BAP layer specification. In addition, such way will prevent the IAB donor CU to do load balancing among different IAB donor DUs.

Proposal 3: RAN2 confirms that for UL, the BAP address in BAP header is to identify the destination IAB donor DU, rather than the IAB donor CU.

- Conclusion

This paper mainly discusses remain issues about routing for IAB networks, then we draw the following observation and proposals:

Proposal 1: RAN2 agrees on option 2 for the UL packet processing in IAB donor DU:

- Option 1: IAB donor DU always deliver the payload of any received BAP data PDU to upper layer, without checking the BAP address .

- Option 2: the IAB donor DU checks the BAP address in received BAP data PDU and deliver those ones which contains same BAP address as its configured BAP address.

Proposal 2: The IAB donor DU is configurable with its BAP address by the IAB donor CU, before receives any UL BAP data PDUs.

Proposal 3: RAN2 confirms that for UL, the BAP address in BAP header is to identify the destination IAB donor DU, rather than the IAB donor CU.

- Reference

- Report of the 3GPP RAN WG2 #107 meeting.

- Report of the 3GPP RAN WG2 #108 meeting.

- R2-1916641, Running CR of TS 38.300 for IAB.

- Report of the 3GPP RAN WG2 #106 meeting.

- Report of the 3GPP RAN WG2 #108 meeting