3GPP TSG-RAN WG2 Meeting #109e	 R2-2000544

# 24 Feb – 6 Mar 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | - | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.321 for HARQ-Based RLF at TX UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-01-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture TP to 38.321 for HARQ-based RLF at TX UE, based on email discussion: [108#99][V2X] HARQ based TX side RLM/RLF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture proposals in email discussion: [108#99][V2X] HARQ based TX side RLM/RLF on top of the latest running CR for NR V2X. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HARQ-based RLF at TX UE is missing in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 5.x.1.3.3	PSFCH reception

The MAC entity shall for each PSSCH transmission:

1> if an acknowledgement corresponding to the transmission in clause 5.x.1.3.2 is obtained from the physical layer:

2> deliver the acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process;

1> else:

2> deliver a negative acknowledgement to the corresponding Sidelink HARQ entity for the Sidelink process;

1> if PUCCH-SL-Config is configured by RRC:

2> instruct the physical layer to signal the acknowledgement corresponding to the transmission on the PUCCH according to clause 16.5 of TS 38.213 [x].

Editor’s Note: FFS whether instructing the L1 to signal the PUCCH is to be specified in 38.321 or RAN1 specification.

Editor’s Note: FFS on PUCCH transmssion when the max number of retransmssions are reached in RAN1.

##### 5.x.1.3.3	HARQ-Based Counting of consecutive DTX

The MAC entity shall maintain a count of the number of consecutive DTX received for each unicast link, in order to detect RLF.

RRC controls the RLF detection condition by configuring the following:

-	[maxNumConsecutiveDTX]

The following UE variable is maintained for each pair of Source Layer-2 ID and Destination Layer-2 ID corresponding to unicast.

-	numConsecutiveDTX

The MAC entity shall reset numConsecutiveDTX for all unicast links upon (re)configuration of maxNumConsecutiveDTX from upper layers.

The MAC entity shall set numConsecutiveDTX to zero upon establishment of the unicast link.

The MAC entity shall for each PSSCH transmission:

1> if the transmission in clause 5.x.1.3.2 is for unicast, and DTX is obtained from the physical layer for this transmission:

2> increment numConsecutiveDTX corresponding to the Source Layer-2 ID and Destination Layer-2 ID pair of the transmission;

2> if numConsecutiveDTX corresponding to this Source Layer-2 ID and Destination Layer-2 ID pair reaches maxNumConsecutiveDTX:

3> indicate to upper layers that the maximum number of consecutive DTX for this Source Layer-2 ID and Destination Layer-2 ID pair has been reached;

1> else if the transmission in clause 5.x.1.3.2 is for unicast and acknowledgement or negative acknowledgement is obtained from the physical layer for this transmission:

2> reset numConsecutiveDTX corresponding to this Source Layer-2 ID and Destination Layer-2 ID pair of the transmission to zero;

Change End