3GPP TSG-RAN WG2 Meeting #109-e	R2-2000624

Electronic meeting, 24th February – 06th March 2020

Agenda Item:	7.2.7

Source:		Huawei, HiSilicon

Title:	NRS presence on non-anchor paging carrier

Document for:	Discussion and Decision

# Introduction

At RAN#80, the Rel-16 work item on additional enhancements for NB-IoT was approved [1]. One of the objectives in this work item is to specify presence of NRS on a non-anchor carrier for paging even when no paging NPDCCH is transmitted.

Improved multi-carrier operation: - Specify support of Msg3 quality reporting for non-anchor access [RAN1, RAN2, RAN4] - Specify support for quality reporting in connected mode for anchor and non-anchor carriers. The quality report is not carried in the physical layer. [RAN1, RAN2, RAN4]. - Specify signalling to indicate on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted [RAN1, RAN2, RAN4]

RAN1 has specified presence of NRS on non-anchor paging carrier for paging early termination for FDD and capture the feature in TS 36.211 section 10.2.6 [4].

RAN1 has agreed that presence of NRS on non-anchor paging carrier is enabled at cell level by a new parameter ‘nrs-NonAnchor-config’ [3].

RAN1 has assumed that the POs associated with NRS presence will be specified by RAN2 in TS 36.304 [2].

In this document we discuss the impact on RAN2 specifications (TS 36.331 and TS 36.304).

# Discussion

## Introduction of NRS presence on non–anchor carrier in TS 36.331

RAN1 has agreed that presence of NRS on non-anchor paging carrier is enabled at cell level by a new parameter ‘nrs-NonAnchor-config’ [3]:

| Parameter name in the spec | New or existing? | Description | Value range | Default value aspect (see note) | UE-specific or Cell-specific | TDD/FDD | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| nrs-NonAnchor-config | New | If enabled, NRS are present on non-anchor carriers even when no paging NPDCCH is transmitted. | enabled | disabled | Cell specific | FDD | 36.331 |

RAN1#96bis:

Agreement The subset of POs that have subframes with NRS even when no NPDCCH is transmitted is determined based at least on nB-r13/nB - FFS: defaultPagingCycle-r13/T - Note: The paging frame and the number of UE groups can be derived from T and nB

RAN1#97:

Agreement With regards to the RAN1#96bis agreement: - If nB >= X, specify the decimation pattern/M/N for each nB. - FFS if the value of M/N is different for different POs within a DRX cycle - For nB < X, all POs have NRS. FFS value of M/N. It is agreed that the value of X is T/2.

As the parameter is cell specific and used by UEs in RRC_IDLE, the obvious way is to signal the parameter in system information.

In addition, as the parameter signals the presence of NRS on non-anchor paging carrier and that the subset of subframes with NRS are determined based on nB and defaultPagingCycle, it seems natural to signal the parameter in systemInformationBlockType2-NB.

Proposal 1: The parameter nrs-NonAnchorConfig is signalled in systemInformationBlockType2-NB.

Proposal 2: Adopt the Text Proposal in section 5.1 for TS 36.331

## Introduction of NRS presence on non–anchor carrier in TS 36.304

RAN1 has assumed that the POs associated with NRS presence will be specified by RAN2 in TS 36.304 [2].

The related agreements are replicated below:

RAN1#97:

Agreement With regards to the RAN1#96bis agreement: - If nB >= X, specify the decimation pattern/M/N for each nB. - FFS if the value of M/N is different for different POs within a DRX cycle - For nB < X, all POs have NRS. FFS value of M/N. It is agreed that the value of X is T/2.

RAN1#99:

Agreement The following existing agreement is revised in red R = (PO_Index+ offset) mod 2, where: - PO_Index = (SFN / T * nB + i_s) mod nB. - The offset depends at least on X = SFN + 1024 * H-SFN. - If R=1, the PO has NRS associated with it. If R=0 the PO does not have NRS associated with it. - Offset = (floor ((SFN + 1024*H-SFN) / T)) mod 2, where T is cell-specific DRX cycle. Where SFN is the SFN corresponding to the PO.

Proposal 3: Introduce a new subclause in TS 36.304 clause 7 Paging to specify the POs associated with NRS.

Proposal 4: Adopt the Text Proposal in section 5.2 for TS 36.304.

# Conclusion

In this document, we have discussed the introduction of NRS presence on non-anchor carrier in RAN2 specifications and made the following proposals:

Proposal 1: The parameter nrs-NonAnchorConfig is signalled in systemInformationBlockType2-NB.

Proposal 2: Adopt the Text Proposal in section 5.1 for TS 36.331

Proposal 3: Introduce a new subclause in TS 36.304 clause 7 Paging to specify the POs associated with NRS.

Proposal 4: Adopt the Text Proposal in section 5.2 for TS 36.304.

# References

RP-193224 “WID revision: Additional enhancements for NB-IoT”, Futurewei, RAN#86, Sitges, Spain, December 2019

R1-1913595 “RAN1 agreements for Rel-16 additional enhancements for NB-IoT”, FutureWei, RAN1#99, Reno, Nevada, December 2019

R1-1913673 “Updated consolidated parameter list for Rel-16 LTE”, FutureWei, RAN1#99, Reno, Neva, December 2019

R1-1913616 “TS 36.211 CR 0505 Introduction of Additional enhancements for NB-IoT”, Ericsson, RAN1#99, Reno, Neva, December 2019

R2-1916566 “Introduction of additional enhancements for NB-IoT in TS 36.331”, Huawei, RAN2#108, Reno, Nevada, USA, November 2019

R2-1916565 “Introduction Introduction of Rel-16 NB-IoT enhancements”, Nokia, RAN2#108, Reno, Nevada, USA, November 2019

# Text proposals

The text proposals in sections 5.1 and 5.2. are based on the corresponding NB-IoT running CRs in [5] and [6].

## Text proposal for TS 36.331

–	RadioResourceConfigCommonSIB-NB

The IE RadioResourceConfigCommonSIB-NB is used to specify common radio resource configurations in the system information, e.g., the random access parameters and the static physical layer parameters.

RadioResourceConfigCommonSIB-NB information element

-- ASN1START

RadioResourceConfigCommonSIB-NB-r13 ::=	SEQUENCE {

rach-ConfigCommon-r13					RACH-ConfigCommon-NB-r13,

bcch-Config-r13							BCCH-Config-NB-r13,

pcch-Config-r13							PCCH-Config-NB-r13,

nprach-Config-r13						NPRACH-ConfigSIB-NB-r13,

npdsch-ConfigCommon-r13					NPDSCH-ConfigCommon-NB-r13,

npusch-ConfigCommon-r13					NPUSCH-ConfigCommon-NB-r13,

dl-Gap-r13								DL-GapConfig-NB-r13			OPTIONAL,		-- Need OP

uplinkPowerControlCommon-r13			UplinkPowerControlCommon-NB-r13,

...,

[[	nprach-Config-v1330					NPRACH-ConfigSIB-NB-v1330	OPTIONAL		-- Need OR

]],

[[	nprach-Config-v1450					NPRACH-ConfigSIB-NB-v1450	OPTIONAL		-- Cond EnhPowerControl

]],

[[	nprach-Config-v1530					NPRACH-ConfigSIB-NB-v1530	OPTIONAL,	-- Need OR

dl-Gap-v1530						DL-GapConfig-NB-v1530		OPTIONAL, 	-- Cond TDD

wus-Config-r15						WUS-Config-NB-r15			OPTIONAL	-- Need OR

]],

[[	nprach-Config-v1550					NPRACH-ConfigSIB-NB-v1550	OPTIONAL	-- Cond TDD1

]],

[[	nrs-NonAnchorConfig-r16				ENUMERATED {true}			OPTIONAL	-- Need OR

]]

}

BCCH-Config-NB-r13 ::=					SEQUENCE {

modificationPeriodCoeff-r13				ENUMERATED {n16, n32, n64, n128}

}

PCCH-Config-NB-r13 ::=					SEQUENCE {

defaultPagingCycle-r13					ENUMERATED {rf128, rf256, rf512, rf1024},

nB-r13									ENUMERATED {

fourT, twoT, oneT, halfT, quarterT, one8thT,

one16thT, one32ndT, one64thT,

one128thT, one256thT, one512thT, one1024thT,

spare3, spare2, spare1},

npdcch-NumRepetitionPaging-r13			ENUMERATED {

r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

}

-- ASN1STOP

RadioResourceConfigCommonSIB-NB field descriptions

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 36.304 [4]. Value rf128 corresponds to 128 radio frames, rf256 corresponds to 256 radio frames and so on.

dl-Gap Downlink transmission gap configuration for the anchor carrier. See TS 36.211 [21], clause 10.2.3.4. If the field is absent, there is no gap.

modificationPeriodCoeff Actual modification period, expressed in number of radio frames= modificationPeriodCoeff * defaultPagingCycle. n16 corresponds to value 16, n32 corresponds to value 32, and so on. The BCCH modification period should be larger or equal to 40.96s.

nB Parameter: nB is used as one of parameters to derive the Paging Frame and Paging Occasion according to TS 36.304 [4]. Value in multiples of 'T' as defined in TS 36.304 [4]. A value of fourT corresponds to 4 * T, a value of twoT corresponds to 2 * T and so on.

npdcch-NumRepetitionPaging Maximum number of repetitions for NPDCCH common search space (CSS) for paging, see TS 36.213 [23], clause 16.6.

nrs-NonAnchorConfig For FDD: This field indicates if NRS are present on non-anchor paging carriers even when no paging NPDCCH is transmitted, see TS 36.211 [21], clause 10.2.6.

wus-Config For FDD: WUS Configuration.

| Conditional presence | Explanation |
| --- | --- |
| EnhPowerControl | This field is optional present, Need OR, if PowerRampingParameters-NB-v1450 is included in SIB2-NB. Otherwise the field is not present. |
| TDD | The field is optionally present, Need OR, for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD1 | The field is mandatory present for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |

## Text proposal for TS 36.304

7.x	NRS presence on non-anchor paging carrier in NB-IoT

For FDD, when nrs-NonAnchorConfig is signalled in system information, the POs associated with NRS are determined using the DRX parameters broadcast in systeminformationBlockType2-NB:

-	T is the value of defaultPagingCycle broadcast in system information.

-	nB is the value of nB broadcast in system information: 4T, 2T, T, T/2, T/4, T/8, T/16, T/32, T/64, T/128, T/256, T/512, and T/1024.

The POs are determined by:

-	Paging Frame (PF) given by: SFN mod T= (T div N) * k

where:

-	N: min(T, nB)

-	k: 0, 1, .., N-1

-	Index i_s pointing to PO from subframe pattern defined in 7.2.

where Ns: max(1,nB/T)

The POs associated with NRS are determined as follows:

-	if nB is equal to 4T, 2T, T or T/2:

POs for which R = 1 have associated NRS

where:

R = (PO_Index+ offset) mod 2

where:

-	PO_Index = (SFN/ T * nB + i_s) mod nB

-	Offset = (FLOOR ((SFN + 1024*H-SFN) / T)) mod 2

-	SFN is the SFN corresponding to the PO

-	H-SFN is the H-SFN corresponding to the PO

-	i_s is the index i_s corresponding to the PO

-	else:

all POs have associated with NRS.