3GPP TSG-RAN WG2 #109-e	R2-200xxxx

# Electronic meeting, 24th February – 6th March, 2020

Agenda Item:	6.7.2.2

Source:	Ericsson

Title:	Summary on [AT109e][033][IIOT] Scheduling Enhancements

Document for:	Discussion, Decision

1	Introduction

In this document, we summarize the outcome of the following at-meeting email discussion.

- [AT109e][033][IIOT] Scheduling Enhancements (Ericsson)

Scope: Treat summary on Scheduling Enhancements

Intended outcome: Resolve issues, Describe Open Issues accurately.

Deadline: Mar 3 1200 CET (conclusions on “easy agreements” by Feb 27 1200 CET)

This document is based on the pre-meeting summary paper R2-2002091 [32]. Compared to the summary paper R2-2002091, the main body remain unchanged. Companies are invited to provide feedbacks on the proposals that need further discussion in the e-meeting. In addition, companies can provide further comments (if any) on “easy agreements” in Section 3 and identify other open issues in Section 4.

2	Discussion

2.1	LCP restriction enhancement

2.1.1 AllowedCGList for dynamic grant

RRC running CR R2-2001657 lists the following open issue for the LCH restriction enhancement allowedCG-List:

Editor’s note: In this implementation, it is assumed that the LCH configured with allowedCG-List is allowed to be mapped to dynamic grant. This requires a confirmation from RAN2.

Confirmation is proposed in R2-2000111, R2-2000706, R2-2001049, R2-2001171, R2-2001290, R2-2001429.

Confirm LCH configured with allowedCG-List is allowed to be mapped to dynamic grant

R2-2001493 discusses the need to consider beside configured grants also uplink grants addressed to CS-RNTI. For Type2 activation, the grant is stored as configured grant, so no further consideration is needed. For retransmission, since HARQ buffer data is retransmitted, no LCP is evaluated, so no consideration within LCH restriction is needed either. Thus, the proposal is not summarized here.

2.1.2 Applicability of PHY priority indication

The following FFS are noted in RRC running CR R2-2001657:

Editor’s note: In this implementation, it is assumed that the LCH configured with allowedPHY-PriorityIndex is allowed to be mapped to dynamic grant without any priority indication. FFS: The mapping restriction between a LCH configured with allowedPHY-PriorityIndex and a grant without any priority indication.

Editor’s note: FFS whether allowedPHY-PriorityIndex applies for configured grant.

These FFS are discussed in R2-2001049, R2-2000788, R2-2001461, R2-2001429, R2-2001171, R2-2001461, R2-2001493, R2-2001033, R2-2000845, R2-2001289, R2-2001029, R2-2000115

In the TS 38.213 V16.0 clause 9.0, it is specified that “If a priority index is not provided for a PUSCH or a PUCCH, the priority index is 0”. Note that priority index 0 means low priority in PHY layer. If we allow high priority traffic to be mapped to a grant without priority indication, it may cause some issues since it would be subsequently treated as low priority in PHY layer.  Therefore, we propose to align with the Ran1 spec:

LCH configured with allowedPHY-PriorityIndex is allowed to be mapped to dynamic grant without any priority indication only in case the configuration allows it to be mapped on low priority grant.

The above contributions consider diverging views regarding allowedPHY-PriorityIndex restrictions to CG.

- allowedPHY-PriorityIndex does not apply for CG: R2-2001171, R2-2001461, R2-2001033, R2-2000845, R2-2001289, R2-2001029

- allowedPHY-PriorityIndex applies for CG R2-2001049, R2-2000788

Some companies argue that, in light of allowedCG-List, it is not necessary to have allowedPHY-PriorityIndex for CG.

On the other hand, as mentioned in R2-2000788, this restrictive mapping between CG and LCH can also be used by the network to steer the traffic to the intended CG configuration (note that the traffic arrival at gNB might have jitter). It is useful to introduce allowedPHY-PriorityIndex for CG too. Furthermore, not applying such LCP restriction to CG will result in further complication in the specification, i.e. extra conditions would be required. Lastly, there seem to be no technical reasons against restricting the allowedPHY-PriorityIndex to dynamic grants.

There are diverging opinions, but this topic has been well discussed and understood. There is a majority support that “allowedPHY-PrioirtyIndex does not apply for configured grant”, and we propose

allowedPHY-PriorityIndex restriction applies only to dynamic grants.

In addition, R2-2001493 proposes to exclude the UL grant scrambled with CS-RNTI, and considers UL grant scrambled with C-RNTI and MCS-RNTI. For Type2 activation, the grant is stored as configured grant, so no further consideration needed. For retransmission, since HARQ buffer data is retransmitted, no LCP is evaluated, so no consideration within LCH restriction needed either. Furthermore, it is also related with the editor’s note that “FFS whether an uplink grant addressed to CS-RNTI with NDI=1 (i.e. retransmission of a configured grant) is a configured grant or not. In this version of running CR, it is assumed that an uplink grant addressed to CS-RNTI with NDI=1 is considered as a dynamic grant”. We expect it to be resolved in the intra-UE prioritization agenda item.  Thus, the proposal is not summarized here.

We have identified that proposal 3 needs further discussion and companies can provide feedback below.

| Company | Support P3 (y/n) | Additional comments |
| --- | --- | --- |
| LG | y |  |

2.2	Multiple SPS/CG enhancements

2.2.1 When multiple entry CG confirmation MAC CE can be generated

The following FFS are noted in MAC running CR R2-2001487:

Editor’s Note: When Multiple Entry Configured Grant Confirmation MAC CE is generated is FFS.

It needs to be specified in which cases the multi entry CG MAC CE is used, in particular when there is only one configuration and also the legacy single-entry MAC CE could be used instead. The options are:

Option 1: Couple with configuredGrantConfigList-r16 configuration. R2-2000111, R2-2000789

Option 2a: When there are at least two CG indices configured R2-2001290

Option 2b: When there are at least two CG pending for confirmation R2-2001555

Option 3: Couple with Type 2 CG configured with ConfiguredGrantConfigIndexMAC R2-2001489

Option 4: The multiple entry CG confirmation MAC CE shall be used in case the UL grant for new transmission can accommodate the MAC CE plus its subheader. R2-2001461

It is our understanding R2-2001489 does not prefer option 2 above and its proposal option 3 is in principle closer to option 1. We propose to go forward with the simplest option with a majority view, i.e. option 1, that conditioned Rel-16 MAC CE usage with using the Rel-16 configuration for multiple configured grants.

If configuredGrantConfigList-r16 is configured in the MAC entity, the multiple entry configured grant confirmation MAC CE is always used.

We have identified that proposal 4 needs further discussion and companies can provide feedback below

| Company | Support P4 (y/n) | Additional comments |
| --- | --- | --- |
| LG | y |  |

Another open issue is when the multi CG MAC CE is generated. Related aspects are discussed in R2-2000111, R2-2001428, R2-2001461, R2-2001489. In summary it is proposed:

As in legacy, the multiple entry configured grant confirmation MAC CE is generated if 1) the MAC entity has UL resources allocated for new transmission; 2) there is at least one triggered but not cancelled confirmation.

2.2.2 The priority of multiple entry CG confirmation MAC CE

The following FFS are noted in MAC running CR R2-2001487:

Editor’s Note: It is assumed that Multiple Entry Configured Grant Confirmation MAC CE has the same priority with Confirmation Grant Confirmation MAC CE in this version. The confirmation of this assumption may be needed.

Since the multiple entry confirmation MAC CE will not be generated together with the single entry, it is okay to assume that they have the same priority. This is also discussed in R2-2000111, R2-2001290, R2-2001428. R2-2001489, Thus, it is proposed:

Confirm that Multiple Entry Configured Grant Confirmation MAC CE has the same priority as Confirmation Grant Confirmation MAC CE.

2.2.3 Detailed contents for multiple entry CG confirmation MAC CE

The following FFS are noted in MAC running CR R2-2001487:

Editor’s Note: In the current version of the running CR, fixed size MAC CE of four octets is assumed as an example. The format in detail should be discussed and updated later. Editor’s Note: It is an FFS whether this MAC CE has a fixed size or not. Editor’s Note: In the current version of the running CR, it is assumed that this MAC CE reports confirmation of type 2 configured grants.

The following FFS are noted in RRC running CR R2-2001657:

Editor’s note: FFS the maximum length of the allowedList, i.e., the maximum number of configured grant configurations per MAC entity.

When it comes to the maximum number of configured grant configurations per MAC entity, the following options are proposed:

Option 16     R2-2001290

Option 32     R2-2001428/R2-2001429, R2-2001613, R2-2000789, R2-2001489

Option 48	  R2-2001049

Option 64     R2-2000111, R2-2000430

Note, in one MAC entity there can be multiple cells with each up to four dedicated BWPs, each can have up to 12 CG configurations. For maximum flexibility in scheduling CGs among those BWPs and cells, in order to support a high number of TSC traffic flows, a large number is preferable. On the other side, the number is limited by UE implementation feasibility, and furthermore a smaller number would decrease the size of the MAC CE for CG confirmation. Given options above, it is proposed to go forward with the compromise of 32.

Maximum 32 CG configurations per MAC entity.

We have identified that proposal 7 needs further discussion and companies can provide feedback below

| Company | Support P7 (y/n) | Additional comments |
| --- | --- | --- |
| LG | y |  |

The size of the MAC CE for multiple CG confirmation must include the maximum number of CG configurations per MAC entity. It can be of fixed size, i.e. always up to maximum number, or variable size, including e.g. only configured CG configurations. The options discussed are:

Variable size:  	R2-2000111, R2-2001049

Fixed size:		R2-2000430, R2-2001290, R2-2001428, R2-2001489, R2-2000789

In summary, it is proposed to agree on the majority view and introduce a fixed size MAC CE of size 4 bytes to cover the 32 proposed configurations.

MAC CE for CG configuration has fixed size of 4 bytes.

We have identified that proposal 8 needs further discussion and companies can provide feedback below

| Company | Support P8 (y/n) | Additional comments |
| --- | --- | --- |
| LG | y |  |

The question is whether CG confirmation MAC CE applies also for type 1 confirmation, or what entries in the MAC CE related to type 1 mean. This is discussed in R2-2000111, R2-2000789, R2-2001613, R2-2001489, confirming:

Confirm that multiple entry configured confirmation MAC CE only confirms configured grant type 2 configurations and other entries can be ignored.

2.2.4 SFN misalignment for CG type 1

This topic was summarized in the last meeting in R2-1916527, but was postponed. The majority of companies are fine with the option 2 in R2-1916527 and summarized in the following two proposals.

In R2-2001049, R2-2000431, R2-2000697 the following is proposed:

For Type-1 CG, after receiving the configuration, UE should first identify the lowest N value corresponding to the nearest available CG occasion, then, N is incremented after each CG occasion starting from the N identified in the first step.

The same problem is discussed but with an alternative proposals in R2-2001627.

For the previous proposal, the following new field is proposed in R2-2001627, R2-2001498(optional), R2-2001428, R2-2001049, R2-2000697, R2-2000431.

Introduce timeReferenceSFN in RRC CG type 1 configuration.

The paper R2-2000789 proposes an alternative solution in which the timeDomainOffset is extended to 5120*16-1 with three extra bits (compared to Proposal 11) to indicate the whole range of the HFN. This solution aligns the resource allocation of type 1 to type 2 CG and SPS, with no change on the MAC formula. A simlar principle is mentioned in the paper R2-2001498.

One company (that hasn’t submitted any contributions to this topic) indicates during the pre-meeting email discussion that the issue can be resolved by gNB implementation, as the option 1 in the summary R2-1916527.

The above two proposals are bundled together to solve the issue, and we propose to discuss them together. These two proposals need further discussion and companies can provide feedback below

| Company | Support P10,11 (y/n) | Additional comments |
| --- | --- | --- |
| LG | y |  |

2.3	Open issues to complete the spec

2.3.1 Simultaneous configuration of type 1 on UL and SUL

Simultaneous configuration of type 1 CG for UL and SUL is discussed related to open issue noted in RRC-running CR R2-2001657

Editor’s note: FFS: WHETHER we follow the legacy restriction that the configured grant type1 can only be configured for either or SUL, OR the configured grant type1 can be configured for both UL and SUL.

The paper R2-2000111 proposes that different configurations can be configured for UL and SUL. This is the assumption by the RRC rapportuer in the RRC running CR discussion and possibly also in the papers R2-2000429 and R2-2001049. Thus, to make the discussion clear, we propose to confirm that

Proposal 12a When multiple configured grant configurations per BWP is supported, different configured grant type 1 configurations can be configured in UL and SUL.

The following options to solve this open issue are being discussed:

Option 1: UL and SUL can be configured simultanously.

R2-2000111, R2-2000429

Option 2: UL and SUL cannot be configured simultanously

R2-2001049

Some technical concerns are mentioned in the R2-20001049. Since this is a configuration option anyway, we propose to go with the majority view

Proposal 12b When multiple configured grant configurations per BWP is supported, the same configured grant type 1 configuration can be configured for both UL and SUL.

We have identified that proposal 12b needs further discussion and companies can provide feedback below

| Company | Support P12b (y/n) | Additional comments |
| --- | --- | --- |
| LG | y |  |

2.3.2 Naming of ”PHY-PriorityIndex”

R2-2001429 propose to change the name of ”PHY-PriorityIndex” to ”Reliablity-PriorityIndex” to reflect its motivation. From RRC spec rapporterur point of view, we want techinques for a feature to be re-usable in other scenarios in later releases and prefer not to be restricted. One can refer MCS C-RNTI as an example. As there were no objections during the email discusison and this is proposed in the email discussion rapporterur summary R2-2000785 and paper R2-2000115, we propose to confirm that

Proposal 13		  Align the terminology and use name “phy-PriorityIndex” in TS 38.300, TS 38.321, TS 38.331 to indicate the priority of the grant/SR-source agreed by RAN1

2.4	Other open issues

Measurement gaps

R2-2000564 proposes UE to transmit during the measurement gaps, since UE cannot transmit/receive during the measurement gap and it impacts the latency performance. However, there were other proposals before this meeting that even if allowing UE to send during measurement gap, it does not solve the issue for DL traffic and one simpler solution is to allow UE to be equipped with two radios.  Therefore, we expect this topic to be contentious and unlikely to converge at e-Meeting.

Confirmation MAC CE to indicate activation/deactivation status

R2-2000699 proposes to change the confirmation from the reception of the DCI to activation/de-activation status. In addition, one company (that hasn’t submitted any contributions to this topic) indicates that they support the proposal in R2-2000699 and prefers further discussion. The proposal is motivated from the case that the network may send an activation DCI quickly followed by a deactivation DCI or vice versa. In this case, it is not clear the confirmation MAC CE is for the first or for the second DCI.  We believe this can be avoided by network configurations that the network does not send two different DCIs closely in time, as it is not typical that network needs to activate and de-active one CG short in time. As a matter of fact, this was discussed in the MAC running CR and agreed among participant companies to go with the reception of the DCI.

Therefore, we expect this topic to be contentious and unlikely to converge at e-Meeting. One way-forward is to include this in an offline email discussion (if there is any) during the RAN2#109e.

Burst arrival time

R2-2000790 proposes a clarification of the burst arrival time defined in SA2 TS 23.501. gNB uses burst arrival time to determine when it has received all payload for inclusion in the next periodic allocated resources (such as DL SPS, UL CG and UL dynamic grant) in support of the corresponding TSN streams. At the moment, the burst arrival time is defined as the beginning of the burst, while it is more beneficial for RAN to obtain from TSCAI a burst arrival time that refers to the end of the burst. Note that, multiple packets from the single TSN stream or an aggregated TSN streams can arrive within the burst.

As this is the first time this issue is identified, we propose to treat this in the next meeting.

3. Companies’ inputs on easy agreements.

The following proposals have an overwhelming majority support and are identified by the rapporteur as easy agreements.

Proposal 1	Confirm LCH configured with allowedCG-List is allowed to be mapped to dynamic grant

Proposal 2	LCH configured with allowedPHY-PriorityIndex is allowed to be mapped to dynamic grant without any priority indication only in case the configuration allows it to be mapped on low priority grant.

Proposal 5	As in legacy, the multiple entry configured grant confirmation MAC CE is generated if 1) the MAC entity has UL resources allocated for new transmission; 2) there is at least one triggered but not cancelled confirmation.

Proposal 6	Confirm that Multiple Entry Configured Grant Confirmation MAC CE has the same priority as Confirmation Grant Confirmation MAC CE.

Proposal 9	Confirm that multiple entry configured confirmation MAC CE only confirms configured grant type 2 configurations and other entries can be ignored.

Proposal 12a When multiple configured grant configurations per BWP is supported, different configured grant type 1 configurations can be configured in UL and SUL.

Proposal 13		  Align the terminology and use name “phy-PriorityIndex” in TS 38.300, TS 38.321, TS 38.331 to indicate the priority of the grant/SR-source agreed by RAN1

Companies can provide comments below on which proposal is not acceptable and the reason.

| Company | Proposal number | Why? |
| --- | --- | --- |
| LG | Proposal 9 | Proposal 9 depends on what is indicated by the Multiple Entry CG Confirmation MAC CE. If the MAC CE indicates activation/deactivation status of each CG, there is no entry that can be ignored. |

4. Other issues

Companies can identify further issues to be discussed at this email discussion:

| Issue number | Proposal company and description |
| --- | --- |
| 1 | [LG] Confirmation MAC CE to indicate activation/deactivation status |
| 2 |  |

Companies can provide comments on the above indicated issue

| Company | Issue number | Support or not, comments and etc. |
| --- | --- | --- |
| LG | 1 | The Multiple Entry CG Confirmation MAC CE should contain the activation/deactivation status of each CG. Otherwise, if the MAC CE confirms reception status of DCI, there is no point of introducing multiple entries in confirmation MAC CE because the confirmation MAC CE would require only 1 entry in most cases. |

5	Conclusion

6	References

R2-2000111	Remaining issues for multiple CG configurations	CATT

R2-2000429	Configured grant configurations for SUL serving cell	Huawei, HiSilicon

R2-2000430	Discussion on the new CG type 2 confirmation MAC CE	Huawei, HiSilicon

R2-2000431	Method to avoid confusion between UE and network for CG type 1	Huawei, HiSilicon

R2-2000564	Consideration on collision of measurement gap and TSN traffic	Spreadtrum Communications

R2-2000697	SFN misalignment issue on periodicities of non-divisor of 10240ms	OPPO

R2-2000699	Left issue on multiple entry confirmation MAC CE	OPPO

R2-2000706	Support mapping LCHs configured with allowedCG-list to dynamic grant	OPPO

R2-2000788	LCP restriction enhancement based on PHY priority indcation	Ericsson

R2-2000789	SPS and CG remaining MAC aspects	Ericsson

R2-2000790	TSC AI clarifications: meaning of arrival time	Ericsson

R2-2000791	Draft LS: TSC AI clarifications for arrival time	Ericsson

R2-2001049	Remaining issues on TSC scheduling	Nokia, Nokia Shanghai Bell

R2-2001171	LCP restrictions in IIoT	Intel Corporation

R2-2001290	Open issues in Scheduling Enhancements	Qualcomm Incorporated

R2-2001428	Remaining Issues for Multiple SPS-CG enhancements	CMCC

R2-2001429	Remaining Issues for LCP restrictions	CMCC

R2-2001461	The considerations on scheduling enhancement	ZTE Corporation, Sanechips

R2-2001476	TP on IIoT Running RRC for Scheduling Enhancements	CMCC

R2-2001489	Remaining Issues on CG Confirmation MAC CE	Samsung

R2-2001493	LCP Restriction for allowedCG-List and allowedPHY-PriorityIndex	Samsung

R2-2001498	Type 1 Configured Grant with Integer Periodicity	Samsung

R2-2001555	Consideration on multiple entry CG confirmation MAC CE	LG Electronics Inc.

R2-2001613	Multiple Entry Configured Grant Confirmation MAC CE	Intel Corporation

R2-2001627	Impact of CG/SPS with periodicities non dividing HF length	Sequans Communications

R2-2000785 Remaining minor issues in [108#32][IIoT] Running CR 38.331 Ericsson

R2-2001033	Remaining issues on Configured Grant, Huawei

R2-2000845 On UL intra-UE prioritisation, MediaTek Inc.

R2-2001289	Open issues in Intra-UE prioritization, Qualcomm

R2-2001029	L1-priority applies for CG	Lenovo, Motorola Mobility

R2-2000115	Remaining issues for intra-UE multiplexing and prioritization	CATT

R2-2002091, Summary on Scheduling Enhancement for IIoT (6.7.2.2), Ericsson