3GPP TSG-RAN WG2 Meeting #109 electronic	R2-xxxx

E-Meeting, 28th Feb– 6th Mar, 2020

Source:	vivo

Title:	Power Control NR DC

Agenda Item:	6.10.2

Document for:	Discussion and Decision

- Introduction

This document is summarized for the following email discussion:

- [AT109e][044][DCCA] Power Control NR DC (vivo)

Scope: Treat Email discussion + additional issues from the other papers to this Agenda item

Intended outcome: Agreed Issues resolutions

Deadline: Mar 3 1200 CET

The document is organized as follows:

- Whether send an LS to RAN4 to inform the agreed new NR-DC power control parameters in RAN2 and the wording of LS if needed

- Discuss whether NR-DC power control mode should be indicated in CG-ConfigInfo message

- Discuss whether TDD pattern of MCG should be indicated in CG-ConfigInfo message

- Discussion

2.1 Send an LS to RAN4

In the email discussion [1], we have the following proposals for NR-DC power control:

Proposal 2: The existing parameter p-UE-FR1 defined in TS 38.331 can be reused to configure the total maximum transmit power to be used by the UE across all cell groups for NR-DC on FR1.

Proposal 3: Introducing a new parameter p-UE-FR2 in the RRCReconfiguration message to configure the total maximum transmit power to be used by the UE across all cell groups for NR-DC on FR2.

Proposal 2 and proposal 3 have been captured in the corresponding endorsed TP. From our understanding the p-UE-FR1 and p-UE-FR2 will impact maximum output power calculation in RAN4. Similar with EN-DC case according to TS 38.101-3, the total maximum transmit power to be used by the UE across all cell groups for EN-DC, is signaled by RRC within the parameter p-MaxUE-FR1 defined in TS 36.331.

6.2B.4.1.1 Intra-band contiguous EN-DC The following requirements apply for one component carrier per CG configured for synchronous DC. … The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds: PCMAX_L_ E-UTRA,c (p) ≤ PCMAX_ E-UTRA,c (p) ≤ PCMAX H _ E-UTRA,c (p) where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] subclause 6.2.5 modified by PLTE as follows: PCMAX_L_ E-UTRA,c = MIN {MIN(PEMAX,c , PEMAX, EN-DC, PLTE) – tC_ E-UTRA, c, (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc + ΔTIB,c + TC_ E-UTRA, c + TProSe, P-MPRc)} PCMAX H _ E-UTRA,c = MIN {PEMAX,c, PEMAX, EN-DC , PLTE, PPowerClass, EN-DC, PPowerClass – ΔPPowerClass} where - PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8]; - PLTE is the value given by the field p-maxEUTRA-r15 of the RRCConnectionReconfiguration-v1510 IE as defined in TS 36.331 [8] which is the same as PLTE in TS 38.213 [10]; - ∆tC_EUTRA, c = 1.5 dB when NOTE 2 in Table 6.2.2-1 of TS 36.101 [4] applies; ∆tC_EUTRA, c = 0 dB otherwise;

So, we suggest to send an LS to RAN4 to inform two new parameters introduced in RAN2. The drafted LS can be found in [3].

Now, companies are invited to provides their comments for the draft LS out [3].

Q1: Do you agree to send an LS to RAN4 to inform two new parameters introduced in RAN2, and do you have any comments for the drafted LS out [3] if the LS is needed?

| Company | Any comments |
| --- | --- |

2.2 NR-DC power control mode indication

Two semi-static power sharing and dynamic power sharing defined by RAN1 are as follows.

- Semi-static power sharing:

In RAN1#98, it was agreed to consider the following two alternatives for semi-static power sharing with PMCG+PSCG≤PTotalNN-DC: - Alt.1: For the UL transmission in MCG, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of SCG, and vice versa. - If such overlapping with UL transmission on the SCG is possible (i.e. collides with semi-static ‘UL’ and ‘flexible’ symbols on some CCs of SCG), UE limits its actual transmission power pMCG in MCG such that pMCG≤PMCG; - Otherwise (i.e. collides with only semi-static ‘DL’ symbols on all CCs of SCG), pMCG can be up to PMCG' and pSCG can be up to PSCG'. - Alt.1-1: PMCG' and PSCG' are configured by RRC signalling. - Alt.1-2: PMCG' and PSCG' are determined by RAN4 requirement. - Alt.2: For the uplink transmission in MCG and in SCG, UE limits its actual transmission power pMCG to be up to PMCG and pSCG to be up to PSCG.

- Dynamic power sharing:

- For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <=min{PSCG, Ptotal– MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG - Otherwise, pwr_SCG <= Ptotal; - UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission. - (working assumption) No new RRC signaling is introduced for T_offset: - Alt.1: T_offset <= T_proc,2 - Alt.2: T_offset <= 2*T_proc,2 - Alt.3: T_offset reasonbly larger than Alt 1. & Alt 2 but <=4ms

In RAN1#99 meeting, NR-DC-PC-mode is introduced in the IE PhysicalCellGroupConfig to indicated UE which mode is selected.

| LTE_NR_DC_CA_enh-Core | NR-DC | PhysicalCellGroupConfig | NR-DC-PC-mode | New | Selects the uplink power control mode to use for NR-DC. |
| --- | --- | --- | --- | --- | --- |

According to the above agreements, UE behaviour is clear, i.e., UE adjusts its transmit power as the way defined for different power control modes. And in the email discussion [1], all companies agreed that it is MN to decide the power control mode. However, whether NR-DC-PC-mode needs to be indicated to the SN should be further discussed.

In this RAN2#109e meeting, 3 companies support that NR-DC-PC-mode is indicated to the SN [2][4][5], and 1 company does not support [6]. The main argument  why the companies support is:

Since that difference from EN-DC power control, it is possible that PMCG+PSCG≤PTotalNN-DC when dynamic power control is set by MN in NR-DC power control. Thus, SN can’t distinguish whether MN sets dynamic or semi-static power sharing via CG-ConfigInfo. As a results, SN cannot determine whether SN is allowed to set the max SCG power such that the sum of MCG and SCG power exceeds the total UE max power.

Q2: Do you think NR-DC-PC-mode is indicated to SN by MN?

| Companies | Yes or No | Comments |
| --- | --- | --- |

2.3 TDD pattern indication

In this RAN2#109e meeting, whether semi-static TDD pattern of MCG should be included in CG-ConfigInfo message is discussed in [2]. The main argument is

If semi-static power control solution Alt 1-2 is selected, SCG UL performance can be improved if SN can also know the semi-static TDD pattern of MCG because SN scheduler can take into account when the UE can allocate larger power to SCG transmissions.

Q3: Do you agree that the TDD pattern of MCG can be indicated to SN?

| Companies | Yes or No | Comments |
| --- | --- | --- |

- Summary

- Reference

- R2-2000293, Report of email discussion power control for NR-DC, vivo, discussion

- R2-2000137, Remaining issues of power control in NR-DC, Qualcomm Incorporated,	discussion

- R2-2000294, Draft LS on NR-DC power control, vivo, LS out, To:RAN4, Cc:RAN1

- R2-2000674, NR DC power control	Nokia, Nokia Shanghai Bell, discussion

- R2-2000872, Remaining issues for NR-DC power control, Ericsson, discussion

- R2-2001391, NR-DC power control	Huawei, HiSilicon, discussion