3GPP TSG-RAN WG2 Meeting #109-e                                                           R2-20xxxxx

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda item:	    6.19

Source:	CMCC (rapporteur)

Title:	Report of [AT109e][050][R16 Other WISI] NR HST

Document for:	Discussion and Decision

# Introduction

This summary document is used to gather the offline comments for the following email discussion:

By Email – Discussion

NR HST

R2-2000040	LS on the UE capability and network assistance signalling for Rel-16 NR HST RRM (R4-1915855; contact: China Mobile)	RAN4	LS in	Rel-16	NR_HST	To:RAN2

R2-2001656	LS on the UE capability and network assistance signalling for Rel-16 NR HST demodulation (R4-1915916; contact: CMCC)	RAN4	LS in	Rel-16	NR_HST	To:RAN2

R2-2000919	Discussion on signalling for R16 NR HST	CMCC	discussion	Rel-16

=> Revised in R2-2002084

R2-2002084	Discussion on signalling for R16 NR HST	CMCC, CATT	discussion	Rel-16

R2-2000920	38.331 CR on introduction of RRC parameters and UE capabilities for Rel-16 NR HST	CMCC, Huawei, HiSilicon	CR	Rel-16	38.331	15.8.0	1464	-	B	NR_HST

=> Revised in R2-2002085

R2-2002085	38.331 CR on introduction of RRC parameters and UE capabilities for Rel-16 NR HST	CMCC, Huawei, HiSilicon, CATT	CR	Rel-16	38.331	15.8.0	1464	1	B	NR_HST

R2-2000921	38.306 CR on introduction of UE capabilities for Rel-16 NR HST	CMCC, Huawei, HiSilicon	CR	Rel-16	38.306	15.8.0	0242	-	B	NR_HST

=> Revised in R2-2002086

R2-2002086	38.306 CR on introduction of UE capabilities for Rel-16 NR HST	CMCC, Huawei, HiSilicon, CATT	CR	Rel-16	38.306	15.8.0	0242	1	B	NR_HST

- [AT109e][050][R16 Other WISI] NR HST (CMCC)

Scope: Treat documents above

Intended outcome: Focus first on LS and discussion doc. Achieve initial agreements, agree what we shall do. Treatment of CRs expected next meeting.

Deadline: Mar 3 1200 CET

# Discussions

RAN4 has sent two LS to RAN2 on the capability and signalling for NR high speed train (HST) in R16. One is for the UE capability and network assistance signalling for Rel-16 NR HST RRM [1]. The other one is for the UE capability and network assistance signalling for Rel-16 NR HST demodulation [2]. In the LS RAN4 asks RAN2 to design the corresponding signalling to support the enhanced RRM requirements, enhanced demodulation requirements and UE capability for R16 NR HST. A discussion paper is provided in [3] with corresponding proposals. We respectfully ask companies to provide their comments.

Network assistant signalling

Q1. As requested by RAN4 in [1], do you agree to introduce network assistant signalling to enable the enhanced RRM requirements for Rel-16 NR HST?

| Company | Y/N | Comments |
| --- | --- | --- |
| Ericsson | Agree |  |

In Rel-14/Rel-16 LTE HST WI, the network signalling to enable the RRM enhancements to support LTE HST is cell specific configured and is included in the IE RadioResourceConfigCommonSIB and IE RadioResourceConfigCommon. For NR HST, the enhanced RRM requirements are also cell specific configured and provided to UE in idle mode and connected mode.

Therefore if the answer to Q1 is yes, companies are kindly asked to provide their further comments on Q2.

Q2. As suggested in [3], do you agree to include the network assistant signalling to enable the enhanced RRM requirements for Rel-16 NR HST in the IE ServingCellConfigCommon and IE ServingCellConfigCommonSIB?

| Company | Y/N | Comments |
| --- | --- | --- |
| Ericsson | Y |  |

Q3. As requested by RAN4 in [2], do you agree to introduce network assistant signalling to enable the enhanced UE demodulation requirements for Rel-16 NR HST?

| Company | Y/N | Comments |
| --- | --- | --- |
| Ericsson | Y |  |

In Rel-14/Rel-16 LTE HST WI, the network signalling to enable the enhanced UE demodulation requirements to support LTE HST is cell specific configured and is included in the IE RadioResourceConfigCommonSIB and IE RadioResourceConfigCommon. For NR HST, the enhanced UE demodulation requirements are also cell specific configured and provided to UE in idle mode and connected mode.

Therefore if the answer to Q3 is yes, companies are kindly asked to provide their further comments on Q4.

Q4. As suggested in [3], do you agree to include the network assistant signalling to enable the enhanced UE demodulation requirements for Rel-16 NR HST in the IE ServingCellConfigCommon and IE ServingCellConfigCommonSIB?

| Company | Y/N | Comments |
| --- | --- | --- |
| Ericsson | Agree |  |

UE capability

Q5. As requested by RAN4 in [1], do you agree to introduce the UE capability for NR HST to indicate whether UE is capable of supporting the enhanced RRM requirements?

| Company | Y/N | Comments |
| --- | --- | --- |
| Ericsson | Y |  |

Q6. As requested by RAN4 in [2], do you agree to introduce the UE capability for NR HST to indicate whether UE is capable of the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h?

| Company | Y/N | Comments |
| --- | --- | --- |
| Ericsson | Y |  |

# Conclusions

TBD.

# References

[1] R2-2000040 LS on the UE capability and network assistance signalling for Rel-16 NR HST RRM

[2] R2-2001656 LS on the UE capability and network assistance signalling for Rel-16 NR HST demodulation

[3] R2-2002084 Discussion on signalling for R16 NR HST