3GPP TSG-RAN WG2 Meeting #109 electronic	draftR2-2001877

Electronic meeting, 24 Feb – 6 Mar 2020

Agenda Item:	7.1.5

Source:	Ericsson (Rapporteur)

Title:	Report of [AT109e][412][eMTC/NB-IoT] Scheduling multiple TBs: Open issues (Ericsson)

Document for:	Report

1	Introduction

This document contains the report of the following offline discussion:

[AT109e][412][eMTC/NB-IoT]  Scheduling multiple TBs: Open issues (Ericsson)

Scope: Further discussion on proposals 3, 4, 6, and 10 and identify potential agreements

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001877

Deadline: Tuesday, Mar 3rd 17:00 CET

Schedule: Wednesday, Mar 4th, 06:30 - 07:30 CET

The referenced proposals from R2-2001862 [1] are the following:

Proposal 3	For unicast, multiple TBs scheduling is enabled separately for uplink and downlink for both LTE-M and NB-IoT.

Proposal 4	Discuss over email the details of unicast configuration for scheduling multiple TBs for 1) NB-IoT and 2) LTE-M.

Proposal 6	Discuss over email whether scheduling gap configuration is in SC-MTCH or in SIB20(-NB) for 1) NB-IoT and 2) LTE-M with details of configuration to be captured.

Proposal 10	Discuss whether multiple TBs scheduling in multicast is optional without capability reporting.

2	Discussion

2.1 	Unicast configuration

In [2] (Huawei) in the text proposals, scheduling of multiple TBs can be enabled/disabled separately for uplink and downlink. In [3] (Ericsson) text proposal, enabling/disabling separately is possible for eMTC but this doesn't seem possible for NB-IoT. The text proposals are provided below for companies to comment on.

In [2] an explicit proposal was presented (for NB-IoT) on this and the following proposal was presented in the summary document:

Proposal 3		For unicast, multiple TBs scheduling is enabled separately for uplink and downlink for both LTE-M and NB-IoT.

This was however not agreed to during the brief online discussion. Companies are asked whether they agree to above proposal or if they have some concerns. Please elaborate in comments especially if not agreeable.

| Company | Agree P3 for NB-IoT? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree | We have agreed to have separate capabilities for UL/DL so it should be possible to enable one or the other only. |

| Company | Agree P3 for eMTC? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree | Same as above. |

Text proposals for NB-IoT on unicast configuration:

The following new information element is already captured in (endorsed) NB-IoT running TS 36.331 CR [4]:

–	MultiTB-Config-NB

The IE MultiTB-Config-NB is used to specify the multiple TBs scheduling configuration for unicast transmission.

MultiTB-Config-NB information element

-- ASN1START

MultiTB-Config-NB-r16 ::=			SEQUENCE {

...

}

-- ASN1STOP

The following text proposal has been provided in [2] (Huawei) for NB-IoT:

-- ASN1START

MultiTB-Config-NB-r16 ::=			SEQUENCE {

ul-MultiTB-Config-r16				ENUMERATED {interleaving, non-interleaving}

OPTIONAL, 	-- Need OR

dl-MultiTB-Config-r16				ENUMERATED {interleaving, non-interleaving}

OPTIONAL, 	-- Need OR

dl-HARQ-ACK-Bundling-r16			ENUMERATED {true}	OPTIONAL, 	-- Cond dl-interleaving

...

}

-- ASN1STOP

And based on RAN1 parameter list, the following is also proposed in [2] (Huawei):

-- ASN1START

MultiTB-Config-NB-r16 ::=			SEQUENCE {

ul-MultiTB-Config-r16				ENUMERATED {interleaving, non-interleaving}

OPTIONAL, 	-- Need OR

dl-MultiTB-Config-r16				ENUMERATED {interleaving, non-interleaving}

OPTIONAL, 	-- Need OR

dl-HARQ-ACK-Bundling-r16			ENUMERATED {true}	OPTIONAL, 	-- Cond dl-interleaving

...

}

-- ASN1STOP

The following text proposal has been proposed in [3] (Ericsson) for NB-IoT:

#### –	MultiTB-Config-NB

The IE MultiTB-Config-NB is used to specify the multiple TBs scheduling configuration for unicast transmission.

MultiTB-Config-NB information element

-- ASN1START

MultiTB-Config-NB-r16 ::=			SEQUENCE {

multi-TB-DL-Unicast-Interleaving-r16		ENUMERATED {on}		OPTIONAL,	-- Need OR

multi-TB-UL-Unicast-Interleaving-r16		ENUMERATED {on}		OPTIONAL,	-- Need OR

multi-TB-HARQ-ACK-Bundling-r16				ENUMERATED {on}		OPTIONAL	-- Need OR

}

-- ASN1STOP

MultiTB-Config-NB field descriptions

multi-TB-DL-Unicast-Interleaving Activation of interleaving of repetitions of separate transport blocks, when multiple downlink TBs are scheduled by one DCI, see TS 36.211 [21], TS 36.212 [22] and TS 36.213 [23].

multi-TB-UL-Unicast-Interleaving Activation of interleaving of repetitions of separate transport blocks, when multiple uplink TBs are scheduled by one DCI, see TS 36.211 [21], TS 36.212 [22] and TS 36.213 [23].

multi-TB-HARQ-ACK-Bundling Activation of HARQ-ACK feedback bundling, when a single DCI schedules multiple transport blocks for DL unicast, see TS 36.212 [22] and TS 36.213 [23]. HARQ-ACK bundling is only supported when interleaving is configured.

| Company | Comments on above text proposals (from [2] and [3]) for NB-IoT: |
| --- | --- |
| Huawei, HiSilicon | 1. We think multi-TB-DL-Unicast-Interleaving-r16 (and for UL) can not cover all cases. If absent, does it mean multiple TBs scheduling is not configured or multiple TBs is configued but interleaved transmission is not allowed? 2. We think condition for HARQ ACK Bundling configuration is needed as it only applies to DL interleaved transmission. |

Text proposals for eMTC on unicast configuration:

The following text proposal has been provided in [3] (Ericsson) for eMTC:

#### –	PDSCH-Config

The IE PDSCH-ConfigCommon and the IE PDSCH-ConfigDedicated are used to specify the common and the UE specific PDSCH configuration respectively.

PDSCH-Config information element

-- ASN1START

< removed unmodified part >

PDSCH-ConfigDedicated-v16xy ::=		SEQUENCE {

ce-PDSCH-MultiTB-AllocConfig-r16		CHOICE {

release				NULL,

setup				SEQUENCE {

multi-TB-DL-Interleaving-r16		ENUMERATED {on}				OPTIONAL,	-- Need OR

multi-TB-DL-HARQ-Bundling-r16		ENUMERATED {on}				OPTIONAL	-- Need OR

}

}

}

< removed unmodified part >

-- ASN1STOP

And following suggestions for field descriptions:

multi-TB-DL-Interleaving Activation of interleaving of repetitions of separate transport blocks, when a single DCI schedules multiple transport blocks for DL unicast in CE mode A/B in RRC_CONNECTED, see TS 36.213 [23].

multi-TB-DL-HARQ-Bundling Activation of HARQ-ACK feedback bundling, when a single DCI schedules multiple transport blocks for DL unicast in CE mode A in RRC_CONNECTED, see TS 36.212 [22] and TS 36.213 [23].

#### –	PUSCH-Config

The IE PUSCH-ConfigCommon is used to specify the common PUSCH configuration and the reference signal configuration for PUSCH and PUCCH. The IE PUSCH-ConfigDedicated is used to specify the UE specific PUSCH configuration.

PUSCH-Config information element

-- ASN1START

< removed unmodified part >

PUSCH-ConfigDedicated-v16xy ::=		SEQUENCE {

ce-PUSCH-MultiTB-AllocConfig-r16		CHOICE {

release				NULL,

setup				SEQUENCE {

multi-TB-UL-Interleaving-r16	ENUMERATED {on}					OPTIONAL	-- Need OR

}

}

}

< removed unmodified part >

-- ASN1STOP

And following suggestion for field description:

multi-TB-UL-Interleaving Activation of interleaving of repetitions of separate transport blocks, when a single DCI schedules multiple transport blocks for UL unicast in CE mode A/B in RRC_CONNECTED, see TS 36.213 [23].

| Company | Comments on above text proposals (from [3]) for eMTC |
| --- | --- |
| Huawei, HiSilicon | We are fine with above TP. |

Rapporteur summary: TBD

2.2	Configuration for scheduling gap

The following proposals related to scheduling gap configuration are proposed by Huawei and Ericsson:

multiTB-Gap is introduced in SC-MTCH-Info-NB-r14 to indicate the scheduling gap for each SC-MTCH configured with multiple TBs scheduling. [2] (Huawei)

Scheduling gaps for multi-TB scheduling with multicast are configured in SIB20 for LTE-M.[3] (Ericsson)

Scheduling gaps for multi-TB scheduling with multicast are configured in SIB20-NB for NB-IoT.[3] (Ericsson)

Thus, the two companies who have provided input on this issue to this meeting have different view on whether the configuration should be in SC-MCCH or in system information. Related proposal in [1] is:

Proposal 6		Discuss over email whether scheduling gap configuration is in SC-MTCH or in SIB20(-NB) for 1) NB-IoT and 2) LTE-M with details of configuration to be captured.

Companies are asked to provide their views and preferences on this:

| Company | Scheduling gap configuration in SC-MCCH or in SIB20(-NB)? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | SC-MCCH | We think SIB20 should be used to configure SC-MCCH transmission. Configuration for SC-MTCH should be siganlled in SC-MCCH. |

The rapporteur suggestion is to first try to achieve consensus on above and then look at possible text proposals, as those should not be very controversial once the location is agreed.

Rapporteur summary: TBD

2.3	Capabilities

The following proposal was also not agreed during online discussion, and companies are welcome to provide their views on this:

Proposal 10		Discuss whether multiple TBs scheduling in multicast is optional without capability reporting.

| Company | Scheduling multiple TBs in multicast is optional without capability reporting? Please elaborate. |
| --- | --- |
| Huawei, HiSilicon | We support this. |

Rapporteur summary: TBD

3	Summary

TBD

References

R2-2001862, Summary of contributions on scheduling multiple Dl/UL transport blocks, Ericsson, RAN2#109-e, February 2020

R2-2000644, Signalling aspect of multiple TBs scheduling for NB-IoT, Huawei, HiSilicon, RAN2#109e, February 2020

R2-2000977, Scheduling enhancements for LTE-M and NB-IoT, Ericsson, RAN2#109e, February 2020

R2-2000620, Introduction of additional enhancements for NB-IoT in TS 36.331, Huawei, RAN2#109-e, February 2020.