3GPP TSG-RAN WG2 Meeting #109-e	R2-20xxxxx

# Online, February 24 – March 06, 2020

Agenda item:	6.8.2.5

Source: 	Qualcomm Incorporated

Title: 	Summary of [AT109e][624][POS] Open issues on UE-based downlink positioning assistance data

Document for:		Discussion and Decision

# 1. 	Introduction

This document summarizes the following email discussion:

[AT109e][624][POS] Open issues on UE-based downlink positioning assistance data (Qualcomm)

Intended outcome: Updated TP reflecting agreement where possible on the open issues:

(a) 	whether beamwidth information can be provided in the assistance data in Rel-16 or 			should be deferred to e.g. Rel-17;

(b) 	whether to support LCS-to-GCS translation parameter for the spatial direction 				information;

(c)		whether to include RTD drift rate in the assistance data;

(d)		whether to include RTD per DL-PRS Resource.

Updated TP in R2-2001949.

Deadline:  Wednesday 2020-03-04 1300 CET

The summary of "[108#89][NR/Pos] UE-based downlink positioning assistance data" email discussion held between RAN2#108 and RAN2#109-e can be found in R2-2001234.

# 2. 	Discussion

## 2.1		Beamwidth information for DL-PRS resources

The inclusion of beamwidth information for DL-PRS resources was proposed by Qualcomm in R2-2001244 ("Remaining details for UE-based downlink positioning assistance data").

The motivation can be briefly summarized as follows:

For DL-AoD positioning, the UE may make DL-PRS RSRP measurements of DL-PRS resources from the same TRP. The TRPs may transmit beamformed DL-PRS in a beam sweeping manner that may be measured by the UE. The UE may identify the beam with the e.g. strongest RSRP and use the corresponding direction (azimuth, elevation) as AoD estimate.  However, each beam has a certain width which essentially determines the AoD uncertainty. A simple first order approximation of the beamshape could be provided using the DL-PRS beamwidth. If the UE measures two adjacent beams, the beamwidth information can be used to determine a better fit of the AoD between the two PRS beams.

More advanced techniques may make use of more accurate parametrized beamshape information. For DL-AoD positioning, the gNB beam pattern needs to be known in advance, as also described in R2-2000969 ("Discussion on DL-AoD positioning procedure", Huawei, HiSilicon). The beamwidth would allow a simple 1st–order approximation of the beam pattern.

Question 1:	Do companies agree that beamwidth information can optionally be provided in the assistance data as part of the spatial direction information of the DL-PRS Resources (IE NR-DL-PRS-BeamInfo)? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |

## 2.2. 	LCS-to-GCS translation parameter for the spatial direction information

The inclusion of LCS-to-GCS translation parameter was originally proposed by Huawei/HiSilicon during the email discussion [108#89][NR/Pos] mentioned in section 1 above.

It was also supported by Qualcomm in R2-2001244 ("Remaining details for UE-based downlink positioning assistance data").

The motivation can be briefly summarized as follows:

As agreed by RAN1, the DL-PRS boresight direction can be provided in either a Global Coordinate System (GCS) or Local Coordinate System (LCS). Therefore, if the DL-PRS Resource AoD is provided in LCS, the conversion parameter need to be provided as well (α (bearing angle), β (downtilt angle),  γ  (slant angle)).

Question 2:	Do companies agree that LCS-to-GCS translation parameter can be provided in the assistance data as part of the spatial direction information of the DL-PRS Resources (IE NR-DL-PRS-BeamInfo)? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |

## 2.3 	RTD drift rate

The inclusion of RTD drift rate as part of the RTD information was proposed by Qualcomm in R2-2001244 ("Remaining details for UE-based downlink positioning assistance data").

The motivation can be briefly summarized as follows:

Existing UE-based TDOA related specifications support a RTD drift rate (e.g., 3GPP TS 25.331, OMA LPPe).

One of the reasons is the base station frequency accuracy requirements, which for NR are specified in TS 38.104 (these requirements are the same as for LTE or UMTS).

3GPP specifications require frequency stability of 0.05-0.1 ppm on the air interface. 0.1 ppm equals 100 ns drift after 1 second, which corresponds to about 30 m/s drift rate (which is the value range supported in UMTS and LPPe specifications for the RTD drift rate).  The DL-PRS periodicity can be up to 10.24 seconds, and therefore, RTD drift can have a significant adverse impact to positioning accuracy.

An RTD drift rate if available can prolong the validity time of the RTD. The RTD value is essentially one "snapshot" of the RTD, valid at the reference time. If the RTD drift rate is available, the UE can take this into account when extrapolating the RTD at a delta-time from the RTD reference time.

Question 3:	Do companies agree that RTD Drift Rate can be provided in the assistance data as part of the RTD information (IE NR-RTD-Info)? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |

## 2.4 	RTD information per DL-PRS Resource

The inclusion of RTD information per DL-PRS Resource as part of the RTD information was proposed by Qualcomm in R2-2001244 ("Remaining details for UE-based downlink positioning assistance data").

The motivation can be briefly summarized as follows:

The DL-PRS RSTD measurements in NR are made per DL-PRS Resource. The UE can report the PRS resource ID(s) or PRS resource set ID(s) used for determining the timing of each TRP in the RSTD measurements. This information may be used by an LMF to e.g., determine the correct transmission reference point and timing required for position calculation.

Thus, it is reasonable for the RTD information to also be indicated per DL-PRS Resource for UE-based. If beams for different PRS resources are formed using different sets of antenna elements, possibly from different sets of panels that are driven by different clocks or have other internal HW delay variations, such information would allow determining the RTD more precisely.

Question 4:	Do companies agree that RTD can optionally be provided per DL-PRS Resource in the assistance data (IE NR-RTD-Info)? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |

## 2.5 	Resolution of Angular Parameter

It was commented during email discussion [108#89][NR/Pos] mentioned in section 1 above, that angular information should be provided in two steps: (a) 1-degree resolution, and (b) optionally 0.1-degree delta resolution (Ericsson).

A corresponding TP could be as follows:

-- ASN1START

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

dl-PRS-Azimuth-r16				INTEGER (0..3599),

dl-PRS-Azimuth-Delta-r16		INTEGER (0..9)					OPTIONAL,	-- Need ON

dl-PRS-Elevation-r16			INTEGER (0..1800)				OPTIONAL,	-- Need ON

dl-PRS-Elevation-Delta-r16		INTEGER (0..9)					OPTIONAL,	-- Need ON

-- FFS for HPBW

--	dl-PRS-HPBW-Az-r16 				INTEGER (0..1200)				OPTIONAL,	-- Need ON

--	dl-PRS-HPBW-El-r16 				INTEGER (0..1200)				OPTIONAL,	-- Need ON

...

}

LCS-GCS-Translation-Parameter-r16 ::= SEQUENCE {

alpha-r16						INTEGER (0..3599),

alpha-delta-r16					INTEGER (0..9)					OPTIONAL,	-- Need ON

beta-r16						INTEGER (0..3599),

beta-delta-r16					INTEGER (0..9)					OPTIONAL,	-- Need ON

gamma-r16						INTEGER (0..3599),

gamma-delta-r16					INTEGER (0..9)					OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

Question 5:	Which option should be used for providing the angular information?

Option 1: 	Single field with 0.1-degrees resolution.

Option 2: 	Two  fields:  One field with 1-degrees resolution, and a second field with 0.1 delta-resolution. Please provide also a brief justification for your answer.

| Company | Option 1 or Option2 | Comments |
| --- | --- | --- |

# 3. 	Summary

# Annex: 	Text Proposal

## LPP Assistance Data Transfer

#### –	NR-PositionCalculationAssistance

The IE NR-PositionCalculationAssistance is used by the location server to provide assistance data to enable UE-based downlink positioning.

-- ASN1START

# NR-PositionCalculationAssistance-r16 ::= SEQUENCE {

nr-trp-LocationInfo-r16 		NR-TRP-LocationInfo-r16				OPTIONAL,	-- Need ON

nr-dl-prs-BeamInfo-r16			NR-DL-PRS-Beam-Info-r16				OPTIONAL,	-- Need ON

nr-rtd-Info-r16					NR-RTD-Info-r16						OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

NR-PositionCalculationAssistance field descriptions

nr-trp-LocationInfo This field provides the location coordinates of the antenna reference points of the TRPs.

nr-dl-prs-BeamInfo This field provides the spatial directions of DL-PRS Resources for TRPs.

nr-rtd-Info This field provides the time synchronization information between the reference TRP and neighbour TRPs.

#### –	NR-TRP-LocationInfo

The IE NR-TRP-LocationInfo is used by the location server to provide the coordinates of the antenna reference points for a set of TRPs. For each TRP, the ARP location can be provided for each associated PRS Resource ID per PRS Resource Set.

-- ASN1START

NR-TRP-LocationInfo-r16 ::= SEQUENCE (SIZE (1..4)) OF NR-TRP-LocationInfoPerFreqLayer-r16

NR-TRP-LocationInfoPerFreqLayer-r16 ::= SEQUENCE {

referencePoint-r16			ReferencePoint-r16				OPTIONAL,	-- Cond NotSameAsPrev

trp-LocationInfoList-r16	SEQUENCE (SIZE (1..64)) OF TRP-LocationInfoElement-r16,

...

}

TRP-LocationInfoElement-r16 ::= SEQUENCE {

trp-id-r16						TRP-ID-r16,

trp-Location-r16				RelativeLocation-r16					OPTIONAL,	-- Need OP

trp-DL-PRS-ResourceSets-r16		SEQUENCE (SIZE(1..2)) OF

DL-PRS-ResourceSets-TRP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-ResourceSets-TRP-Element-r16 ::= SEQUENCE {

dl-PRS-ResourceSetARP-r16			RelativeLocation-r16				OPTIONAL,	-- Need OP

dl-PRS-Resource-ARP-List-r16		SEQUENCE (SIZE(1..64)) OF

DL-PRS-Resource-ARP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-Resource-ARP-Element-r16 ::= SEQUENCE {

dl-PRS-Resource-ARP-location-r16	RelativeLocation-r16				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsPrev | The field is mandatory present in the first entry of the NR-TRP-LocationInfoPerFreqLayer list; otherwise it is optionally present, need OP. |

NR-TRP-LocationInfo field descriptions

referencePoint This field specifies the reference point used to define the TRP location in the trp-LocationInfoList. If this field is absent, the reference point is the same as in the previous entry of the NR-TRP-LocationInfoPerFreqLayer list.

trp-LocationInfoList This field provides the antenna reference point locations of the DL-PRS Resources for the TRPs and comprises the following sub-fields: - trp-id: This field provides an identity of the TRP. - trp-Location: This field provides the location of the TRP relative to the referencePoint location. If this field is absent the TRP location coincides with the referencePoint location. - trp-DL-PRS-ResourceSets: This field provides the antenna reference point location(s) of the DL-PRS Resource Set(s) associated with this TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resource Set(s) coincides with the trp-Location location. This field comprises the following sub-fields: - dl-PRS-ResourceSetARP: This field provides the antenna reference point location of the DL-PRS Resource Set relative to the trp-Location location. If this field is absent, the antenna reference point location of this DL-PRS Resource Set coincides with the trp-Location location. - dl-PRS-Resource-ARP-List: This field provides the antenna reference point location(s) of the DL-PRS Resource(s) associated with this resource set of the TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resources coincides with the dl-PRS-ResourceSetARP location. This field comprises the following sub-fields: - dl-PRS-Resource-ARP-location: This field provides the antenna reference point location of the DL-PRS Resource associated with the DL-PRS Resource Set of the TRP relative to the dl-PRS-ResourceSetARP location. If this field is absent, the antenna reference point location of this DL-PRS Resource coincides with the dl-PRS-ResourceSetARP location.

#### –	ReferencePoint

The IE ReferencePoint provides a well defined location relative to which other locations may be defined.

-- ASN1START

# ReferencePoint-r16 ::= SEQUENCE {

referencePointGeographicLocation-r16 		CHOICE {

location3D-r16 			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ha-location3D-r16 		HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

...

},

...

}

-- ASN1STOP

ReferencePoint field descriptions

referencePointGeographicLocation This field provides the geodetic location of the reference point.

#### –	RelativeLocation

The IE RelativeLocation provides a location relative to some known reference location.

-- ASN1START

# RelativeLocation-r16 ::= SEQUENCE {

milli-arc-second-units-r16 	ENUMERATED { mas0-03, mas0-3, mas3, mas30, ...},

height-units-r16			ENUMERATED {mm, cm, m, ...},

delta-latitude-r16			Delta-Latitude-r16,

delta-longitude-r16			Delta-Longitude-r16,

delta-height-r16			Delta-Height-r16,

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

...

}

Delta-Latitude-r16 ::= SEQUENCE {

delta-Latitude-r16						INTEGER (-1024..1023),

coarse-delta-Latitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Longitude-r16 ::= SEQUENCE {

delta-Longitude-r16						INTEGER (-1024..1023),

coarse-delta-Longitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Height-r16 ::= SEQUENCE {

delta-Height-r16						INTEGER (-1024..1023),

coarse-delta-Height-r16					INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

LocationUncertainty-r16 ::= SEQUENCE {

horizontalUncertainty-r15				INTEGER (0..255),

horizontalConfidence-r15				INTEGER (0..100),

verticalUncertainty-r15					INTEGER (0..255),

verticalConfidence-r15					INTEGER (0..100)

}

-- ASN1STOP

RelativeLocation field descriptions

milli-arc-second-units This field provides the units and scale factor for the delta-latitude and delta-longitude fields. Enumerated values mas0-03, mas0-3, mas3, and mas30, correspond to 0.03, 0.3, 3, and 30 milliarcseconds, respectively.

height-units This field provides the units and scale factor for the delta-height field. Enumerated values mm, cm, and m correspond to 10-3 metre, 10-2 metre, and 1 metre, respectively.

delta-latitude This field specifies the delta value in latitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Latitude specifies the delta value in latitude in the unit provided in milli-arc-second-units field. - coarse-delta-Latitude specifies the delta value in latitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Latitude field. If this field is absent, the value for coarse-delta-Latitude is zero. I.e., the full delta-latitude is given by: (delta-Latitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-longitude This field specifies the delta value in longitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Longitude specifies the delta value in longitude in the unit provided in milli-arc-second-units field. - coarse-delta-Longitude specifies the delta value in longitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Longitude field. If this field is absent, the value for coarse-delta-Longitude is zero. I.e., the full delta-longitude is given by: (delta-Longitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-height This field specifies the delta value in ellipsoidal height of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Height specifies the delta value in ellipsoidal height in the unit provided in height-units field. - coarse-delta-Height specifies the delta value in ellipsoidal height in 1024 times the size of the unit provided in height-units field and with the same sign as in the delta-Height field. If this field is absent, the value for coarse-delta-Height is zero. I.e., the full delta-height is given by: (delta-Height × height-units) ± (coarse-delta-Height × 1024 × height-units) [metres]

locationUNC This field specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The 'verticalUncertainty' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'verticalConfidence' corresponds to confidence as defined in TS 23.032 [15]. If this field is absent, the uncertainty is the same as for the associated reference point location.

Editor’s NOTE:	Inclusion of HPBW parameter in DL-PRS-Beam-Info is FFS.

#### –	NR-DL-PRS-BeamInfo

The IE NR-DL-PRS-BeamInfo is used by the location server to provide spatial direction information of the DL-PRS Resources.

-- ASN1START

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE (SIZE (1..4)) OF NR-DL-PRS-BeamInfoPerFreqLayer-r16

# NR-DL-PRS-BeamInfoPerFreqLayer-r16 ::= SEQUENCE (SIZE (1..64)) OF NR-DL-PRS-BeamInfo-r16

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE {

trp-id-r16							TRP-ID-r16,

lcs-gcs-translation-parameter-r16	LCS-GCS-Translation-Parameter-r16	OPTIONAL,	-- Need OP

dl-prs-BeamInfoSet-r16				DL-PRS-BeamInfoSet-r16,

...

}

DL-PRS-BeamInfoSet-r16 ::= SEQUENCE (SIZE(1..2)) OF DL-PRS-BeamInfoResourceSet-r16

DL-PRS-BeamInfoResourceSet-r16 ::= SEQUENCE (SIZE(1..64)) OF DL-PRS-BeamInfoElement-r16

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

dl-PRS-Azimuth-r16				INTEGER (0..3599),

dl-PRS-Elevation-r16			INTEGER (0..1800)				OPTIONAL,	-- Need ON

-- FFS for HPBW

--	dl-PRS-HPBW-Az-r16 				INTEGER (0..1200)				OPTIONAL,	-- Need ON

--	dl-PRS-HPBW-El-r16 				INTEGER (0..1200)				OPTIONAL,	-- Need ON

...

}

LCS-GCS-Translation-Parameter-r16 ::= SEQUENCE {

alpha-r16						INTEGER (0..3599),

beta-r16						INTEGER (0..3599),

gamma-r16						INTEGER (0..3599),

...

}

-- ASN1STOP

NR-DL-PRS-Beam-Info field descriptions

trp-id This field provides an identity of the TRP.

lcs-gcs-translation-parameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [x]. If this field is absent, the dl-PRS-Azimuth and dl-PRS-Elevation are provided in a GCS.

dl-prs-BeamInfoSet This field provides the DL-PRS beam information for each DL-PRS Resource of the DL-PRS Resource Set associated with this TRP.

dl-PRS-Azimuth This field specifies the azimuth angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the azimuth angle is measured counter-clockwise from geographical North. For a Local Coordinate System (LCS), the azimuth angle is measured measured counter-clockwise from the x-axis of the LCS. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

dl-PRS-Elevation This field specifies the elevation angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the elevation angle is measured relative to zenith and positive to the horizontal direction (elevation 0 deg. points to zenith, 90 deg to the horizon). For a Local Coordinate System (LCS), the elevation angle is measured relative to the z-axis of the LCS (elevation 0 deg. points to the z-axis, 90 deg to the x-y plane). Scale factor 0.1 degrees; range 0 to 180 degrees.

dl-PRS-HPBW-Az This field specifies the half-power beamwidth (HPBW) in the horizontal plane of the beam in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. HPBW is the angle subtended by the half-power points of the mainlobe in the horizontal plane. Scale factor 0.1 degrees; range 0 to 120 degrees.

dl-PRS-HPBW-El This field specifies the half-power beamwidth (HPBW) in the vertical plane of the beam in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. HPBW is the angle subtended by the half-power points of the mainlobe in the vertical plane. Scale factor 0.1 degrees; range 0 to 120 degrees.

alpha This field specifies the bearing angle α for the translation of the LCS to a GCS as defined in TR 38.901 [x]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

beta This field specifies the downtilts angle β for the translation of the LCS to a GCS as defined in TR 38.901 [x]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

gamma This field specifies the slant angle γ for the translation of the LCS to a GCS as defined in TR 38.901 [x]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

Editor’s NOTE:	Inclusion of rtd-DriftRate and delta-rtd-set parameter in RTD-Info is FFS.

#### –	NR-RTD-Info

The IE NR-RTD-Info is used by the location server to provide time synchronization information between a reference TRP and a list of neighbour TRPs.

-- ASN1START

# NR-RTD-Info-r16 ::= SEQUENCE {

referenceTRP-RTD-Info-r16		ReferenceTRP-RTD-Info-r16,

rtd-InfoList-r16				RTD-InfoList-r16,

...

}

ReferenceTRP-RTD-Info-r16 ::= SEQUENCE {

ref-trp-id-r16					TRP-ID-r16,

refTime-r16						CHOICE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utc-r16						UTCTime,

...

},

rtd-RefQuality-r16				NR-MeasQuality-r16				OPTIONAL,	-- Need ON

...

}

RTD-InfoList-r16 ::= SEQUENCE (SIZE (1..4)) OF RTD-InfoListPerFreqLayer-r16

RTD-InfoListPerFreqLayer-r16 ::= SEQUENCE (SIZE(1..63)) OF RTD-InfoElement-r16

RTD-InfoElement-r16 ::= SEQUENCE {

trp-id-r16					TRP-ID-r16,

subframeOffset-r16			INTEGER (0..1966079),

rtd-Quality-r16				NR-MeasQuality-r16,

-- FFS on drift rate

--	rtd-DriftRate-r16			INTEGER (-256..255)					OPTIONAL,	-- Need ON

-- FFS on delta-rtd-set

--	delta-rtd-set-r16			SEQUENCE (SIZE (1..2)) OF

--										Delta-RTD-ResourceSet-r16	OPTIONAL,	-- Need ON

...

}

Delta-RTD-ResourceSet-r16 ::= SEQUENCE {

delta-rtd-r16				INTEGER (-64..63)					OPTIONAL,	-- Need ON

delta-rtd-ResourceList-r16	SEQUENCE (SIZE (1..64)) OF

Delta-RTD-ResourceElement-r16	OPTIONAL,	-- Need ON

...

}

Delta-RTD-ResourceElement-r16 ::= SEQUENCE {

delta-rtd-r16				INTEGER (-64..63)					OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

NR-RTD-Info field descriptions

referenceTRP-RTD-Info This field defines the reference TRP for the RTD and comprises the following sub-fields: - ref-trp-id: This field specifies the identity of the reference TRP. - refTime: This field specifies the reference time at which the rtd-InfoList is valid. The systemFrameNumber choice refers to the SFN of the reference TRP. - rtd-RefQuality: This field specifies the quality of the timing of reference TRP, used to determine the RTD values provided in rtd-InfoList.

trp-id-r16 This fields provides the identity of the TRP for which the RTD-InfoElement is applicable.

subframeOffset This field specifies the subframe boundary offset at the TRP antenna location between the reference TRP and this neighbour TRP in time units where Δfmax=480∙103 Hz and (TS 38.211 [x]). The offset is counted from the beginning of a subframe #0 of the reference TRP to the beginning of the closest subsequent subframe of this neighbour TRP. Scale factor 1 Tc.

rtd-Quality This field specifies the quality of the RTD.

rtd-DriftRate This field specifies the drift rate of the RTD between the reference TRP and this neighbour TRP in units of 0.5 Tc per second. A positive value indicates that the reference TRP clock is running at a greater frequency than the neighbour TRP clock. Scale factor 0.5 Tc/sec.

delta-rtd This field provides a delta RTD value to be added to the RTD of the TRP for the corresponding DL-PRS Resource in the Resource Set. Scale factor 0.5 Tc

## posSIB Types Element Definitions

#### –	NR-UEB-TRP-LocationData

The IE NR-UEB-TRP-LocationData is used in the assistanceDataElement if the posSibType in IE PosSIB-Type defined in TS 38.331 [x] indicates 'posSibTypeX-y'.

-- ASN1START

NR-UEB-TRP-LocationData-r16 ::= SEQUENCE {

nr-trp-LocationInfo-r16 			NR-TRP-LocationInfo-r16,

nr-dl-prs-BeamInfo-r16				NR-DL-PRS-Beam-Info-r16			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

NR-UEB-TRP-LocationData field descriptions

nr-trp-LocationInfo LPP IE NR-TRP-LocationInfo as defined in sub-clause x.y.z.b.

nr-dl-prs-BeamInfo LPP IE NR-DL-PRS-Beam-Info as defined in sub-clause x.y.z.c.

#### –	NR-UEB-TRP-RTD-Info

The IE NR-UEB-TRP-RTD-Info is used in the assistanceDataElement if the posSibType in IE PosSIB-Type defined in TS 38.331 [x] indicates 'posSibTypeX-z'.

-- ASN1START

NR-UEB-TRP-RTD-Info-r16 ::= SEQUENCE {

nr-rtd-Info-r16						NR-RTD-Info-r16,

...

}

-- ASN1STOP

NR-UEB-TRP-RTD-Info field descriptions

nr-rtd-Info LPP IE NR-RTD-Info as defined in sub-clause x.y.z.c.