3GPP TSG-RAN WG2 #109-e	 R2-2000882

# Electronic Meeting, 24th February – 6th March 2020	(Revision of R2-1915378)

Agenda Item:	6.4.2.2

Source:	Ericsson

Title:	Discussion on SL Mode 2 left issues

Document for:	Discussion, Decision

# Introduction

Two sidelink resource allocation (RA) modes are defined for NR-V2X sidelink communication, i.e. mode 1 RA, where base station schedules sidelink resource(s) to be used by UE, and mode 2 RA, where UE determines sidelink transmission resource(s) within sidelink resources configured by base station/network or pre-configured sidelink resources.

During RAN1#96, the following related to mode 2 RA was agreed:

- Agreements: - Blind retransmissions of a TB are supported for SL by NR-V2X - Details are for the WI phase - Agreements: - NR V2X Mode-2 supports reservation of sidelink resources at least for blind retransmission of a TB - Whether reservation is supported for initial transmission of a TB is to be discussed in the WI phase - Whether reservation is supported for potential retransmissions based on HARQ feedback is for the WI phase

During RAN1#96bis, the following related to mode 2 RA was agreed:

- Agreements: - NR V2X supports an initial transmission of a TB without reservation, based on sensing and resource selection procedure - NR V2X supports reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB, based on sensing and resource selection procedure - This functionality can be enabled/disabled by (pre-)configuration - FFS Standalone PSCCH transmissions for resource reservations are supported in NR V2X

During RAN1#97, the following related to mode 2 RA was agreed:

- Agreements: - NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmissions by signaling associated with a prior transmission of the same TB - FFS impact on subsequent sensing and resource selection procedures - At least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported - No additional signaling is defined for the purpose of release of unused resources by the transmitting UE

In this paper, we will discuss left issues related to SL Mode 2. These include left issues on resource selection for HARQ retransmissions. In the context of sidelink resource allocation, the focus is on resource reservation in mode 2 resource allocation. Further, issues related to multiple resource pool configuration and mode 2 resource pool selection are discussed.

# Discussion

## Resource pool configuration and selection

In the RAN2#106 meeting, it was confirmed that only single carrier is assumed for SL transmission in release 16. However, it is still open if one carrier can be configured with single or multiple resource pools.

RAN2#106 Agreements on LCP: 1: As, in release 16, only single carrier is used for SL transmission, RAN2 assumes mapping restriction between SCS and Sidelink LCH should not be considered in SL LCP procedure.

It is unclear if multiple resource pools can be configured in a single carrier for SL transmission.

In another aspect, NR SL supports different HARQ configurations, and the transmitted TB may request ACK/NACK feedback, or NACK only feedback, or no feedback at all. Transmitting a HARQ feedback must use the PSFCH resource configured in the same resource pool as agreed in RAN1#96bis meeting. Therefore, selection of resource pool must consider if the resource pool can support demanded HARQ procedure.

RAN1#96bis (Xi’an, China) Agreements: - It is supported, in a resource pool, that within the slots associated with the resource pool, PSFCH resources can be (pre)configured periodically with a period of N slot(s) - N is configurable, with the following values - 1 - At least one more value >1 - FFS details - The configuration should also include the possibility of no resource for PSFCH. In this case, HARQ feedback for all transmissions in the resource pool is disabled - HARQ feedback for transmissions in a resource pool can only be sent on PSFCH in the same resource pool

To enable HARQ feedback transmission, the selected resource pool must be configured with PSFCH resource.

In Mode 1, when gNB assigns SL grant to a UE, the resource pool selection is implicitly considered and whether the given SL grant is expecting a HARQ feedback is indicated in the DCI as proposed in our companion paper [4].

In Mode 2, the UE must autonomously select resources for transmission, which may be organized in multiple transmit pools. As stated above, a transmission pool may or may not be configured with feedback PHY resources (i.e., PSFCH resources). Thus, the UE must first select a pool. The selection of the pool may be restricted, at least, by the HARQ mode required for transmission. Once the pool is selected, the resource allocation procedure determines the resources to be used for transmission. In this way, by appropriately selecting the pool and resources from within that pool, the resulting grant is suitable for requesting SL HARQ feedback, if so required. Example restrictions could be:

- For SL transmissions triggered by data that requires HARQ feedback, a resource pool with PSFCH resource is selected.

- For SL transmissions triggered by data that does not require HARQ feedback, any resource pool can be selected.

To obtain a Mode-2 grant, the UE first selects a pool and then applies the resource allocation procedure to the selected pool.

RAN2 discusses Mode 2 pool selection restriction, at least, by the HARQ mode required for transmission. 

a.	For SL transmissions triggered by data that requires HARQ feedback, a resource pool with PSFCH resource is selected. 
b.	For SL transmissions triggered by data that does not require HARQ feedback, any resource pool can be selected.

## Resource selection for HARQ retransmissions

In NR SL mode 2, after selecting resources for the initial transmission of a MAC PDU, UE will then select resources for required HARQ retransmissions as highlighted in yellow. However, it’s not clear what will happen if the available resources are not enough for the required number of HARQ retransmissions. For example, N times HARQ retransmission is selected for one MAC PDU, however the available resources can only support N-1 times HARQ retransmission. In our view, the simplest way out is to not do any HARQ retransmission. Another alternative is to only perform those number of HARQ retransmissions that can be supported and forget about those left.

38.321 running CR: 5.x.1.1: If the MAC entity is configured by RRC to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [xy] based on sensing or random selection, the MAC entity shall for each Sidelink process: 1> if the MAC entity selects to create a configured sidelink grant corresponding to transmissions of multiple MAC PDUs, and SL data is available in a logical channel: 2> perform the TX resource (re-)selection check as specified in clause 5.x.1.2; 2> if the TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check; 3> select one of the allowed values configured by RRC in reservationPeriodAllowed and set the resource reservation interval with the selected value; 3> randomly select, with equal probability, an integer value in the interval [5, 15] for the resource reservation interval higher than or equal to 100ms and set SL_RESOURCE_RESELECTION_COUNTER to the selected value; … 3> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer according to clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 3> use the randomly selected resource to select a set of periodic resources spaced by the resource reservation interval for transmissions of PSCCH and PSSCH corresponding to the number of transmission opportunities of MAC PDUs determined in TS 38.214 [7]; 3> if one or more HARQ retransmissions are selected: 4> if there are available resources left in the resources indicated by the physical layer according to clause 8.1.4 of TS 38.214 [7] for more transmission opportunities: … 5> consider the set of new transmission opportunities and retransmission opportunities as the selected sidelink grant. 3> else: 4> consider the set as the selected sidelink grant; 3> use the selected sidelink grant to determine the set of PSCCH durations and the set of PSSCH durations according to TS 38.214 [7]; 3> consider the selected sidelink grant to be a configured sidelink grant.

UE behavior in case of not enough resources for required HARQ retransmissions is unclear.

RAN2 discusses the UE behavior in case of not enough resources for the required number of HARQ retransmissions considering the following alternatives:

  - Do not select resources for any HARQ retransmission

  - Select resources for as many HARQ retransmissions as possible

Besides, when selecting resources for periodic MAC PDUs, UE first selects resources for the initial transmission and required HARQ retransmissions for the first MAC PDU, then extends the selected set of resources for future MAC PDUs by based on the selected reservation interval as highlighted in blue. For example, as illustrated in Figure 1, the initial transmission and HARQ retransmission of MAC PDU#1 are selected within the scheduling window bounded by T1 and T2, then the initial transmission of the MAC PDU#2 will take place P slots (i.e., the period) after the initial transmission of MAC PDU#1, so as for HARQ retransmissions. However, there is a chance that the resources selected for the retransmission of MAC PDU#1 collides with the resources reserved for the retransmission of MAC PDU#2. For example, if the first and second retransmission of MAC PDU#1 are scheduled at slot 10 and 20, given a period of 10 slot the first retransmission of MAC PDU#2 will occur at slot 20 which collides with the second retransmission of MAC PDU#1.

To resolve this issue, when selecting the resource reservation interval from reservationPeriodAllowed, the duration of the selection window (i.e. T2-T1) must be taken into account. In particular, the resource reservation interval has to be larger than the selection window duration (i.e. T2-T1), such that the initial transmission of the next MAC PDU will be scheduled after the HARQ retransmissions of the current MAC PDU.

Figure 1: Illustration of resource reservation collision for period MAC PDU transmissions

Without restriction on resource reservation interval selection, the HARQ retransmissions for periodic MAC PDUs might collide.

The selected resource reservation interval is larger than the selection window duration, i.e., T2-T1.

## Other issues related to resource reservation

When reservation is enabled, a PSCCH for resource reservation is transmitted in a resource which is deemed to be idle. When a surrounding UE wants to transmit, the UE first detects and decodes the PSCCH for resource reservation, and avoids to select resources indicated in the PSCCH for its own transmission, so as to avoid a collision. It can be seen that sensing is the foundation to make reservation really useful. Without sensing it is useless to have reservation, for both initial transmission and retransmission, as a UE will neglect whether a resource is reserved or not when it selects resource for its own transmission.

Without sensing it is useless to have reservation, for both initial transmission and retransmission.

On the other hand, in some cases the sensing functionality is not available. For instance, in exceptional resource pool only random resource selection is supported, and in a resource  pool used for V2X communication, it could be configured so that only random resource selection is allowed [1]. Therefore, resource reservation should be disabled for the pools where sensing is not supported or allowed.

Resource reservation should be disabled for the pools where sensing is not used, i.e. exceptional pool.

For a connected UE adopting either mode 1 or mode 2 RA, the NW could configure dedicated and exclusive Tx resource pool for it. In this case and when mode 2 RA the UE knows what resource is available in the resource pool and is able to select a collision free resource for (re)transmission even without sensing, naturally reservation is also not needed in this case.

Resource reservation is not necessary in a resource pool dedicated and exclusive for a specific UE.

Another aspect worth discussion is whether/how to support resource reservation for initial transmission. In our companion paper [2], methods are proposed to enable reservation for initial transmission. On the other hand, it may be too late to use the reserved resources if the services require very low latency. Indeed, reservation of the initial transmissions would add a delay to the transmission of a TB. However, for latency critical cases, the delays in transmissions of a TB have to be minimized. Considering this, resource reservation of initial transmission could be disabled for services for which the required latency is below a certain level.

It may be too late to use the reserved resources for initial transmission if the services require very low latency.

Resource reservation of initial transmission could be disabled for services requiring low latency.

# Conclusion

In section 2 we made the following observations:

Observation 1	It is unclear if multiple resource pools can be configured in a single carrier for SL transmission.

Observation 2	To enable HARQ feedback transmission, the selected resource pool must be configured with PSFCH resource.

Observation 3	To obtain a Mode-2 grant, the UE first selects a pool and then applies the resource allocation procedure to the selected pool.

Observation 4	UE behavior in case of not enough resources for required HARQ retransmissions is unclear.

Observation 5	Without restriction on resource reservation interval selection, the HARQ retransmissions for periodic MAC PDUs might collide.

Observation 6	Without sensing it is useless to have reservation, for both initial transmission and retransmission.

Observation 7	Resource reservation is not necessary in a resource pool dedicated and exclusive for a specific UE.

Observation 8	It may be too late to use the reserved resources for initial transmission if the services require very low latency.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 discusses Mode 2 pool selection restriction, at least, by the HARQ mode required for transmission.   
a.  For SL transmissions triggered by data that requires HARQ feedback, a resource pool with PSFCH resource is selected.  
b.  For SL transmissions triggered by data that does not require HARQ feedback, any resource pool can be selected.

Proposal 2	RAN2 discusses the UE behavior in case of not enough resources for the required number of HARQ retransmissions considering the following alternatives:

a.	Do not select resources for any HARQ retransmission

b.	Select resources for as many HARQ retransmissions as possible

Proposal 3	The selected resource reservation interval is larger than the selection window duration, i.e., T2-T1.

Proposal 4	Resource reservation should be disabled for the pools where sensing is not used, i.e. exceptional pool.

Proposal 5	Resource reservation of initial transmission could be disabled for services requiring low latency.

# References

TS 36.331, V15.5.1, “Radio Resource Control (RRC); Protocol specification (Release 15)”

R1-1908913, Resource allocation for Mode-2 transmissions, Ericsson, RAN1#98, Prague, October 2019.

R2-1906495, Summary of 105bis#31 NR V2X Resource pool configuration and selection (ZTE)

R2-1915272, Support on HARQ procedure over sidelink, 3GPP TSG RAN2 Meeting #108, November 2019