3GPP TSG-RAN WG2 Meeting #109e	R2-2001147

# Online, 24 February – 6 March 2020

Agenda item:	6.12.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Per DRB Measurements in 38.314

WID/SID:	NR_SON_MDT-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In this document, we present our point of view on per DRB measurement as currently written in the draft 38.314.

# 2	Discussion

In the current 38.314 draft contribution, following the email discussion, some measurements are defined “Per DRB”:

##### 4.1.1.2.1 Average over-the-air interface packet delay in the UL per DRB per UE ##### 4.1.1.2.2 Average RLC packet delay in the UL per DRB per UE ##### 4.1.1.2.3 Average PDCP re-ordering delay in the UL per DRB per UE #### 4.2.1.1 UL PDCP Packet Excess Delay per DRB #### 4.2.1.2 UL PDCP Packet Average Delay per DRB ##### 4.1.1.3.1 Mean number of Active UEs in the DL per DRB per cell ##### 4.1.1.3.2 Max number of Active UEs in the DL per DRB per cell ##### 4.1.1.3.3 Mean number of Active UEs in the UL per DRB per cell ##### 4.1.1.3.4 Max number of Active UEs in the UL per DRB per cell ##### 4.1.1.5.1 Packet Loss Rate in the DL per DRB per UE

## 2.1 Per DRB per Cell

There is a NOTE in 4.1.1.2, saying that: “Per DRB refers to per mapped 5QI for NR SA or per QCI for EN-DC”

This note applies to the measurement “4.1.1.2.2” Average RLC packet delay in the UL per QoS level per UE

Indeed, the DRB used in the definition itself is open for interpretation, if there is no explicit UE context associated. For example in a Cell, some UE could have DRB1 configured for Voice and some other UE DRB1 configures for Non- GBR Data. In this case, it makes no sense to mix the results for the same DRB ID. But we agree that the measurement should be done per DRB, and then aggregated together for the similar DRB in the cell

Proposal 1: RAN2 clarifies in 38.314 that per DRB Per Cell means that the measurement can be aggregated per QoS level in the cell.

## 2.2 Per DRB per UE

When a measurement is defined per DRB per UE, this mean that it applies to a specific UE, in this case, UE context may be known and it makes sense in this case to collect measurement for each DRB of the UE, individually

## 2.3 Per DRB

It is not written for measurements 4.2.1.1	and 4.2.1.2, if the measurement are per UE, but as they are performed by a UE, it makes sense to clarify that they are performed and signaled per DRB per UE.

Proposal 2: RAN2 clarifies in 38.314 that measurements 4.2.1.1	and 4.2.1.2, are performed and signaled per DRB per UE.

# 3	Conclusion

In this contribution, we made the following proposals:

Proposal 1 : RAN2 clarifies in 38.314 that per DRB Per Cell means that the measurement can be aggregated per QoS level in the cell.

Proposal 2: RAN2 clarifies in 38.314 that measurements 4.2.1.1	and 4.2.1.2, are performed and signaled per DRB per UE.