3GPP TSG-RAN WG2 Meeting #109-e	R2-2001608

# Electronic meeting, 24th February - 6th March, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0202 | rev |  | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unsecured UE capability handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-2-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For unsecured UE capability, SA3 agreed not to either store them locally for later use or send them to other network entities except for unauthenticated emergency calls (S3-194488). This CR reflects the agreements and to add a requirement for gNB in clause 14 UE capabilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 14 UE capabilities, add a requirement that with the exception of unauthenticated emergency calls, if the gNB had acquired UE capabilities using RRC UE capability transfer procedure before AS security activation, then the gNB shall not store them locally for later use and shall not send them to other nodes. Impact analysis Impacted functionality: UE capability. Inter-operability: none |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | With the exception of unauthenticated emergency calls, gNB may store the UE capabilities aquired before AS security activation or send them to other nodes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

14	UE Capabilities

The UE capabilities in NR do not rely on UE categories: UE categories associated to fixed peak data rates are only defined for marketing purposes and not signalled to the network. Instead, the network determines the UL and DL data rate supported by a UE from the supported band combinations and from the baseband capabilities (modulation scheme, MIMO layers, …).

To limit signalling overhead, the gNB can request the UE to provide NR capabilities for a restricted set of bands. When responding, the UE can skip a subset of the requested band combinations when the corresponding UE capabilities are the same.

With the exception of unauthenticated emergency calls, if the gNB had acquired UE capabilities using RRC UE capability transfer procedure before AS security activation, the gNB shall

-	release them when UE transits from RRC_CONNECTED to either RRC_IDLE or RRC_INACTIVE.

-	not send them to other RAN nodes or MME on handover or retrieve UE context.

-	not initiate UE CAPABILITY INFO INDICATION procedure

End of change