3GPP TSG-RAN WG2 Meeting#109e									 		R2-2001864

e-meeting, 24th February - 6th March, 2020

Source: 			Intel Corporation

Title: 	Summary of contributions on Idle mode mobility

Agenda item:		7.1.8.1

Document for: 		Discussion and Decision

# Introduction

This document summarizes the proposals of the contributions [1]-[4] under this agenda item. For simplicity, the contribution numbers [1]-[4] are referred by their last 4 digits between parenthesis e.g. (xxxx).

# Operation of Rel-16 non-BL UE camping on enhanced coverage mode in a cell (non-standalone case) when S criteria for normal coverage is fulfilled

## Is a Rel-16 non-BL UE allowed to camp in a cell (non-standalone case) on enhanced coverage mode when S criteria for normal coverage is fulfilled?

| Company | Views |
| --- | --- |
| Thales (0251) | Proposal1: Non-BL UEs with S criterion in normal coverage fulfilled are allowed to camp in coverage enhancement mode, when indicated by the network to be allowed. Amongst other, see benefits of smaller bandwidth for WUS and paging |
| Ericsson (0979) | Proposal1: Except for standalone LTE-M cell, non-BL UE shall camp in enhanced coverage only if S-criterion for normal coverage is not fulfilled and S-criterion for enhanced coverage is fulfilled. Proposal2: In case Proposal 1 is not agreed, a non-BL UE is allowed to camp in enhanced coverage when the S-criterion for normal coverage is fulfilled only if such behavior is enabled in SI. |
| Huawei (1067) | Proposal1: Non-BL UE is allowed to camp on enhanced coverage mode when S criteria for normal coverage is fulfilled. See no impact to UE and network in supporting this. |
| Intel (1098) | As in legacy, a Rel-16 non-BL UE can camp in a cell in normal coverage either in BR mode (if SIB1-BR is used) or in WB, i.e. non-BR mode (if SIB1 is used). |

Observation 1: Majority of companies (3 out of 4) seem to support allowing non-BL UE to camp on enhanced coverage mode when S criteria for normal coverage is fulfilled. 1 company is willing to compromise if the behavior can be controlled/enabled by the network (which is a topic discussed in next section 2.1).

## Is network control needed to allow non-BL UE to camp on a cell in normal coverage in BR mode?

| Company | Views |
| --- | --- |
| Thales (0251) | Proposal 2: The network may indicate whether non-BL UEs are allowed to camp in CE mode even when S criterion in normal coverage is fulfilled. Proposal 3: The network may indicate whether the fulfilment of the S-criterion for CE mode is required for all non-BL devices or whether it only applies for the non-BL UEs trying to newly camp (select or re-select) to said cell. |
| Ericsson (0979) | Proposal 2: In case Proposal 1 is not agreed, a non-BL UE is allowed to camp in enhanced coverage when the S-criterion for normal coverage is fulfilled only if such behavior is enabled in SI. |
| Huawei (1067) | Proposal2: NW control can be discussed further. |
| Intel (1098) | Proposal 2: RAN2 to discuss if a new mechanism is needed to control non-BL UEs about camping in normal LTE cells in WB mode (using normal SIB1) when these UEs can satisfy the S criteria for normal coverage in both BR mode or WB mode |

Observation 2:  All companies seems to be ok or are ok to have or discuss about the network control on whether non-BL UE is allowed or not to camp on cell in CE mode when S-criteria in normal coverage is fulfilled.

## Summary on the operation of Rel-16 non-BL UE camping on enhanced coverage mode in a cell (non-standalone case) when S criteria for normal coverage is fulfilled

Taken into consideration the details discussed in previous sections 2.1 and 2.2, a 1st proposal is suggested for agreement, and a 2nd one for discussion:

Proposal 1: Rel-16 non-BL UE is allowed to camp in a cell (non-standalone case) in enhanced coverage when S-criterion for normal coverage is fulfilled only if indicated by the network via broadcast signalling (FFS on whether the presence or absence of the broadcast indication enables the feature).

Proposal 2: If proposal 1 is agreed, FFS on whether the new broadcast indication described in proposal 1 is (a) applicable only for non-BL UE entering (i.e. (re)selecting) the cell or (b) all non-BL UEs in the cell.

# Miscellaneous proposals

| Company | Views |
| --- | --- |
| Huawei (1067) | Proposal3: Absolute priorities rather than ranking applies for non-BL UEs using enhanced coverage when S criteria for normal coverage is fulfilled. |

There are no sufficient contributions to make a decision on the proposals therefore we suggest a proposal for discussion:

Proposal 3: If proposal 1 is agreed, FFS on whether priority based cell reselection rather than ranking should be applied for non-BL UEs using enhanced coverage when S criteria for normal coverage is fulfilled.

# Conclusions and Potential proposals

Observation 1: Majority of companies (3 out of 4) seem to support allowing non-BL UE to camp on enhanced coverage mode when S criteria for normal coverage is fulfilled. 1 company is willing to compromise if the behavior can be controlled/enabled by the network (which is a topic discussed in next section 2.1).

Observation 2:  All companies seems to be ok or are ok to have or discuss about the network control on whether non-BL UE is allowed or not to camp on cell in CE mode when S-criteria in normal coverage is fulfilled.

Proposal 1: Rel-16 non-BL UE is allowed to camp in a cell (non-standalone case) in enhanced coverage when S-criterion for normal coverage is fulfilled only if indicated by the network via broadcast signaling (FFS on whether the presence or absence of the broadcast indication enables the feature).

Proposal 2: If proposal 1 is agreed, FFS on whether the new broadcast indication described in proposal 1 is (a) applicable only for non-BL UE entering (i.e. (re)selecting) the cell or (b) all non-BL UEs in the cell.

Proposal 3: If proposal 1 is agreed, FFS on whether priority based cell reselection rather than ranking should be applied for non-BL UEs using enhanced coverage when S criteria for normal coverage is fulfilled.

# References

- R2-2000979   S-Criterion interpretation for non-BL UEs   Ericsson   discussion         LTE_eMTC5-Core

- R2-2001067   Enhancements to idle mode mobility for non-BL UEs   Huawei, HiSilicon           discussion         Rel-16   LTE_eMTC5-Core

- R2-2000251   Clarification to idle mode mobility for non-BL UEs   THALES   discussion

- R2-2001098   Non-BL UE in normal and enhanced coverage       Intel Corporation          discussion         Rel-16  LTE_eMTC5-Core