3GPP TSG-RAN WG2 Meeting #109bis-e		R2-2002575

Electronic, April 20 – 30, 2020

Source: 			Qualcomm Incorporated

Title: 				ULSUP applicability to FDD bands

Document for: 	Decision

Agenda Item: 	6.19

# Introduction

RAN2 received RAN4 LS on FDD band capability signalling for uplink sharing, in which RAN4 stated as follows [1].

- RAN4#93 has agreed to use the UE capability signalling ‘ul-SharingEUTRA-NR’ for the NR FDD bands in addition to the NR SUL […]

In this document we discuss about the forward compatibility issues of the proposed approach and propose an alternative solution for UE capability signalling.

# Discussion

Today ULSUP is supported only for SUL for which RAN4 defines specific frequency bands. The existing UE capability ‘ul-SharingEUTRA-NR’ is signaled per band combination. It is our understanding that the underlying assumption here is that there is going to be a single SUL configured within a band combination.

Now RAN4 discussed the applicability of FDD bands in the context of their WI on “Introduction of NR FDD bands with variable duplex and corresponding framework”. In addition, RAN4 agreed on the following as RAN4’s common understanding.

- “It is RAN4 understanding that RAN4 specification does not include any band combination with FDD bands with fixed duplex for ULSUP requirements”

We should consider the following possibilities in the future in developing the UE capability signaling.

- RAN4 defines a band combination where ULSUP is supported for a SUL and FDD band(s) within the band combination.

- RAN4 defines ULSUP with fixed duplex FDD bands.

The UE capability signaling should take into account the fact that the UE requirement for supporting ULSUP for different band types above can be different. We therefore propose the following generic approach for the UE capability signaling, with the intention to provide forward compatibility to the future possibilities discussed above.

Proposal 1:	To keep the existing capability parameter ‘ul-SharingEUTRA-NR’ only to represent the UE capability for ULSUP with a SUL band.

Proposal 2:	To introduce a new UE capability parameter with per band granularity to represent the UE capability for ULSUP with an FDD band.

# Conclusion

In this document we discussed about the forward compatibility issues of the proposed approach and proposed an alternative solution for UE capability signalling.

Proposal 1:	To keep the existing capability ‘ul-SharingEUTRA-NR’ only to represent the UE capability for ULSUP with a SUL band.

Proposal 2:	To introduce a new UE capability with per band granularity to represent the UE capability for ULSUP with an FDD band.

RAN2 is kindly requested to discuss the corresponding CRs provided in [2] and [3].

# Reference

[1]		R2-2002124/R4-1916180	LS on FDD band capability signalling for uplink sharing		RAN4 (To: RAN2)

[2]		CR to 38.331 on Introduction of UE capability for ULSUP with FDD band

[3]		CR to 38.306 on Introduction of UE capability for ULSUP with FDD band