3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2002604

Electronic, 20 April – 30 April 2020

Agenda Item:	6.4.2.2

Source:	Intel Corporation

Title:	Open aspects on mode 2 operation

Document for:	Discussion, Decision

Introduction

During the last meeting, there was not much discussion on mode 2 operation. While there was an email discussion on several mode 1 aspects, there are still some unresolved issues that we think require further discussion. In this contribution, we analyze some of these aspects and present our view.

Discussion

  - Cast dependent exceptional pools

One of the outstanding aspects for SL resource pool design is whether or not cast-depending exceptional pools should be considered. In previous discussions, this aspect has been left pending on RAN1 discussions. In this regard, RAN1 made the following agreements in RAN1#97 [2]:

Agreements: - In physical layer perspective, a (pre-)configured resource pool can be used for all of unicast, groupcast, and broadcast for a given UE. - There is no (pre-)configuration to inform which cast types are used for the resource pool.

In short, since cast-dependent common resource pools are not supported according to RAN1 agreement, it makes little sense to support cast-dependent exceptional pools. Exceptional resource pools are only expected to be used in specific scenarios (as the name implies) and so there is no real need to define them in a cast specific fashion. In addition, doing so would also lead to segmentation of resources and decreased resource efficiency. Finally, given the lack of time for this WI, it seems prudent to not open up this issue for discussion anyway. Therefore, we propose to not have cast dependent exceptional resource pools for NR V2X for this release.

Proposal 2:	No exceptional pools are separately allocated for unicast and non-unicast traffic for Rel-16 (same as common resource pools).

  - On Resource (Re-)Selection

On the other hand, one could argue that in some sense, there is still some resource pool usage differentiation based on cast type due to HARQ feedback and provision of PSFCH resources. Specifically, it was agreed previously in RAN1 that HARQ feedback shall only be sent on PSFCH in the same resource pool as the original transmission.

Agreements: - It is supported, in a resource pool, that within the slots associated with the resource pool, PSFCH resources can be (pre)configured periodically with a period of N slot(s) - N is configurable, with the following values - 1 - At least one more value >1 - FFS details - The configuration should also include the possibility of no resource for PSFCH. In this case, HARQ feedback for all transmissions in the resource pool is disabled - HARQ feedback for transmissions in a resource pool can only be sent on PSFCH in the same resource pool

This means that individual resource pools must be configured which have associated PSFCH resources for HARQ feedback transmissions, such that only UEs involved in unicast and groupcast operation with HARQ feedback enabled can utilize those pools. Then, for UEs performing SL communication without the need for HARQ feedback, the question arises as to whether they shall be allowed to use resource pools with dedicated PSFCH resources in addition to pools with no such resources.

In our view, the decision on which resource pool shall be used by the UE in mode 2 depends on the type of V2X transmission the UE performs. Based on (pre-)configuration of SLRB HARQ feedback enabled/disabled, the UE can select the applicable pool (i.e. with or without PSFCH resources). Specifically, for the case when the transmission corresponds to no HARQ feedback, it should be left to UE implementation on which particular resource pool to select. We think that this would avoid the problem of resource fragmentation and have less specification impact compared to defining some scheme or configured criteria for resource pool selection depending on HARQ feedback enabled/disabled. It is also in line with earlier discussions in RAN1 and RAN2 on not dividing resource pools into being cast type specific since many companies were of the view that it would lead to inefficient resource utilization. In our view, the same principle applies here as well, i.e. the number of UEs requiring HARQ feedback resources is expected to be quite varied and unpredictable to warrant the need for defining such restriction.

Proposal 3:	For broadcast or groupcast sidelink transmission without the need of HARQ feedback, resource pool selection for (initial) transmission is left to UE implementation.

Moreover, in case of new incoming data which requires HARQ feedback, a resource pool reselection might be required, i.e. selecting a new resource pool with PSFCH resources. In this case, we also need to consider if the UE behavior needs to be specified. Using similar reasoning as above and considering the limited time left for discussion in this WI, we think that we can also leave this resource pool re-selection case to UE implementation. Note that the specification already currently has a clause for the case when the UE cannot meet the latency requirement for the V2X transmission using existing resources. We can simply add the reliability requirement therein and the UE can trigger resource reselection if it cannot meet the requirements based on its current set of resources. The corresponding change is included in the text proposal in section 5 below

- Proposal 4:	For mode 2 operation, in case the outgoing traffic whose reliability requirement cannot be met with the currently selected set of resources, the UE can trigger resource reselection (as per the TP in section 5).

Conclusion

This contribution discusses outstanding issues on mode 2 operation and makes the following proposals:

Proposal 1:	RAN2 is suggested to consider reusing the geographical zone-based resource pool configuration for NR V2X sidelink operation, similar to LTE V2X.

Proposal 2:	No exceptional pools are separately allocated for unicast and non-unicast traffic for Rel-16 (same as common resource pools).

Proposal 3:	For broadcast or groupcast sidelink transmission without the need of HARQ feedback, resource pool selection for (initial) transmission is left to UE implementation.

- Proposal 4:	For mode 2 operation, in case the outgoing traffic whose reliability requirement cannot be met with the currently selected set of resources, the UE can trigger resource reselection (as per the TP in section 5).

References

| [1] | "RAN2#108, Chairman Notes". |
| --- | --- |
| [2] | "RAN1#97, Chairman Notes". |

Text Proposal

START OF THE CHANGE

#### 5.22.1.2	TX resource (re-)selection check

If the TX resource (re-)selection check procedure is triggered for a Sidelink process according to clause 5.22.1.1, the MAC entity shall for the Sidelink process:

1>	if SL_RESOURCE_RESELECTION_COUNTER = 0 and when SL_RESOURCE_RESELECTION_COUNTER was equal to 1 the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is above the probability configured by upper layers in sl-ProbResourceKeep; or

1>	if a pool of resources is configured or reconfigured by upper layers; or

1>	if there is no configured sidelink grant; or

1>	if neither transmission nor retransmission has been performed by the MAC entity on any resource indicated in the configured sidelink grant during the last [second]; or

1>	if sl-ReselectAfter is configured and the number of consecutive unused transmission opportunities on resources indicated in the configured sidelink grant is equal to sl-ReselectAfter; or

1>	if the configured sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by upper layers in sl-MaxMCS-PSSCH and the MAC entity selects not to segment the RLC SDU; or

NOTE 1:	If the configured sidelink grant cannot accommodate the RLC SDU, it is left for UE implementation whether to perform segmentation or sidelink resource reselection.

1>	if transmission(s) with the configured sidelink grant cannot fulfil the latency or reliability requirement of the data in a logical channel according to the associated priority, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU; or

NOTE 2:	If the latency or reliability requirement is not met, it is left for UE implementation whether to perform transmission(s) corresponding to single MAC PDU or sidelink resource reselection.

1>	if a sidelink transmission is scheduled by any received SCI indicating a higher priority than the prority of the logical channel and expected to overlap with a resource of the configured sidelink grant, and a measured result on SL-RSRP associated with the sidelink transmission is higher than [threshold]:

2>	clear the configured sidelink grant associated to the Sidelink process, if available;

2>	trigger the TX resource (re-)selection.

START OF THE CHANGE