3GPP TSG-RAN WG2 #109bis-e                                                                                 R2-2002769

eMeeting, 20th – 30th, April 2020

Agenda Item:	    6.10.2

Source: 	    MediaTek Inc.

Title:		    Remaining issue on DCCA capability

Document for:	    Discussion and decision

# 1 Introduction

In RAN2#109e, the DCCA related capabilities are discussed and result in endorsed CRs as following.

Feature List and UE capabilities

R2-2001189	Summary of [108#48][DCCA] DCCA R2 feature list	Huawei	discussion	Rel-16	LTE_NR_DC_CA_enh-Core	Late

- Noted

R2-2001190	TP for 38.331 on introducing UE capability for eDDCA	Huawei	draftCR	Rel-16	38.331	15.8.0	LTE_NR_DC_CA_enh-Core	Late

- [AT109e][039][DCCA] Endorsed

R2-2001191	TP for 36.331 on introducing UE capability for eDCCA	Huawei	draftCR	Rel-16	36.331	15.8.0	LTE_NR_DC_CA_enh-Core	Late

- [AT109e][039][DCCA] Endorsed

R2-2001192	Running CR for 38.306 on introducing UE capability for eDCCA	Huawei	CR	Rel-16	38.306	15.8.0	0247	-	B	LTE_NR_DC_CA_enh-Core	Late

- [AT109e][039][DCCA] Endorsed (not for approval RP)

R2-2002335	Running CR for 36.306 on introducing UE capability for eDCCA	Huawei	CR	Rel-16	36.306	15.7.0	1744	-	B	LTE_NR_DC_CA_enh-Core

- [AT109e][039][DCCA] Endorsed (not for approval RP)

However, there are still some FFS issues left in the endorsed CR. The paper address the open issues and proposed our suggestions to resolve them.

# 2 Discussion

## 2.1 Early measurement

In 36.306 endorsed CR [1], the following FFS issues related to “early measurement” are identified.

#### 4.3.6.31	ca-IdleModeMeasurements-r15

This field defines whether the UE supports performing eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE mode, including reporting them when requested by eNB while in RRC_CONNECTED, as specified in TS 36.331 [5].

#### 4.3.6.32	ca-IdleModeValidityArea-r15

This field defines whether the UE supports configuration of validityArea for performing eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE mode, as specified in TS 36.331 [5]. A UE that supports this feature shall also support ca-IdleModeMeasurements-r15.

#### 4.3.6.x1          ca-IdleInactiveMeasurements-r16

This field defines whether the UE supports:

-	(if the UE supports inactiveState-r15), performing eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_INACTIVE, including reporting them when requested by the eNB while resuming from RRC_INACTIVE or in RRC_CONNECTED, as specified in TS 36.331 [5];

-	(if the UE supports RRC connection suspension), reporting eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE while resuming the RRC connection from RRC_IDLE, as specified in TS 36.331 [5];

A UE that supports this feature shall also support ca-IdleInactiveMeasurements-r15.

#### 4.3.6.x2	endc-IdleInactiveMeasurements-r16

This field defines whether the UE supports performing eNB-configured NR SSB-based RRM measurements for configured carrier(s) in RRC_IDLE and in RRC_INACTIVE (if the UE supports inactiveState-r15), including reporting them when requested by the eNB while resuming from RRC_IDLE/RRC_INACTIVE or in RRC_CONNECTED, as specified in TS 36.331 [5].

FFS1: Is the UE that supports this feature required to also support ca-IdleInactiveMeasurements-r16?

FFS2: Separate capability for RRC_IDLE and RRC_INACTIVE.

#### 4.3.6.x3	idleInactiveValidityAreaList-r16

This field defines whether the UE supports configuration of validityAreaList-r16 for performing eNB-configured measurements for configured carrier(s) in RRC_IDLE and in RRC_INACTIVE, as specified in TS 36.331 [5].

A UE that supports this feature shall also support ca-IdleInactiveMeasurements-r16 and/or endc-IdleInactiveMeasurements-r16.

FFS: A UE that supports this feature shall also support ca-IdleModeValidityArea-r15.

First, for a UE supporting the NR early measurement (endc-IdleInactiveMeasurements-r16), we do not see the necessary to mandate UE to support LTE early measurement at the same time. The measurement applies to different targets and it is used for different architecture (LTE CA and EN-DC). We understand that the two feature could work independently, there is no reason to combine them. Thus we propose that no dependence on the two sub features.

Proposal 1: In LTE, a UE that supports NR early measurement (endc-IdleInactiveMeasurements-r16) is not required to support LTE early measurement (ca-IdleInactiveMeasurements-r16).

Regarding to whether to separate the NR early measurement (endc-IdleInactiveMeasurements-r16) according to LTE IDLE and INACTIVE mode (it should also include RRC connection suspension mode), we do not have strong view on this. For simplicity, we propose the use one capability for now but open for discussion on this.

Proposal 2: In LTE, the NR early measurement (endc-IdleInactiveMeasurements-r16) capability applies to LTE IDLE, INACTIVE, and RRC connection suspension states.

For validity area, our understanding is that the LTE R15 validity area is somehow broken. Based on the LTE ASN.1 definition, the LTE validity area is configured within the target LTE early measurement frequencies while the validity area should be configure based on serving cell frequencies. It is not suitable to reuse the same R15 configuration also for NR early measurement. That’s why we have new R16 validity area design. Therefore, we should not force a UE supports R16 validity area to support R15 validity area.

Proposal 3: In LTE, a UE that supports R16 validity area (idleInactiveValidityAreaList-r16) is not required to support R15 validity area (ca-IdleModeValidityArea-r15).

In 38.306 endorsed CR [1], the following FFS issues related to “early measurement” are identified.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| idle-inactive-MeasReport-r16 Indicates whether the UE supports configuration of NR SSB measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9]. FFS: Is separate capability needed for E-UTRA measurements or does the UE which supports idle-inactive-MeasReport and NE-DC also support E-UTRA measurements? | UE | No | No | FFS |
| idle-inactive-ValidityArea-r16 Indicates whether the UE supports configuration of a validity area for NR measurements in RRC_IDLE/RRC_INACTIVE as specified in TS 38.331 [9]. | UE | No | No | No |

During the discussion, some companies argue that in NR, we should use single capability bit for both LTE early measurement and NR early measurement. We don’t think this design is flexible enough. For a UE supports NR CA and NR early measurement, it should not be forced to support LTE early measurement if it design to support NE-DC first. We think that combining of these two feature is somehow unreasonable and suggest to separate it as two capabilities (e.g. nedc-idle-inactive-MeasReport-r16 and ca-idle-inactive-MeasReport-r16).

Proposal 4: In NR, separate the early measurement capability (idle-inactive-MeasReport-r16) into two different sub-features according to measurement targets.

- One for LTE CRS measurement (for NE-DC configuration, e.g. nedc-idle-inactive-MeasReport-r16);

- other for NR SSB based measurement (for NR CA configuration, e.g. ca-idle-inactive-MeasReport-r16).

And on whether to have FR1/FR2 difference for NR early measurement, we prefer to have separate capability to include implementation flexibility. The measurement on NR FR2 is a changeling design and has higher power consumption. So, it may make sense that to separate FR1/FR2 on NR measurement. Note that we should have the same design principle for both NR early measurement in LTE or NR.

Proposal 5: The NR early measurement in LTE (endc-IdleInactiveMeasurements-r16) and NR (ca-idle-inactive-MeasReport-r16) should be defined as FR1/FR2 separation capability.

## 2.2 RRC Resume with DC/CA

In 36.306 endorsed CR [1], the following FFS issues related to “RRC Resume with DC/CA” are identified.

#### 4.3.15.x1	resumeWithSCells -r16

This parameter defines whether the UE supports not deleting the stored MCG SCell configuration when initiating the resume procedure and (re-)configuration of MCG SCells in the RRCConnectionResume message as specified in TS 36.331 [5].

FFS 1: Split in "not deleting the stored SCell configuration" and "(re-)configuration of an SCG during the resume procedure".

FFS 2: Split in "resume from RRC_IDLE" and "resume from RRC_INACTIVE".

#### 4.3.15.x2	resumeWithSCG -r16

This parameter defines whether the UE supports not deleting the stored SCG configuration when initiating the resume procedure and (re-)configuration of SCG in the RRCConnectionResume message as specified in TS 36.331 [5].

FFS 1: Split in "not deleting the stored SCG configuration" and "configuration of an SCG during the resume procedure" (it is assumed that if both are supported, the network can reconfigure the stored SCells).

FFS 2: Split "resume from RRC_IDLE" and "resume from RRC_INACTIVE".

In 38.306 endorsed CR [2], the following FFS issues related to “RRC Resume with DC/CA” are identified.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| resumeWithStoredSCells-r16 Indicates whether the UE supports not deleting the stored MCG SCell configuration when initiating the resume procedure. | UE | No | No | No |
| resumeWithSCG-r16 Indicates whether the UE supports not deleting the stored SCG configuration when initiating resume and (re-)configuration of an SCG during the resume procedure. FFS: Split in two separate capabilities i.e. "not deleting the stored SCG configuration" and "(re-)configuration of an SCG during the resume procedure". | UE | No | No | No |

The first FFS applies both to LTE and NR. It is about whether to combine the “not deleting the stored SCG configuration” and “(re-)configuration of an SCG during the resume procedure” into single capability. In our understanding, the latter part is blind configuration on SCG upon resume, it can work without the former one. The “not deleting” part is used to support delta configuration for further signalling optimization. Based on this, we think that UE should not be forced to support the delta configuration (not deleting part) in order to support the blind reconfiguration. We thus suggest to have separate capability for that.

Proposal 6: In both LTE and NR, define two separate capabilities for "not deleting the stored SCG configuration" and "(re-)configuration of an SCG during the resume procedure".

For SCell configuration, following the same logic as in SCG configuration, we should also have separate capability on “not deleting the stored SCell configuration” and “(re-)configuration of SCell(s) during the resume procedure”. Note that in NR, blind SCell configuration in Resume is already supported in R15. So, “not deleting SCell configuration” is already a separate capability.

Proposal 7: In LTE, define two separate capabilities for "not deleting the stored SCell configuration" and "(re-)configuration of SCell(s) during the resume procedure".

On the other hand, we agree that the supporting of delta configuration (not deleting part) depends on the blind configuration. Thus we should specify some dependence on the two capabilities.

Proposal 8: In both LTE and NR, a UE supports “not deleting the stored SCG configuration” should also support “re-configuration of SCG during the resume procedure”.

Proposal 9: In LTE, a UE supports “not deleting the stored SCell(s) configuration” should also support “re-configuration of SCell(s) during the resume procedure”.

For the LTE parts, on whether to define different capability based on different LTE state (i.e. LTE INACTIVE and RRC Connection with suspension), we do not have strong view. For simplicity, we assume that single capability is fine for now.

Proposal 10: In LTE, the capability on “not deleting the stored SCell/SCG” and "(re-)configuration of an SCG/SCell(s) during resume" apply to LTE INACTIVE and RRC connection suspension states.

## 2.3 Direct SCell activation

In 38.306 endorsed CR [2], the following FFS issues related to “direct SCell activation” are identified.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| directSCellActivation-r16 Indicates whether the UE supports direct MCG/SCG SCell activation upon SCell addition, upon reconfigurationWithSync and upon resume. FFS: Separate capabilities for resume? | UE | No | No | FFS |

For direct SCell activation in NR, we think that it would be fine to separate the capability for RRC Resume and RRC Reconfiguration, as anyway it would be different IE in different message. We do not have strong preference to have FR1/FR2 differentiation on this capability but thinking that it may not be necessary.

Proposal 11: In NR, define separate capabilities of direct SCell activation for RRC Resume and RRC reconfiguration. The capabilities are not FR1 and FR2 differentiation.

## 2.4 Fast Recovery

In 36.306 endorsed CR [1], the following TBC issue for “fast recovery” is identified.

#### 4.3.15.x3	mcgRLF-RecoveryViaSCG-r16

This parameter defines whether the UE supports recovery from MCG RLF via split SRB1 (if supported) and via SRB3 (if supported) as specified in TS 36.331 [5].

TBC: No need to separate split SRB1 from SRB3, or NR-DC and NE-DC.

In 38.306 endorsed CR [2], the following TBC issue for “fast recovery” is identified.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| mcgRLF-RecoveryViaSCG-r16 Indicates whether the UE supports recovery from MCG RLF via split SRB1 (if supported) and via SRB3 (if supported) as specified in TS 38.331[9]. TBC: No need to separate split SRB1 from SRB3, or NR-DC and NE-DC. | UE | No | No | No |

For fast recovery, basically same FFS issues on both LTE and NR. In our view, it seems not necessary to separate the capability for split SRB1 and SRB3. Also separation on NR-DC and NE-DC is not critical. So, we would propose to confirm the TBC item.

Proposal 12: In both LTE and NR, RAN2 confirms that it is not necessary to separate the fast recovery capability by “split SRB1 and SRB3” or by “NR-DC and NE-DC”.

# 3 Conclusions

Base on the discussion in section 2, we propose the following:

For early measurement:

Proposal 1: In LTE, a UE that supports NR early measurement (endc-IdleInactiveMeasurements-r16) is not required to support LTE early measurement (ca-IdleInactiveMeasurements-r16).

Proposal 2: In LTE, the NR early measurement (endc-IdleInactiveMeasurements-r16) capability applies to LTE IDLE, INACTIVE, and RRC connection suspension states.

Proposal 3: In LTE, a UE that supports R16 validity area (idleInactiveValidityAreaList-r16) is not required to support R15 validity area (ca-IdleModeValidityArea-r15).

Proposal 4: In NR, separate the early measurement capability (idle-inactive-MeasReport-r16) into two different sub-features according to measurement targets.

- One for LTE CRS measurement (for NE-DC configuration, e.g. nedc-idle-inactive-MeasReport-r16);

- other for NR SSB based measurement (for NR CA configuration, e.g. ca-idle-inactive-MeasReport-r16).

Proposal 5: The NR early measurement in LTE (endc-IdleInactiveMeasurements-r16) and NR (ca-idle-inactive-MeasReport-r16) should be defined as FR1/FR2 separation capability.

For RRC Resume with DC/CA:

Proposal 6: In both LTE and NR, define two separate capabilities for "not deleting the stored SCG configuration" and "(re-)configuration of an SCG during the resume procedure".

Proposal 7: In LTE, define two separate capabilities for "not deleting the stored SCell configuration" and "(re-)configuration of SCell(s) during the resume procedure".

Proposal 8: In both LTE and NR, a UE supports “not deleting the stored SCG configuration” should also support “re-configuration of SCG during the resume procedure”.

Proposal 9: In LTE, a UE supports “not deleting the stored SCell(s) configuration” should also support “re-configuration of SCell(s) during the resume procedure”.

Proposal 10: In LTE, the capability on “not deleting the stored SCell/SCG” and "(re-)configuration of an SCG/SCell(s) during resume" apply to LTE INACTIVE and RRC connection suspension states.

For Direct SCell activation:

Proposal 11: In NR, define separate capabilities of direct SCell activation for RRC Resume and RRC reconfiguration. The capabilities are not FR1 and FR2 differentiation.

Fast recovery:

Proposal 12: In both LTE and NR, RAN2 confirms that it is not necessary to separate the fast recovery capability by “split SRB1 and SRB3” or by “NR-DC and NE-DC”.

# 4 References

[1] R2-2002335, “Running CR for 36.306 on introducing UE capability for eDCCA”, Huawei

[2] R2-2001192, “Running CR for 38.306 on introducing UE capability for eDCCA”, Huawei

# Annex: RAN2 feature table for DCCA

The following is the DCCA feature tables based on current endorsed CR and our proposals. Note that we also include R15 euCA capabilities for comparison.

| Sub-feature | ASN.1 | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- | --- |
| Early measurement in LTE |  |  |  |  |  |
| R15 Validity Area | ca-IdleModeValidityArea-r15 | UE | No | No | No |
| LTE CRS based measurement in LTE IDLE | ca-IdleModeMeasurements-r15 | UE | No | No | No |
| LTE CRS based measurement in LTE IDLE, INACTIVE, and RRC connection suspension | ca-IdleModeMeasurements-r16 | UE | No | No | No |
| NR SSB based measurement in LTE IDLE, INACTIVE, and RRC connection suspension | endc-IdleInactiveMeasurements-r16 | UE | No | No | Yes |
| R16 Validity Area | idleInactiveValidityAreaList-r16 | UE | No | No | No |
| Early measurement in NR |  |  |  |  |  |
| NR SSB based measurement in NR IDLE/INACTIVE | ca-idle-inactive-MeasReport-r16 | UE | No | No | Yes |
| LTE CRS based measurement in NR IDLE/INACTIVE | nedc-idle-inactive-MeasReport-r16 | UE | No | No | No |
| R16 Validity Area | idle-inactive-ValidityArea-r16 | UE | No | No | No |
| Resume with DC/CA in LTE |  |  |  |  |  |
| MCG SCell configuration in Connection Resume | resumeWithSCells-r16 | UE | No | No | No |
| SCG configuration in Connection Resume | resumeWithSCG-r16 | UE | No | No | No |
| Maintain MCG SCell configuration in LTE INACTIVE | resumeWithStoredSCells-r16 | UE | No | No | No |
| Maintain SCG configuration in LTE INACTIVE | resumeWithStoredSCG-r16 | UE | No | No | No |
| Resume with DC/CA in NR |  |  |  |  |  |
| SCG configuration in Resume | resumeWithSCG-r16 | UE | No | No | No |
| Maintain MCG SCell configuration in NR INACTIVE | resumeWithStoredSCells-r16 | UE | No | No | No |
| Maintain SCG configuration in NR INACTIVE | resumeWithStoredSCG-r16 | UE | No | No | No |
| Direct SCell activation in LTE |  |  |  |  |  |
| Direct SCell activation in RRC Connection Reconfiguration | directSCellActivation-r15 | UE | No | No | No |
| Direct SCell activation in RRC Connection Resume | directSCellActivationResume-r16 | UE | No | No | No |
| Direct SCell activation in NR |  |  |  |  |  |
| Direct SCell activation in RRC Reconfiguration | directSCellActivation-r16 | UE | No | No | No |
| Direct SCell activation in RRC Resume | directSCellActivationResume-r16 | UE | No | No | No |
| Fast recovery in LTE |  |  |  |  |  |
| Fast recovery from MCG RLF via split SRB1 or via SRB3 in (NG)EN-DC | mcgRLF-RecoveryViaSCG-r16 | UE | No | No | No |
| Fast recovery in NR |  |  |  |  |  |
| Fast recovery from MCG RLF via split SRB1 or via SRB3 in NR-DC and NE-DC | mcgRLF-RecoveryViaSCG-r16 | UE | No | No | No |