3GPP TSG RAN WG2 Meeting #109b-e	R2-2002771

Electronic Meeting, 20th April – 30th April, 2020

Agenda Item:	  6.4.2.2 Others

Source: 	ITRI

Title: 	Remaining Issues on NR V2X Resource Allocation for Zone Configuration

Document for:	Discussion and Decision

Introduction

In RAN2# 108, zone ID based on the formulae specified in 3GPP TS 36.331 [1] is agreed, the related agreements are shown as follows [2].

Agreements on MAC:

1: 	For all cast-types, Layer-1 Destination ID corresponds to the 16 bit LSB of the Destination Layer-2 ID, and the Layer-1 Source ID corresponds to the 8 bit LSB of the Source Layer-2 ID.

2:	The DST field includes 8 bit MSB of the Destination Layer-2 ID and the SRC field includes 16 bit MSB of the Source Layer-2 ID for the SL-SCH subheader of a MAC PDU to be transmitted to the peer UE.

3a:	Zone ID determination is based on the formulae specified in 36.331 as UE’s location information. And we will ask to RAN1 if any problem with this way.

3b:	Formulae is specified in 38.331 and 38.321 specification refers 38.331, and MAC will indicate zone id to L1.

4a:	For V2X, the number of LCH priorities is 3bits.

4b:	The priority indicated in a SCI is derived from the value of the highest priority of logical channels served by a MAC PDU to be transmitted on PSSCH scheduled by the SCI.

5:	If there is no unoccupied Sidelink process in the Sidelink HARQ entity, when a new TB arrives, how to handle RX buffer management is up to UE implementation.

6:	For each new transmission, UE selects a Destination having the logical channel with the highest priority, among the logical channels having data available for transmission and having no mapping restrictions to a sidelink grant. Then, UE allocate the sidelink grant to the logical channels from the logical channel with the highest priority.

7:	The logical channel with disabling the HARQ feedback cannot be multiplexed with a logical channel which enabling the HARQ feedback.

8:	Up to 8 configured grants (including both configured grant type1 and type2) in mode1 can be configured and active for NR sidelink communication (working assumption). Running 38.321 will specify something like “[8]”

9:	Configured grant Type 1 and Type 2 can be simultanesouly configured for a UE in NR sidelink communication.

10:	The Sidelink Configured Grant Confirmation MAC CE is identified by a MAC subheader with a new LCID value.

Zone ID can be used to calculate the Tx Rx distance between the transmitter UE and recevier UE. And, the HARQ feedback is provided when the calculated Tx Rx distance is less than a pre-configured threshold. Otherwise, HARQ feedback is not provided. However, zone ID based on the formulae specified in 3GPP TS 36.331 is peridically reused in the geographical area for frquency resue, and hence an ambiguity issue for the Tx Rx distance calculation bsed on zone ID will be introduced. Besides, it is still unclear about the intended usage of zones. In this contribution, we discuss the potential usage of zones in NR V2X, including the Tx Rx distance calculation based on zone ID and NR V2X resource allocation for zone configuration, and present our views.

The Tx Rx Distance Calculation based on Zone ID

### The formulation of Zone ID specified in 3GPP TS 36.331 is shown as follows [1],

The UE shall determine an identity of the zone (i.e. Zone_id) in which it is located using the following formulae, if zoneConfig is included in SystemInformationBlockType21 or SystemInformationBlockType26 or in SL-V2X-Preconfiguration:

x1= Floor (x / L) Mod Nx;

y1= Floor (y / W) Mod Ny;

Zone_id = y1 * Nx + x1.

The parameters in the formulae are defined as follows:

L is the value of zoneLength included in zoneConfig in SystemInformationBlockType21 or SystemInformationBlockType26 or in SL-V2X-Preconfiguration;

W is the value of zoneWidth included in zoneConfig in SystemInformationBlockType21 or SystemInformationBlockType26 or in SL-V2X-Preconfiguration;

Nx is the value of zoneIdLongiMod included in zoneConfig in SystemInformationBlockType21 or SystemInformationBlockType26 or in SL-V2X-Preconfiguration;

Ny is the value of zoneIdLatiMod included in zoneConfig in SystemInformationBlockType21 or SystemInformationBlockType26 or in SL-V2X-Preconfiguration;

x is the geodesic distance in longitude between UE's current location and geographical coordinates (0, 0) according to WGS84 model [80] and it is expressed in meters;

y is the geodesic distance in latitude between UE's current location and geographical coordinates (0, 0) according to WGS84 model [80] and it is expressed in meters.

### From above description, zone ID specified in 3GPP TS 36.331 is peridically reused in the geographical area, as shown in Figure 1. In this figure, the zone ID is from zero to Nx*Ny-1, and there will be Nx*Ny zones in a cluster. Besides, the zone ID will be reused in another cluster, e.g., zone_id=0 is reused in different clusters.

Figure 1: An illustration of Zone ID based on the formulae specified in 36.331

### The Tx Rx distance is calculated as the distance between the geometric centers of the Tx UE’s Zone and RX UE’s Zone. For example, the Tx Rx distance between NR V2X Tx UE1 and NR V2X Rx UE2 is 1 zone unit, as shown in Figure 2. In the figure, the zone_id information (x1, y1) for NR V2X Tx UE 1 and NR V2X Rx UE 2 are (1, Ny-1) and (1, Ny-2), respectively, and the Tx Rx distance is the Euclidean distance between the zone_id information (1, Ny-1) and (1, Ny-2).

### However, when the NR V2X Tx UE and NR V2X Rx UE are located in different clusters, an ambiguity issue will occur for the Tx Rx distance calculation. For example, NR V2X Tx UE3 and NR V2X Rx UE4 are located in different clusters, as shown in Figure 2. The zone_id information (x1, y1) for NR V2X Tx UE 3 and NR V2X Rx UE 4 are (Nx-1, 1) and (0, 1), respectively, and then the Euclidean distance between the zone_id information (Nx-1, 1) and (0, 1) is Nx-1 zone unit. In fact, the Tx Rx distance is 1 zone unit between NR V2X Tx UE3 and NR V2X Rx UE4. Therefore, the ambiguity issue occurs. The ambiguity issue is resulted from zone ID reuse in different clusters. Hence, some modification mechanisms are needed for the ambiguity issue.

Figure 2: Ambiguity issue for the Tx Rx distance calculation based on Zone ID specified in 36.331

Proposal 1: The Tx Rx distance can be calculated based on the zone ID specified in 36.331. However, the ambiguity issue due to zone ID reuse in different clusters should be considered.

Zone-based Resource Allocation for NR V2X

### The zone-based resource allocation is similar to the radio resource sharing and reusing, i.e., reuse of radio resources at different geographical locations. For the NR V2X use cases, the zone-based resource allocation may improve packet reception performance by reducing the impact of in-band emission and co-channel interference under the latency and reliability requirement. The major idea of zone-based resource allocation is that managing target vehicles to different geo-zones. Each geo-zone uses difference subset resource to reduce the interference impact between vehicles of different geo-zone. As a result, the interference of geo-zones apart from the others can be low. And it makes the benefit from the resource-reuse scheme possible. The major impacts of zone-based resource allocation can be discussed in two parts, Intra-geo-zone interference and Inter-geo-zone interference The intra-geo-zone interference means that vehicles in the same geo-zone which use the same resource to transmit. The inter-geo-zone interference is that vehicles in different zone impact each other.

Observation 1: For the NR V2X use cases, the zone-based resource allocation may improve packet reception performance by reducing the impact of in-band emission and co-channel interference.

Proposal 2: The major idea of zone-based resource allocation is that managing target vehicles to different geo-zones. Each geo-zone uses difference subset resource to reduce the interference impact between vehicles of different geo-zone.

Conclusion

Based on the discussion in section 2 & 3, we have following observation and proposals:

Observation 1: For the NR V2X use cases, the zone-based resource allocation may improve packet reception performance by reducing the impact of in-band emission and co-channel interference.

Proposal 1: The Tx Rx distance can be calculated based on the zone ID specified in 36.331. However, the ambiguity issue due to zone ID reuse in different clusters should be considered.

Proposal 2: The major idea of zone-based resource allocation is that managing target vehicles to different geo-zones. Each geo-zone uses difference subset resource to reduce the interference impact between vehicles of different geo-zone.

References

- 3GPP TS 36.331 V15.9.0 (2020-03), “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 15)”

- R2-2000009, “Report of 3GPP TSG RAN WG2 meeting #108”, Reno, USA, 18 - 22 November, 2019