3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2002843

20-30 April 2020

Agenda item:	6.2.3

Source: 	Qualcomm Incorporated

Title:  	Report of [Post109e#38][NR-U] RRC open issues (Qualcomm)

Document for:	Discussion and decision

# Introduction

The following email discussion was agreed in RAN2#109e to discuss the current and possibly new open issues in RRC:

[Post109e#38][NR-U] RRC open issues (Qualcomm)

Address known stage-3 remaining open issues from 109e for 38.331 and update E-UTRAN (36.331) with relevant agreements and any open issues that need to be discussed

Capture identified NEW, if any, stage-3 corrections/issues.  Issues that have already been discussed and not pursued should not be brought up again.

Intended outcome: Agreable proposals.  CR for 38.331 addressing open issues and CR for 36.331 (including editorials received offline) (Deadline above)

Intended outcome 2: Open Issues list with RRC impact (April 1)

This report captures the outcome of the discussion on the above issues with proposals for resolution. In addition, based on the endorsed ASN.1 review plan in R2-2001709, for each issue, the following will be captured:

WI Rapporteurs produces a list of known open issues that has RRC impact, by email on the R2 reflector after R2 109e. For each issue, expected RRC impact shall be listed if possible. For each known open issue, WI rapporteur should suggest if to treat WI open issue in the scope of the ASN.1 review or not. Some guidance:

    - Issues isolated to procedure text or ASN.1 specific to the WI could be handled in WI-specific CR

    - Issues with impact on other WIs and/or legacy (Rel-15) procedure text or ASN.1 should be handled in ASN.1 review.

# Changes needed due to new RAN1 agreements

RAN1#100e has made several agreements which impact RRC. These are discussed in this section.

Note that there are still RAN1 parameters with FFS values which will need further decision by RAN1. These are captured in Annex 3 as a reference.

Also, RAN2 has changed the names of several RAN1 parameteres to comply with ASN.1 guidelines. These are captured in Annex 2 as a reference.

All the issues in this section will be addressed in the WI specific CR but not in the ASN.1 review.

### Issue A1: Clarification of SS/PBCH

RAN1 has agreed on further clarifications relaed to the SS/PBCH block index and sent an LS to RAN2 (and RAN4) in R1-2001357. The LS text is included in Annex 1 for reference. In this LS, RAN1 suggested several changes to the field descriptions of ssb-PositionsInBurst and mediumBitmap as follows:

ServingCellConfigCommon field descriptions

ssb-PositionsInBurst For operation without shared spectrum channel access, iIndicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB. For operation with shared spectrum channel access, only mediumBitmap is used, and as described in the TS 38.213 [13], clause 4.1, UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the discovery burst transmission window and have candidate SS/PBCH blocks indexes corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst. If MSB k, k≥1, of ssb-PositionsInBurst is set to 1, the UE assumes that one or more SS/PBCH blocks within the discovery burst transmission window with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k-1 may be transmitted; if MSB k is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. The UE expects that a bit at position k > ssb-PositionQCL-Relationship-16 is 0, and the number of actually transmitted SS/PBCH blocks equal to the number of 1’s in the bitmap. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB.

ServingCellConfigCommonSIB field descriptions

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, only mediumBitmap is used, and UE interpret this field the same as in the corresponding field in ServingCellConfigCommon.

SSB-ToMeasure field descriptions

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, a bit set to 1 at position k (indexing starts at 1) in the bitmap indicates SS/PBCH block index k-1. A UE can derive the time domain positions of the candidate SS/PBCH blocks within the SMTC measurement duration based on this bitmap. From a value 0 at position k in the bitmap, the UE can derive the candidate SS/PBCH block(s) with index corresponding to the SS/PBCH block index k-1 are not to be measured, while from a value 1 at position k in the bitmap, the UE can derive the candidate SS/PBCH block(s) with index corresponding to the SS/PBCH block index k-1 are to be measured.

Note that these parts of the suggested changes in ssb-PositionsInBurst were already in the RRC CR:

For operation with shared spectrum channel access, only mediumBitmap is used, and as described in the TS 38.213 [13], clause 4.1.

The UE expects that a bit at position k > ssb-PositionQCL-Relationship-16 is 0,

Question A1a: Do you agree to the suggested changes by RAN1 in the LS R1-2001357? If not, please provide justification and alternative text if applicable.

| Company | Response | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE | Yes,but | For ServingCellConfigCommonSIB field description, inOneGroup should be used instead of mediumBitmap. |
| Samsung | Yes | Agree with ZTE |
| MediaTek | Yes |  |
| CMCC | Yes |  |
| OPPO | Yes | Agree with ZTE |
| Apple | Yes, In addition | Suggest to also capture the description “The UE expects that a bit at position k > ssb-PositionQCL-Relationship-16 is 0, and the number of actually transmitted SS/PBCH blocks equal to the number of 1’s in the bitmap.” into SSB-ToMeasure. |
| Google | Yes | 1. Agree with ZTE 2. Typo and naming: the “ssb-PositionQCL-Relationship-16” in ServingCellConfigCommon should be “SSB-PositionQCL-Relationship-r16” |
| Nokia | Yes |  |
| Ericsson |  | In other parts of RRC, we used ”For operation in licensed spectrum” instead of “without shared spectrum channel access”. Prefer to keep it consistent in RRC. “MSB k”: it is not quite clear in RRC how this relates exactly to the position in the bitmap. The existing language that is used is provided below, and we strongly recommend to be consistent to avoid that bits could be interpreted differently. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1. Propose not to refer to specific fields defining “SSB-QCL-Relationship-r16” because we have different parameters in SIB2, SIB3, SIB4 and for the measurement report (“ssb-Position-QCL-Common” and “ssb-Position-QCL”). And in MIB there is even no explicit parameter. Therefore, it does not seem appropriate to indicate all the fields and locations.We propose this to be rewritten, see proposal below. ServingCellConfigCommon ssb-PositionsInBurst For operation with shared spectrum channel access, only mediumBitmap is used, and as described in the TS 38.213 [13], clause 4.1, the UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the discovery burst transmission window and have candidate SS/PBCH blocks indexes corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst, where the number of actually transmitted SS/PBCH blocks is equal to the number of 1’s in the bitmap. kk≥1k-1kThe first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on, and the k-th bit corresponds to SS/PBCH block index k-1. For value 1 for the k-th bit, the UE assumes that one or more SS/PBCH blocks within the discovery burst transmission window with candidate SS/PBCH block indexes corresponding to SS/PBCH block index k-1 may be transmitted; for value 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. For k > Q, the UE expects that the bit for the k-th bit is set to value 0, where Q is the value set for SSB-Position-QCL-Relationship-r16. . The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB. ServingCellConfigCommonSIB ssb-PositionsInBurst For operation with shared spectrum channel access, only inOneGroup is used, and the UE interprets this field the same way as the corresponding field in ServingCellConfigCommon. SSB-ToMeasure -> mediumBitmap For operation with shared spectrum channel access, the leftmost/first bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on, and the k-th bit corresponds to SS/PBCH block index k-1. A UE can derive the time domain positions of the candidate SS/PBCH blocks within the SMTC measurement duration based on the positions within this bitmap. Value 0 for the k-th bit indicates that the candidate SS/PBCH block(s) with index corresponding to the SS/PBCH block index k-1 are not to be measured. Value 1 at the k-th position in the bitmapindicates that the candidate SS/PBCH block(s) with index corresponding to the SS/PBCH block index k-1 are to be measured. |
| Huawei | Yes |  |
| vivo | Yes |  |
| Qualcomm | Yes |  |

Summary: Out of 13 companies, only 1 does not agree with the description sent by RAN1.

ZTE pointed out that inOneGroup should be used instead of mediumBitmap in ServingCellConfigCommonSIB field description. 3 companies supported this. Since this is an obvious error so should be corrected.

Apple suggested to repeat a statement in “The UE expects that a bit at position k > ssb-PositionQCL-Relationship-16 is 0, and the number of actually transmitted SS/PBCH blocks equal to the number of 1’s in the bitmap.” in SSB-ToMeasure. Since SSB-ToMeasure is used in measurement object as well, this seems a useful clarification.

Google suggested to use the IE name instead of the field name. However, the convention in RRC is to use the field name.

Ericsson proposed a major rewrite of the RAN1 text. RAN2 should not discuss technical aspects of these IEs and any such request should be brought up in RAN1. However, the following editorial suggestions are acceptable as they are based on the RRC convention:

- Use “for operation in licensed spectrum” instead of “without shared spectrum channel access”

- Use “leftmost” instead of “MSB”

Proposal 1: Introduce the field descriptions communicated by RAN1 with the following changes:

- Use inOneGroup instead of mediumBitmap in ServingCellConfigCommonSIB

- Include the statement in “The UE expects that a bit at position k > ssb-PositionQCL-Relationship-16 is 0, and the number of actually transmitted SS/PBCH blocks is not larger the number of 1’s in the bitmap.” in SSB-ToMeasure

- Use “For operation in licensed spectrum” instead of “without shared spectrum channel access”

- Use “leftmost” instead of “MSB”

Question A1b: Do you think any additional changes to RRC are needed in regards to the RAN1 agreements on this?

| Company | Response | Comments |
| --- | --- | --- |
| Intel | No |  |
| ZTE | No |  |
| Samsung | No |  |
| MediaTek | No |  |
| CMCC | No |  |
| OPPO | Maybe | According to the agreements in the coming LS, specifically: “Agreement: RAN1 recommends the following terminology to be used consistently across RAN1 (and RAN2/RAN4) specifications for operation with shared spectrum channel access: - “SS/PBCH block index” can be expressed as modulo(PBCH DMRS sequence index, Q) or modulo(Candidate SS/PBCH block index, Q) - Note: These two modulo operations yield the same result and which index should be applied depends on the specific scenario - Note: This is applicable for cases in the specification where “SS/PBCH block” instead of “SS/PBCH block index” - “Candidate SS/PBCH block index” within a DRS transmission window was previously agreed in RAN1 - Include this agreement in an LS to RAN2/RAN4 ” RAN1 has the following recommendation on the above agreement: RAN WG1 kindly asks RAN WG2 and WG4 to take the above agreements on SS/PBCH block indexing into account for developing their specifications for operation with shared spectrum channel access. It’s understood that the SSB index in NR-U should be the index for several candidate position considering the introduction of Q, we are wondering whether RAN2 should capture something in 331, e.g.,: the following yellow-higlighted part in 331 field description related to SS block index. For operating with shared spectrum channel access, the IE SSB-Index identifies the SS/PBCH block index, which corresponds to one or more candidate SS/PBCH block indexes as defined in TS 38.213 [13], clause 4.1. |
| Apple | Yes | We would like to suggest that in NR-U, the SSB-ToMeasure is always configured together with ssb-PositionQCL-Common-r16, which can help UE to avoid decoding the PBCH from the neighbor cells. |
| Google | No |  |
| Nokia | No |  |
| Ericsson | No |  |
| Huawei | No |  |
| vivo | No |  |
| Qualcomm | No |  |

Summary: Out of 13 companies, 11 do not see any additional changes to the RRC. Vivo suggests that a general statement on SSB index corresponding to several candidate locations (due to Q) may be beneficial. Apple proposed that SSB-ToMeasure should always be configured together with ssb-PositionQCL-Common-r16 to help the UE measurements. None of these proposals were supported by any other company.

Proposal 2: No other changes are introduced to RRC to address the recommendations and agreements in RAN1 LS (R2-2001357).

### Issue A2: COT duration parameters

RAN1#100e had the following agreements on COT duration:

Agreement: The length of each COT duration indicator is not configured explicitly by RRC, but is derived from the number X of values configured in the co-DurationList-r16, e.g. length=ceil[log2(X)]. Agreement: The size of the list of configured channel occupancy durations for a serving cell (in the field co-DurationList-r16) can range from 1 to 64 channel occupancy duration values (co-Duration-r16). Agreement: The value for a single channel occupancy duration value (co-Duration-r16) can range from 0 to 20 ms with a granularity of one symbol.

Based on these agreements, the following changes are needed in the RRC:

- Add field description for co-DurationList-r16 which states that “The UE determines the length of the COT duration indicator based on the number of entries in the co-DurationList-r16”.

- Replace ffsValue with 64 in:

co-DurationList-r16       SEQUENCE (SIZE(1..ffsValue)) OF CO-Duration-r16 -- FFS size upper limit 64

- Replace ffsValue below with 1120 to support 20ms duration (the new upper limit is changed from 560 to 1120 as it is needed for SCS 60Khz):

CO-Duration-r16 ::=    INTEGER (0..ffsValue) -- FFS upper limit 560

Question A2: Do you agree with the above changes to implement the RAN1 agreements on COT duration? If not, please provide justification and suggestions.

| Company | Response | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE | Yes |  |
| Samsung | Yes |  |
| MediaTek | Yes |  |
| CMCC | Yes |  |
| OPPO |  | It’s ok to replace ffsValue with 1120, it seems there is no field description for CO-Duration-r16 in the current NR-U 331 CR. We may need to capture something in the field description that 1120 is for the case when SCS is 60KHz similar as the following for searchSpaceSwitchingTimer “For 15 KHz SCS, {1..20} are valid. For 30KHz SCS, {1..40} are valid. For 60Khz SCS, {1..80} are valid” |
| Apple | Yes |  |
| Google | Yes |  |
| Nokia | Yes |  |
| Ericsson | 1. No 2. Yes 3. Yes | General comment/question: as indicated in the title, this is about the duration of COT. COT is associated with the successful channel access of a node, while CO in RAN2 has more often been associated with RSSI/CO measurements. However, this parameter uses CO-Duration instead of COT-Duration. Is there a difference between CO duration and channel occupancy time? Maybe something to clarify with RAN1? 1. No This is an agreement related to the number of COT duration indicators described in TS 38.212 and should be fixed there, see TDRA example in DCI 0_1: The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationList… Instead, the following is proposed for the field description of CO-DurationList: “List of channel occupancy [time] durations that can be indicated in DCI format 2_0, see TS 38.213, clause 11.1.1.” 2. Yes 3. The restrictions can be consistently structured as in Issue A6 for configured grant. Note that in other places, we have introduced a CHOICE structure, e.g. for interlace. This could potentially be done consistenty and needs some further thinking: CO-Duration-r16 ::= CHOICE { scs15 INTEGER (0.. 280), scs30 INTEGER (0.. 560), scs30 INTEGER (0.. 1120) } Similar structure as agreed for interlace 0: interlace0 CHOICE { scs15 INTEGER (0..9), scs30 INTEGER (0..4) } |
| Huawei | Yes for 2,3 | For 1, there is no need to caputre since anyway this will be captured in the DCI description in 38.212/38.213 |
| vivo | Yes for 2,3 | Agree with Huawei. |
| Qualcomm | Yes |  |

Summary: Out of 13 companies, 10 support all the suggestions. 3 companies think that the first change is already includes in RAN1 specifications. Ericsson suggests that “COT” instead of “CO” should be used and a CHOICE structure is preferable. It is not clear to the rapporteur why RAN1 chose CO-duration in this instance; one plausible speculation is that T already stands for time and time-duration might have seemed as repetition. The second Ericsson suggestion on using CHOICE is pure ASN.1 and can be moved to ASN.1 discussion. At this point, we can make the changes agreed by the majority. Oppo suggested to have a field description for CO-Duration which is reasonable.

Proposal 3. Introduce the following changes:

- Replace ffsValue with 64 in:

co-DurationList-r16       SEQUENCE (SIZE(1..ffsValue)) OF CO-Duration-r16 -- FFS size upper limit 64

- Replace ffsValue below with 1120 to support 20ms duration (the new upper limit is changed from 560 to 1120 as it is needed for SCS 60Khz):

CO-Duration-r16 ::=    INTEGER (0..ffsValue) -- FFS upper limit 560

- Add field description for CO-Duration

### Issue A3: Search space switching

RAN1#100e had the following agreements on search space switching:

Agreement: Higher layer parameter searchSpaceSwitchingTimer can take values in the range of 1 to 20 ms. Agreement: For processing times for search space switching, P1 and P2 have the same value.

Based on these agreements, the following changes are suggested in the RRC:

- Replace ffsValue below with 80 (maximum needed for 20ms with SCS of 60khz):

searchSpaceSwitchingTimer-r16       INTEGER (1..ffsValue)

- Put in the field description of searchSpaceSwitchingTimer that “For 15 KHz SCS, {1..20} are valid. For 30KHz SCS, {1..40} are valid. For 60Khz SCS, {1..80} are valid. Note that this is in slots as used in 38.213.

Question A3: Do you agree with the above changes to implement the RAN1 agreements on search space switching? If not, please provide justification and suggestions.

| Company | Response | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE | Yes |  |
| Samsung | Yes |  |
| MediaTek | Yes |  |
| CMCC | Yes |  |
| OPPO | Yes |  |
| Apple | Yes |  |
| Google | Yes |  |
| Nokia | Yes and No | It might be better to code this with choice structure fro different SCS instead of limiting values for SCS in the field description? searchSpaceSwitchingTimer-r16 CHOICE { scs15 INTEGER(1..20), scs30 INTEGER(1..40)), scs60 INTEGER(1..80)) } We don’t have strong view on this but it might be worthwhile to consider this kind of coding. |
| Ericsson | In principle yes | OK, but could consider whether a CHOICE structure is better, see comment for A2. Maybe to be discussed as part of the ASN.1 review. Editorial: should be “kHz” |
| Huawei | Yes | What has been mentioned by Nokia is a general issue and can be handled in the more general setting, e.g., Class2 ASN.1 discussion |
| vivo | Yes |  |
| Qualcomm | Yes |  |

Summary: Out of 13 companies, 11 support the suggested changes. 2 companies are fine in principle but prefer a CHOICE structure. As commented by Huawei, that is an ASN.1 only issue and can be discussed separately.

Proposal 4. Introduce the following changes:

- Replace ffsValue below with 80 (maximum needed for 20ms with SCS of 60khz):

searchSpaceSwitchingTimer-r16       INTEGER (1..ffsValue)

- Put in the field description of searchSpaceSwitchingTimer that “For 15 kHz SCS, {1..20} are valid. For 3kKHz SCS, {1..40} are valid. For 60kHz SCS, {1..80} are valid. Note that this is in slots as used in 38.213.

### Issue A4: CP extension

RAN1#100e had the following agreements on CP extension:

Agreement: For UL transmissions prior to dedicated RRC configuration or in CBRA, C2 and C3 before RRC configuration are set to the maximum integers which satisfy following: - C2*symbol length – 16 us – TA < symbol length - C3*symbol length – 25 us – TA < symbol length Agreement: The value ranges for C2 and C3 are updated as follows: - C2 values - 1, 2, 3, .. , 28 for 15/30 kHz SCS - 2, 3, .., 28 for 60 kHz SCS - C3 values - 1, 2, 3, .. , 28 for 15 kHz SCS - 2, 3, .. , 28 for 30 kHz SCS - 3, 4, .. , 28 for 60 kHz SCS

The agreement which impacts RRC are the new range of values for C2 and C3. The existing field description in the NR-U CR is as follows:

cp-ExtensionC2, cp-ExtensionC3

Configures the cyclic prefix (CP) extension (see TS 38.211 [16], clause 5.3.1). For 15 and 30KHz SCS, {1..28} are valid. For 60KHz SCS, {2..28} are valid.

Based on these agreements, the following changes are needed in the field description of these IEs:

Configures the cyclic prefix (CP) extension (see TS 38.211 [16], clause 5.3.1). For 15 KHz SCS, {1..28} are valid. For 30Khz SCS, {1..28} are valid for cp-ExtensionC2 and {2..28} are valid for cp-ExtensionC3. For 60KHz SCS, {2..28} are valid for cp-ExtensionC2 and {3..28} are valid for cp-ExtensionC3.

Question A4: Do you agree with the above changes to implement the RAN1 agreements on new value ranges for cp-ExtensionC2 and cp-ExtensionC3? If not, please provide justification and suggestions.

| Company | Response | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE | Yes |  |
| Samsung | Yes |  |
| MediaTek | Yes |  |
| CMCC | Yes |  |
| OPPO | Yes |  |
| Apple | Yes |  |
| Google | Yes |  |
| Nokia | Yes |  |
| Ericsson | Yes | Would be nice to use same list structure as in issue 6. - Editorial: add the following for completeness: - For 15 kHz SCS, {1..28} are valid for both cp-ExtensionC2 and cp-ExtensionC3. Typo: should be “kHz” |
| Huawei | Yes |  |
| vivo | Yes |  |
| Qualcomm | Yes |  |

Summary: All the companies are fine with the changes. Ericsson pointed out an editorial change which is acceptable and a typo.

Proposal 5: Introduce the following in the field description of cp-ExtensionC2, cp-ExtensionC3:

Configures the cyclic prefix (CP) extension (see TS 38.211 [16], clause 5.3.1). For 15 kHz SCS, {1..28} are valid for both cp-ExtensionC2 and cp-ExtensionC3. For 30 kHz SCS, {1..28} are valid for cp-ExtensionC2 and {2..28} are valid for cp-ExtensionC3. For 60 kHz SCS, {2..28} are valid for cp-ExtensionC2 and {3..28} are valid for cp-ExtensionC3.

### Issue A5: RMTC configuration

RAN1#100e had the following agreements on RMTC configuration:

Agreement: The RMTC-Config can indicate a reference SCS and CP as one of {15 kHz, 30 kHz, 60 kHz-NCP, 60 kHz-ECP} Agreement: The L1 averaging duration of RSSI measurements (within a configured measurement duration) is limited to 1 OFDM symbol of a configured reference subcarrier spacing. Agreement: Keep value range for rmtc-Period-r16 and rmtc-SubframeOffset-r16 the same as for LTE-LAA. Agreement: Keep value range for rssi-Result-r16 and channelOccupancyThreshold-r16 the same as for LTE-LAA and inform RAN2 of this decision. Agreement: For RSSI (and channel occupancy measurements) layer 3 filtering is not applied.

For rssi-Result-r16 and channelOccupancyThreshold-r16, the RAN1 recommendation is to keep the same values as LTE LAA. However, in LTE LAA, these are integer values which are determined by the table in RAN4 LTE specs as follows.

-- ASN1START

RSSI-Range-r13 ::=						INTEGER(0..76)

-- ASN1STOP

MeasRSSI-ReportConfig-r13 ::=	SEQUENCE {

channelOccupancyThreshold-r13			RSSI-Range-r13				OPTIONAL	-- Need OR

}

MeasResultForRSSI-r13 ::=			SEQUENCE {

rssi-Result-r13							RSSI-Range-r13,

channelOccupancy-r13					INTEGER (0..100),

...

}

The RAN1 agreement above was based on the contribution R1-2000827 and it seems that RAN1 was assuming RAN4 would agree to the same table as follows:

Another item that is FSS in the running CR for 38.331 and where RAN2 needs RAN1 input is the value range for rssi-Result-r16 and channelOccupancyThreshold-r16. Both of these fields are inherited from LTE-LAA and have the value range INTEGER(0..76). The value is then mapped to dBm based on Table 9.1.18.5.1-1 in 36.133. Given that the RSSI measurement is defined in the same way for NR as for LTE-LAA, we see no reason to change the value range.

Such a table is not available for NR-U yet. Since RAN4 is still discussing this, it is prudent to wait for their conclusion.

Based on the aboave agreements, the following changes to the RRC are suggested:

- Introduce a new IE in RMTC-Config called ref-SCS-CP with the values of {15 kHz, 30 kHz, 60 kHz-NCP, 60 kHz-ECP}.

- Remove the Editor’s Note on L3 filtering for RSSI

- Wait for RAN4 conclusion on actual values for rssi-Result-r16 and channelOccupancyThreshold-r16 before introducing the indices corresponding to RAN4 table

Question A5: Do you agree with the above changes to implement the RAN1 agreements on RMTC configuration? If not, please provide justification and suggestions.

| Company | Response | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE | Yes |  |
| Samsung | Yes |  |
| MediaTek | Yes |  |
| CMCC | Yes |  |
| OPPO | Yes |  |
| Apple | Yes |  |
| Google | Yes |  |
| Nokia | Yes |  |
| Ericsson | Yes, but | Propose to use more self-descriptive name for channelOccupancyThreshold (rssi-ThresholdForCO), see comments for Section 4. |
| Huawei | Yes |  |
| vivo | Yes |  |
| Qualcomm | Yes |  |

Summary: All companies agree with the suggested changes. Ericsson proposes to use a more descriptive name for these IEs. That can be discussed as part of ASN.1.

Proposal 6: Introduce the following changes to RRC:

- Introduce a new IE in RMTC-Config called ref-SCS-CP with the values of {15 kHz, 30 kHz, 60 kHz-NCP, 60 kHz-ECP}

- Remove the Editor’s Note on L3 filtering for RSSI

- Wait for RAN4 conclusion on actual values for rssi-Result-r16 and channelOccupancyThreshold-r16 before introducing the indices corresponding to RAN4 table

### Issue A6: Configured Grant

RAN1#100e had the following agreements on Configured Grant parameters:

Agreement: The value range for cg-nrofSlots-r16 is {1, 2, …, 40} slots Agreement: Value ranges for cg-minDFIDelay-r16 in symbols with the step size of [14] symbols except for the first two values for different SCS are {[7], [14], [28], …, M} where M is as follows: - 1 *4 * 14 = 56 (symbols)} for 15 kHz SCS - 2 * 4 * 14 = 112 (symbols)} for 30 kHz SCS - 4 * 4 * 14 = 224 (symbols) for 60 kHz SCS Agreement: For values of CP extension 7 possible starting positions are introduced - The indices to 7 possible starting positions will be specified - A UE is configured with indices to values from the indices to the 7 values. Agreement: The set of Beta-offset values used for CG-UCI is the same as the set of Beta-offset values used for HARQ-ACK - When CG-UCI is jointly encoded with HARQ, use the Beta-offset values configured for HARQ-ACK

These agreements impact the following IEs:

cg-minDFI-Delay-r16                    INTEGER (1..ffsValue)   OPTIONAL,   -- Need R Upper limit 7 FFS

cg-nrofSlots-r16                       INTEGER (1..ffsValue)   OPTIONAL,   -- Need R

cg-StartingFullBW-InsideCOT-r16        ENUMERATED (ffs)        OPTIONAL,   -- Need R

cg-StartingFullBW-OutsideCOT-r16       ENUMERATED (ffs)        OPTIONAL,   -- Need R

cg-StartingPartialBW-InsideCOT-r16     ENUMERATED (ffs)        OPTIONAL,   -- Need R

cg-StartingPartialBW-OutsideCOT-r16    ENUMERATED (ffs)        OPTIONAL,   -- Need R

betaOffsetCG-UCI-r16                   INTEGER (1..ffsValue)   OPTIONAL,   -- Need R

For the cg-startingXX IEs which point to CP extension, the agreement is to have an index to 7 possible values. Therefore INTEGER (0..6) is reasonable. However, 38.214 Section 6.1.2.3 refers to a “set of values” for FullBW case where the UE picks one randomly within the set and a single value for “PartialBW” case. Therefore, a sequence of FFS size for FullBW can be introduced at this point.

For betaOffsetCG-UCI, the RAN1 agreement is to use the same values as HARQ-ACK. The BetaOffsets IEs in RRC are all of INTEGER (0..31) and map to the Table 9.3-1 in 38.213. Therefore, the same range should be used for betaOffsetCG-UCI.

Based on these agreements, the following changes are suggested:

- Change the value range for cg-nrofSlots-r16 to {1,2, ..., 40}

- Change the value range for cg-minDFIDelay-r16 to ENUMERATED {sym7, sym1x14, sym2x14, sym3x14, sym4x14, sym5x14, sym6x14, sym7x14, sym8x14, sym9x14, sym10x14, sym11x14, sym12x14, sym13x14, sym14x14} and introduce additional text in the field description as:

cg-minDFIDelay

Indicates the minimum duration (in unit of symbols) from the ending symbol of the CG-PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH. UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n - cg-minDFIDelay-r16, where n is the time corresponding to the beginning of the start symbol of the DFI (see TS 38.213 [13], clause 10.3).

The following minimum delay values are supported depending on the configured subcarrier spacing [symbols]:

15 kHz:	7, n*14, where n={1, 2, 3, 4}

30 kHz:	7, n*14, where n={1, 2, 3, 4, 5, 6, 7, 8}

60 kHz:	7, n*14, where n={1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14}

- Change the value ranges as follows:

cg-StartingPartialBW-InsideCOT-r16 and cg-StartingPartialBW-OutsideCOT-r16 to to INTEGER (0..6)

cg-StartingFullBW-InsideCOT-r16 and cg-StartingFullBW-OutsideCOT-r16r to SEQUENCE (SIZE (1..ffsValue)) OF INTEGER (0..6)

- Change the ffsValue in betaOffsetCG-UCI-r16 value range to 31

Question A6: Do you agree with the above changes to implement the RAN1 agreements on configured grant parameters? If not, please provide justification and suggestions.

| Company | Response | Comments |
| --- | --- | --- |
| Intel | Yes | Some editorials: 1. There are 2 ‘n’ s in the field description. Perhaps we could change one to ‘m’ … 2. 7,n*14 is a range, so, it should look something like below: 15 kHz: {7,.., n*14}, where n={1, 2, 3, 4} 30 kHz: {7, …, n*14}, where n={1, 2, 3, 4, 5, 6, 7, 8} 60 kHz: {7, …, n*14}, where n={1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14} Alternatively we could just provide the range without using n (which would also solve the issue with 1. above). 3. 60 kHz: {7, …, n*14}, where n={1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14} is missing 2 values 15 and 16 to go to M=224 |
| ZTE | Yes, but | For cg-minDFIDelay, two values are missing. Firstly, the value range of cg-minDFIDelay-r16 should be ENUMERATED {sym7, sym1x14, sym2x14, sym3x14, sym4x14, sym5x14, sym6x14, sym7x14, sym8x14, sym9x14, sym10x14, sym11x14, sym12x14, sym13x14, sym14x14, sym15x14, sym16x14 }. Secondly, for the field description, when 60kHz is used, the value range of n should be {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16}. |
| Samsung | Yes | Agree with Intel and ZTE. |
| MediaTek | Yes | We agree with ZTE that two values “sym15x14”, “sym16x14” are missing in the value range of cg-minDFIDelay-r16 and field description should include “15” and “16”. |
| CMCC | Yes | Agree with Intel and ZTE on the editorials and missing values. |
| LG |  | One editorial comment for value range of cg-nrofSlots-r16 is to allow up to 16 slots (as per RAN1 agreements), i.e., 60 kHz: 7, n*14, where n={1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16} |
| OPPO | Yes | Agree with ZTE |
| Apple | Yes | Agree with Intel and ZTE. |
| Google | Yes | Agree with Intel and ZTE |
| Nokia | Yes for 1,3,4 No for 2 | 60khz is missing values – it is not going all the way to 224 but stops at 196 => add sym14x15 and sym14x16 |
| Ericsson | Yes, but | Agree with ZTE. Alternatively, a CHOICE structure for the 3 subcarrier spacings could be used. Field name: should be cg-minDFI-Delay-r16 (missing hyphen). Field description: - DFI should be spelled out. Prefer to replace “n – minDFI-Delay-r16 …” with “T1 - T2, where T1 is the time corresponding to the beginning of the start symbol of the DFI and T2 is provided by cg-minDFI-Delay (see TS 38.213 [13], clause 10.3). |
| Huawei |  | Agree with Intel and ZTE on item 2. For item 3., the understanding of the group in RAN1 is that same mechanism of FeLAA will be used yet with indeces defined for CPE values rather than defining the offset values. Therefore, at least cg-StartingFullBW-InsideCOT-r16 should be SEQUENCE (SIZE (1..5)) OF INTEGER (0..6) and cg-StartingFullBW-OutsideCOT-r16 should be SEQUENCE (SIZE (1..7)) OF INTEGER (0..6) aul-StartingFullBW-InsideMCOT-r15 BIT STRING (SIZE (5)), aul-StartingFullBW-OutsideMCOT-r15 BIT STRING (SIZE (7)), aul-StartingPartialBW-InsideMCOT-r15 ENUMERATED {o34, o43, o52, o61, oOS1}, aul-StartingPartialBW-OutsideMCOT-r15 ENUMERATED {o16, o25, o34, o43, o52, o61, oOS1}, |
| vivo | Yes | Agree with ZTE |

Summary: ZTE pointed out two missing values and acknowledged by all companies. This is correct.

Intel suggested to use another letter than “n” to prevent confusion which is also fine.

Intel also suggested to use “{7,.., n*14} to represent the range which was supported by 6 companies. Note that the the “periodicity” IE field description does not have the curly bracket so I would suggest to keep the same format as in current RRC for safety. The RRC rapporteur can change both if this is brought up in ASN.1.

Huawei proposed to replace the FFS values with same as feLAA and that is the RAN1 intention. This may be correct but it would be better to see an official agreement in RAN1.

Ericsson proposed other editorials not related to this discussion which can also be moved to the ASN.1 discussion.

Proposal 7: Introduce the following changes in RRC:

- Change the value range for cg-nrofSlots-r16 to {1,2, ..., 40}

- Change the value range for cg-minDFIDelay-r16 to ENUMERATED {sym7, sym1x14, sym2x14, sym3x14, sym4x14, sym5x14, sym6x14, sym7x14, sym8x14, sym9x14, sym10x14, sym11x14, sym12x14, sym13x14, sym14x14, sym15x14, sym16x14} and introduce additional text in the field description as:

cg-minDFIDelay

Indicates the minimum duration (in unit of symbols) from the ending symbol of the CG-PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH. UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n - cg-minDFIDelay-r16, where n is the time corresponding to the beginning of the start symbol of the DFI (see TS 38.213 [13], clause 10.3).

The following minimum delay values are supported depending on the configured subcarrier spacing [symbols]:

15 kHz:	7, m*14, where m={1, 2, 3, 4}

30 kHz:	7, m*14, where m={1, 2, 3, 4, 5, 6, 7, 8}

60 kHz:	7, m*14, where m={1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16}

- Change the value ranges as follows:

cg-StartingPartialBW-InsideCOT-r16 and cg-StartingPartialBW-OutsideCOT-r16 to to INTEGER (0..6)

cg-StartingFullBW-InsideCOT-r16 and cg-StartingFullBW-OutsideCOT-r16r to SEQUENCE (SIZE (1..ffsValue)) OF INTEGER (0..6)

- Change the ffsValue in betaOffsetCG-UCI-r16 value range to 31

# Open issues from RAN2#109e

These are issues which were not resolved in RAN2#109e due to limited discussion and/or captured in the NR-U RRC CR (R2-2001981) as FFS.

All the issues in this section will be addressed in the WI specific CR but not in the ASN.1 review.

### Issue B1: Signaling of intra-cell guard bands

RAN2#109e has discussed the signaling of guard bands and agreed on the following:

- The guard bands for a cell are signalled by using a starting index and length for each guard band, only when the network wants to configure it.

(FFS – move to offline) RAN2 should further discuss the signalling for the cases when there is no guard band, when RAN4 specs should be used, and when/if the UE does not support guard bands.

This is captured in the CR as follows for the case RRC signals an actual guard band:

IntraCellGuardBand-r16 ::=         SEQUENCE (SIZE (1..ffsValue)) OF GuardBand-r16 -- FFS upper size 4, assuming 100Mhz cell

GuardBand-r16       ::= SEQUENCE {

startCRB-r16         INTEGER (0..ffsValue), --FFS upper range 275

nrofCRBs-r16         INTEGER (1..ffsValue)

}

Several options have come up during the email discussion on how to signal the default case (when RAN4 specs are used) and when there is “no guard band” (e.g. 20Mhz).

During the email discussion in RAN2#109e, there was no resolution to this issue as there was no real majority for any of the options.

To recap the issue, these are the two intra-cell guard options which need to be signaled:

- No guard band (e.g. 80 Mhz cell without any guards between each 20 Mhz sub-band)

- Instead of the guard bands explicitly provided via RRC, RAN4 specs should be used instead (will be called “default” case)

One company brought up the possibility that the UE may not support guard bands. However, such an option is missing in the ongoing UE features discussion in RAN1 and therefore will not be addressed here; in other words, it is assumed that it is mandatory for the NR-U capable UE to support guard bands.

To simplify the discussion, we can focus on the three options:

Option 1: Explicit IE for “no guard band”; “default” if signaling is absent

Option 2: Explicit IE for “default”, “no guard band” if signaling is absent

Option 3: Explicit IEs for both “no guard band” and “default”

Note that the explicit IEs can just be NULL when using a CHOICE structure between them and the agreed IE of IntraCellGuardBand-r16.

Question B1: Please provide which option you prefer for the signaling of no guard and default for intra-cell guard bands.

| Company | Response | Comments |
| --- | --- | --- |
| Intel | Option 2 | This is more aligned with the existing RRC signalling, where default is provided via a CHOICE while absent indicates no configuration. |
| ZTE | Option1 |  |
| Samsung | Option 2 | Agree with Intel that absent can be used for release of the configuration. |
| MediaTek | Option 2 | We agree with Intel. |
| CMCC | Option 2 | Agree with Intel. |
| LG | Option 1 | Prefer to keep RAN1 agreement, and current RAN1/2 specifications referring to RAN4 specification when intra-cell guard bands are not configured. |
| OPPO | Option1 | According to running CR, if the parameter is not configured, UE regards it as default behaviour, i.e., referring to RAN4 spec. So, We prefer option1 which is aligned with the current running CR. |
| Apple | Option 2 | Agree with Intel. |
| Nokia | Option 1 | In our understanding this was already decided by RAN1 – i.e no need to discuss in RAN2. Anyway RAN4 needs to define guard bands and defautls for those – it seems logical that one utilizes defaults ones if nothing is signalled. And as the most common case is anyway default GB it seems unnecessary to require any extra signalling for this purpose like we normally do in RAN2. |
| Ericsson | Option 2 | Agree with Intel. It is RAN2 responsiblility to decide on signalling details, and it is recommended to follow the general RRC signalling. - Absent indicates no configuration. - Default configuration is provided in the CHOICE Furthermore, related capabilities are discussed in RAN1, see feature groups 10-19, 10-20, and 10-29 related to wideband operation with guard bands as copied below for better readability. It is not our understanding that operation with guard bands is mandatory. If a UE does not support operation with multiple RB sets, i.e. operation with guard bands, it should not be required to read this field. |
| Huawei | Option2 | Same as what we have proposed in the last meeting. |
| vivo | Option1 |  |

Summary: Out of 12 companies, 7 support Option 2 and 5 support Option 1. Even though there is no clear majority, a decision needs to be made. Since this is just an ASN.1 optimization, it is suggested to go with the slight majority.

Proposal 8: For signalling of intra-cell guard bands, an explicit IE is used for “default” case and no guard bands are used if signaling is absent.

### Issue B2: Multi-TTI grant

In RAN2#100e, there were some papers on implementing multi-TTI grant but this topic was skipped due to limited input and time as well as to wait for conclusion in RAN1.

RAN1 had the following agreements on this:

RAN1#98bis Agreement: For signaling the number of scheduled PUSCHs and TDRA in one DCI scheduling multiple PUSCHs, the TDRA table is extended such that each row indicates multiple PUSCHs (continuous in time-domain) - Alt. 1: Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - FFS: Separate k2 - Alt. 2: - A separate SLIV for - the starting slot, - the ending slot - The slots in between have a SLIV of S=0, L=14 - A separate mapping type for - the starting slot, - the ending slot, - the slots in between (a single mapping type for all of them) - The total number of PUSCHs for the row. - FFS: Extension to non-contiguous PUSCHs RAN1#99 Agreement: - The TDRA table configuration allows indicating single or multiple continuous PUSCHs in any slot of the multiple scheduled slots - For signaling the number of scheduled PUSCHs and TDRA in one DCI format 0_1 scheduling multiple PUSCHs, the TDRA table is extended such that each row indicates multiple PUSCHs (continuous in time-domain) - Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - Note: For the fallback DCI, Rel-15 TDRA table is used Agreement: For scheduling multiple PUSCHs by a single DCI Format 0_1: - The same DCI Format 0_1 can schedule a single PUSCH or multiple PUSCHs - Maximum number of PUSCHs that can be configured in a row of the TDRA table: 8 - The number of NDI bits and RV bits in DCI format 0_1 is determined based on the configured TDRA table - 1 RV bit per PUSCH in case multiple PUSCHs are scheduled - 2 RV bits for the PUSCH in case only a single PUSCH is scheduled RAN1#100e (based on R1-200170) Agreement: - TS38.214 and TS38.212 editors to implement the missing RAN1#97 agreement: - HARQ process ID signaled in the DCI applies to the first scheduled PUSCH. HARQ process ID is then incremented by 1 for subsequent PUSCHs in the scheduled order (with modulo operation as needed) - The corresponding ordering of bits in RV and NDI fields also needs to be specified for the multiple PUSCHs, taking into account BWP switching where MSB are added with zeros.

The agreements in RAN#99 show that the first alternative in RAN2#98bis (Alt-1) has been chosen as the solution. RAN1#100e agreements have no impact on RRC since they are all related to PHY level signalling.

The relevant part in 38.214 v16.0.0 (Section 6.1.2.1) which can impact RRC signalling is the following:

If pusch-TimeDomainAllocationList in pusch-Config contains row indicating resource allocation for two to eight contiguous PUSCHs, K2 indicates the slot where UE shall transmit the first PUSCH of the multiple PUSCHs. Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the pusch-TimeDomainAllocationList signalled in DCI format 0_1.

As the 38.214 text shows, a variable length of PUSCH time domain resource allocation (SLIV and mapping type) is needed for each row. The number of rows, as in Rel-15, will determined the bit-width of the DCI field. Note that there is a single K2 value for each row. This is in contrast to the Rel-15 signaling where each row has one element with a separate K2 as follows:

PUSCH-TimeDomainResourceAllocation information element

-- ASN1START

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PUSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation

PUSCH-TimeDomainResourceAllocation ::=  SEQUENCE {

k2                                      INTEGER(0..32)          OPTIONAL,   -- Need S

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

Here, the logical choice seems to be to have a new IE to replace PUSCH-TimeDomainResourceAllocation in Rel-16 which corresponds to the new row vector described above. This was suggested in R2-200964 (Huawei). A possible ASN.1 based on the Ericsson proposal in RAN2#109e is as follows:

In PUSCH-Config:

pusch-TimeDomainAllocationList-r16   SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

The new PUSCH-TimeDomainResourceAllocationList-r16:

PUSCH-TimeDomainResourceAllocationList-r16 ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation-r16

PUSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k2-r16                                              INTEGER (0..32)                                    	                           OPTIONAL,   -- Need S

multiplePUSCH-Allocations-r16            	        SEQUENCE (SIZE(2..maxNrofMultiplePUSCHs-r16)) OF PUSCH-TimeDomainResourceAllocation-r16

}

PUSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

Here, the value of maxNrofMultiplePUSCHs-r16 would be 8.

Question B2: Is the above ASN.1 text to introduce multi-TTI grant for NR-U acceptable? If not, please provide reason and possible alternatives.

| Company | Response | Comments |
| --- | --- | --- |
| Intel | Yes | An editorial on the ASN.1. There are 2 PUSCH-TimeDomainResourceAllocation-r16 being used. The latter should be changed to Single-PUSCH-TimeDomainResourceAllocation-r16 as it provides a single TDRA. |
| ZTE | Yes | We agree with Intel |
| Samsung | Yes | Agree with Intel |
| MediaTek | Yes | We agree with Intel. |
| CMCC | Yes | Agree with Intel |
| OPPO | Yes | Agree with Intel |
| Apple | Yes | Agree with Intel. |
| Nokia | Yes | We agree with Intel |
| Ericsson | Yes; either use Intel’s correction or legacy PUSCH TDRA IE. | Our original proposal was to reuse legacy PUSCH-TimeDomainResourceAllocation (without suffix) to define the Single-PUSCH. This requires clarification in the field description: If PUSCH-TimeDomainResourceAllocation-r16 is configured, k2-r16 is applied for the PUSCH (if only one PUSCH is configured) or for the first PUSCH of the multiple contiguous PUSCHs (if more than one PUSCH is configured), and k2 (without suffix) is not configured. Using a new IE as proposed by Intel is also fine and simplifies the field description. |
| Huawei | Yes | should be SinglePUSCH-TimeDomainResourceAllocation-r16. Without the dash |
| vivo | Yes | Agree with Intel and Huawei. |

Summary: All companies agree on the changes. Intel pointed out that the same name was used for both multiple and single allocation and suggested to add “single”. This was agreed by all companies.

Proposal 9: Add the following ASN.1 to introduce multiple uplink grants:

In PUSCH-Config:

pusch-TimeDomainAllocationList-r16   SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

The new PUSCH-TimeDomainResourceAllocationList-r16:

PUSCH-TimeDomainResourceAllocationList-r16 ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation-r16

PUSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k2-r16                                              INTEGER (0..32)                                    	                           OPTIONAL,   -- Need S

multiplePUSCH-Allocations-r16            	        SEQUENCE (SIZE(2..maxNrofMultiplePUSCHs-r16)) OF SinglePUSCH-TimeDomainResourceAllocation-r16

}

SinglePUSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

### Issue B3: Paging monitoring

The procedural text for reception of short message with paging stop indication is currently captured in section “5.2.2.2.2 SI change indication and PWS notification” as follows:

1> if the stopPagingMonitoring bit of Short Message is set:

2>	stop monitoring PDCCH monitoring occasion(s) for paging in this Paging Occasion (PO).

It was commented that the title of the section is not compatible with this procedure. The companies are invited to provide input whether they see this as an issue and if so how to resolve it. Following options can be considered:

Option 1: No changes to the spec

Option 2: Change the title of the section, e.g. to “Reception of Short Message”

Option 3: A new section for the new short message reception

Question B3: Please select which option is preferable. If neither, please provide an alternative one.

| Company | Response | Comments |
| --- | --- | --- |
| Intel | None of the above Option | It is better to include a text in Section 6.5. For example, For operation with shared spectrum channel access, once a Short Message is received, the UE shall stop monitoring PDCCH monitoring occasion(s) for paging in this Paging Occasion (PO). |
| ZTE | Option2 |  |
| Samsung | None of the above Option | The current text in RRC is misleading (i.e. it does not cover all the cases which RAN2 agreed), and TS 38.304 already covers all the behaviors regarding paging reception, so the text in RRC can be removed. TS 38.304: "If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO." In case of short message, the PDCCH is also addressed to P-RNTI. |
| MediaTek | Option 2 | We agree with ZTE that the title should be changed. |
| CMCC | Option 2 | Agree with ZTE and MediaTek |
| OPPO | Option 2 |  |
| Apple | Option 2 |  |
| Ericsson | Samsung proposal | Agree with Samsung. The text from Section 5.2.2.2.2 can be removed. This section describes more detailed UE behaviour on further SIB acquisition. Note that Section 5.2 in 38.331 is about System Information. Paging monitoring behaviour is already described in TS 38.304 and in TS 38.331 Section 6.5, which is about the Short Message. Anything related to Short Message should therefore be included in Section 6.5 if really needed. |
| Huawei | Option2 |  |
| vivo | None of the above Options | Agree with Samsung and Ericsson. The text from Section 5.2.2.2.2 is misleading and can be removed. |
| Qualcomm | Option 2 |  |

Summary: Out of 13 companies, 7 support Option 2. 5 companies want to remove the existing sentence and 1 company wants to move a similar text to another section. This topic was discussed for a long time and the sentence was agreed in RAN2#109. The rapporteur suggests not to discuss or revert previous agreements and focus on editorial changes. It is likely that short message will be used for other signalling in the coming releases so the title change will be future-proof.

Proposal 10:  Change the title of the section 5.2.2.2.2 to “Reception of Short Message”. If this is allowed by specification rules, move the statement in section 5.2.2.2.2 to section 6.5.

### Issue B4: Additional values for multiple paging occasions

For nrofPDCCHMonitoringOccasionPerSSB-r16, the value range is INTEGER (2..4) and the following Editor’s Note is in the RRC.

- Editor’s Note: Additional values for nrofPDCCHMonitoringOccasionPerSSB-r16 are FFS.

Question B4: Are additional values needed for nrofPDCCHMonitoringOccasionPerSSB-r16? If yes, please suggest them with justification.

| Company | Response | Comments |
| --- | --- | --- |
| MediaTek | No |  |
| OPPO | No |  |
| Nokia | No | We do not identify reason but we are open if there is use case |
| Ericsson | Yes | 4 bits: INTEGER (2..10) or at least 3 bits: INTEGER (2..8) There should be possibilities to exploit more positions within discovery burst window, which has up to Y=20 candidate SSB positions.. Given that we specified early termination of page monitioring, large values will not necessarily drain UE battery consumption. |
| vivo | No |  |
| Qualcomm | No |  |

Summary: Out of 6 companies, 1 wants to introduce more values while the rest are fine with the existing values.

Proposal 11: No additional values are introduced for nrofPDCCHMonitoringOccasionPerSSB-r16 and the Editor’s Note on this can be removed.

### Issue B5: Additional values for LBT failure detection parameters

The values for LBT Failure detection parameters are as follows with the Editor’s Note that additional values are FFS.

LBT-FailureRecoveryConfig information element

-- ASN1START

-- TAG-LBT-FAILURERECOVERYCONFIG-START

LBT-FailureRecoveryConfig-r16 ::=            SEQUENCE {

lbt-FailureInstanceMaxCount                         ENUMERATED {n4, n8, n16, n32}                              OPTIONAL,   -- Need R

lbt-FailureDetectionTimer                           ENUMERATED {ms10, ms20, ms40, ms80, ms160, ms320}          OPTIONAL,   -- Need R

...

}

-- TAG-LBT-FAILURERECOVERYCONFIG-STOP

-- ASN1STOP

LBT-FailureRecoveryConfig field descriptions

lbt-FailureDetectionTimer Timer for consistent uplink LBT failure detection (see TS 38.321 [3]). Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms, and so on.

lbt-FailureInstanceMaxCount This field determines after how many consistent uplink LBT failure events the UE triggers uplink LBT failure recovery (see TS 38.321 [3]).

- Editor’s Note: Additional values for lbt-FailureDetectionTimer and lbt-FailureInstanceMaxCount are FFS.

Question B5: Are additional values needed for lbt-FailureInstanceMaxCount-r16 and lbt-FailureDectectionTimer-r16? If yes, please suggest them with justification.

| Company | Response | Comments |
| --- | --- | --- |
| ZTE | Yes | For lbt-FailureInstanceMaxCount, we think more values should be defined, such as 64, 128. |
| Samsung | Yes | No strong view, but agree with ZTE that more values would be useful to avoid false alarm. |
| MediaTek | No Preference | In general we agree with the rapporteur. However, if companies see a need for further values, that can also be acceptable |
| CMCC | No Preference | We understand the motivation of avoiding false alarm. To us configuring appropriate lbt-FailureDetectionTimer may solve this issue. Considering the backoff mechanism already existed in LBT procedure (e.g. CCA backoff duration increases as contention happens), we wonder if it is necessary to extend values for lbt-FailureInstanceMaxCount to avoid false alarm. |
| OPPO | No preference |  |
| Apple | No preference |  |
| Nokia | No | We do not identify reason but we are open if there is use case |
| Ericsson | Yes | lbt-FailureInstanceMaxCount It seems reasonable to avoid false alarm and also support larger values, e.g. to cover max preamble transmissions (preambleTransMax is {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}) lbt-FailureDetectionTimer We can take the beamFailureDetectionTimer as a reference. The value is dependent on the CSI-RS/SSB-periodicity. SSB-periodicity can be up to 160 ms. So the beamFailureDetectionTimer could be {ms160, ms320, …, ms1600}. So it seems reasonable to consider larger values (> than 320 ms) also for the lbt-FailureDetectionTimer. |
| Huawei | No |  |
| vivo | No preference |  |

Summary: ZTE proposes to add 64 and 128 which was supported by one company. Ericsson wants longer values than 320ms for lbt-FailureDetectionTimer. 2 companies are against any new values while 5 companies have no preference. Considering that there is not a majority request for any single value, it is not possible to agree on them. One option would be to have spare values so that new values can be added more easily; this can be discussed during ASN.1 review.

Proposal 12: No additional values are introduced for lbt-FailureInstanceMaxCount-r16 and lbt-FailureDectectionTimer-r16 and the Editor’s Note on this can be removed.

# ASN.1 related Issues

Please suggest if there are any ASN.1 related issues such as changes to Need codes, clarifications on field descriptions etc.

All the issues here should be within the scope of the ASN.1 review and a list will be provided for that discussion.

| INDEX | Subclause | IE name | Class (0: Trivial; 1: Minor; 2: ASN.1 specific) | Issue | Rapporteur comments |
| --- | --- | --- | --- | --- | --- |
| SS001 | 5.3.10.3 |  | 0 | “; or” is missing before the second change | Accepted, corrected in CR1528 |
| SS002 | 6.3.1 | SIB2 | 2 | the field ‘ssb-PositionQCL-Common-r16’ should have separate double-square brackets, instead of reusing the existing one. | Accepted, however it was already corrected in 38.331 16.0.0. |
| SS003 | 6.3.2 | BWP-UplinkCommon | 2 | double-square brackets should be added after extension marker. | Accepted, however it was already corrected in 38.331 16.0.0. |
| SS004 | 6.3.2 | ConfiguredGrantConfig | 0 | The field name in the table should be corrected to ‘cg-minDFI-Delay’ (i.e. ‘-‘ is missing) | Accepted, corrected in CR1528 |
| SS005 | 6.3.2 | LogicalChannelConfig | 0 | typo: channellAccessPriority in the table should be channelAccessPriority (ucces l) | Accepted, corrected in CR1528 |
| SS006 | 6.3.2 | MeasObjectNR | 2 | the field rmtc-SubframeOffset-r16 does not have to be optional: it should be mandatory | Move to open issues. This was optional for LAA. However, it was optional for inter-frequency case due to the following statement “For inter-frequency measurements, this field is optional present and if it is not configured, the UE chooses a random value as rmtc-SubframeOffset for measDuration which shall be selected to be between 0 and the configured rmtc-Period with equal probability.”. RAN2 should discuss whether the same applies to NR-U. |
| SS007 | 6.3.2 | MeasObjectNR | 0 | typo: the reference of rmtc-Periodicity is TS 38.215 (i.e. 5 is missing) | Accepted, however it was already corrected in 38.331 16.0.0. |
| SS008 | 6.3.2 | PUCCH-Config | 0 | The field name can be updated to ‘dl-DCI-triggered-UL-ChannelAccess-CP-ext-r16’ (i.e. to add ‘-‘ after acronym CP). | Move to ASN.1 |
| SS009 | 6.3.2 | PUSCH-Config | 0 | The field name can be updated to ‘ul-dci-triggered-UL-ChannelAccessCP-ext-CAPC-r16’ (i.e. to remove ‘-‘ after Access and add ‘-‘ after acronym CP). | Accepted to remove “-“. ; corrected in CR1528 |
| SS010 | 6.3.2 | BWP-UplinkDedicated | 2 | The field ‘useInterlacePUCCH-PUSCH-r16’ should be possible to release: can be changed to BOOLEAN with need M. | Move to open issues. It is not clear if waveform change dynamically is feasible or preferable. Will need RAN1 input. |
| SS011 | 6.3.2 | ConfiguredGrantConfig | 2 | (Many) new NR-U parameters can be grouped together, and can be SetupRelease with need M. Also, some fields (e.g. cg-RetransmissionTimer-r16) within the group can be mandatory to save bits for optional fields. | Move to ASN.1. Note that retransmission may be adopted for licensed as an optional feature. |
| SS012 | 6.3.2 | ControlResourceSet | 2 | Need code for ‘rb-Offset’ should be ‘Need S’, and it has to refer RAN1 specification (to use value 0 in case of absence). | Accepted; corrected in CR1528 |
| SS013 | 6.3.2 | MAC-CellGroupConfig | 2 | Both ‘lbt-FailureRecoveryConfig-r16’ and ‘schedulingRequestID-LBT-Scell-r16’ should use SetupRelease with need M | Move to ASN.1 |

| Index | Subclause | IE Name | Class |  |  |
| --- | --- | --- | --- | --- | --- |
| MTK001 | 6.3.1 | SIB2, SIB4 | 2 | Our understanding is that Q is always needed for measurements on an NR-U cell. However in RRC CR, it appears to be optional (perhaps because it’s not needed for licensed operation). To make it clear that Q always needs to be uccessfu for an NR-U cell/frequency, perhaps “ssb-PositionQCL-Common-r16” in SIB2, SIB4 and MeasObjectNR should be made “Cond Mandatory” (mandatory for NR-U) for NR-U. Without mandatory configuration value of Q, a NR-U UE might not be able to correctly combine measurements from detected SSBs, which would affect RRM. | Move to open issues. |
| MTK002 | 6.3.2 | MeasObjectNR | 2 |  | According to 38.213, it doesn’t have to be signalled for cell access as below. However, this should be discussed for measobject where the UE does not have to read neighbour MIB. An alternative for measobject is also to have a default value (8) when not signalled. and NSSBQCL is either provided by ssbPositionQCL-Relationship-r16 or, if ssbPositionQCL-Relationship-r16 is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4-1. for cell access |
| A001 | 6.3.2 | MeasObjectNR | 2 | Agree with MTK002 that the “ssb-PositionQCL-Common-r16” should be “conditional mandatory” in SIB2/SIB4 and MeasObjectNR for NR-U. | See above |
| A002 | 6.3.1 | SIB2, SIB4 | 2 |  |  |
| A003 | 6.3.1 | SIB2, SIB4 |  | For the following statement “The UE expects that a bit at position k > ssb-PositionQCL-Relationship-16 is 0, and the number of actually transmitted SS/PBCH blocks equal to the number of 1’s in the bitmap.”, we understand the “acutally transmitted” is the “intentionally transmitted” as NW may not be able to transmit all those SSB(s) uccessfully due to LBT failure. Thus, we suggest to change “equal to” to “is no more than”. Note that if the same sentence can be copied to SSB-ToMeasure as we proposed in Issue A1a, the same change is expected. | This is not in the spec so not an ASN.1 issue yet. The comment should have been put in Issue A1. Changed Proposal 1 with “not larger than” as in 38.213. |

| ERI001 | 6.3.2 | PUCCH-Config | 1 | ‘dl-DCI-triggered-UL-ChannelAccess-CP-ext-r16’ List of the combinations of CP extension and UL channel access type (See TS 38.212 [17], Clause 7.3.1). Propose to use generic name instead of listing all parameters configured by the corresponding IE. “list” missing in the field/IE name, compare also PUSCH-TimeDomainResourceAllocationList channelAccessConfigListForDCI1-1 Proposal: Configures a set of channel access parameter combinations, where each list entry indicates a combination of UL channel access type and CP extension. The actual combination for the PUCCH transmission can be indicated in DCI format 1_1 (see TS 38.212 [17], Table 7.3.1.2.2-6). | Accepted to add “list”. Move the IE name change to ASN.1.; partially corrected in CR1528. |
| --- | --- | --- | --- | --- | --- |
| ERI002 | 6.3.2 | PUSCH-Config | 1 | ‘ul-dci-triggered-UL-ChannelAccessCP-ext-CAPC-r16’ List of the combinations of CP extension and UL channel access mode (See TS 38.212 [17], Table 7.3.1-2-35). See above; should be channel access type. channelAccessConfigListForDCI0-1 Configures a set of channel access parameter combinations, where each list entry indicates a combination of UL channel access type, CP extension, and channel access priority class (CAPC). The actual combination for the PUSCH transmission can be indicated in DCI format 0_1 (see TS 38.212 [17], Table 7.3.1.1.2-35). | Accepted the list; move IE name change to ASN.1. |
| ERI003 | 6.3.2 | ServingCellConfigCommon | 0 | discoveryBurst-WindowLength-r16 ENUMERATED {s0dot5, s1, s2, s3, s4, s5} - The field name should be without hyphens (discoveryBurstWindowLength-r16) - As indicated in the field description, the values are prvodied in ms. Thus, the prefixes should be “ms0dot5, ms1, ms2, ms3, ms4, ms5”. | Accepted; corrected in CR1528. |
| ERI004 | 6.3.2 | SearchSpace | 1 | Propose to start with a general description of the field, then define what the bit positions correspond to, and then describe what a value means for a bit. Propose to consistently use RB instead of mixing PRB and RB. freqMonitorLocations Defines an association of the search space to multiple monitoring locations in the frequency domain and indicates whether the pattern configured in the associated CORESET is replicated to a specific RB set. Each bit corresponds to one RB set, and the rightmost bit corresponds to the lowest RB set in the BWP. Value 1 indicates that a frequency domain resource allocation replicated from the pattern configured in the associated CORESET is mapped to the RB set. For this RB set, the first RB of the frequency domain monitoring location confined within the RB set is aligned with {the first RB of the RB set + rb-Offset provided by the associated CORESET. | Accepted the first paragraph. The second sentence is from RAN1 and RAN2 can’t decide if the suggested sentence is technically correct. I will change “implies” to “indicates” as suggested. Partially corrected in CR1528. |
| ERI005 | 6.3.2 | MeasRSSI-ReportConfig | 1 | Propose to use self-descriptive field name that matches with the field description: ‘rssi-ThresholdForCO’ instead of ’channelOccupancyThreshold’ rssi-ThresholdForCO RSSI threshold which is used for channel occupancy evaluation. | Move to ASN.1. Same name has been used in LAA. |
| ERI006 | 6.3.2 | PDCCH-Config | 1 | searchSpaceSwitchingTimer The duration in slots for monitoring search space group 1 before switching to the default search space group (see TS 38.213 [13], clause 11.5.2). Justification: see TS 38.213, clause 11.5.2 | Changed to the “value of the timer in slots…”. Checking 38.213, the suggestion here does not match the description there and the current text is correct. |
| ERI007 | 6.3.2 | CG-COT-Sharing | 0 | Field description of channelAccessPriority-r16 should be moved to CG-COT-Sharing | Accepted. Corrected in CR1528. |
| ERI008 | 6.3.2 | AvailableRB-SetPerCell | 0 | - ASN.1 naming: should be plural: availableRB-SetsPerCell - propose to move to SlotFormatIndicator IE Justification: - Is only used in SlotFormatIndicator - More logical to nest it there because it contains positionInDCI used in the SlotFormatIndicator - Follows RAN1 recommendation | Accepted. Corrected in CR1528. |
| ERI009 | 6.3.2 | RMTC-Config |  | rmtc-Frequency Indicates the center frequency of the pre-defined measurement bandwidth (see TS 38.215, clause 5.1.aa). - Propose to use ”frequency” in the field name to provide more descriptive field name and align with other parameters in the MeasObjectNR: ssbFrequency ARFCN-ValueNR refFreqCSI-RS ARFCN-ValueNR - Propose to clarify that the bandwidth is pre-defined in TS 38.215, clause 5.1.aa. - Clause 5.1.aa can also be included for rmtc-Periodicity and also for rmtc-SubframeOffset | Accepted. Corrected in CR1528 |
| ERI010 | 6.3.2 | SemiStaticChannelAccessConfig |  | period Indicates the period in ms from which the maximum channel occupancy time (COT) is derived and also the periodicity at which the COT can be initiated in of the semi-static channel access mode (see TS 37.213 [XX], clause 4.3). Value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, value ms2dot5 corresponds to 2.5 ms, and so on. - IE description: remove redundant “mode2 (channel accces mode mode) - field description: propose to clarify as indicated above. Closing square bracket missing (also in other several other places, wrong closing bracket is used) | Accepted the reference correction and second sentence starting with “value”. Move the first sentence to open issues (RAN1 should confirm if the suggestion is correct). I don’t see a redundant “mode” in the field description. Partially corrected in CR1528 |
| ERI011 | 6.3.2 | ra-ResponseWindow |  | ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when ra-ResponseWindow (without suffix) is configured and 40 ms when ra-ResponseWindow-r16 is configured (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-r16 is signalled, the UE shall ignore the ra-ResponseWindow (without suffix). - The new parameter has been introduced exactly for extending the RAR window. Therefore, we propose to make the value range dependent on the used parameter version rather than link it to the spectrum. | Move to open issues. |
| v001 | 5.7.3.1 |  | 2 | The purpose of SCG failure information procedure needs to be extended to inform NR MN about PSCell consistent UL LBT failure. | Accepted; corrected in CR 1528 |

| Index | Class | IE | Spec with issue | Change reason |  |
| --- | --- | --- | --- | --- | --- |
| H001 | Class1 | ControlResourceSet | rb-Offset Indicates the RB level offset from the first RB of the first 6RB group to the first PRB of BWP (see 38.213 [13], clause 10.1). When the field is absent, The UE applies the value 0. | Based on the field description of rb-Offset, the field should be OPTIONAL needS | Accepted. See SS012 for the same proposal. |
| H002 | Class1 | PCCH-Config | numPagingMonitoringOccasionPerSSB The number of paging monitoring occasions corresponding to an SSB, see TS 38.304 [20], clause 7.1. | More accurately, it should be "within a PO for an SSB" | Accepted. Corrected in CR1528. Note that the IE name is nrofPDCCHMonitoringOccasionPerSSB-InPO and it is actually in DownlinkConfigCommonSIB |
| H003 | Class1 | PUCCH-Config | rbSetIndex ndicates the RB set where the first interlace allocated for a PUCCH resource. | no need to say that this is the first interlace, just say the RB set allocated for a PUCCH resource | Accepted. I will change to “"Indicates the RB set where PUCCH resource is allocated”. Corrected in CR1528. |
| H004 | Class1 | ServingCellConfigCommon | semiStaticChannelAccessConfig The parameters for semi-static channel access. The network configures this only when channelAccessType-r16 is set to semistatic. | no need for the sentence “The network configures this only when channelAccessType-r16 is set to semistatic.”. With the choice structure this is self-explanatory | Accepted. Corrected in CR1528. |
| H005 | CLass0 |  | Names like channelAccess-Config-r16 and discoveryBurst-Win dowLength-r16 should follow the ASN.1 naming rules |  | Accepted. The first one was corrected in 16.0.0; the second one was corrected in CR1528. |
| H006 | Class1 |  | pdsch-HARQ-ACK-Codebook-r16 ENUMERATED {enhancedDynamic , spare1 } | why there is a spare bit here? | Accepted; will remove; Corrected in CR1528. |
| H007 | Class1 | PUCCH-COnfig | dl-DataToUL-ACK-r16 and dl-DataToUL-ACK cannot be configured at the same time. |  | Will add the conventional sentence that the UE will ignore the legacy if r16 is configured. |

# New Open Issues

Please suggest if there are any other issues not capture above. For each one, we will also need to determine if it should be addressed within the WI RRC or as part of ASN.1 review.

### 5.1 Ericsson input

The following issues have been raised before, but have not been resolved yet:

#### 1a) Search space switching has to be configured also for Type-3 CSS. Currently only defined for USS. freqMonitorLocations is not limited to USS, either.

Agreement:

For search space switching, limit the switching to USS and Type-3 CSS.

[Rapporteur] Same as Huawei 1. Move to open issues; need RAN1 confirmation. My understanding is that Type-3 CSS is still UE specific.

#### 1b) SearchSpaceSwitchingTrigger for cell groups: In 38.213, no group ID is indicated, but RRC parameter sheet has not been updated yet.

searchSpaceSwitchTrigger-r16     SEQUENCE {

positionInDCI        INTEGER(0..maxSFI-DCI-PayloadSize-1),

servingCellId          ServCellIndex

OPTIONAL, -- Need N

[Rapporteur] This seems like a RAN1 problem. We can wait for their update.

#### 2) CO-DurationPerCell:

From RAN1 parameter list: Add in SlotFormatIndicator.

A list of CO-DurationPerCell-r16 objects is configured for one or more serving cells.

It should be possible to configure multiple COT duration, for multiple cells. Currently, only one single cell can be configured.

[Rapporteur] Agree; will change this to a list. Corrected in CR1528

#### 3) RSSI/CO reporting: some open issues need to be resolved, e.g. whether only RSSI/CO measurements are provided for a report ID or whether those are provided with existing quantities.

Improved specification text / clarifications:

#### 4) RSSI measurements: Section 5.5.1:

Current text: for RSSI and channel occupancy measurements, the UE measures and reports on any reception on the indicated frequency.

Proposed clarification: for RSSI and channel occupancy measurements, the UE measures and reports on the defined measurement bandwidth and configured time domain measurement resources on the indicated frequency.

Justification: the current text is too vague, given that the RMTC-Config configures the time domain resources and the measurement bandwidth is specified in 38.215:

“the configured measurement bandwidth over N number of resource blocks corresponding to LBT bandwidth with the center frequency of configured ARFCN”

[Rapporteur] Even though this was discussed and not agreed, I’ll move to open issues for one last time. Again, the current text is same as LAA. If the text in 38.215 is clear, why have the same in RRC?

#### 5) RMTC configuration in Section 5.5.2.X can be improved as follows:

The UE shall setup the RSSI measurement timing configuration (RMTC) in accordance with the received rmtc-Periodicity, and if configured with rmtc-SubframeOffset, otherwise rmtc-SubframeOffset is determined by the UE randomly, i.e. the first symbol of each RMTC occasion occurs at first symbol of an SFN and subframe of the PCell meeting the following condition:

SFN mod T = FLOOR(rmtc-SubframeOffset/10);

subframe = rmtc-SubframeOffset mod 10;

with T = rmtc-Periodicity/10;

The length of the RMTC occasion is defined by measDuration. On theconfigured rmtc-Frequency, the UE shall perform RSSI and channel occupancy measurements within the configured RMTC occasion, where the measurement bandwidth is defined in [TS 38.215, clause 5.1.aa] and the time domain measurement resources are defined by the RMTC occasion.

Justification:

As all symbols within the measDuration are considered for the measurements. Thus, the sentence above can be simplified. Further clarification on the RSSI time/frequency domain resources is useful for inter-operability.

It would be beneficial to clarify that only rmtc-SubframeOffset is optional.

For completeness, it would be beneficial to add rmtc-Frequency (currently ‘rmtc-measARFCN’, see ERI009), to clarify that the frequency is configured as well.

[Rapporteur] See the ASN.1 list regarding your suggestion to use “frequency”. Will move the rest to open issues.

#### 6) Overhead due to extension of IntraFreqNeighCellInfo / InterFreqNeighCellInfo:

As previously indicated, the extension of the above lists has significant signaling costs. If there is no need to signal any of the parameters related to cell offset, the following will result in additional signaling overhed.

  - adding extensions after “. . .” costs 2-3 bytes.

  - signaling of the mandatory cell individual offset costs 5 bits

  - signaling of the optionality bits for the three other optional fields costs 3 bits

-> 3-4 bytes of unnecessary signaling FOR EACH CELL in the neighboring cell list in system information compared to defining new lists in order to signal 2-bit SSB-Position-QCL-Relationship (compared to 24-32 bits of overhead). And this is even multiplied by the number of cells.

IntraFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,		10 bits

q-OffsetCell                        Q-OffsetRange,  5 bits

q-RxLevMinOffsetCell                INTEGER (1..8)                 OPTIONAL,   -- Need R   1 bit

q-RxLevMinOffsetCellSUL             INTEGER (1..8)                 OPTIONAL,   -- Need R   1 bit

q-QualMinOffsetCell                 INTEGER (1..8)                 OPTIONAL,   -- Need R   1 bit

...,

[[  2-3 bytes

ssb-Position-QCL-r16           SSB-Position-QCL-Relationship-r16           OPTIONAL    -- Need R      -- 2 + 1 bits

]]

}

Proposal: Use separate list as for eample below. Similar list can be used for configuring ssb-PositionQCL-CellList in the measurement object.

SIB3:

[[

intraFreqWhiteCellList-r16          IntraFreqWhiteCellList-r16   OPTIONAL,    -- Need R

ssb-PositionQCL-CellList-r16       SSB-PositionQCL-CellList-r16     OPTIONAL    -- Need R

]]

SSB-PositionQCL-CellList-r16 ::=    SEQUENCE (SIZE (1..maxCellIntra)) OF SSB-PositionQCL-Info-r16

SSB-PositionQCL-Info-r16 ::=   SEQUENCE {

physCellId-r16                  PhysCellId,

ssb-PositionQCL-Cell-r16        SSB-PositionQCL-Relationship-r16

}

SIB4:

[[

interFreqWhiteCellList-r16          InterFreqWhiteCellList-r16                  OPTIONAL,   -- Need R

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relationship-r16            OPTIONAL,    -- Need R

ssb-PositionQCL-CellList-r16       SSB-PositionQCL-CellList-r16     			OPTIONAL    -- Need R

]]

SSB-PositionQCL-CellList-r16 ::=    SEQUENCE (SIZE (1..maxCellInter)) OF SSB-PositionQCL-Info-r16

Justification:

It is typically not necessary to define a cell-specific offset. Typically used only for heterogenous network deployments. 
SSB-Position-QCL-Relationship itself only requires 2 bits.

[Rapporteur] This was discussed in RAN2#109e and not agreed. Will move to ASN.1.

#### 7) For the parameter useInterlacePUCCH-PUSCH within the BWP-UplinkCommon IE:

the field description should be as follows:

If the field is present, the UE uses uplink frequency domain resource allocation Type 2 for cell-specific PUSCH, e.g., PUSCH scheduled by RAR UL grant (see 38.213 clause 8.3 and 38.214 clause 6.1.2.2) and uses interlaced PUCCH Format 0, 1, 2, and 3 for cell-specific PUCCH (see TS 38.213 [13], clause 9.2.1).

since only PUCCH formats 0,1 are allowed for cell-specific PUCCH, i.e., PUCCH prior to dedicated configuration.

[Rapporteur] Corrected in CR1528

#### 8) Further comments on Issue A1

The issue is that Q can be defined in MIB (no field name), in ssb-PositionQCL-Common, and ssb-PositionQCL, depending on what the gNB provides in its system information or in the measurement object, which would make the field description unnecessarily complex.
Therefore, referring to the Information Element instead of the field could make sense, maybe something like “k > Q, where Q is the configured value for SSB-PositionQCL-Relationship.”

[Rapporteur] Will add to the open issue list.

### 5.3 Huawei Input

Issue#1:

searchSpaceGroupIdList is under ue-specific. However, common search space can also use search space group

RAN1#100e has the following Agreement: For search space switching, limit the switching to USS and Type-3 CSS.

[Rapporteur] See Ericsson 1a. Move to open issues; need RAN1 confirmation. My understanding is that Type-3 CSS is still UE specific.

Issue#2:

field description for cg-COT-SharingList is missing.

[Rapporteur] Will add. Corrected in CR1528

Issue#3

searchSpaceGroupIdList-r16            SEQUENCE (SIZE (1.. 2)) OF INTEGER (0..1)               OPTIONAL,    -- Need R

searchSpaceGroupIdList

List of search space group IDs which the search space set is associated with.

description not clear. which bit corresponds to which group and what does 0/1 stands for should be explained.

[Rapporteur] Will add “The network configures at most 2 search space groups per PWP where the group ID is either 0 or 1.”

Corrected in CR1528

Issue#4

dl-DataToUL-ACK-r16                            SEQUENCE (SIZE (1..8)) OF INTEGER (-1 ..15)

what does the value "-1" means should be explained

[Rapporteur] Will add to the field description “If dl-DataToUL-ACK-r16 is signalled, UE shall ignore the dl-DataToUL-ACK (without suffix). The value -1 corresponds to “non-numerical value” for the case where the A/N feedback timing is not explicitly included at the time of scheduling PDSCH”.

Corrected in CR1528

Issue#5

cg-minDFIDelay

Indicates the minimum duration (in unit of symbols) from the ending symbol of the CG-PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH. UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n-cg-DFIDelay-r16, where n is the time corresponding to the beginning of the start symbol of the DFI (see TS 38.213 [13], clause 10.3)

The description is not accurate enough and it is only for CG. For DG, the descritpion is different.

[Rapporteur] This is for CG only as indicated by the prefix. Note that DFI is only applicable to CG. Please provide the issue for DG with suggestion on how to resolve it.

Issue#6

"    [[

searchSpaceSwitchingTimer -r16     INTEGER (1..ffsValue)                                         OPTIONAL,   -- Need R

searchSpaceSwitchingGroupList-r16   SEQUENCE(SIZE (1..ffsValue2)) OF SearchSpaceSwitchingGroup-r16 OPTIONAL   -- Need R

]]"

these two configurations are cell group level configuration. The intention is that when the group is switched, all the PDCCH monitoring of cells within a SS swtiching group is switched.

[Rapporteur] Please describe the problem.

Issue#7

PUCCH-Resource-r16

within PUCCH-Resource-r16, the PUCCH-ResourceID should share the common space of numbering as the R15 PUCCH resource Id

[Rapporteur] Please describe the problem. It does share the same resource ID as Rel-15 in the CR/spec.

# Conclusion

Based on the discussion on the above issues, the following are proposed:

Proposal 1: Introduce the field descriptions communicated by RAN1 with the following changes:

- Use inOneGroup instead of mediumBitmap in ServingCellConfigCommonSIB

- Include the statement in “The UE expects that a bit at position k > ssb-PositionQCL-Relationship-16 is 0, and the number of actually transmitted SS/PBCH blocks is not larger the number of 1’s in the bitmap.” in SSB-ToMeasure

- Use “For operation in licensed spectrum” instead of “without shared spectrum channel access”

- Use “leftmost” instead of “MSB”

Proposal 2: No other changes are introduced to RRC to address the recommendations and agreements in RAN1 LS (R2-2001357).

Proposal 3. Introduce the following changes:

- Replace ffsValue with 64 in:

    - co-DurationList-r16       SEQUENCE (SIZE(1..ffsValue)) OF CO-Duration-r16 -- FFS size upper limit 64

- Replace ffsValue below with 1120 to support 20ms duration (the new upper limit is changed from 560 to 1120 as it is needed for SCS 60Khz):

    - CO-Duration-r16 ::=    INTEGER (0..ffsValue) -- FFS upper limit 560

- Add field description for CO-Duration

Proposal 4. Introduce the following changes:

- Replace ffsValue below with 80 (maximum needed for 20ms with SCS of 60khz):

    - searchSpaceSwitchingTimer-r16       INTEGER (1..ffsValue)

- Put in the field description of searchSpaceSwitchingTimer that “For 15 kHz SCS, {1..20} are valid. For 3kKHz SCS, {1..40} are valid. For 60kHz SCS, {1..80} are valid. Note that this is in slots as used in 38.213.

Proposal 5: Introduce the following in the field description of cp-ExtensionC2, cp-ExtensionC3:

Configures the cyclic prefix (CP) extension (see TS 38.211 [16], clause 5.3.1). For 15 kHz SCS, {1..28} are valid for both cp-ExtensionC2 and cp-ExtensionC3. For 30 kHz SCS, {1..28} are valid for cp-ExtensionC2 and {2..28} are valid for cp-ExtensionC3. For 60 kHz SCS, {2..28} are valid for cp-ExtensionC2 and {3..28} are valid for cp-ExtensionC3.

Proposal 6: Introduce the following changes to RRC:

- Introduce a new IE in RMTC-Config called ref-SCS-CP with the values of {15 kHz, 30 kHz, 60 kHz-NCP, 60 kHz-ECP}

- Remove the Editor’s Note on L3 filtering for RSSI

- Wait for RAN4 conclusion on actual values for rssi-Result-r16 and channelOccupancyThreshold-r16 before introducing the indices corresponding to RAN4 table

Proposal 7: Introduce the following changes in RRC:

- Change the value range for cg-nrofSlots-r16 to {1,2, ..., 40}

- Change the value range for cg-minDFIDelay-r16 to ENUMERATED {sym7, sym1x14, sym2x14, sym3x14, sym4x14, sym5x14, sym6x14, sym7x14, sym8x14, sym9x14, sym10x14, sym11x14, sym12x14, sym13x14, sym14x14, sym15x14, sym16x14} and introduce additional text in the field description as:

cg-minDFIDelay

Indicates the minimum duration (in unit of symbols) from the ending symbol of the CG-PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH. UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n - cg-minDFIDelay-r16, where n is the time corresponding to the beginning of the start symbol of the DFI (see TS 38.213 [13], clause 10.3).

The following minimum delay values are supported depending on the configured subcarrier spacing [symbols]:

15 kHz:	7, m*14, where m={1, 2, 3, 4}

30 kHz:	7, m*14, where m={1, 2, 3, 4, 5, 6, 7, 8}

60 kHz:	7, m*14, where m={1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16}

- Change the value ranges as follows:

cg-StartingPartialBW-InsideCOT-r16 and cg-StartingPartialBW-OutsideCOT-r16 to to INTEGER (0..6)

cg-StartingFullBW-InsideCOT-r16 and cg-StartingFullBW-OutsideCOT-r16r to SEQUENCE (SIZE (1..ffsValue)) OF INTEGER (0..6)

- Change the ffsValue in betaOffsetCG-UCI-r16 value range to 31

Proposal 8: For signalling of intra-cell guard bands, an explicit IE is used for “default” case and no guard bands are used if signaling is absent.

Proposal 9: Add the following ASN.1 to introduce multiple uplink grants:

In PUSCH-Config:

pusch-TimeDomainAllocationList-r16   SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

The new PUSCH-TimeDomainResourceAllocationList-r16:

PUSCH-TimeDomainResourceAllocationList-r16 ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation-r16

PUSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k2-r16                                              INTEGER (0..32)                                    	                           OPTIONAL,   -- Need S

multiplePUSCH-Allocations-r16            	        SEQUENCE (SIZE(2..maxNrofMultiplePUSCHs-r16)) OF SinglePUSCH-TimeDomainResourceAllocation-r16

}

SinglePUSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

Proposal 10:  Change the title of the section 5.2.2.2.2 to “Reception of Short Message”. If this is allowed by specification rules, move the statement in section 5.2.2.2.2 to section 6.5.

Proposal 11: No additional values are introduced for nrofPDCCHMonitoringOccasionPerSSB-r16 and the Editor’s Note on this can be removed.

Proposal 12: No additional values are introduced for lbt-FailureInstanceMaxCount-r16 and lbt-FailureDectectionTimer-r16 and the Editor’s Note on this can be removed.

# Annex 1 (RAN1 LS R1-2001357)

1. Overall Description:

RAN WG1 has made the following agreements related to the candidate SS/PBCH block index and SS/PBCH block index for NR-U:

Agreement:

UE determines serving cell timing from the detected SSB candidate position, where the SSB candidate positions within the DRS transmission window are indexed from 0,…,Y-1 (Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS).

Agreement:

RAN1 recommends the following terminology to be used consistently across RAN1 (and RAN2/RAN4) specifications for operation with shared spectrum channel access:

- “SS/PBCH block index” can be expressed as modulo(PBCH DMRS sequence index, Q) or modulo(Candidate SS/PBCH block index, Q)

  - Note: These two modulo operations yield the same result and which index should be applied depends on the specific scenario

  - Note: This is applicable for cases in the specification where “SS/PBCH block” instead of “SS/PBCH block index”

- “Candidate SS/PBCH block index” within a DRS transmission window was previously agreed in RAN1

- Include this agreement in an LS to RAN2/RAN4

Agreement:

The interpretation of ssb-PositionsInBurst in SIB1 or ServingCellConfigCommon is as follows:

- A bit set to 1 at position k (indexing starts at 1) indicates SS/PBCH block index k-1.

- The UE expects that a bit at position k > Q is set to 0

Please note: in RAN1 specifications, the normative term used for “DRS transmission window” in the above agreements is “Discovery burst transmission window,” where “Discovery burst” is defined in Clause 4 of TS 37.213.

RAN WG1 kindly asks RAN WG2 and WG4 to take the above agreements on SS/PBCH block indexing into account for developing their specifications for operation with shared spectrum channel access.

A list of examples of the expected changes to the specifications are provided in the Appendix, and more examples can be found in R1-1913538.

2. Actions:

To RAN WG2 and WG4.

ACTION: RAN WG1 kindly asks RAN WG2 and WG4 to take above agreements for developing their specifications.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-WG1 Meeting #100bis 	20th – 24th Apr 2020		Busan, KR

TSG-WG1 Meeting #101		25th – 29th May 2020		Athens, GR

Appendix: Example changes to TS 38.331

6.3.2	Radio resource control information elements

ServingCellConfigCommon field descriptions

ssb-PositionsInBurst For operation without shared spectrum channel access, iIndicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB. For operation with shared spectrum channel access, only mediumBitmap is used, and as described in the TS 38.213 [13], clause 4.1, UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the discovery burst transmission window and have candidate SS/PBCH blocks indexes corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst. If MSB k, k≥1, of ssb-PositionsInBurst is set to 1, the UE assumes that one or more SS/PBCH blocks within the discovery burst transmission window with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k-1 may be transmitted; if MSB k is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. The UE expects that a bit at position k > ssb-PositionQCL-Relationship-16 is 0, and the number of actually transmitted SS/PBCH blocks equal to the number of 1’s in the bitmap. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB.

ServingCellConfigCommonSIB field descriptions

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, only mediumBitmap is used, and UE interpret this field the same as in the corresponding field in ServingCellConfigCommon.

SSB-ToMeasure field descriptions

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access, a bit set to 1 at position k (indexing starts at 1) in the bitmap indicates SS/PBCH block index k-1. A UE can derive the time domain positions of the candidate SS/PBCH blocks within the SMTC measurement duration based on this bitmap. From a value 0 at position k in the bitmap, the UE can derive the candidate SS/PBCH block(s) with index corresponding to the SS/PBCH block index k-1 are not to be measured, while from a value 1 at position k in the bitmap, the UE can derive the candidate SS/PBCH block(s) with index corresponding to the SS/PBCH block index k-1 are to be measured.

# Annex 2 Changes to RAN1 parameter names by RAN2

RAN2 decided to use a single useInterlacePUCCH-PUSCH IE which is signaled in BWP-UplinkCommon and BWP-UplinkDedicated instead of separate PUSCH and PUCCH used in RAN1 specifications.

The following RAN1 parameters were modified to comply with ASN.1 guidelines:

cg-CG-UCI-Multiplexing -> cg-UCI-Multiplexing

ChannelAccess-Config-r16 -> channelAccess-Config-r16

ChannelAccessMode-r16   -> channelAccessMode-r16

CO-DurationList-r16 - > co-DurationList-r16

CO-DurationPerCell-r16 - > co-DurationPerCell-r16

CP-ExtensionC2-r16 -> cp-ExtensionC2-r16

CP-ExtensionC3-r16 -> cp-ExtensionC3-r16

DLDCI-triggered-UL-ChannelAccess-CPext-r16 - > dl-DCI-triggered-UL-ChannelAccess-CPext-r16

EnableConfiguredUL-r16 -> enableConfiguredUL-r16

Interlace1 -> interlace1

InterlaceAllocation-r16 -> interlaceAllocation-r16

NFI-TotalDAI-Included-r16 -> nfi-TotalDAI-Included-r16

OCC-Index -> occ-Index

OCC-Length -> occ-Length

Period -> period

rbOffset-r16 -> rb-Offset-r16

SearchSpaceSwitchTrigger-r16 -> searchSpaceSwitchTrigger-r16

ssb-positionsInBurst -> ssb-PositionsInBurst

ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-r16 -> ul-dci-triggered-UL-ChannelAccess-CPext-CAPC-r16

ULtoDL-CO-SharingED-Threshold-r16 - > ul-toDL-COT-SharingED-Threshold-r16

UL-TotalDAI-Included-r16 -> ul-TotalDAI-Included-r16

rmtc-MeasARFCN-r16 -> rmtc-Frequency

# Annex 3 RAN1 parameters with FFS values

cg-COT-SharingOffset-r16               INTEGER (1..ffsValue)   OPTIONAL,   -- Need R

cg-nrofPUSCH-InSlot-r16                INTEGER (1..ffsValue)

cg-COT-SharingList-r16                 SEQUENCE (SIZE (1..ffsValue)) OF CG-COT-Sharing-r16 OPTIONAL,   -- Need R

CG-COT-Sharing-r16 ::= SEQUENCE {

duration-r16                      INTEGER (1..ffsValue),

offset-r16                        INTEGER (1..ffsValue),

channelAccessPriority-r16         INTEGER (1..4)

}

IntraCellGuardBand-r16 ::=         SEQUENCE (SIZE (1..ffsValue)) OF GuardBand-r16 -- FFS upper size 4, assuming 100Mhz cell

GuardBand-r16       ::= SEQUENCE {

startCRB-r16         INTEGER (0..ffsValue), --FFS upper range 275

nrofCRBs-r16         INTEGER (1..ffsValue)

}

searchSpaceSwitchingTimer-r16       INTEGER (1..ffsValue)