3GPP TSG-RAN WG2 Meeting #109bis	electronic							    R2-2002872

E-Meeting, 20th Apr – 30th Apr, 2020

Source: 	vivo

Title:  	Discussion on the SCell BFD on the deactivated SCell

Agenda Item:	6.16.3

Document for:	Discussion and Decision

# Introduction

According to the agreed MAC CR [1] for the eMIMO WI, the SCell BFR is cancelled while the SCell is deactivated. In this contribution, we discuss the potential issue on the measurement of the BFD RS for the deactivated SCell.

# Discussion

## Measurement on the BFD RS for the deactivated SCell

According the agreed MAC CR for the eMIMO WI as quoted below, the SCell BFR is cancelled when the SCell is deactivated.

1> else if an SCell Activation/Deactivation MAC CE is received deactivating the SCell; or 1> if the sCellDeactivationTimer associated with the activated SCell expires: 2> deactivate the SCell according to the timing defined in TS 38.213 [6]; 2> stop the sCellDeactivationTimer associated with the SCell; 2> stop the bwp-InactivityTimer associated with the SCell; 2> deactivate any active BWP associated with the SCell; 2> clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively; 2> clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell; 2> suspend any configured uplink grant Type 1 associated with the SCell; 2> cancel all the triggered BFRs (see clause 5.17) for this Serving Cell; 2> flush all HARQ buffers associated with the SCell.

However when the SCell is deactivated, the UE is still required to measurement the BFD (Beam Failure Detection) RS to send the “beam failure instance indication” for the deactivated SCell according to the current specification. When the SCell is deactivated, the UE is supposed to return off the RF chain for the deactivated SCell to save the UE power. If the measurement of the BFD RS is not stopped when the SCell is deactivated, the power saving benefit of the deactivated SCell would be lost. As such we consider that the UE should stop the measurement of the BFR RS when the SCell is deactivated. The related text proposal can be found in the Annex.

Proposal: The UE stops the BFD RS monitoring when the SCell is deactivated.

# Conclusion

According to the analysis given above, we have the following proposal:

Proposal: The UE stops the BFD RS monitoring when the SCell is deactivated.

# Reference

[1] R2-2001706, Samsung, “MAC Running CR for NR eMIMO”.

# Annex

## 5.9	Activation/Deactivation of SCells

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry.

The MAC entity shall for each configured SCell:

1>	if an SCell Activation/Deactivation MAC CE is received activating the SCell:

2>	activate the SCell according to the timing defined in TS 38.213 [6]; i.e. apply normal SCell operation including:

3>	SRS transmissions on the SCell;

3>	CSI reporting for the SCell;

3>	PDCCH monitoring on the SCell;

3>	PDCCH monitoring for the SCell;

3>	PUCCH transmissions on the SCell, if configured;

3>	BFD RS monitoring on the SCell.

2>	if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE:

3>	activate the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively;

2>	start or restart the sCellDeactivationTimer associated with the SCell according to the timing defined in TS 38.213 [6];

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this SCell according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2;

2>	trigger PHR according to clause 5.4.6.

1>	else if an SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2> cancel all the triggered BFRs (see clause 5.17) for this Serving Cell;

2>	flush all HARQ buffers associated with the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

2>	not monitor BFD RS on the SCell.

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

When SCell is deactivated, the ongoing Random Access procedure on the SCell, if any, is aborted.