3GPP TSG-RAN WG2 Meeting #109bis	electronic							    R2-2002875

E-Meeting, 20th Apr – 30th Apr, 2020

Source: 	vivo

Title:  	Remaining control plane issues of DAPS

Agenda Item:	7.3.2.2

Document for:	Discussion and Decision

# Introduction

In this contributions we provide some our understandings on some of the remaining control plane issues for the DAPS handover, for both the DAPS DRB and the non-DAPS DRB.

# Discussion

## Fallback of the non-DAPS DRB at handover failure

According to the discussion on the non-DAPS DRB handling in the RAN2#109e meeting, RAN2 made the following agreements:

1: Upon DAPS handover failure, UE reverts back to the source configuration prior to the reception of the handover command (including RLC and PDCP state) for the DRB that is not configured with DAPS.

2: For non DAPS DRB, upon DAPS HO failure, the reverted PDCP/RLC state includes data stored in transmission and reception buffers in PDCP and RLC entities prior to the reception of the handover command.

3: For non DAPS DRB, upon DAPS HO failure, the reverted source configuration also includes SDAP (for NR) configuration and logical channel configuration.

4: If the data is reverted for non-DAPS DRBs in case of DAPS HO failure, the data stored in transmission and reception buffers should NOT be discarded.

5: RRC re-establishment shall not be triggered due to source link RLF after successful RA and before the release of source link.

7: for NR, the state variables of the target SRB PDCP should be set to the latest ones kept in the source SRB PDCP if security key is unchanged.

9: for SRBs and non-DAPS DRBs, the PDCP COUNT is maintained when DAPS HO without key change and also at fallback to source cell when DAPS handover is performed without key change.

For the LTE non-DAPS DRB, as the RLC entity has stored packets in the RLC entity upon the DAPS HO failure, the RLC entity should submit the stored RLC packet to the PDCP entity and the PDCP entity should process those packets delivered from the RLC entity. As delivering the stored RLC packet to the PDCP entity can only be done via the RLC re-establishment procedure according to the current specification, we could have the following two ways to allow the RLC entity to deliver the stored RLC packets:

- Option 1: LTE RLC entity is re-established before reverting back to the source. configuration.

- Option 2: Define a new RLC procedure.

As Option 2 would require more discussion and more specification efforts on the new procedure, we would suggest to adopt Option 1. For the RLC UM entity, we consider that the packets in the UM entity should be discarded as the current re-establishment procedure of the RLC. Otherwise we need to define a new procedure to flush the packets in the reordering/reassembly buffer of the UM entity to avoid stuck packets occupying the RLC buffer.

Observation 1: The packets stored the LTE RLC AM entity should not be lost.

Observation 2: The packets stored in the LTE/NR RLC entity should not occupy the RLC buffer.

Proposal 1: Upon DAPS HO failure, LTE/NR RLC UM/AM entity of DRB is re-established before reverting back to the source configuration (i.e. RLC states before the reception of the handover command).

According to the current PDCP procedure, the PDCP entity will only deliver the packets after reordering. If the PDCP states at DAPS HO failure revert back to the source PDCP states before the reception of the handover command, then the PDCP entity is not able to deliver the target packets to the upper layer as the states of the reordering window of the PDCP entity would be changed when the reverting happens. Then one solution would be to allow the PDCP to deliver all the stored packets in-order after deciphering and decompression by ignoring all the gaps in the re-ordering window.

Observation 3: If the PDCP states reverts back to the source connection, the PDCP re-establishment procedure cannot be reused to avoid the packet loss in PDCP as the PDCP re-establishment procedure can only support in-order delivery after reordering and without changing the PDCP states.

Proposal 2: Upon DAPS HO failure, LTE/NR PDCP entity of DRB delivers all stored packets in-order by ignoring all the gaps in the re-ordering window after deciphering and decompression using the target context, before reverting back to the source configuration (i.e. PDCP states before the reception of the handover command).

## Stuck old RRC message

According to the discussion on “Issue 2.3-8: Handling on the stored old RRC message in source when fallback upon DAPS failure”, most companies support to discard the old RRC message when the source SRB is resumed at the DAPS handover failure. According to the current LTE/NR specification, the old RRC message is stored in the PDCP, and could also be stored in the RLC buffer due to the NR pre-processing or the uplink grant arrival. Thus to discard the old RRC message in the PDCP/RLC buffer, the UE can flush its PDCP/RLC buffer by reusing the current re-establishment procedure.

Proposal 3: The PDCP/RLC buffer of the source SRB is flushed by reusing the re-establishment procedure at the DAPS handover failure.

# Conclusions

Based on the analysis given above, we have the following Observations and Proposals:

Observation 1: The packets stored the LTE RLC AM entity should not be lost.

Observation 2: The packets stored in the LTE/NR RLC entity should not occupy the RLC buffer.

Observation 3: If the PDCP states reverts back to the source connection, the PDCP re-establishment procedure cannot be reused to avoid the packet loss in PDCP as the PDCP re-establishment procedure can only support in-order delivery after reordering and without changing the PDCP states.

Proposal 1: Upon DAPS HO failure, LTE/NR RLC UM/AM entity of DRB is re-established before reverting back to the source configuration (i.e. RLC states before the reception of the handover command).

Proposal 2: Upon DAPS HO failure, LTE/NR PDCP entity of DRB delivers all stored packets in-order by ignoring all the gaps in the re-ordering window after deciphering and decompression using the target context, before reverting back to the source configuration (i.e. PDCP states before the reception of the handover command).

Proposal 3: The PDCP/RLC buffer of the source SRB is flushed by reusing the re-establishment procedure at the DAPS handover failure.

# Reference

[1] R2-200xxxx, Intel Corporation, “Report of [Post109e#11][MOB] Resolving open issues for DAPS (Intel)”.