3GPP TSG-RAN WG2 #109bis-e	R2-2003041

Electronic meeting, April 2020	Revision of R2-2000337

Agenda Item:	6.2.3

Source:	Ericsson

Title:	Remaining Control Plane Issues

Document for:	Discussion, Decision

# 1	Introduction

This contribution will discuss and/or provide clarifications for the following issues:

- RSSI/CO measurement reporting

- SearchSpace modifications to include search space switching related parameters and the bitmap for freqMonitoringLocations

- Optimizing signaling overhead for signalling of SSB-Position-QCL-Relationship

# 2	Discussion

## 2.1 	RSSI measurements

For RSSI/CO measurement and reporting, we have the following agreements:

RAN2#105bis Xi’an:

RSSI and Channel Occupancy configuration and reporting, in particular measurements over an interval (at least for CO) and periodical reporting, are used as a baseline for NR-U

RAN2#106 Reno:

RSSI and CO measurement quantities can be reported with existing triggers as in LAA

RAN2#107bis Chongqing:

For normal HO and CHO, no new event triggers will be introduced.  RRSI CO measurements can be included in the measurement reports.

RAN2#109e, eMeeting:

Change the location of measRSSI-ReportConfig-r16 so that it is located in both PeriodicalReportConfig and EventTriggerConfig

In addition to periodical reporting, RAN2 has agreed to support RSSI/CO reporting for both periodical and event-triggered reporting.

For both reporting types, the measurements are performed as described in Section 5.5.3.1:

4>	if the measRSSI-ReportConfig is configured in the associated reportConfig:

5>	perform the RSSI and channel occupancy measurements on the frequency indicated in the associated measObject;

### 2.1.1 	Periodical RSSI/CO measurement reporting

In LTE, the field description for measRSSI-ReportConfig is as follows (TS 36.331, V15.8.0):

measRSSI-ReportConfig If this field is present, the UE shall perform measurement reporting for RSSI and channel occupancy and ignore the triggerQuantity, reportQuantity and maxReportCells fields.. E-UTRAN only sets this field to true when setting triggerType to periodical and purpose to reportStrongestCells.

LTE only supports periodical reporting, since the triggerType is supposed to be set to periodical when RSSI/CO measurement reporting is configured.

LTE only supports periodical reporting, since the triggerType is supposed to be set to periodical when RSSI/CO measurement reporting is configured.

Furthermore, RSSI/CO measurement reports are configured separately from other cell measurements, since the UE shall ignore the triggerQuantity, reportQuantity and maxReportCells fields.

RSSI/CO measurement reports are configured separately from other cell measurements, since the UE shall ignore the triggerQuantity, reportQuantity and maxReportCells fields.

If we want to follow LAA as a baseline, similar restrictions need to be included in the field description for periodical reporting.

Periodical reporting configuration for NR Rel-16:

PeriodicalReportConfig ::=                  SEQUENCE {

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                    MeasReportQuantity                                             OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

useWhiteCellList                            BOOLEAN,

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          BT-NameListConfig-r16                                          OPTIONAL,   -- Need R

includeWLAN-Meas-r16                        WLAN-NameListConfig-r16                                        OPTIONAL,   -- Need R

includeSensor-Meas-r16                      Sensor-NameListConfig-r16                                      OPTIONAL,   -- Need R

ul-DelayValueConfig-r16                     SetupRelease { UL-DelayValueConfig-r16 }                       OPTIONAL    -- Need R

]]

}

measRSSI-ReportConfig If this field is present, the UE shall perform measurement reporting for RSSI and channel occupancy and ignore the rsType and maxReportCells fields. For reportQuantityCell, the UE expects that all values are set to FALSE.

Where

- rsType must be set to SSB or CSI-RS (1 bit)

- reportQuantityCell can configure a BOOLEAN for RSRP, RSRQ, and/or SINR (3 bits)

- maxReportCells can be configured with an INTEGER between 1…8 (3 bits)

As in LTE-LAA periodical RSSI/CO reporting, the UE shall ignore the rsType, reportQuantityCell and maxReportCells for this reportConfigNR when configured with measRSSI-ReportConfig

### 2.1.2 	Correction for measurement report triggering

In contrast to LTE, where only periodical RSSI/CO measurement reporting is supported, NR also supports event-triggered reporting using existing event triggers. The following measurement report triggering procedures in Section 5.5.4.1 therefore need further clarification, as the report should only be sent when measurement reporting is periodical, as highlighted by the red text color.

Furthermore, the reference for the L1 measurement period is missing. In LTE, the L1 measurement period is defined in TS 36.133, Section 8.11, where RSSI measurement duration refers to rmtc-MeasDuration, and RSSI measurement period is the L1 measurement period, which are not identical. We would therefore propose to use “L1 measurement period“ instead of “L1 measurement duration“. Note that “L1 measurement period“ is also used  in Section 5.5.3.2. Without the second change, there could be ambiguity between both specifications.

2>	if the corresponding reportConfig includes a measRSSI-ReportConfig, a reportType set to periodical, and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure as specified in 5.5.5 immediately when RSSI sample values are reported by the physical layer after the first L1 measurement duration period as defined in TS 38.133 [14].

Otherwise, the measurement report would be triggered also for event-triggered measurement reporting, even if none of the reporting conditions was fulfilled.

Clarify that measurement report triggering is only valid for a reportType set to periodical. For event-triggered measurement reporting, the measurement report would otherwise be triggered even if none of the reporting conditions were fulfilled.

Clarify that the report is triggered after the “L1 measurement period” (and not “L1 measurement duration” which in TS 36.133 is defined by the rmtc-MeasDuration).

Include a reference to TS 38.133, where the L1 measurement period is defined.

### 2.1.3 	Configuration of RSSI measurements

For clarification purposes, we propose the following changes for Section 5.5.2.10a on RSSI measurement timing configuration:

The UE shall setup the RSSI measurement timing configuration (RMTC) in accordance with the received rmtc-Periodicity, and if configured with rmtc-SubframeOffset, otherwise rmtc-SubframeOffset is determined by the UE randomly, i.e. the first symbol of each RMTC occasion occurs at first symbol of an SFN and subframe of the PCell meeting the following condition:

SFN mod T = FLOOR(rmtc-SubframeOffset/10);

subframe = rmtc-SubframeOffset mod 10;

with T = rmtc-Periodicity/10;

The length of the RMTC occasion is defined by measDuration. On the configured rmtc-Frequency, the UE shall perform RSSI and channel occupancy measurements within the configured RMTC occasion, where the measurement bandwidth is defined in [TS 38.215, clause 5.1.aa] and the time domain measurement resources are defined by the RMTC occasion.

Simplify the specification text to state that RSSI measurements are performed within the RMTC occasion.

Clarify how the measurement resources in the time domain and frequency domain are derived.

Similarly, in TS 38.331 Section 5.5.1, the specification text in LTE for RSSI measurements was not aligned with the specification is TS 36.133. Even though clearly described in TS 36.133, the statements in TS 38.331 should not contradict with the statements in other specifications.

For NR measurement object(s), the UE measures and reports on the serving cell(s), listed cells and/or detected cells. For inter-RAT measurements object(s) of E-UTRA, the UE measures and reports on listed cells and detected cells and, for RSSI and channel occupancy measurements, the UE measures and reports on any reception on the indicated frequency.

For RSSI measurements, the measurement resources are clearly specified. Stating that they can be “on any reception” contradicts with the agreement that the measurement resources are specified in the rmtc-Config. It is acceptable if different parts have different level of detail, but in order to avoid any ambiguity, different parts in the same/different specifications should be consistent/aligned.

“RSSI measurements and reporting on any reception” does not correctly reflect the agreement that RSSI measurements and reporting are performed according to the rmtc-Config in MeasObjectNR.

Therefore, the above part should be changed as follows:

For NR measurement object(s), the UE measures and reports on the serving cell(s), listed cells and/or detected cells. For inter-RAT measurements object(s) of E-UTRA, the UE measures and reports on listed cells and detected cells and, for RSSI and channel occupancy measurements, the UE measures and reports on any reception on the configured RSSI measurement resources on the indicated frequency.

To avoid contradicting parts in the RRC spec, clarify that RSSI measurements and reports are not “on any reception”, but rather “on the configured RSSI measurement resources”.

## 2.2		Modifications for SearchSpace

There are new fields applicable for both UE-specific search space (USS) and common search space (CSS):

- searchSpaceGroupIdList-r16

- freqMonitorLocations-r16

RAN1#100e

Agreement:

For search space switching, limit the switching to USS and Type-3 CSS.

As specified in TS 38.213, clause 10.1, the above search spaces are defined in the UE-specific PDCCH-Config as follows:

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, or TPC-SRS-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, or CS-RNTI(s).

However, the current implementation only covers the UE-specifc search space (USS):

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

-- irrelevant parts removed

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

-- irrelevant parts removed

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

searchSpaceGroupIdList-r16            SEQUENCE (SIZE (1.. 2)) OF INTEGER (0..1)               OPTIONAL,    -- Need R

freqMonitorLocations-r16              BIT STRING (SIZE (5)) )                                 OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup

}

Due to missing extension markers

- for the SearchSpace IE as such and also

- for the common search space configuration,

neither of them can be extended.

Instead, searchSpacesToAddModList-r16 can be used to configure SearchSpace-v16xy, which can provide additional parameters associated with the SearchSpace. We therefore propose to move the searchSpaceGroupIdList and freqMonitorLocations fields from USS in SearchSpace (without suffix) to SearchSpace-v16xy as follows:

PDCCH-Config information element

-- ASN1START

-- TAG-PDCCH-CONFIG-START

PDCCH-Config ::=                    SEQUENCE {

controlResourceSetToAddModList      SEQUENCE(SIZE (1..3)) OF ControlResourceSet                 OPTIONAL,   -- Need N

controlResourceSetToReleaseList     SEQUENCE(SIZE (1..3)) OF ControlResourceSetId               OPTIONAL,   -- Need N

searchSpacesToAddModList            SEQUENCE(SIZE (1..10)) OF SearchSpace                       OPTIONAL,   -- Need N

searchSpacesToReleaseList           SEQUENCE(SIZE (1..10)) OF SearchSpaceId                     OPTIONAL,   -- Need N

downlinkPreemption                  SetupRelease { DownlinkPreemption }                         OPTIONAL,   -- Need M

tpc-PUSCH                           SetupRelease { PUSCH-TPC-CommandConfig }                    OPTIONAL,   -- Need M

tpc-PUCCH                           SetupRelease { PUCCH-TPC-CommandConfig }                    OPTIONAL,   -- Need M

tpc-SRS                             SetupRelease { SRS-TPC-CommandConfig}                       OPTIONAL,   -- Need M

...,

[[

controlResourceSetToAddModList-r16  SEQUENCE (SIZE (1..5)) OF ControlResourceSet                 OPTIONAL,   -- Need N

controlResourceSetToReleaseList-r16 SEQUENCE (SIZE (1..5)) OF ControlResourceSetId-r16           OPTIONAL,   -- Need N

searchSpacesToAddModList-r16        SEQUENCE(SIZE (1..10)) OF SearchSpace-v16xy                 OPTIONAL,   -- Need N

searchSpaceSwitchingTimer-r16       INTEGER (1..ffsValue)                                       OPTIONAL,    -- Need R

searchSpaceSwitchingGroupList-r16   SEQUENCE(SIZE (1..ffsValue)) OF SearchSpaceSwitchingGroup-r16 OPTIONAL, -- Need R

uplinkCancellation-r16              SetupRelease { UplinkCancellation-r16 }                    		OPTIONAL,    -- Need M

monitoringCapabilityConfig-r16      ENUMERATED { r15monitoringcapability,r16monitoringcapability } 	OPTIONAL

]]

}

SearchSpaceSwitchingGroup-r16 ::=       SEQUENCE(SIZE (1..16)) OF ServCellIndex

-- TAG-PDCCH-CONFIG-STOP

-- ASN1STOP

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

-- irrelevant parts removed

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

-- irrelevant parts removed

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

searchSpaceGroupIdList-r16            SEQUENCE (SIZE (1.. 2)) OF INTEGER (0..1)               OPTIONAL,    -- Need R

freqMonitorLocations-r16              BIT STRING (SIZE (5)) )                                 OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup

}

SearchSpace-v16xy ::=                   SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId-r16                ControlResourceSetId-r16                                    OPTIONAL,   -- Cond SetupOnly

searchSpaceGroupIdList-r16            	SEQUENCE (SIZE (1..2)) OF SearchSpaceGroupId				OPTIONAL,    -- Need R

freqMonitorLocations-r16              	BIT STRING (SIZE (5)) )                                 	OPTIONAL     -- Need R

searchSpaceType-r16                     CHOICE {

common-r16                              SEQUENCE {

dci-Format2-4-r16                       SEQUENCE {

nrofCandidates-CI-r16                   SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

},

dci-Format2-5-v16xy                     SEQUENCE {

nrofCandidates-IAB-r16                  SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

},

dci-Format2-6-r16                       SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...

},

mt-Specific-v16xy                           SEQUENCE {

dci-Formats-r16                             ENUMERATED {formats2-0-And-2-5},

...

}

}                                                                                                   OPTIONAL    -- Cond Setup2

}

SearchSpaceGroupId	::= INTEGER (0..1)

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

Move the fields searchSpaceGroupIdList and freqMonitorLocations from USS in SearchSpace (without suffix) to SearchSpace-v16xy.

## 2.3 	Reducing unnecessary overhead for signalling of SSB-Position-QCL-Relationship

### 2.3.1	Overhead due to extension of IntraFreqNeighCellInfo / InterFreqNeighCellInfo in SIB3 and SIB4, respectively

The following code shows the signalling costs when IntraFreqNeighCellInfo is extended.

IntraFreqNeighCellList ::=          SEQUENCE (SIZE (1..maxCellIntra)) OF IntraFreqNeighCellInfo

IntraFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,		-- 10 bits

q-OffsetCell                        Q-OffsetRange,    -- 5 bits

q-RxLevMinOffsetCell                INTEGER (1..8)                 OPTIONAL,   -- Need R   1 bit

q-RxLevMinOffsetCellSUL             INTEGER (1..8)                 OPTIONAL,   -- Need R   1 bit

q-QualMinOffsetCell                 INTEGER (1..8)                 OPTIONAL,   -- Need R   1 bit

...,

[[  2-3 bytes

ssb-Position-QCL-r16           SSB-Position-QCL-Relationship-r16           OPTIONAL    -- Need R      -- 2 + 1 bits

]]

}

As previously indicated, the extension of the above lists has significant signaling costs. If there is no need to signal any of the parameters related to cell offset, the following will result in additional signaling overhead.

  - adding extensions after “. . .” costs 2-3 bytes.

  - signaling of the mandatory cell individual offset costs 5 bits

  - signaling of the optionality bits for the three other optional fields costs 3 bits

There will be up to 4 bytes of unnecessary signaling FOR EACH CELL in the neighboring cell list in system information compared to defining new lists in order to signal 2-bit SSB-Position-QCL-Relationship (compared to 24-32 bits of overhead). And this is even multiplied by the number of cells.

FOR EACH CELL in the neighboring cell list in SIB3/SIB4, there is 24-32 bits of overhead to signal the 2-bit SSB-Position-QCL-Relationship.

As a general RRC convention for SIB extensions, it is not recommended to extend information elements within a list to avoid overhead that comes along with each single list element.

Therefore, we propose to define separate lists for signal of cell-specific SSB-PositionQCL-Relationships.

Define a separate list in SIB3 and SIB4 for signaling of the cell-specific SSB-PositionQCL-Relationships instead of integrating it to the neighbor cell list to avoid unnecessary broadcasting overhead.

Change proposal for SIB3:

[[

intraFreqWhiteCellList-r16          IntraFreqWhiteCellList-r16   	OPTIONAL,    -- Need R

ssb-PositionQCL-CellList-r16       SSB-PositionQCL-CellList-r16     OPTIONAL     -- Need R

]]

SSB-PositionQCL-CellList-r16 ::=    SEQUENCE (SIZE (1..maxCellQCL)) OF SSB-PositionQCL-CellInfo-r16

SSB-PositionQCL-CellInfo-r16 ::=   		SEQUENCE {

physCellId-r16                  	PhysCellId,

ssb-PositionQCL-Cell-r16        	SSB-PositionQCL-Relationship-r16

}

Note that we use the IE name SSB-PositionQCL-CellInfo based on the IntraFreqNeighCellInfo.

Change proposal for SIB4:

[[

interFreqWhiteCellList-r16          InterFreqWhiteCellList-r16                  OPTIONAL,   -- Need R

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relationship-r16            OPTIONAL,   -- Need R

ssb-PositionQCL-CellList-r16        SSB-PositionQCL-CellList-r16     			OPTIONAL    -- Need R

]]

It is typically not necessary to define a cell-specific offset. It is used only for heterogenous network deployments. 
SSB-Position-QCL-Relationship itself only requires 2 bits.

### 2.3.2	Reducing unnecessary overhead for signalling of cell-specific SSB-PositionQCL-Relationships in measObjectNR

Current implementation in measObjectNR:

[[

rmtc-Config-r16                     	SetupRelease {RMTC-Config-r16}                                  OPTIONAL,   -- Need M

ssb-PositionQCL-Common-r16          	SSB-PositionQCL-Relationship-r16                                OPTIONAL,   -- Need M

ssb-PositionQCL-CellsToAddModList-r16   SSB-PositionQCL-CellsToAddModList-r16                       	OPTIONAL,   -- Need N

ssb-PositionQCL-CellsToRemoveList-r16   PCI-List                                                    	OPTIONAL    -- Need N

]]

Since SSB-PositionQCL-Relationship is related to SSB-ConfigMobility, which is a sub-field within MeasObjectNR, it should be included there. This also has a similar structure as we propose for the corresponding MeasObjectNR in LTE, see text proposals in the annex below.

Furthermore, we propose to use a SetupRelease command to add a modified list or remove it completely rather than supporting adding/releasing single list elements as indicating one Q value is only 2 bits, while indicating the physCellId (PCI), which is defined as INTEGER (0..1007), costs 10 bits. Indicating the removal of a list entry would thus also cost 10 bits.

There is basically no gain in signalling overhead with an AddMod/Remove structure, since adding/modifying a Q value for a cell costs 12 bits, while removing the config for a cell costs 10 bits.

For connected mode mobility, we propose the following:

In order to provide frequency specific Q values in the MeasObjectNR, include ssb-PositionQCL-Common in SSB-ConfigMobility.

In order to provide cell specific Q values in MeasObjectNR, include a SetupRelease structure for SSB-QCL-RelationCellList in SSB-ConfigMobility.

The same applies also for LTE.

The information about the frequency specific Q value, ssb-PositionQCL-CommonNR, should be included in the existing RS-ConfigSSB-NR-r15 IE as ssb-PositionQCL-CommonNR is .

In order to provide frequency specific Q values in the MeasObjectNR for E-UTRAN, include ssb-PositionQCL-CommonNR in the existing IE RS-ConfigSSB-NR-r15.

Similar as explained above, using 10-bit PCI information to remove the 2-bit SSB-Position-QCL-RelationshipNR does not save signaling overhead. Therefore, it is better to use a setup/release structure also for LTE.

In order to provide cell specific Q values in MeasObjectNR, include a SetupRelease structure for SSB-QCL-RelationCellListNR in SSB-ConfigMobility.

Proposed implementation within measObjectNR:

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                           OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                       OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                 OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                           OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

blackCellsToRemoveList              PCI-RangeIndexList                                              OPTIONAL,   -- Need N

blackCellsToAddModList              SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

whiteCellsToRemoveList              PCI-RangeIndexList                                              OPTIONAL,   -- Need N

whiteCellsToAddModList              SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

...,

[[

freqBandIndicatorNR                 FreqBandIndicatorNR                                             OPTIONAL,   -- Need R

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

]],

[[

smtc3list-r16                     SSB-MTC3List-r16                                                  OPTIONAL,   -- Cond FFS

rmtc-Config-r16                     SetupRelease {RMTC-Config-r16}                                  OPTIONAL,   -- Need M

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relationship-r16                                OPTIONAL,   -- Need M

ssb-PositionQCL-CellsToAddModList-r16   SSB-PositionQCL-CellsToAddModList-r16                       OPTIONAL,   -- Need N

ssb-PositionQCL-CellsToRemoveList-r16   PCI-List                                                    OPTIONAL,   -- Need N

t312-r16                            SetupRelease { T312-r16 }                                       OPTIONAL    -- Need M

]]

}

SSB-MTC3List-r16::=               SEQUENCE (SIZE(1..4)) OF SSB-MTC3-r16

T312-r16 ::=                        ENUMERATED { ms0, ms50, ms100, ms200, ms300, ms400, ms500, ms1000}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                           SetupRelease { SSB-ToMeasure }                              OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             	BOOLEAN,

ss-RSSI-Measurement                     SS-RSSI-Measurement                                     	OPTIONAL,   -- Need M

...,

[[

ssb-PostionQCL-Common-r16          		SSB-Position-QCL-Relationship-r16                           OPTIONAL,   -- Need R

ssb-PositionQCL-CellList-r16        	SetupRelease { SSB-PositionQCL-CellList-r16 }               OPTIONAL    -- Need M

]]

}

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	LTE only supports periodical reporting, since the triggerType is supposed to be set to periodical when RSSI/CO measurement reporting is configured.

Observation 2	RSSI/CO measurement reports are configured separately from other cell measurements, since the UE shall ignore the triggerQuantity, reportQuantity and maxReportCells fields.

Observation 3	“RSSI measurements and reporting on any reception” does not correctly reflect the agreement that RSSI measurements and reporting are performed according to the rmtc-Config in MeasObjectNR.

Observation 4	FOR EACH CELL in the neighboring cell list in SIB3/SIB4, there is 24-32 bits of overhead to signal the 2-bit SSB-Position-QCL-Relationship.

Observation 5	There is basically no gain in signalling overhead with an AddMod/Remove structure, since adding/modifying a Q value for a cell costs 12 bits, while removing the config for a cell costs 10 bits.

Based on the discussion in the previous sections we propose the following:

Proposal 1	As in LTE-LAA periodical RSSI/CO reporting, the UE shall ignore the rsType, reportQuantityCell and maxReportCells for this reportConfigNR when configured with measRSSI-ReportConfig

Proposal 2	Clarify that measurement report triggering is only valid for a reportType set to periodical. For event-triggered measurement reporting, the measurement report would otherwise be triggered even if none of the reporting conditions were fulfilled.

Proposal 3	Clarify that the report is triggered after the “L1 measurement period” (and not “L1 measurement duration” which in TS 36.133 is defined by the rmtc-MeasDuration).

Proposal 4	Include a reference to TS 38.133, where the L1 measurement period is defined.

Proposal 5	To avoid contradicting parts in the RRC spec, clarify that RSSI measurements and reports are not “on any reception”, but rather “on the configured RSSI measurement resources”.

Proposal 6	Move the fields searchSpaceGroupIdList and freqMonitorLocations from USS in SearchSpace (without suffix) to SearchSpace-v16xy.

Proposal 7	Define a separate list in SIB3 and SIB4 for signaling of the cell-specific SSB-PositionQCL-Relationships instead of integrating it to the neighbor cell list to avoid unnecessary broadcasting overhead.

Proposal 8	In order to provide frequency specific Q values in the MeasObjectNR, include ssb-PositionQCL-Common in SSB-ConfigMobility.

Proposal 9	In order to provide cell specific Q values in MeasObjectNR, include a SetupRelease structure for SSB-QCL-RelationCellList in SSB-ConfigMobility.

Proposal 10	In order to provide frequency specific Q values in the MeasObjectNR for E-UTRAN, include ssb-PositionQCL-CommonNR in the existing IE RS-ConfigSSB-NR-r15.

Proposal 11	In order to provide cell specific Q values in MeasObjectNR, include a SetupRelease structure for SSB-QCL-RelationCellListNR in SSB-ConfigMobility.

# 4	References

RP-190706, “Revised WID on NR-based Access to Unlicensed Spectrum”, Qualcomm Inc., RAN#83, Shenzhen, China, 18-21 March, 201

# 5 	Annex

## 2.5 Text proposals for TS 36.331 (LTE)

Start of change proposals for MeasObjectNR in TS 36.331, clause 6.3.5

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for inter-RAT NR neighbouring cells.

MeasObjectNR information element

-- ASN1START

MeasObjectNR-r15 ::=				SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

rs-ConfigSSB-r15					RS-ConfigSSB-NR-r15,

threshRS-Index-r15					ThresholdListNR-r15				OPTIONAL,		-- Need OR

maxRS-IndexCellQual-r15				MaxRS-IndexCellQualNR-r15		OPTIONAL,		-- Need OR

offsetFreq-r15						Q-OffsetRangeInterRAT			DEFAULT 0,

blackCellsToRemoveList-r15			CellIndexList					OPTIONAL,		-- Need ON

blackCellsToAddModList-r15			CellsToAddModListNR-r15			OPTIONAL,		-- Need ON

quantityConfigSet-r15				INTEGER (1.. maxQuantSetsNR-r15),

cellsForWhichToReportSFTD-r15		SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellIdNR-r15	OPTIONAL,	-- Need OR

...,

[[	cellForWhichToReportCGI-r15			PhysCellIdNR-r15				OPTIONAL,	-- Need ON

deriveSSB-IndexFromCell-r15			BOOLEAN							OPTIONAL,	-- Need ON

ss-RSSI-Measurement-r15				SS-RSSI-Measurement-r15			OPTIONAL,	-- Need ON

bandNR-r15					CHOICE {

release						NULL,

setup						FreqBandIndicatorNR-r15

}																OPTIONAL	-- Need ON

]]

}

RS-ConfigSSB-NR-r15 ::=			SEQUENCE {

measTimingConfig-r15			MTC-SSB-NR-r15,

subcarrierSpacingSSB-r15		ENUMERATED {kHz15, kHz30, kHz120, kHz240},

...,

[[	ssb-ToMeasure-r15			CHOICE {

release						NULL,

setup						SSB-ToMeasure-r15

}											OPTIONAL	-- Need ON

]],

[[

ssb-PositionQCL-CommonNR-r16		SSB-Position-QCL-Relationship-NR-r16  OPTIONAL,	-- Need OR

ssb-PositionQCL-CellList-r16    	CHOICE {

release								NULL,

setup								SSB-PositionQCL-CellList-r16

}																		OPTIONAL	-- Need ON

]]

}

CellsToAddModListNR-r15 ::=			SEQUENCE (SIZE (1..maxCellMeas)) OF CellsToAddModNR-r15

CellsToAddModNR-r15 ::=			SEQUENCE {

cellIndex-r15					INTEGER (1..maxCellMeas),

physCellId-r15					PhysCellIdNR-r15

}

-- ASN1STOP

End of change proposals for MeasObjectNR

Start of change proposals for SystemInformationBlockType24 in TS 36.331, clause 6.3.1

#### –	SystemInformationBlockType24

The IE SystemInformationBlockType24 contains information relevant only for inter-RAT cell re-selection i.e. information about NR frequencies and NR neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

SystemInformationBlockType24 information element

-- ASN1START

SystemInformationBlockType24-r15 ::=	SEQUENCE {

carrierFreqListNR-r15				CarrierFreqListNR-r15				OPTIONAL, 		-- Need OR

t-ReselectionNR-r15					T-Reselection,

t-ReselectionNR-SF-r15				SpeedStateScaleFactors				OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...

}

CarrierFreqListNR-r15 ::=		SEQUENCE (SIZE (1..maxFreq)) OF CarrierFreqNR-r15

CarrierFreqNR-r15 ::=				SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

multiBandInfoList-r15				MultiFrequencyBandListNR-r15		OPTIONAL,	-- Need OR

multiBandInfoListSUL-r15			MultiFrequencyBandListNR-r15		OPTIONAL,	-- Need OR

measTimingConfig-r15				MTC-SSB-NR-r15						OPTIONAL,	-- Need OR

subcarrierSpacingSSB-r15			ENUMERATED {kHz15, kHz30, kHz120, kHz240},

ss-RSSI-Measurement-r15				SS-RSSI-Measurement-r15		OPTIONAL, 		-- Cond RSRQ2

cellReselectionPriority-r15			CellReselectionPriority		OPTIONAL,		-- Need OP

cellReselectionSubPriority-r15		CellReselectionSubPriority-r13	OPTIONAL,	-- Need OR

threshX-High-r15					ReselectionThreshold,

threshX-Low-r15						ReselectionThreshold,

threshX-Q-r15						SEQUENCE {

threshX-HighQ-r15				ReselectionThresholdQ-r9,

threshX-LowQ-r15				ReselectionThresholdQ-r9

}																OPTIONAL,	-- Cond RSRQ

q-RxLevMin-r15						INTEGER (-70..-22),

q-RxLevMinSUL-r15					INTEGER (-70..-22)				OPTIONAL, 		-- Need OR

p-MaxNR-r15							P-MaxNR-r15,

ns-PmaxListNR-r15					NS-PmaxListNR-r15					OPTIONAL,	-- Need OR

q-QualMin-r15						INTEGER (-43..-12)				OPTIONAL,		-- Need OP

deriveSSB-IndexFromCell-r15			BOOLEAN,

maxRS-IndexCellQual-r15				MaxRS-IndexCellQualNR-r15		OPTIONAL,		-- Need OR

threshRS-Index-r15					ThresholdListNR-r15				OPTIONAL,		-- Need OR

...,

[[	multiBandNsPmaxListNR-v1550		MultiBandNsPmaxListNR-1-v1550	OPTIONAL,	-- Need OR

multiBandNsPmaxListNR-SUL-v1550	MultiBandNsPmaxListNR-v1550		OPTIONAL,	-- Need OR

ssb-ToMeasure-r15				SSB-ToMeasure-r15				OPTIONAL		-- Need OR

]],

[[

ssb-PositionQCL-CommonNR-r16		SSB-PositionQCL-RelationshipNR-r16	OPTIONAL,	-- Need OR

ssb-PositionQCL-CellListNR-r16		SSB-PositionQCL-CellListNR-r16		OPTIONAL	-- Need OR

]]

}

MultiBandNsPmaxListNR-1-v1550	::=	SEQUENCE (SIZE (1.. maxMultiBandsNR-1-r15)) OF NS-PmaxListNR-r15

MultiBandNsPmaxListNR-v1550	::=	SEQUENCE (SIZE (1.. maxMultiBandsNR-r15)) OF NS-PmaxListNR-r15

-- ASN1STOP

End of change proposals for SystemInformationBlockType24 in TS 36.331

New IE SSB-PositionQCL-RelationshipNR in TS 36.331

#### –	SSB-PositionQCL-RelationshipNR

The IE SSB-PositionQCL-RelationshipNR is used to indicate the QCL relationship between SSB positions on the indicated frequency or cell (see TS 38.213 [88], clause 4.1). Value n1 corresponds to 1, value n2 corresponds to 2 and so on.

-- ASN1START

SSB-PositionQCL-RelationshipNR-r16 ::=		ENUMERATED {n1,n2,n4,n8}

SSB-PositionQCL-CellListNR-r16 ::=  		SEQUENCE (SIZE (1..maxCellMeas)) OF SSB-PositionQCL-CellInfoNR-r16

SSB-PositionQCL-CellInfoNR-r16 ::=   		SEQUENCE {

physCellId-r16          	        		PhysCellIdNR-r15,

ssb-PositionQCL-CellNR-r16        			SSB-PositionQCL-RelationshipNR-r16

}

-- ASN1STOP

End of change proposal for IE SSB-PositionQCL-RelationshipNR in TS 36.331