3GPP TSG-RAN WG2 #109e-bis	R2-2003073

Electronic meeting, 20th April - 30th April 2020

Agenda Item:	6.12.3

Source:	Ericsson

Title:	Open issues of L2 measurements

Document for:	Discussion, Decision

# Introduction

During the RAN2#109e meeting, all the functional open issues associated to L2 measurements were sorted out and the associated agreements were implemented in 38.314 specification. In this contribution, we discuss some of the open issues (no new functionality) associated to the L2 measurements in the current specifications.

# Discussion

## Delay measurements in EN-DC

During the RAN2#109e meeting it was agreed that the MN can configure the delay measurements for MN terminated bearers and the SN can configure the delay measurements for SN terminated bearers in EN-DC case.

5	For EN-DC UL D1 delay measurement configuration for non-split bearer,

-	D1 measurement of MN terminated bearer(including non-split bearer) can be configured by MN,

-	D1 measurement of SN terminated bearer(including non-split bearer) can be configured by SN via RRC message (SRB3 or SRB1).

-	For the SN terminated bearers, it is the SN to configure and calculate the UL/DL delay.

So, the agreements were taken for all the direct bearers. However, we believe there are some issues with the above agreements for the SN terminated MCG bearers.

LTE RLC

LTE MAC + PHY

NR MAC + PHY

NR RLC

NR PDCP

LTE/NR PDCP

Figure 1 : Different direct bearer related configurations, MN terminated MCG bearer (blue), SN terminated MCG bearer (Orange) and SN terminated SCG bearer (black)

In the case of SN terminated MCG bearer (as shown in orange color in the figure above), the NR PDCP is used and the EUTRA related RLC, MAC and PHY is used. The UE receives the packets from the MCG in the DL. In the UL, when the UE needs to transmit the packet associated to a SN terminated MCG bearer, it sends the scheduling request to the MN. So, the PDCP queueing delay measurement, D1, is associated to the UE implementation on the MN related chipset (assuming dual chip set case) and the MN related scheduling grant.

In such a scenario, if we are to follow the current agreements, then the SN will configure the D1 measurements for the MCG bearer which includes MCG related aspects. The SN CU-CP receives the D1 measurement report associated to a DRB whose measurements have nothing to do with the SCG at all. This is very strange outcome in our opinion. If it is identified that such a D1 measurement is large, then either the UE’s implementation needs to be improved or faster scheduling grant needs to be provided. Thus the nodes that can impact this D1 measurements are the UE and the MN’s DU. Therefore, we do not think that reporting the D1 measurement associated to SN terminated MCG bearer should be reported to SN.

Therefore, we propose to follow the principle wherein the node associated to the scheduling request that impacts the D1 measurement to be the one that receives the D1 measurement report. That implies the following:

- D1 measurement for MN terminated MCG bearer is configured by and reported to MN.

- D1 measurement for SN terminated MCG bearer is configured by and reported to MN.

- D1 measurement for SN terminated SCG bearer is configured by and reported to SN.

- D1 measurement for split bearers to be discussed in rel-17.

Based on the above, we propose the following.

- D1 measurement for MN terminated MCG bearer is configured by and reported to MN.

- D1 measurement for SN terminated MCG bearer is configured by and reported to MN.

- D1 measurement for SN terminated SCG bearer is configured by and reported to SN.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	D1 measurement for MN terminated MCG bearer is configured by and reported to MN.

Proposal 2	D1 measurement for SN terminated MCG bearer is configured by and reported to MN.

Proposal 3	D1 measurement for SN terminated SCG bearer is configured by and reported to SN.

# References