3GPP TSG-RAN WG2 #109bis-e	R2-2003115

Electronic meeting, 20 – 30 April, 2020

Agenda item:		6.20.2.1	Open / ongoing proposals

Source:		NEC

Title:		Further details on Secondary DRX group

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#108, the basic concept of secondary DRX group was agreed as below and the LS was sent to RAN1 and RAN4 for asking its potential impact [1]. In the LS, it is further clarified that the length of Long DRX cycle and Short DRX cycle, if configured, are common for both FR1 and FR2.

Conditional on R1 acceptance: A separate drx-InactivityTimer and drx-onDurationTimer can be configured for the secondary DRX group. R2 understands that this has zero or almost zero impact in R1 and R4 The combination of cross-carrier scheduling and secondary DRX group is not supported FFS if timers for FR2 DRX configuration are shorter than timers for FR1 DRX configuration. The intention is to apply secondary DRX configuration to FR2 and existing DRX configuration to FR1 We send an LS to R1, ask whether there is impact, and if so whether the impact is acceptable.

Although RAN1 had discussed the RAN2 LS in RAN1#100e, there seemed to be no consensus on impact or feasibility for some functions to be used with secondary DRX (e.g. WUS, dormancy SCell). Accordingly, some contributions were submitted to RAN#87e but not treated [2,3]. In this contribution, we discuss more details on the DRX handling in two groups based on the previous agreements. In addition, we also discuss remaining issues and provide our views to complete the work in Rel-16.

# 2. Discussion

2.1	DRX handling

As agreed in RAN2#108, drx-InactivityTimer and drx-onDurationTimer are configured for each DRX group. This can be understood that the Active Time is independent in two DRX groups (but start timings of On-duration are aligned), as shown in the figure of the outgoing LS [1].

Observation 1. RAN2 implicitly agreed the independent Active Time in two DRX groups.

Fig.1: Example of timer handling in two DRX groups [1]

In the last RAN2#109e, there were some contributions proposing the independent DRX state (short or long) handling, i.e. drx-ShortCycleTimer can be handled per DRX group [4][5]. The motivation is further power saving according to the traffic in each DRX group. This is aligned with our original proposal, where independent DRX handling is applied [6]. Even if the independent DRX state handling is applied, when the network want to use both DRX groups for one service, there is no issue anyway. The drawback of different DRX state handling is causing additional complexity due to one more timer. However, the drx-ShortCycleTimer is (re)started once per DRX cycle at most, so the additional complexity would be just marginal.

One issue is whether to configure separate drx-ShortCycleTimer for the second DRX cycle. Differently from drx-InactivityTimer and drx-onDurationTimer which determines the Active Time dynamically, short or long can be determined relatively longer period. Thus, we consider the length of drx-ShortCycleTimer can be the same value for both groups just like short and long DRX cycle.

Proposal 1: DRX state (short or long) is determined per DRX group by handling drx-ShortCycleTimer independently.

Proposal 2: Similar to the length of long DRX cycle and the length of short DRX cycle, drx-ShortCycleTimer is common for two DRX groups (if configured), i.e. not to configure separate value.

2.2	Other remaining issues

There are some open issues in relation to the secondary DRX group.

WUS

Firstly, it is not yet decided whether the WUS can be configured with the secondary DRX group. This was discussed in the past Email discussion [7], but there was no consensus. Some companies consider and proposal in [4] says that the simple solution without additional enhancement (due to support of secondary DRX group) can be introduced, while some other companies consider the combination of secondary DRX group and the WUS seems not necessary as both can achieve good power saving.

On top of the past RAN2 discussions and the LS, RAN1 also discussed this issue as the WUS is more or less L1 oriented function. However, there was no consensus, either.

Considering the current situation and this is the work under TEI, what RAN2 can do at most is to introduce a simple solution with no optimization. Regarding “simple” solution, we consider the simplest way is to reuse the existing mechanism, i.e. the WUS is configured on PCell and the UE does not monitor the WUS during Active Time. Then, the action upon receiving the WUS indication can be extended to the secondary DRX group.

Based on the observations so far, we consider the feasible way forward can be either of the following two options:

- Option 1: Reuse the existing WUS handling and extend it to secondary DRX group

  - WUS is configured on PCell only

  - UE does not monitor the WUS during Active Time on PCell (regardless whether Active Time or not in secondary DRX group)

  - When the WUS is received during non-Active Time on PCell, the UE applies it to all serving cells regardless of DRX group.

  - No more enhancement is supported.

- Option 2: The WUS is not configured with the secondary DRX group in Rel-16.

Proposal 3: For the combination of the WUS and the secondary DRX group, RAN2 to agree either Option 1 (reuse and extend existing WUS handling) or Option 2 (not support).

Dormant BWP

Another issue is the support of SCell dormancy (i.e. dormant BWP configuration) in the secondary DRX group. This was also discussed in RAN1 and brought up to RAN#87e [2,3]. However, from RAN2 point of view, we do not see any specific issue in the combination of SCell dormancy and the secondary DRX.

Proposal 4: RAN2 to agree that the dormant BWP can be configured with the secondary DRX group.

# 3. Conclusion

In this contribution we discussed further details on the secondary DRX group and made the following proposals.

Proposal 1: DRX state (short or long) is determined per DRX group by handling drx-ShortCycleTimer independently.

Proposal 2: Similar to the length of long DRX cycle and the length of short DRX cycle, drx-ShortCycleTimer is common for two DRX groups (if configured), i.e. not to configure separate value.

Proposal 3: For the combination of the WUS and the secondary DRX group, RAN2 to agree either Option 1 (reuse and extend existing WUS handling) or Option 2 (not support).

Proposal 4: RAN2 to agree that the dormant BWP can be configured with the secondary DRX group.

# References

[1] R2-1916597, “LS on secondary DRX group”, RAN2

[2] RP-200130, “Views on NR TEI for secondary DRX group”, vivo

[3] RP-200324, “Views on two DRX groups in the same cell group”, Apple Inc.

[4] R2-2000345, “Introduction of secondary DRX group”, Ericsson, Qualcomm, Samsung, Deutsche Telekom, Verizon

[5] R2-2000407, “Further considerations on secondary DRX group”, OPPO

[6] R2-1916187, “General aspects on cDRX enhancement”, NEC

[7] R2-1915292, “Email report [107bis#49][NR TEI16] cDRX enhancement for CA”, Ericsson (Rapporteur)

[8] R2-2001433, “Supporting WUS in multiple DRX groups”, Samsung