3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003158

Elbonia, 20 – 30 April 2020

Agenda item:	6.12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Resolving MDT Stage 2 open issues

WID/SID:	NR_SON_MDT-Core - Release 16

Document for:	Discussion and Decision

# Introduction

This contribution addresses known MDT stage 2 open issues, i.e. FFSs, Editor’s Notes listed in the latest TS37.320 v16.0.0, as well as issues that remain to be clarified jointly with issues discovered as class 3, in WI-specific items that need to be resolved to complete the NR MDT functionality.

# 2	Discussion

## 2.1	FFS

TS37.320 list two FFS points:

- Section 5.1.1.1.1:

Editor’s note: For the serving cell downlink pilot strength measurements logging configuration is FFS.

- Section 5.1.2.3:

For NR, the MDT configuration received by signalling based trace messages for a specific UE will propagate during intra-PLMN handover, and may propagate during inter-PLMN handover if the Signalling Based MDT PLMN List is available and includes the target PLMN. This behaviour applies also for MDT configuration that includes area scope, regardless of whether the source or target cell is part of the configured area scope. [FFS]

The FFS in 1. has been raised “to decide if serving cell DL pilot is always measured whenever the UE has periodical event configured”. (R2-2002006). Based on the RRC CR introducing MDT support, 38.331 v16.0.0 in 5.5a.3 Measurement logging section, defines the UE does always follow the following procedure for measurement logging (including periodical based event):

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

Observation 1: RRC (38.331 v16.0.0 in 5.5a.3) resolves the FFS and clarifies the UE always logs serving cell for periodical event.

Proposal 1: Remove Editor’s note and FFS within: “For the serving cell downlink pilot strength measurements logging configuration is FFS.

The FFS in 2. concerns MDT configurations handling across network interfaces. There are 2 RAN3 BL CRs and latest 32.422 that cover MDT signalling in handovers:

- 32.422 in section 4.4 on Handling the MDT trace sessions at handover, has not been updated yet for 5GC, but the LTE principle was aligned with the principle stated in 37.320:

- For LTE, the MDT configuration received by signalling based trace messages for a specific UE will propagate during intra-PLMN handover, and may propagate during inter-PLMN handover if the Signalling Based MDT PLMN List is available and includes the target PLMN. This behaviour applies also for MDT configuration that includes area scope, regardless of whether the source or target cell is part of the configured area scope.

- When the management based MDT activation is sent to eNodeB/RNC, eNodeB/RNC shall check the availability of the Management Based MDT Allowed IE before making the UE selection. In case the Management Based MDT Allowed IE is not available, the eNodeB/RNC shall not select the UE. In case the Management Based MDT Allowed IE is available, the eNodeB/RNC shall verify if the UE’s RPLMN matches the PLMN where TCE resides – Trace Reference PLMN (PLMN portion of the Trace Reference). In case of a mismatch, the eNodeB/RNC shall not select the UE.The eNB/RNC shall forward the received Management Based MDT Allowed IE during X2/Iur based handovers to the target node. The Management Based MDT Allowed IE is stored in the eNB/RNC as part of the UE context. If the user consent information is updated while a UE context is already set up in the eNB, the changed user consent should be taken into account in the next call/session setup.

Similarly:

- R3-201478 CR to 38.413, reads:

If the Management Based MDT PLMN List IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN shall use it, if supported, to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [11].

- R3-201480 CR to 38.423 reads:

If the Management Based MDT PLMN List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

The source NG-RAN node shall, if supported and available in the UE context, include the Management Based MDT PLMN List IE in the HANDOVER REQUEST message, except if the source NG-RAN node selects a serving PLMN in the target NG-RAN node which is not included in the Management Based MDT PLMN List.

Observation 2: Trace Management (32.422 v16.0.0 in 4.4) and 38.4x3 CRs define MDT Trace configuration propagation in handovers with respect to PLMN (list), i.e. regardless of area scope.

Proposal 2: Remove FFS within Section 5.1.2.3.

## 2.2	Editors’ notes

TS37.320 list six Editor’s notes:

- Section 3.1 and 4.1 cover three, same type of editor’s note, that ensures the LTE definitions apply also for NR MDT framework:

Editor's NOTE: Definition applicability to NR needs checking with SA5.

- Section 5.1.1.1.1:

Editor’s note: For the serving cell downlink pilot strength measurements logging configuration is FFS.

Editor’s note: Trace relevant parameters in NR need SA5 confirmation.

- Section 5.4.2:

Editor’s NOTE: Above agreement for signalling based logged MDT received by the NG-RAN when UE is in RRC_INACTIVE should be confirmed by RAN3.

RAN3 endorsed baseline CRs incorporate two methods for MDT activation Signalling and Management based MDT (e.g. R3-201476 CR to 38.401, R3-201478 CR to 38.413). Similarly, SA5 specification introduced already MDT support in 5GC, and use the same activation methods (32.422) Therefore, it can be understood both: Signalling and Management based MDT will be followed in NR (as it was in LTE) and both methods are supported in NR.

Observation 3: Both: Signalling and Management based MDT Trace are supported in NR.

Proposal 3: Remove Editor’s notes ‘Definition applicability to NR needs checking with SA5’ and Trace relevant parameters in NR need SA5 confirmation in section 3.1 and 4.1.

The MDT context handling (5.1.1.4) defines that  “MDT context is assumed to be released in the RAN nodes when the UE is in IDLE and INACTIVE if corresponding MDT configuration has already been configured to UE.”

Furthermore, 32.422 section 4.5 reads:

In logged MDT (…), no propagation of the MDT configuration is performed.

None of the BL CRs in RAN3 nor SA5  support for signalling based MDT the requirement “when the UE resumes the RRC connection in one new NG-RAN, the last serving NG-RAN can propagate the logged MDT configuration to the new NG-RAN.”

Observation 4: Default requirement for the network has been to not propagate logged MDT configuration.

The assumption that the MDT configuration context should propagate after resuming the UE connection may be seen as contradictive requirement. For simplicity, we believe the configuration continuity can be ensured by signalling based MDT principles and propagation to the entire Signalling Based MDT PLMN List:

Proposal 4: To revise the agreement for signalling based MDT from:

”When the UE resumes the RRC connection in one new NG-RAN, the last serving NG-RAN can propagate the logged MDT configuration to the new NG-RAN.”

To”

”When the UE resumes the RRC connection in one new NG-RAN, the new NG-RAN can configure the MDT configuration for the UE, only if the signalling based logged MDT was received by the NG-RAN. It is not required to propagate the logged MDT configuration.”

Proposal 5: Based on the revised agreement, remove the Editor’s note in 5.4.2.

## 2.3	Terminology alignment

37.320 uses diverse abbreviations for:

- gNB/RAN node/(e/g)NB

- RRC IDLE/INACTIVE/CONNECTED state

- NCGI/Cell Identity (5.1.2.2)

It is proposed to unify the terms and use aligned wording according to:

Proposal 6: Use gNB and NR for NR specific RAN actions.

Proposal 7: Use IDLE/INACTIVE/CONNECTED state (instead of mode)

Proposal 8: Use NR Cell Global Identifier for NR specific cell id reference.

## 2.4	RLFreport clean-up

37.320 section 5.4.1.2 Radio Link Failure report defines NR RLF report content. While the LTE baseline has been followed, it differs in a few aspects:

- TS38.300 (Baseline RAN3 CR in R3-201475) will not define new contents for SON. The RLFreport is the same for MDT and SON use cases. Therefore the sentence: “ In addition, the RLF report may include additional information required for MRO solutions, as specified in TS [TS 38.300].” does not reflect NR approach.

- the “indication per SSB/CSI-RS beams reporting whether it is configured to RLM purpose” has been agreed in RAN2#109e to be a full RLM configuration per SSB/CSI-RS beams

- the detailed location information does not consist of the listed (as in LTE) vectors, i.e.:

-	Latitude, longitude (mandatory);

-	Altitude (conditional on availability);

-	Velocity (conditional on availability);

-	Uncertainty (conditional on availability);

-	Confidence (conditional on availability);

-	Direction (conditional on availability).

38.331 list detailed location information differently, including e.g. locationError, locationSource. Since stage 3 defines more accurate fields, it is proposed to align stage 2 description with stage 3:

Proposal 9: Radio Link Failure report section is cleaned up to reflect RLFreport contents can be used for MRO purposes as proposed in R2-2003159.

## 2.5	Accessibility measurements clean-up

37.320 section 5.1.6 defining Accessibility measurements makes a connection to 38.300. Similarly, to NR RLFreport TS38.300 (Baseline RAN3 CR in R3-201475) will not define new contents for SON for RA-report.

Proposal 10: Accessibility measurements section is cleaned up to reflect CEFreport contents can be used for RACH Optimization purposes as proposed in R2-2003159.

# 3	Conclusion

Observation 1: RRC (38.331 v16.0.0 in 5.5a.3) resolves the FFS and clarifies the UE always logs serving cell for periodical event.

Proposal 1: Remove Editor’s note and FFS within: “For the serving cell downlink pilot strength measurements logging configuration is FFS.

Observation 2: Trace Management (32.422 v16.0.0 in 4.4) and 38.4x3 CRs define MDT Trace configuration propagation in handovers with respect to PLMN (list), i.e. regardless of area scope.

Proposal 2: Remove FFS within Section 5.1.2.3.

Observation 3: Both: Signalling and Management based MDT Trace are supported in NR.

Proposal 3: Remove Editor’s notes ‘Definition applicability to NR needs checking with SA5’ and Trace relevant parameters in NR need SA5 confirmation.

Observation 4: Default requirement for the network has been to not propagate logged MDT configuration.

Proposal 4: To revise the agreement for signalling based MDT from:

”When the UE resumes the RRC connection in one new NG-RAN, the last serving NG-RAN can propagate the logged MDT configuration to the new NG-RAN.”

To:

”When the UE resumes the RRC connection in one new NG-RAN, the new NG-RAN can configure the MDT configuration for the UE, only if the signalling based logged MDT was received by the NG-RAN. It is not required to propagate the logged MDT configuration.”

Proposal 5: Based on the revised agreement, remove the Editor’s note in 5.4.2.

Proposal 6: Use gNB and NR for NR specific RAN actions.

Proposal 7: Use IDLE/INACTIVE/CONNECTED state (instead of mode)

Proposal 8: Use NR Cell Global Identifier for NR specific cell id reference.

Proposal 9: Radio Link Failure report section is cleaned up to reflect RLFreport contents can be used for MRO purposes as proposed in R2-2003159.

Proposal 10: Accessibility measurements section is cleaned up to reflect CEFreport contents can be used for RACH Optimization purposes as proposed in R2-2003159.