3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003172

Elbonia, Online, 20 – 30 April 2020

Agenda item:	6.7.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarification on Ethernet frame handling by EHC

WID/SID:	NR_IIOT - Release 16

Document for:	Discussion and Decision

# 1	Introduction

The current PDCP specifications describes the main principles of how EHC compresses Ethernet frames, e.g. by stating which of the fields are removed from the Ethernet header. However, the current description remains vague, especially for the person not involved directly in developing and specifying EHC. Thus, we think it would be beneficial to provide an explanation of how the EHC is expected to process Ethernet frames of different types. Section 2 provides a description of such operation while Annex A comprises the TP to the PDPCP specifications.

# 2		Processing of different Ethernet header structures

There are basically three different Ethernet frame header formats that are widely used in real networks:

- Ethernet II format with DESTINATION MAC, SOURCE MAC and ETHERTYPE header fields

- Ethernet II format extended with a single 802.1Q VLAN header/tag

- Ethernet II format extended with one or more 802.1ad header/tag (known as S-tags) followed by a single 802.1Q VLAN header (known as a C-tag).

These formats are depicted, respectively, in the figure below.

In addition to Ethernet II format also IEEE 802.3 format is used to some (very small nowadays) extent. Its main difference to the baseline Ethernet II format is that the ETHERTYPE field is interpreted as LENGTH (which may be variable even within a single flow of frames between two end stations).

It is possible for the compressor and decompressor to parse all the four above frame header structures (three based on Ethernet II with or without 802.1Q and 802.1ad and also the IEEE 802.3 baseline) independently and based on that conclude which header fields are (to be) removed, without explicit signalling or multiple profiles. To achieve that, the compressor would have to work according to the following rules:

- If TYPE/LEN value is equal or smaller than 1500:

  - Identification: This is an IEEE 802.3 format frame

  - Compressor/decompressor operation: Remove/restore DESTINATION MAC, SOURCE MAC and TYPE/LEN

- If TYPE/LEN value is larger than 1500 but not (0x8100 OR 0x88a8)

  - Identification: This is an Ethernet II format frame without IEEE 802.1Q or IEEE 802.1ad headers/tags

  - Compressor/decompressor operation: Remove/restore DESTINATION MAC, SOURCE MAC and TYPE/LEN

- If TYPE/LEN value is 0x8100

  - Identification: This is an Ethernet II format frame with an IEEE 802.1Q VLAN header/tag

  - Compressor/decompressor operation: Remove/restore DESTINATION MAC, SOURCE MAC, IEEE 802.1Q VLAN header and TYPE/LEN

- If TYPE/LEN value is 0x88a8

  - Identification: This is an Ethernet II format frame with one or more IEEE 802.1ad headers/tags (S-tags) followed by an IEEE 802.1Q VLAN header/tag (C-tag)

  - Compressor/decompressor operation: Remove/restore DESTINATION MAC, SOURCE MAC, one or more IEEE 802.1ad headers, IEEE 802.1Q VLAN header and TYPE/LEN

  - Note: The existence of multiple 802.1ad headers (S-tags) is identified by inspecting if the TPID field value in the 802.1ad header:

    - If TPID value is 0x8a88 the next header is 802.1ad header

    - If TPID value is 0x8100 the next header is 802.1Q VLAN header

The TP describing such exemplary operation of EHC compressor is captured in Annex A.

Proposal 1: Adopt the TP captured in Annex A comprising the informative example of EHC compressor operation.

It should be also noted that even though the above four frames are the most common nowadays, also other types of frames may be encountered in TSCN networks, e.g. 802.1CB, i.e. FRER (Frame Replication and Elimination for Reliability). It is unclear what the behaviour of the EHC compressor in such case would be, e.g. should such frames be always sent uncompressed or can they be compressed up to the point to which EHC recognizes the Ethernet fields, e.g. Destination/Source MAC addresses?

Proposal 2: RAN2 should clarify how EHC handles Ethernet frames which contain fields unrecognizable by EHC.

# 3	Conclusion

Based on the discussion in section 2, it is proposed:

Proposal 1: Adopt the TP captured in Annex A comprising the informative example of EHC compressor operation.

Proposal 2: RAN2 should clarify how EHC handles Ethernet frames which contain fields unrecognizable by EHC.

# Annex A – TP for TS 38.323

Annex A (normative):
Ethernet Header Compression (EHC) protocol

## A.1 	EHC principle

The Ethernet header compression (EHC) protocol compresses Ethernet header as shown in figure A.1-1 [xx]. The fields that are compressed (i.e. removed from the Ethernet header) by the EHC protocol are: DESTINATION ADDRESS, SOURCE ADDRESS, 802.1Q TAG, and LENGTH/TYPE. The fields PREAMBLE, SFD, and FCS are not transmitted in 3GPP system, and thus not considered in EHC protocol. There may be more than one 802.1Q TAG fields in the Ethernet header, and all are compressed by the EHC protocol. The padding (PAD) is not compressed by the EHC protocol. A detailed example of EHC compressor operation is presented in section A.3.

Figure A.1-1: Ethernet packet format [xx]

The EHC compressor and the EHC decompressor store original header field information as a "EHC context". Each EHC context is identified by a unique identifier, called Context ID (CID). The EHC context must be synchronized between the EHC compressor and the EHC decompressor; otherwise, the EHC decompressor erroneously decompresses the "Compressed Header (CH)" packets.

For an Ethernet packet stream, the EHC compressor establishes the EHC context and associates it with the CID. Then, the EHC compressor transmits the "Full Header (FH)” packet to the EHC decompressor including the associated CID. The EHC compressor keeps transmitting the FH packets until the EHC feedback is received from the EHC decompressor.

NOTE:	If the maximum number of EHC contexts are already established for the compressed flows and a new Ethernet flow does not match any established EHC context, the compressor should associate the new Ethernet flow with one of the EHC CIDs allocated for the existing compressed flows or send PDCP SDUs belonging to the Ethernet flow as uncompressed packet.

When the EHC decompressor receives the FH packet, the EHC decompressor establishes the EHC context identified by the CID, and transmits the EHC feedback to the EHC compressor to indicate that the EHC context associated with the CID is successfully established in the EHC decompressor.

After receiving the EHC feedback, the EHC compressor starts to transmit the CH packets to the EHC decompressor including the associated CID. The CH packet includes only the header fields not stored in the EHC context.

When the EHC decompressor receives the CH packet, the EHC decompressor restores original header fields based on the stored EHC context identified by the associated CID.

Figure A.1-2 represents a conceptual view of EHC operation.

Figure A.1-2: EHC operation

<UNCHANGED TEXT OMITTED>

## A.3 Detailed example of operation on different Ethernet header structures (informative)

EHC operates on different Ethernet frame formats as follows:

- If TYPE/LEN value is different from 0x8100 or 0x88a8:

  - Compression context is: DESTINATION MAC, SOURCE MAC and TYPE/LEN

- If TYPE/LEN value is 0x8100 (IEEE 802.1Q VLAN header)

  - Compression context is: DESTINATION MAC, SOURCE MAC, IEEE 802.1Q VLAN header and TYPE/LEN

- If TYPE/LEN value is 0x88a8 (IEEE 802.1ad header)

  - Compression context is: DESTINATION MAC, SOURCE MAC, IEEE 802.1ad headers, any additional IEEE 802.1ad or IEEE 802.1Q VLAN headers and TYPE/LEN

  - The existence of additional 802.1ad or 802.1Q VLAN headers is recursively identified by inspecting the TYPE/LEN value following the current header:

    - If TYPE/LEN (TPID) value is 0x8a88 the next header is 802.1ad header

    - If TYPE/LEN (TPID) value is 0x8100 the next header is 802.1Q VLAN header

    - If TYPTE/LEN (TPID) value is different from 0x8100 or 0x88a8 there are no additional 802.1ad or 802.1Q VLAN headers

Compressor compares the resulting compression context with all existing compression contexts to determine if there is an existing context the frame belongs to, or whether a new context needs to be established.