3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003246

# Online, 20th – 30th April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.322 | CR | 0145 | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on RLC UM SN size for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RLC UM for DRB was introduced as part of Further NB-IoT enhancements in Rel-15. At RAN2#101bis, it was agreed: - Agreements: RLC SN is 5 bits. No change to PDCP SN, but applicability should be clarified to also apply to UM. Allow the option of configure RLC UM to be unidirectional (or bidirectional). RLC UM is not supported for SRB in Rel-15.. Can capture above agreements in running CRs However, the size of the RLC SN for RLC UM for DRB in not captured in the specification. |
| Summary of change: |  | Capture in section 7.4., that for NB-IoT, the UM SN field size is fixed to 5 bits. Impact analysis Impacted functionality: RLC UM for DRB Inter-operability: If the UE is implemented according tho the CR and the NW is not, then the RLC PDU will not be decoded properly and the data will not be received correctly. If the NW is implemented according tho the CR and the UE is not, then the RLC PDU will not be received correctly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RLC SN field size is not specified for RLC UM in NB-IoT. leading to data loss. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.4	Configurable parameters

The following parameters are configured by RRC [5]:

a) maxRetxThreshold

This parameter is used by the transmitting side of each AM RLC entity to limit the number of retransmissions of an AMD PDU (see clause 5.2.1).

b) pollPDU

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollPDU PDUs (see clause 5.2.2).

c) pollByte

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollByte bytes (see clause 5.2.2).

d) sn-FieldLength

This parameter gives the UM SN field size in bits (see clause 7.1). In NB-IoT, the UM SN field size is 5 bits.