3GPP TSG-RAN WG2 Meeting #109b electronic	R2-2003318

Elbonia, 20th - 30th April 2020

Agenda item:	6.8.2.3

Source:	Intel Corporation

Title:	Handling on TRP-ID

Document for:	Discussion and Decision

# Introduction

In last meeting, based on [1], RAN2 discussed open issues for LPP. However there was no consensus on how to handle TRP ID.

In this contribution, we futher discuss the issue.

# Discussion

Cited from [1]:

Issue 2: TRP ID handling; R2-2000476/2000966 - Rap: The issue was raised during email discussion on running CR. Assume some changes are needed to make it clear. RAN2 need to decide how to make it clear. The IE TRP-ID provides the IDs to identify the TRP. (in current running CR) -- ASN1START TRP-ID-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255) OPTIONAL, nr-PhysCellId-r16 NR-PhysCellId-r16 OPTIONAL, nr-CellGlobalId-r16 NCGI-r15 OPTIONAL, -- Need ON nrARFCNRef-r16 ARFCN-ValueNR-r15 OPTIONAL -- Cond NotSameAsRefServ0 } -- ASN1STOP - Option 1: remove the TRP ID IE, and indicate fields inside it separately; R2-2000476 - Option 2: change the name of TRP-ID to TRP-ID-Set, and remove nrARFCN;R2-2000966 Based RAN3 assumption, a TRP is identified by a TRP ID and Cell ID. Based on RAN1 agreement, an ID, which is currently written as dl-PRS-ID, is also introduced to identify the TRP. So in our understanding, {TRP ID, Cell ID} is cell-specific and UE common, which can be used for broadcast assistance data, while dl-PRS-ID may be UE specific, which means that the same TRP can be associated with the different dl-PRS-ID for different UEs. In current running CR, TRP-ID is not same as RAN3 definition, and will lead confusion. Option 1: Proposal: Remove TRP-ID IE, indicates PRS-ID, PCI, CGI and ARFCN individually. R2-2000476 Option 2: Proposal: Adopt the following change to the TRP id configuration 2000966 - Change the name of the IE to TRP-ID-Set - Remove nrARFCNRef.

Based on RAN1 defintion as below, dl-PRS-ID can be used to identify at least the TRPs configured by the network in assistance data.

“This ID can be used along with a DL PRS Resource Set ID and a DL PRS Resources ID to uniquely identify a DL PRS Resource. This ID can be associated with multiple DL PRS Resource Sets associated with a single TRP.Each TRP should only be associated with one such ID.”

Observation 1 dl-PRS-ID can be used to identify at least the TRPs configured by the network in assistance data.

In [2], TRP-ID is used in:

1 Measurement report for Multi-RTT;

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16	NR-Multi-RTT-MeasList-r16,

...

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16							TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

Observation 2: dl-PRS-ID is needed for NR-Multi-RTT-MeasElement. Other fields in TRP-ID are not needed for measurement report of Multi-RTT.

2 Measurement report for DL-AoD

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

...

}

NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs)) OF NR-DL-AoD-MeasElement-r16

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16							TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

Observation 3: dl-PRS-ID is needed for NR-DL-AoD-MeasElement. Other fields in TRP-ID are not needed for measurement report of DL-AoD.

3 Measurement report for DL-TDOA

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-IdInfo-r16,

nr-DL-TDOA-MeasList-r16	NR-DL-TDOA-MeasList-r16,

...

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16						TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

Observation 4: dl-PRS-ID is needed for NR-DL-TDOA-MeasElement. Oher fields in TRP-ID are not needed for measurement report of DL-TDOA.

4 Measurement report for ECID

NR-ECID-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-PrimaryCellMeasuredResults-r16	NR-MeasuredResultsElement-r16,

nr-MeasuredResultsList-r16			NR-MeasuredResultsList-r16	OPTIONAL,

...

}

NR-MeasuredResultsList-r16 ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement-r16

NR-MeasuredResultsElement-r16 ::= SEQUENCE {

systemFrameNumber				BIT STRING (SIZE (10)),

trp-ID-r16						TRP-ID-r16			OPTIONAL,

Observation 5: pci, CGI and ARFCN are needed for ECID. dl-PRS-ID is not needed.

5 NR-TimeStamp

NR-TimeStamp-r16 ::= SEQUENCE {

trp-ID-r16					TRP-ID-r16			OPTIONAL,-- Cond NotSameAsRefServ0

nr-SFN-r16			INTEGER (0..1023),

nr-Slot-r16 		CHOICE {

scs15			INTEGER (0..9),

scs30			INTEGER (0..19),

scs60			INTEGER (0..39),

scs120			INTEGER (0..79)

},

...

}

NR-TimeStamp is provided together with measurement result (with PRS-ID, etc.) The TRP-ID is not needed inside the NR-TimeStamp.

Observation 6: TRP-ID is not needed for NR-TimeStamp.

6 DL PRS id information to provides the IDs of the reference and neighbour TRPs DL-PRS Resources.

-- ASN1START

DL-PRS-IdInfo-r16 ::= SEQUENCE {

trp-ID-r16					TRP-ID-r16		OPTIONAL,

nr-DL-PRS-ResourceID-List-r16	(SIZE (1..nrMaxResourceIDs)) OF NR-DL-PRS-ResourceId-r16 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL

}

DL-PRS-IdInfo is to indicate which PRS configurations are for reference TRP. dl-PRS-ID along with a DL PRS Resource Set ID and a DL PRS Resources ID can uniquely identify a DL PRS Resource. Therefore other fields in TRP-ID are not needed for DL-PRS-IdInfo.

Observation 6: dl-PRS-ID is needed for DL-PRS-IdInfo. Other fields in TRP-ID are not needed for DL-PRS-IdInfo.

7 DL PRS AD

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

nr-DL-PRS-expectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-expectedRSTD-uncerainty-r16	INTEGER (-246..246),

trp-ID-r16					TRP-ID-r16		OPTIONAL,

NR-DL-PRS-AssistanceDataPerTRP is to indicate per freq PRS configurations of a TRP. dl-PRS-ID and ARFCN are needed. Other fields in TRP-ID are not needed for NR-DL-PRS-AssistanceDataPerTRP.

Observation 7: dl-PRS-ID and ARFCN are needed. Other fields in TRP-ID are not needed for NR-DL-PRS-AssistanceDataPerTRP.

8 NR SSB config

NR-SSB-Config-r16 ::= SEQUENCE {

trp-ID-r16					TRP-ID-r16,

As indicated by RAN1, PCI and ARFCN are needed for NR-SSB-Config. Other fields in TRP-ID are not needed.

Observation 8: PCI and ARFCN are needed for NR-SSB-Config. Other fields in TRP-ID are not needed.

9 UE based positioning: RTD information

ReferenceTRP-RTD-Info-r16 ::= SEQUENCE {

ref-trp-id-r16					TRP-ID-r16,

RTD-InfoElement-r16 ::= SEQUENCE {

trp-id-r16					TRP-ID-r16,

subframeOffset-r16			INTEGER (0..1966079),

rtd-Quality-r16				NR-TimingMeasQuality-r16,

...

}

Trp-id here is to provide the identity of the TRP for which the RTD-InfoElement is applicable. Same as measurement report, dl-PRS-ID is sufficient.

Observation 9: dl-PRS-ID is needed for ReferenceTRP-RTD-Info and RTD-InfoElement. Other fields in TRP-ID are not needed for NR-DL-PRS-AssistanceDataPerTRP.

10 UE based positioning: DL Beam Information is used by the location server to provide spatial direction information of the DL-PRS Resources.

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE {

trp-id-r16							TRP-ID-r16,

lcs-gcs-translation-parameter-r16	LCS-GCS-Translation-Parameter-r16	OPTIONAL,	-- Need OP

dl-prs-BeamInfoSet-r16				DL-PRS-BeamInfoSet-r16,

...

}

Trp-id here is to provide an identity of the TRP. Same as measurement report, dl-PRS-ID is sufficient.

Observation 10: dl-PRS-ID is needed for NR-DL-PRS-BeamInfo. Other fields in TRP-ID are not needed for NR-DL-PRS-AssistanceDataPerTRP.

11 UE based positioning: TRP location

TRP-LocationInfoElement-r16 ::= SEQUENCE {

trp-id-r16						TRP-ID-r16,

trp-Location-r16				RelativeLocation-r16					OPTIONAL,	-- Need OP

Trp-id here is to provide an identity of the TRP. Same as measurement report, dl-PRS-ID is sufficient.

Observation 11: dl-PRS-ID is needed for TRP-LocationInfoElement. Other fields in TRP-ID are not needed for NR-DL-PRS-AssistanceDataPerTRP.

Based on above analysis, we could see that it is unclear if we use TRP-ID for all cases, especially for most of cases only few fields are needed.

Proposal: Remove the TRP ID IE, and indicate fields for below cases separately.

| IE name | Required fields |
| --- | --- |
| NR-Multi-RTT-MeasElement | dl-PRS-ID |
| NR-DL-AoD-MeasElement | dl-PRS-ID |
| NR-DL-TDOA-MeasElement | dl-PRS-ID |
| NR-MeasuredResultsElement | pci, CGI and ARFCN |
| NR-TimeStamp | None |
| DL-PRS-IdInfo | dl-PRS-ID |
| NR-DL-PRS-AssistanceDataPerTRP | dl-PRS-ID and ARFCN |
| NR-SSB-Config | PCI and ARFCN |
| ReferenceTRP-RTD-Info | dl-PRS-ID |
| RTD-InfoElement | dl-PRS-ID |
| NR-DL-PRS-BeamInfo | dl-PRS-ID |
| TRP-LocationInfoElement | dl-PRS-ID |

# Conclusion

The followings are proposed:

Proposal: Remove the TRP ID IE, and indicate fields for below cases separately.

| IE name | Required fields |
| --- | --- |
| NR-Multi-RTT-MeasElement | dl-PRS-ID |
| NR-DL-AoD-MeasElement | dl-PRS-ID |
| NR-DL-TDOA-MeasElement | dl-PRS-ID |
| NR-MeasuredResultsElement | pci, CGI and ARFCN |
| NR-TimeStamp | None |
| DL-PRS-IdInfo | dl-PRS-ID |
| NR-DL-PRS-AssistanceDataPerTRP | dl-PRS-ID and ARFCN |
| NR-SSB-Config | PCI and ARFCN |
| ReferenceTRP-RTD-Info | dl-PRS-ID |
| RTD-InfoElement | dl-PRS-ID |
| NR-DL-PRS-BeamInfo | dl-PRS-ID |
| TRP-LocationInfoElement | dl-PRS-ID |

# References

[1] R2-2001959	Report of [AT109e][615][POS] Update RAN1 parameters part of 37.355 CR (Intel)	Intel Corporation

[2] TS37.355