3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003320

# Electronic meeting, 20 – 30 April 2020

Agenda item:		6.7.4.1

Source:	Intel Corporation

Title:	Remaining issues in PDCP duplication enhancements

Document for:		Discussion and Decision

# Introduction

In RAN2 #109-e meeting, following was agreed regarding PDCP duplication enhancements:

Agreements [AT109e][037][IIOT]

- …

- When a secondary RLC entity is deactivated (but PDCP duplication is still activated), the UE shall discard duplicated PDCP PDUs in the deactivated secondary RLC entity.

- If Rel-16 MAC CE indicates all secondary RLC entities are deactivated for a DRB, the UE shall deactivate PDCP duplication for the DRB. FFS whether and how this has TS impact.

- DRBdup ID in Rel-16 MAC CE is set to 5bits full DRB ID.

- FFS if and how Rel-15 MAC CE is used for Rel-16 Duplication

In this contribution, we discuss the remaining open issues in PDCP duplication enhancements.

# Discussion

## Whether Rel-15 MAC CE can be used for Rel-16 duplication

One open issue from RAN2#109-e meeting is “FFS if and how Rel-15 MAC CE is used for Rel-16 Duplication”. In RAN2#108 meeting, one of the main reasons to select the per DRB approach (as in current MAC running CR) instead of all DRB approach (similar to Rel-15 MAC CE) is to minimize the network coordination. Using Rel-15 MAC CE to turn on and off Rel-16 PDCP duplication configurations is not consistent with the agreement of per DRB MAC CE due to the required network coordination. In addition, per DRB approach was agreed in RAN2#108 meeting with the understanding that signaling overhead of multiple Rel-16 MAC CEs is not an issue. Therefore there is no clear benefit of using Rel-15 MAC CE to turn on and off Rel-16 PDCP duplication configurations.

Proposal 1: Rel-15 MAC CE is not used to turn on and off Rel-16 PDCP duplication configurations.

## PDCP duplication terminology

Another open issue is "FFS whether and how the following agreement impacts specifications: “If Rel-16 MAC CE indicates all secondary RLC entities are deactivated for a DRB, the UE shall deactivate PDCP duplication for the DRB.” This issue is actually related to the PDCP duplication terminology.

In latest MAC running CR R2-2002341 [2], handling of activation/deactivation is specified as below:

The MAC entity shall for each DRB configured with PDCP duplication: 1> if a Duplication Activation/Deactivation MAC CE is received activating the PDCP duplication of the DRB: 2> indicate the activation of PDCP duplication of the DRB to upper layers. 1> if a Duplication Activation/Deactivation MAC CE is received deactivating the PDCP duplication of the DRB: 2> indicate the deactivation of PDCP duplication of the DRB to upper layers. Editor’s Note: It is an FFS whether and how Rel-15 MAC CE turns on and off PDCP duplication with more than 2 RLC entities. 1> if a Duplication RLC Activation/Deactivation MAC CE is received activating PDCP duplication for associated RLC entities of a DRB configured with PDCP duplication: 2> indicate the activation of PDCP duplication for the indicated secondary RLC entity(ies) of the DRB to upper layers. 1> if a Duplication RLC Activation/Deactivation MAC CE is received deactivating PDCP duplication for associated RLC entities of a DRB configured with PDCP duplication: 2> indicate the deactivation of PDCP duplication for the indicated secondary RLC entity(ies) of the DRB to upper layers.

It can be seen that for Rel-15 MAC CE, the terminology of “activation/deactivation of PDCP duplication of the DRB” is used, while for Rel-16 MAC CE, the terminology of “activation/deactivation of PDCP duplication for the indicated secondary RLC entity(ies)” is used.

In latest running PDCP CR R2-2002366 [1], terminology “if the PDCP duplication is activated” is used in clause 5.2.1 (transmit operation) and clause 5.6 (data volume calculation). It is understood that “if the PDCP duplication is activated” means “if the PDCP duplication of the PDCP entity is activated” (note that MAC only handles PDCP duplication activation/deactivation for DRB, while in PDCP spec, the activation status is applicable for both SRB and DRB). However, there is no definition in PDCP CR R2-2002366 [1] regarding “PDCP duplication is activated” for DRBs. Currently, clause 5.11.1 “Activation/Deactivation of PDCP duplication” of PDCP running CR is as below. For SRB, “activate the PDCP duplication” means “activate the PDCP duplication for the SRB / PDCP entity”. For DRB, only “activate/deactivate the PDCP duplication for the indicated associated RLC entities” is specified. It is obvious that PDCP duplication for the DRB is activated if PDCP duplication is activated for at least one secondary RLC entity. It is proposed to add one clarifying sentence (as highlighted below) regarding “activation of PDCP duplication” for DRB.

For the PDCP entity configured with pdcp-Duplication, the transmitting PDCP entity shall: - for SRBs: - activate the PDCP duplication; - for DRBs: - if the activation of PDCP duplication is indicated: - activate the PDCP duplication for the indicated associated RLC entities; - if the deactivation of PDCP duplication is indicated: - deactivate the PDCP duplication for the indicated associated RLC entities. PDCP duplication for DRB is activated if PDCP duplication is activated for at least one associated secondary RLC entity.

Proposal 2: In TS 38.323 clause 5.11.1, add sentence “PDCP duplication for DRB is activated if PDCP duplication is activated for at least one associated secondary RLC entity” to clarify the meaning of “activation of PDCP duplication” for DRB.

# Conclusion

In this contribution, we discuss the remaining open issues in PDCP duplication enhancements, and propose the following:

Proposal 1: Rel-15 MAC CE is not used to turn on and off Rel-16 PDCP duplication configurations.

Proposal 2: In TS 38.323 clause 5.11.1, add sentence “PDCP duplication for DRB is activated if PDCP duplication is activated for at least one associated secondary RLC entity” to clarify the meaning of “activation of PDCP duplication” for DRB.

# References

[1] R2-2002366, PDCP Rapporteur (LG Electronics Inc.), “PDCP running CR for NR IIOT”

[2] R2-2002341, Samsung, “Introduction of NR IIOT”