3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003329

Online, 20 April - 30 2020

Agenda item:	7.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Text Proposal for TS 36.304 for GWUS

Document for:	Discussion and Decision

# Introduction

As part of post e-mail discussion [Post-RAN-109e-32] draft text proposal to caputre GWUS related changes in TS36.304  was reviewed and comments were provided to the TP. As outcome of the meeting, the TP was modified based on the comments received. In this document we provide the consolidated text proposal which includes the changes suggested during e-mail discussion.

# Text Proposal

Next Change

## 7.x		Paging with Group Wake Up Signal

### 7.x.1	General

When the UE supports GWUS and GWUS configuration (gwus-Config)  is provided in system information, the UE shall monitor WUS using the GWUS parameters provided in System Information. A UE supporting GWUS can be configured to monitor a group WUS and a common WUS. Upon detecting either, UE shall monitor POs as defined in subclause 7.4.

For NB-IoT, E-UTRAN may configure up to 2 WUS resources (numbered 0 and 1). The time offset, g0, from the end of WUS resource 0 to the start of corresponding PO is determined as defined in subclasue 7.4. When both wus-Config-r15 and gwus-Config-r16  are present, WUS resource 0 shares radio resources with wus-Config-r15.The time offset from the end of WUS resource 1 to the start of corresponding PO is sum of the time offset g0 and the maximum WUS duration.

For BL UEs and UEs in enhanced coverage, E-UTRAN may configure up to 4 WUS resources. The resource number, time and frequency location of these resources is determined as specified in subclause 7.x.4.

After  the UE has determined the gap between end of WUS and associated PO as specified in subclause 7.4, the  UE selects oneWUS group set for the corresponding gap as specified in subclause 7.x.2.  From the selected WUS group set, the UE selects one WUS group as defined in subclause 7.x.3. If gwus-GroupAlternation is not present in gwus-Config, the UE monitors the selected the WUS group for each PO. Otherwise, the UE determines the WUS group to monitor for each PO as specified in subclause 7.x.5.

### 7.x.2	WUS group sets selection

The total number of WUS groups configured for a gap is given by:

maxWG= i=0maxWR-1maxWGi

Where:

maxWR is the total number of WUS resources configured in gwus-NumGroupsList for the gap.

maxWG[i] is the value of gwus-NumGroupsList[i] provided in gwus-Config for the gap.

Using gwus-NumGroupsList for the gap, the UE builds the list of WUS groups as an ordered list of pairs (NIDresource, NgroupWUS) where the first entry corresponds to the first WUS group on the first WUS resource and the last entry corresponds to the last WUS group on the last configured WUS resource.  The total number of entries in the list is maxWG.

For a NB-IoT UE, if  gwus-ResourcePosition provided in gwus-Config is set to secondary, NIDresource = 0 is not used and the first entry in the list corresponds to NIDresource = 1. Otherwise, NIDresource is the index of the WUS resources in gwus- NumGroupsList.

For a BL UE or UE in enhanced coverage, UE determines NIDresourceof the configured resources as specified in subclause 7.x.4.

If gwus-ProbThreshList is present in gwus-Config, UE determines the WUS group set corresponding to its probability PNAS, if configured, as defined in Table 7.x-1. The total number of WUS groups set is equal to the number of entries in gwus-ProbThreshList + 1. The WUS groups are first assigned to WUS group set 1, followed by WUS group set 2, and so on.

Table 7.x.2-1: WUS group set definition when wus-ProbThreshList is configured

| WUS group set | gwus-ProbThreshList | WUS group index in WUS groups list |  |
| --- | --- | --- | --- |
|  |  | Lower bound | Upper bound |
| 1 | PNAS ≤ Thresh1 | 0 | Nth1 -1 |
| 2 | Thresh1 < PNAS ≤ Thresh2 | Nth1 | Nth1 + Nth2 -1 |
| 3 | Thresh2 < PNAS ≤ Thresh3 | Nth1 + Nth2 | Nth1 +Nth2 + Nth3 -1 |
| 4 | PNAS > Thresh3 or PNAS not configured | Nth1 +Nth2 + Nth3 | maxWG-1 |
| where Threshi is the value signalled in the ith entry of wus-ProbThreshList Nthi is the values signalled in the ith entry of gwus-GroupsForServiceList |  |  |  |

If wus-ProbThreshList is not present in gwus-Config, there is only WUS group set 1, including WUS groups from index 0 to maxWG-1.

### 7.x.3		WUS group selection

After selection of the WUS Group set as specified in sub clause 7.x.2, the UE selects the WUS group to monitor as below.

.

For BL UE, UE in enhanced coverage, the UE determines wg, index of the WUS group within the WUS groups set determined in subclause 7.x.2, as below:

wg=floorfloorUE_IDN×NsNn mod Nw

For NB-IoT, the UE determines wg, index of the WUS group within the WUS groups set determined in subclause 7.x.2, as below:

wg=floorUE_IDN×Ns×W mod Nw

where:

UE_ID, N, Ns, Nn and W are defined in subclause 7.1

Nw is the number of WUS groups in the selected WUS groups set.

wg is the index of the WUS group in the selected WUS groups set, 0 .. Nw-1

Then, the UE determines WG, the index of the corresponding WUS group within the WUS groups list, as below:

Table 7.x.3-1: Index of the WUS group to monitor

| WUS group set | WG |
| --- | --- |
| 1 | wg |
| 2 | wg + Nth1 |
| 3 | wg + Nth1 + Nth2 |
| 4 | wg + Nth1 + Nth2 + Nth3 |
| Where Nthi is defined in table 7.x.1 |  |

The entry corresponding to WG  in the WUS groups list defined in subclause 7.x.2 provides (NIDresource, NgroupWUS)  as specified in TS 36.211 [xx].

### 7.x.4		WUS Resource Location for BL UEs and UEs in Enhanced coverage

A BL UE or UE in enhanced coverage determines the time/frequency location of WUS resources based on FreqLocation IE in wus-Config-r15 if Rel-15 WUS is configured, otherwise based on FreqLocation IE in wus-Config-r16.  In both cases FreqLocation IE indicates the Frequency location of WUS Resource ID 0 (NIDresource=0). Frequency location of other WUS Resources (Resource ID 1,2,3) based on FreqLocation IE is given in the below table if gwus-ResourcePattern value is other than rp-ID-7.

Table 7.x.4-1: WUS Resource frequency location

| WUS Resource (NIDresource) | FreqLocation IE (Frequency location of WUS Resource ID) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NB below centre frequency |  |  | NB above centre frequency |  |  |
|  | n0 | n2 | n4 (Note) | n0 | n2 | n4 (Note) |
| WUS Resource 1 | n2 | n0 | n2 | n2 | n4 | n2 |
| WUS Resource 2 | n0 | n2 | n4 | n0 | n2 | n4 |
| WUS Resource 3 | n2 | n0 | n2 | n2 | n4 | n2 |
| Note 4: This coloumn is applicable if wus-Config-r15 is present |  |  |  |  |  |  |

For the above table, The time time offset, g0 for WUS-Resource-ID 0 and 1, from the end of WUS resource to the start of corresponding PO is determined as defined in subclasue 7.4. The time offset for WUS-Resource-ID 2 and 3 from the end of WUS resource to the start of corresponding PO is sum of the time offset g0 and the maximum WUS duration.

WUS Resource frequency location of other WUS Resources for gwus-ResourcePattern = 7 is given in below table.

Table 7.x.4-2: WUS Resource frequency location (For Resource pattern rpID=7)

| WUS Resource (NIDresource) | FreqLocation in wus-Config-Rel-15 (WUS Resource ID 0 Location ) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | NB below centre frequency |  |  | NB above centre frequency |  |  |
|  | n0 | n2 | n4 (Note) | n0 | n2 | n4 (Note) |
| WUS Resource 1 | n2 | n0 | n2 | n2 | n4 | n2 |
| WUS Resource 2 | n4 | n4 | n0 | n4 | n0 | n0 |

For the above table, for all WUS Resources time time offset, g0 for WUS-Resource-ID 0 and 1, from the end of WUS resource to the start of corresponding PO is determined as defined in subclasue 7.4

The WUS Resource locations where WUS Groups are configured is determined based on gwus-ResourcePattern is given in below Table. ( X indicates the WUS Resource ID is used for WUS Group mapping ).

| Gwus-ResourcePattern | Number of WUS Resources |  | WUS Resource ID |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NIDresource=0 | NIDresource=1 | NIDresource=2 | NIDresource=3 |
| 0 |  | 1 |  |  |  |  | X |  |  |  |
| 1 |  | 1 |  |  |  |  |  | X |  |  |
| 2 |  | 2 |  |  |  |  | X | X |  |  |
| 3 |  | 2 |  |  |  |  |  | X | X |  |
| 4 |  | 3 |  |  |  |  | X | X | X |  |
| 5 |  | 3 |  |  |  |  |  | X | X | X |
| 6 |  | 4 |  |  |  |  | X | X | X | X |
| 7 |  | 3 |  |  |  |  | X | X | X |  |

### Temp table for reference for Cross Checking the above tables

| pattern | NB below centre frequency |  |  | NB below centre frequency |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | n0 | n2 | n4 | n0 | n2 | n4 |
| 0 | 0. | 0. | 0. | 0. | 0. | 0. |
| 1 | 1. | 1. | 1. | 1. | 1. | 1. |
| 2 | 0. 1. | 1. 0. | 1. 0. | 0. 1. | 0. 1. | 1. 0. |
| 3 | 2. 1. | 1. 2. | 1. 2; 0. | 2. 1. | 2. 1. | 1. 2. |
| 4 | 2; 0. 1. | 1. 2; 0. | 1. 2; 0. | 2; 0. 1. | 2; 0. 1. | 1. 2; 0. |
| 5 | 2. 3; 1. | 3; 1. 2. | 3; 1. 2. | 2. 3; 1. | 2. 3; 1. | 3; 1. 2. |
| 6 | 2; 0. 3; 1. | 3; 1. 2; 0. | 3; 1. 2; 0. | 2; 0. 3; 1. | 2; 0. 3; 1. | 3; 1. 2; 0. |
| 7 | 0. 1. 2. | 1. 0. 2. | 2. 1. 0. | 0. 1. 2. |  | 2. 1. 0. |

### 7.x.5		WUS Group Alternation

If gwus-GroupAlternation is present in gwus-Config:

-	if gwus-ProbThreshList is not present in gwus-Config  and gwus-CommonSequence is set to legacyWUS, the UE determines the WUS group to monitor for the current PO as follows:

WGcurrent=WGinitial+ Gmin·divSFN+1024HSFNTcellmod maxWG

where:

Tcell is the default DRX cycle for the cell.

maxWG is the total number of WUS group configured in gwus-NumGroupsList for the gap.

Gmin is the lowest number of groups configured amongst all the WUS resources for the gap.

WGcurrent is the index of the WUS Group to monitor for the current PO.

WGinitial is the index, WG, of the WUS Group determined in sub-clause 7.x.3

The entry corresponding to WGcurrent in the WUS groups list defined in subclause 7.x.2 provides (NIDresource, NgroupWUS)  as specified in TS 36.211 [xx].

-	else, the UE determines the WUS group to monitor for the current PO as follows:

mcurrent=minitial+divSFN+1024HSFNTcellmod maxWR

where:

Tcell is the default DRX cycle for the cell.

maxWR is the total number of WUS resources configured in gwus-NumGroupsList for the gap.

minitial:

For a NB-IoT UE :  NIDresource in the entry corresponding to the index WG determined in sub-clause 7.x.3 .

For a BL UE or UE in enhanced coverage:

if NIDresource= 0 is configured:

NIDresource  - 1,  where NIDresource  is given in the entry corresponding to the index WG determined in sub-clause 7.x.3

else:

NIDresource  , where NIDresource  is given in the entry corresponding to the index WG determined in sub-clause 7.x.3

mcurrent is used to determine NIDresource of the WUS group to monitor for the current PO

For a NB-IoT UE :  NIDresource = mcurrent

For a BL UE or UE in enhanced coverage:

if NIDresource=0 is configured:

NIDresource = mcurrent

else:

NIDresource  =  mcurrent +1

NgroupWUS of the WUS group to monitor for the current PO is given in the entry corresponding to the index WG determined in sub-clause 7.x.3

# References

[1] R2-2003328 E-mail-Discussion-Summary for Post109e-32 : Finalise TP for TS36.304 for WUS , RAN2-109bis-e