3GPP TSG-RAN WG2 Meeting #109bis-e	Tdoc R2-2003354

Electronic meeting, 20th – 30th April

Agenda Item:	7.1.7

Source:	Ericsson

Title:	Remaining issues for LTE-M Standalone deployment

Document for:	Discussion, Decision

# 1	Introduction

The latest agreement from RAN2#109-e on LTE-M standalone deployment is:

Agreements

- In standalone deployment, if a UE considers itself to be in enhanced coverage with S criteria of normal coverage fulfilled, absolute priorities for cell reselection are used (i.e. UE does not switch to ranking as it would when in enhanced coverage due to S-criteria).

In this contribution we discuss the remaining issue of how the conditions and camping in standalone cell should be captured in TS 36.304 and also propose RAN2 to clarify

# 2	Discussion

The following is currently captured in TS 36.304:

If cell selection criteria S in normal coverage is fulfilled for a cell, UE [may] consider itself to be in enhanced coverage if SystemInformationBlockType1 cannot be acquired but UE is able to acquire MasterInformationBlock, SystemInformationBlockType1-BR and SystemInformationBlockType2.

Let's first consider the condition "if SystemInformationBlockType1 cannot be acquired…":

- Case 1, the cell is a stand-alone cell: In this case it is clear that SIB1 cannot be acquired because the cell is stand-alone, as it would not be transmitted (only SIB1-BR version would).

- Case 2, the cell is a normal cell: In this case, the condition is not actually meaningful as the UE would supposedly fill the normal coverage S-criterion even though it has not been able to acquire SIB1. Therefore, the condition "if SystemInformationBlockType1 cannot be acquired" would not applicable for this case, but only applicable for a standalone cell.

The condition for not being able to acquire SIB1 after UE considers itself to be in normal coverage is applicable only in a stand-alone LTE-M cell.

The remaining question then is whether the UE should always consider to be in enhanced coverage in such case or if it is up to UE to decide.

There has been discussion on that UE should not get stuck in a standalone cell when e.g. a neighboring cell provides full service – therefore an argument has been presented that if UE always selects enhanced coverage it would not be possible to camp in such neighboring cell for example in the case UE would like to camp in a normal LTE cell instead. However, we don't think this is an issue for couple of reasons:

- Based on the agreement in RAN2#109-e, absolute priorities can be used for cell reselection, that is, UE can be guided to other frequencies even if such provide worse coverage at the time.

- In conditions in section 5.3.1 in TS 36.304, it is clarified the UE may camp in enhanced coverage in the case SIB1 could not be acquired and cell considered initially as barred – however, there is also option for the UE to exclude the cell for re-selection for up to 300 seconds, therefore we think it is possible for the UE to consider some other cell before camping in the stand-alone cell.

If the specification text  in clause 5.2.3.2 in TS 36.304 mandates the UE to camp in enhanced coverage in the standalone case, it would be possible for the UE to re-select to another cell based on priority or by excluding the standalone cell and re-selecting to another cell on same frequency.

Based on the discussion and observations above, we prefer to update the referenced text in TS 36.304 to "shall":

Change "[may]" in TS 36.304 to "shall".

However, we understand there may still be some concerns for example in the case the neighboring cells are on the same frequency, or different frequency but same priority, where ranking-based cell reselection criterion would apply. Thus, we propose RAN2 to discuss whether a clarification should be made for these cases or whether for example the existing text in 5.3.1 in TS 36.304 already applies and UE may select another cell on the same frequency.

RAN2 to discuss whether further clarification is needed in TS 36.304 for cases where neighboring cells are on same frequency and other cell provides normal coverage operation or for cases with different frequencies but same priority.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The condition for not being able to acquire SIB1 after UE considers itself to be in normal coverage is applicable only in a stand-alone LTE-M cell.

Observation 2	If the specification text  in clause 5.2.3.2 in TS 36.304 mandates the UE to camp in enhanced coverage in the standalone case, it would be possible for the UE to re-select to another cell based on priority or by excluding the standalone cell and re-selecting to another cell on same frequency.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Change "[may]" in TS 36.304 to "shall".

Proposal 2	RAN2 to discuss whether further clarification is needed in TS 36.304 for cases where neighboring cells are on same frequency and other cell provides normal coverage operation or for cases with different frequencies but same priority.

# References

R2-2001881, "Standalone deployments – Discussion on remaining Issues", Nokia, Nokia Shanghai Bell, RAN2#109-e, February-March 2020.

TS 36.300, v16.0.0

TS 36.304, v16.0.0