3GPP TSG-RAN WG2 #109bis_e	Tdoc R2-2003356

Emeeting, April 20th – 30th 2020

Agenda Item:	6.13.2

Source:	Ericsson

Title:	Handling of invalid POs for MsgA transmission.

Document for:	Discussion, Decision

# Introduction

Transmission of msgA in the 2-step Random Access procedures is equivalent to preamble transmission on PRACH followed by a transmission on PUSCH. As has been noted there are configurations where certain PUSCH occasions (POs) are not valid for certain preambles. In this case, if a preamble is selected which is mapped to an invalid PO, only the PRACH can be transmitted. In this contribution we give our solution to this issue.

# Discussion

The handling of invalid POs has been discussed in [1], where it was proposed that the UE should be able to select from the preambles which are mapped to valid POs, i.e. ignoring the preambles that are mapped to invalid POs. This possibility would be an optimisation with some complexity; although at high loads there could be benefits of having UEs selecting preambles that do not have any valid PO and rely on FallbackRAR to send the MsgA PUSCH content. In any aspect, the 2-step procedure would be functional with or without this change.

Restricting the preamble selection to only preambles that are mapped to valid POs is not needed.

However, the MAC procedure when a preamble not mapping to a valid PO is selected is not described in 38.321. The only option described is for when both the preamble and msgA PUSCH are transmitted. We believe that this should be corrected in 38.321. In case the PO is invalid, we believe that only the relevant parameters should be passed to lower layers and that the lower layers should be instructed to transmit only the msgA preamble.

An accompanying CR is supplied in [2]. We propose

.

Agree the accompanying CR [2].

# 2	Conclusion

In the previous sections we made the following observations:

Observation 1	Restricting the preamble selection to only preambles that are mapped to valid POs is not needed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Agree the accompanying CR [2].

# References

- R2-2000225, “Handling Preambles not associated with PRUs”, Samsung, 3GPP TSG-RAN WG2 Meeting#109 electronic

- R2-2003362	38321_CR_ 0725_(Rel-16) Correction for Handling of invalid POs for MsgA transmissions