3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003446

# Electronic, 20th – 30th April, 2020

Agenda Item:	6.19

Source: 	Huawei, Hisilicon

Title: 	Discussion on UL sharing for variable-duplex FDD bands

Document for:	Discussion and decision

1	Introduction

In RAN4#93 meeting, an LS on FDD band capability signalling for uplink sharing was sent to RAN2, in which RAN4 requested RAN2 to take the following RAN4 agreements into account in its work [1]:

- RAN4#93 has agreed to use the UE capability signalling ‘ul-SharingEUTRA-NR’ for the NR FDD bands in addition to the NR SUL for indicating UE support for E-UTRA – NR UL sharing from the UE perspective from the Rel-15 onwards.

| ul-SharingEUTRA-NR Indicates whether the UE supports EN-DC with EUTRA-NR coexistence in UL sharing via TDM only, FDM only, or both TDM and FDM from UE perspective as specified in TS 38.101-3 [4]. | BC | No | No | FR1 only |
| --- | --- | --- | --- | --- |

In last RAN2 meeting, an email discussion regarding UL sharing for variable-duplex FDD bands is triggered and discussed as below [2]:

[AT109e][052][R16 Other WISI] UL sharing for variable-duplex FDD bands (Nokia)

Scope: Treat documents above

CLOSED

[AT109e][052][R16 Other WISI] Postpone to next meeting (if still different interpretations of R4 agreement, send LS to R4 from next meeting)

It seems that companies’ views were not converged and this paper is to further provide the analysis based on RAN4 previous agreements.

2	Discussion

## 2.1 ULSUP band combinations in RAN4

RAN#84 agreed in RP-191567 on new WID on NR FDD bands with variable duplex to specify the following new NR FDD frequency bands for Europe in release independent manner from the Rel-15 onwards, which are to address FDD bands with variable duplex only:

a)	Band nA: 1427 – 1432 MHz DL / 832 – 862 MHz UL (only for Local Area BS operation)

b)	Band nB: 1432 – 1517 MHz DL / 832 – 862 MHz UL

c)	Band nC: 1427 – 1432 MHz DL / 880 – 915 MHz UL (only for Local Area BS operation)

d)	Band nD: 1432 – 1517 MHz DL / 880 – 915 MHz UL

As per TS 38.101-1[4], RAN4 defined the above FDD bands in their specification, which correspond to NR operating band from n91 to n94 highlighted in green:

Table 5.2.1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n14 | 788 MHz – 798 MHz | 758 MHz – 768 MHz | FDD |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n303 | 2305 MHz – 2315 MHz | 2350 MHz – 2360 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD1 |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD4 |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n89 | 824 MHz – 849 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD5 |
| n91 | 832 MHz – 862 MHz | 1427 MHz – 1432 MHz | FDD9 |
| n92 | 832 MHz – 862 MHz | 1432 MHz – 1517 MHz | FDD9 |
| n93 | 880 MHz – 915 MHz | 1427 MHz – 1432 MHz | FDD9 |
| n94 | 880 MHz – 915 MHz | 1432 MHz – 1517 MHz | FDD9 |
| n958 | 2010 MHz – 2025 MHz | N/A | SUL |
| NOTE 1: UE that complies with the NR Band n50 minimum requirements in this specification shall also comply with the NR Band n51 minimum requirements. NOTE 2: UE that complies with the NR Band n75 minimum requirements in this specification shall also comply with the NR Band n76 minimum requirements. NOTE 3: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 4: A UE that complies with the NR Band n65 minimum requirements in this specification shall also comply with the NR Band n1 minimum requirements. NOTE 5: Unless otherwise stated, the applicability of requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the requirements for Band n41. NOTE 6: A UE that supports NR Band n66 shall receive in the entire DL operating band. NOTE 7: A UE that supports NR Band n66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_n66B and CA_n66(2A) in the current version of the specification. NOTE 8: This band is applicable in China only. NOTE 9: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. |  |  |  |

Observation 1:	 only new FDD bands with variable duplex are agreed and captured in RAN4 specification, which expressed as NR operation band n91~n94.

In addition, the following agreements are made as RAN4’s common understanding:

- “It is RAN4 understanding that RAN4 specification does not include any band combination with FDD bands with fixed duplex for ULSUP requirements”

Consequently RAN4 does not support ULSUP with fixed duplex FDD.

Observation 2:	RAN4 does not support ULSUP with fixed duplex FDD.

The above is consistent with what RAN4 has agreed on the band combinations. The approved band combinations for UL sharing was introduced to 38.101-3 by RP-193097, uplink EN-DC configurations for ULSUP band combinations are all captured in RAN4 specification. New added band combinations supporting ULSUP are highlighted in the table below.

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |
| DC_1A_n5A | DC_1A_n5A | No |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A | No |
| … | ... | … |
| DC_8A_n93A | DC_8A_n93A_ULSUP-TDM | N/A |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |
| … | ... | … |
| DC_20A_n91A | DC_20A_n91A_ULSUP-TDM | N/A |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |
| … | ... | … |

Observation 3:	 All band combinations supporting ULSUP are clearly defined in RAN4 and no band combination with fixed duplex FDD bands are defined.

## 2.2 ULSUP capability support in RAN2

RAN4 agrees to reuse the existing capability because the variable FDD bands are newly defined bands with new band numbers, and thus there is no backward compatibility issue since no legacy UE would report any of these new bands. In RAN2 specification, the UL sharing capability field description is defined as below:

ul-SharingEUTRA-NR Indicates whether the UE supports EN-DC with EUTRA-NR existence in UL sharing via TDM only, FDM only, or both TDM and FDM from UE perspective as specified in TS 38.101-3[X].

As per observation 3, obviously, if UE reports ul-SharingEUTRA-NR and supported operation band, it can be learnt whether the supported operation band is for ULSUP. In other words, the ul-SharingEUTRA-NR field description has already referred to the RAN4 specifications which clearly defined the bands supporting UL sharing. Therefore, there is no need to change the current RAN2 text of ul-SharingEUTRA-NR since the current text is already generic enough.

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |
| DC_1A_n5A | DC_1A_n5A | No |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A | No |
| … | ... | … |
| DC_8A_n93A | DC_8A_n93A_ULSUP-TDM | N/A |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |
| … | ... | … |
| DC_20A_n91A | DC_20A_n91A_ULSUP-TDM | N/A |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |
| … | ... | … |

Proposal 1: There is no need to update the existing UE capability description for “ ul-SharingEUTRA-NR”.

3	Conclusions

In this contribution, we discussed the support of UL sharing for variable-duplex FDD bands, and we have the following observations and proposals:

Observation 1:	 only new FDD bands with variable duplex are agreed and captured in RAN4 specification, which expressed as NR operation band n91~n94.

Observation 2:	RAN4 does not support ULSUP with fixed duplex FDD.

Observation 3:	 All band combinations supporting ULSUP are clearly defined in RAN4 and no band combination with fixed duplex FDD bands are defined.

Proposal 1: There is no need to update the existing UE capability description for “ ul-SharingEUTRA-NR”.

4	References

- R4-1916180 LS on FDD band capability signalling for uplink sharing, Reno, USA, 18-22 Nov,2019

- R2-2001661 Report of 3GPP TSG RAN WG1 #109e, 24Feb-6 Mar 2020

- R2-2000864 Support of UL sharing from the UE perspective for FDD bands, 24 Feb-6 Mar 2020

[4].   TS 38.101-1-g20