3GPP TSG-RAN WG2 #109bis-e	R2-20xxxxx

Electronic Meeting, April 20th – 30th 2020

Agenda Item:	5.4.1.1

Source:	Qualcomm

Title:	[AT109bis-e][007][NR15] Security

Document for:	Discussion, Decision

# 1	Introduction

This document is the report of the following email discussion:

[AT109bis-e][007][NR15] Security (Qualcomm, Nokia, Huawei)

Scope: Treat R2-2003334, R2-2003335, R2-2003336, R2-2003337, R2-2002985, R2-2002986, R2-2003697, R2-2003698.

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 23 0700 UTC

Part 2: For the parts that are agreeable, discussion will continue to agree on CRs.

As described above in the scope, the following Tdocs are covered here:

R2-2003334	Clarification on avoiding keystream repeat due to COUNT reuse	Qualcomm Incorporated, Ericsson, Vodafone, NTT DOCOMO	CR	Rel-15	38.331	15.9.0	1555	-	F	NR_newRAT-Core

R2-2003335	Clarification on avoiding keystream repeat due to COUNT reuse	Qualcomm Incorporated, Ericsson, Vodafone, NTT DOCOMO	CR	Rel-16	38.331	16.0.0	1556	-	A	NR_newRAT-Core

R2-2003336	Clarification on avoiding keystream repeat due to COUNT reuse	Qualcomm Incorporated, Ericsson, Vodafone, NTT DOCOMO	CR	Rel-15	36.331	15.9.0	4257	-	F	TEI15

R2-2003337	Clarification on avoiding keystream repeat due to COUNT reuse	Qualcomm Incorporated, Ericsson, Vodafone, NTT DOCOMO	CR	Rel-16	36.331	16.0.0	4258	-	A	TEI15

Moved from 5.4.2

R2-2002985	Avoiding security risk for RLC AM bearers during termination point change	Nokia, Nokia Shanghai Bell, Deutsche Telekom	CR	Rel-15	38.331	15.9.0	1539	-	F	NR_newRAT-Core

R2-2002986	Avoiding security risk for RLC AM bearers during termination point change	Nokia, Nokia Shanghai Bell, Deutsche Telekom	CR	Rel-15	36.331	15.9.0	4241	-	F	NR_newRAT-Core

R2-2003697	Potential issue on the Counter Check in (NG)EN-DC and NR standalone	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

R2-2003698	Draft LS to SA3 on potential issue of Counter Check	Huawei, HiSilicon	LS out	Rel-15	NR_newRAT-Core	To:SA3

# 2	Discussion

Companies are requested to add their comments for each of the treated documents of this email discussion in the boxes below (one for each document to be treated).

## 2.1 Security risk related to COUNT reuse

### 2.1.1 Discussion on the CRs for Clarification on avoiding keystream repeat due to COUNT reuse, R2-2003334, R2-2003335, R2-2003336, R2-2003337

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Nokia | Agree about the COUNT reuse problem in general | Agree that there is something to clarify but just to confirm once again. Is this a different issue than what we have also identified in R2-2002985, R2-2002986? [Qualcomm]: The issues are in the same area. However, we think changes are not overlapping, and changes from both sets are needed/beneficial. |
| Qualcomm | Agree to CRs 😊 |  |
| CATT | Agree |  |
| Ericsson | Agree |  |
| Huawei, HiSilicon | CRs are useful but we have comments for the changes and disagree with the coversheets | There is no relation with UL Valid "reason for change" could be that: - 36/38.331 rewords requirements from 33.501 on avoidance of keystream repeat without actually referring to 33.501, which could be misunderstood that 38.331 is the reference and result in not respecting the requirements from 33.501 - the meaning of "different RB identities for RB establishments" is not clear With the change, 38.331 is anyway "misaligned" in the sense that what is quoted here is "e.g." so this is misaligned by definition. Consequences if not approved could be: "RAN2 specification rewords requirements from 33.501 without quoting the original requirements, which can be misunderstood as replacing the requirements from 33.501". About the changes: - in 36.331 and 38.331, suggest removing "different" - in 38.331: suggest adding "successive" like in 36.331 |

### 2.1.2	Discussion on the CRs for Avoiding security risk for RLC AM bearers during termination point change, R2-2002985, R2-2002986

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Qualcomm | Agree in general | See comments above in 2.1.1. We agree with the intention. However, there are some minor suggestions to the CR - “and” should be put in front of the last (added) example and existing “and” to be removed. - In coversheet, other specs impacted, not sure if we need to add 36.xxx in 38.xxx and vice versa, but if yes, then CR numbers missing. - If Agreed, Rel-16 mirrors are required. - It may also make sense to merge this to respective CRs in 2.1.1 above since changes are in the same section but we are fine either way. |
| CATT | Not really needed | We agree with the scenario, but there are a lot of scenarios that will cause similar security risk, e.g. the SN may release the DRB with this very DRB ID and then add another DRB with the same DRB ID, or release the DRB and provide the DRB ID back to the MN by XnAP signalling… We need not list each of them. |
| Ericsson | Agree | The scenario that Nokia describes is relevant, even though fullConfig cannot be signalled for the SCG configuration, this is handled by SN indicating that it is using full configuration to MN and then MN setting endc-ReleaseAndAdd for EN-DC towards the UE, so that the UE releases the old SCG configuration before applying the new one. Maybe what is described is may sound like a corner case, but it does not hurt to add Nokia’s clarification. There seems to be a typo though on the cover sheet reason for change. Step #4 should say “that the MN did not yet have a key refresh” |
| Huawei, HiSilicon | Not really needed | Same view like CATT, this is certainly not the only scenario. If there is strong support, we can accept this but the "consequences if not approved" is certainly unsuitable. |

## 2.2	Discussion on the Potential issue on the Counter Check in (NG)EN-DC and NR standalone, R2-2003697, R2-2003698

In this discussion paper, following is proposed. A draft LS is also provided.

Proposal: Send a LS to SA3 to check whether it is acceptable for the counter check procedure to check less than the 25 MSBs and indicate the minimum number of bits to be checked.

| Company | Agree/Disagree | Comments |
| --- | --- | --- |
| Nokia | Agree, but… | Just for our understanding do you consider full scheduling active during the COUNTER CHECK procedure? Can you please confirm. |
| CATT | Disagree | SA3 does not say that any COUNT is provided back by the UE means a traffic-insertion attack (it uses “may” instead). The RAN node can handle it based on its own implementation, e.g. comparing the 23 MSBs of the COUNT provided by the UE with the 23 MSBs of the 25 MSBs it sends. As per current status, we prefer not adopting any NBC changes unless there is a fatal issue and impossible to be handled based on implementation. |
| Ericsson | Disagree | With the proposed change, the flexibility for a potential attacker would increase and hence should be avoided. A much better approach to avoid intercepted packets is for the network to enable integrity protection. This seems like an optimization which is not required. The network could address this by implementation, in our view. |
| Qualcomm v4 | Disagree | This is NBC as UE behavior change is required to accommodate for this CR, as UE will expect now to receive and compare less than 25 MSB. And as commented above, the proposal do not address the potential concerns. |
| Huawei, HiSilicon | Agree | To Ericsson's comment: in EN-DC, integrity protection is NOT supported, this mechanism is the only one available. To CATT's comment: whether the check is done by the UE or the network, we think SA3 should confirm that this is acceptable. About NBC UE changes: well, maybe security is a valid reason for some NBC change (potentially with a UE capability). About Qualcomm's comment "not address the potential concerns": well, the group which can appreciate the concerns is SA3, hence why we would like to hear their views. |

# Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections following is proposed:

# References

[1]