3GPP TSG-RAN WG2 Meeting #109bis-e		R2-200xxxx

Electronic, April 20 – 30, 2020

Source: 			Qualcomm Incorporated

Title: 		Summary of email discussion [AT109bis-e][014][NR15] UE Cap Miscellaneous II

Document for: 	Decision

Agenda Item: 	6.19

# Introduction

This document summarizes the following email discussion.

(The list of companies below is incorrect. It should be “Ericsson, Huawei”.)

[AT109bis-e][015][NR15] UE Cap Miscellaneous II (Qualcomm, ZTE, Mediatek, Huawei)

Scope: Treat R2-2003306, R2-2003307, R2-2003280, R2-2003281, R2-2003459, R2-2003460, R2-2003461, R2-2003462

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 23 0700 UTC

Part 2: For the parts that are agreeable, discussion will continue to agree on CRs.

# Discussion: Part 1 (by April 23 0700 UTC)

It is proposed to try to come to a set of agreeable proposals out of the documents listed above.

## Undefined band combinations in UECapabilityInformation (R2-2003306)

This document requests RAN2 to confirm that band combinations advertised by UE in NR and E-UTRA UECapabilityInformation are supported by the UE and defined in RAN4 specifications (36.101, 38.101). The document also requests RAN2 to discuss if anything need to be captured in specifications on that regard.

| Company name | Agree / Disagree | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Agree, but. | We do not consider it is necessary to capture anything in our specifications. Band combinations defined by RAN4 is moving target and there are always cases where the legacy network in the field would see unknown band combinations reported by the UE. |
| Nokia | Disagree | Yes, we did not understand the real point of this contribution. |

## Bands in supportedBandListNR (R2-2003307)

This document requests RAN2 to confirm the UE that indicate support for certain band in supportedBandCombinationList (in RF-Parameters or RF-ParametersMRDC) also indicates this band in supportedBandListNR.

| Company name | Agree / Disagree | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Agree |  |
| Nokia | Agree | That was obvious and no CR was needed ;-) |

## Missing "Optional features without UE radio access capability parameters" (R2-2003280, R2-2003281)

These CRs try to clarify that CMAS and ETWS are optional feature without corresponding UE capability parameters.

| Company name | Support / Not support | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Support |  |
| Nokia | Not Support | Regulatory features are mandated by regulators/operators. It is just not needed to implement *both* CMAS and ETWS with the same region. We think the UE vendors understand this. Also, from NW perspective, everything is optional and what gets implemented depends on operators request. So, it does not make sense to mark it as optional now since anyway PWS is practically mandated by regulators in all regions. |

## Correction on default Power class for FR2 (R2-2003459, R2-2003460)

These CRs try to specify that the UE not signalling the power class for FR2 means the UE  supports the default power class as defined by RAN4, i.e. implement the same behaviour as FR1 today.

| Company name | Support / Not support | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Not support | This is not backward compatible to legacy network. Default UE capability in absence of signalled parameter should be avoided as much as possible. |
| Nokia | Not support | RAN4 has specified for FR2 in TS38.101-2 that the Power class 3 is default power class (also for FR2). So, the clarification is not required. Agree also that default UE capability in absence of signalled parameter should be avoided as much as possible for BC reasons. |

## Correction to the serving cell number for ENDC power class (R2-2003461, R2-2003462)

These CR try make the power class (the one signalled per band combination) to be applicable to three UL CA case.

| Company name | Support / Not support | Comments |
| --- | --- | --- |
| Nokia | Not support | This CR seems to implement a draft LS that was never agreed to be sent to RAN2 and how can we make an agreement in RAN2 to correct something that was only Noted in RAN4? We should just wait for RAN4 discussions to conclude, no? |

Proposal 1:	xxxx

# Discussion: Part 2

xxxxxxxxxx

# Conclusion

xxxxxxxxxx

# Reference

[1]