3GPP TSG-RAN WG2 #109bis-e	R2-20xxxxx

Electronic Meeting, April 20th – 30th 2020

Agenda Item:	6.10.7

Source:	Ericsson

Title:	[AT109bis-e][039][DCCA] Fast MCG Link Recovery

Document for:	Discussion, Decision

# 1	Introduction

This document is to kick off the following email discussion:

[AT109bis-e][039][DCCA] Fast MCG Link Recovery (Ericsson)

Scope: Treat topics in 6.10.6, based on R2-2003199 and ASN.1 issues and RRC corrections. Can start discussion on non-controversial proposals immediately, if any. Wait for on-line discussion for controversial proposal.

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 24 0700 UTC

# 2	Discussion

## 2.1	Summary of [Post109e#27][DCCA] Fast MCG recovery (R2-2003199)

Regarding the fast MCG recovery, the email discussion in R2-2003199 covered two main aspects related to the SN change as result of fast MCG recovery procedure and the support of the inter-RAT HO and other handover scenarios as described in Table B-1 of TS 37.340. According to this, companies are request to provide inputs about the summary of the email discussion.

| Company | Proposal (Agree/Disagree) | Comments |
| --- | --- | --- |
| Nokia | P1, P2, P3 – agree P4 – Disagree in its current wording | P4 ends: “(inter-RAT HO is excluded)”. This is misleading. The origin is probably Q5 of the discussion (that lead to P4) where inter-RAT handover was out of scope. Therefore, we propose the following re-wording: Proposal 4 Apart from inter-RAT HO, according to Table B-1 of TS 37.340, all handover scenario that have a DC option in the column “from” are supported in fast MCG recovery. |

## 2.2	Other contributions for fast MCG recovery

The following contributions addressed topic that have been treated during the email discussion and thus we propose to not formulate any specific proposal.

R2-2002647	Remaining issues in Fast MCG Recovery	Qualcomm Incorporated	discussion	LTE_NR_DC_CA_enh-Core

R2-2002700	Support of Inter-RAT handover upon MCG failure recovery	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

R2-2002992	CR37340 on fast MCG recovery support	vivo	CR	Rel-16	37.340	16.1.0	0191	-	B	LTE_NR_DC_CA_enh-Core

However, in R2-2002700, the proposal 3 and 4 were not really discussed in the email discussion and therefore companies are asked to provide their input on those. The mentioned proposals are as follow:

Proposal 3   Inter-RAT handover towards GERAN/UTRAN via SRB3 is supported upon MCG failure recovery, including the following scenarios:

         Case 6: EN-DC to GERAN/UTRAN;

         Case 7: NE-DC to UTRAN-FDD (i.e. SRVCC from 5G to 3G)

         Case 8: NR-DC to UTRAN-FDD (i.e. SRVCC from 5G to 3G)

Proposal 4   UE can include UTRAN-FDD measurement results in MCG Failure Information message.

| Company | Proposal (Agree/Disagree) | Comments |
| --- | --- | --- |
| Nokia | P3 - Agree P4 - Agree |  |

## 2.3	ASN.1 issues and RRC Corrections

According to chairman notes, the following contributions are classified as ASN.1 issues and RRC Corrections.

R2-2003425	[Z301] Correcction for SCG RLC failure during fast MCG recovery	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

R2-2002790	Correction on the Configuration of T316 [C103] [C104]	CATT	draftCR	Rel-16	38.331	16.0.0	F	LTE_NR_DC_CA_enh-Core	Late

R2-2002984	Erroneous instances of “the procedure ends” impacting reception over SRB3	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.0.0	1538	-	F	LTE_NR_DC_CA_enh-Core

Companies are encouraged to provide comments on those:

### 2.3.1	[Z301] Correction for SCG RLC failure during fast MCG recovery (R2-2003425)

| Company | Proposal (Agree/Disagree) | Comments |
| --- | --- | --- |
| Nokia | We would prefer Alt1, i.e.: Alt1: The UE shall not trigger the failure information procedure | We prefer Alt1. This is in line with the current specification that disallows use of the SCG for FailureInformation. Otherwise we risk SCG RLF -> RRC re-establishment just before the UE would receive response to MCGFailureInfo. |

### 2.3.2	Correction on the Configuration of T316 [C103] [C104] (R2-2002790)

| Company | Comments |
| --- | --- |
| Nokia | Some sympathy for the intention. The current way is indeed a bit ugly |

### 2.3.3	Erroneous instances of “the procedure ends” impacting reception over SRB3 (R2-2002984)

| Company | Comments |
| --- | --- |
| Nokia | (Nokia CR) – we agree. |

# Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

# References

[1]