3GPP TSG-RAN WG2 Meeting #109-e-bis	R2-200xxxx

Electronic meeting, 20 – 30 Apr 2020

Title:	[DRAFT] LS on Conflicting configurations

Response to:

Release:	Rel-16

Work Item:	NR_L1enh_URLLC-Core, NR_eMIMO-Core, NR_unlic-Core, others?

Source:	[Huawei, to be changed to RAN2]

To:	RAN1

CC:

Contact Person:

#### Name:	David Lecompte

Email Address:	david.lecompte@huawei.com

Attachments:	None

1. Overall Description:

RAN2 identified that there are additions in Rel-16 for physical layer configuration for different features/WIs that may conflict with each other and would like to raise the following questions:

1)	DMRS-UplinkConfig can be extended with pi2BPSK-ScramblingID0/pi2BPSK-ScramblingID1, on the condition that "tp-pi2BPSK is included in PUSCH-Config". DMRS-UplinkConfig is use for several fields:

-	in PUSCH-Config: dmrs-UplinkForPUSCH-MappingTypeA/B and dmrs-UplinkForPUSCH-MappingTypeA/B-ForDCI-Format0-2-r16

-	in ConfiguredGrantConfig: for cg-DMRS-Configuration

Change to only ask what is not possible to do. Add one combination

Q1-1)	Can pi2BPSK-ScramblingID0/pi2BPSK-ScramblingID1 be configured in the 5 above listed fields and, for each field, in which instance of PUSCH-Config does tp-pi2BSK need to be included for pi2BPSK-ScramblingID0/pi2BPSK-ScramblingID1to be possible? (note that in ConfiguredGrantConfig is not included in PUSCH-Config but in BWP-UplinkDedicated, where PUSCH-Config is also included).

Q1-2)	Is the usage of pi2BPSK-ScramblingID0/pi2BPSK-ScramblingID1 in each of the 5 above listed fields (or in the ones among them in which it can be used according to the answer to Q1-1) in PUSCH-Config independent of its usage in the other fields or is there any constraint (e.g. can it be used in mappingTypeA and not in mappingTypeB, etc)?

2)	DMRS-DownlinkConfig can be extended with dmrs-Downlink-r16. DMRS-DownlinkConfig is used for several fields in PDSCH-Config: dmrs-DownlinkForPDSCH-MappingTypeA/B and dmrs-DownlinkForPDSCH-MappingTypeA/BForDCI-Format1-2-r16.

Q2-1) Can dmrs-Downlink-r16 be used in the 4 above listed fields?

Q2-2)	Is the usage of dmrs-Downlink-r16 in the 4 above fields (or in in the ones among them in which it can be used according to the answer to Q2-1) in PUSCH-Config fully independent of its usage in the other fields or is there any constraint (e.g. can it be used in mappingTypeA and not in mappingTypeB, etc)?

3)	PDSCH-TimeDomainResourceAllocation can be extended with repetitionNumber. Meanwhile, pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 was introduced in PDSCH-Config.

Q3-1)	Should pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 support the use of repetitionNumber?

Q3-2)	If the answer to Q3-1 is yes, is the usage of repetitionNumber in pdsch-TimeDomainResourceAllocationList independent of its usage in pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 fully independent or can it only be used in both or in none of them?

4)	PUSCH-TimeDomainResourceAllocationNew-r16 (name will have to be changed to avoid "New") was defined which includes the parameters of PUSCH-TimeDomainResourceAllocationList plus startSymbol, length and numberOfRepetitions (also, it is making mappingType and startSymbolAndLength optional). It is used in PUSCH-Config for pusch-TimeDomainAllocationListForDCI-Format0-2-r16 and pusch-TimeDomainAllocationListForDCI-Format0-1-r16 while PUSCH-TimeDomainResourceAllocationList is used for pusch-TimeDomainAllocationList in PUSCH-Config and in PUSCH-ConfigCommon.

Q4-1)	Can startSymbol, length and numberOfRepetitions be used in pusch-TimeDomainAllocationList in PUSCH-Config? Same question for pusch-TimeDomainAllocationList in PUSCH-ConfigCommon?

Q4-2)	If the answer to Q4-1 is "yes", is the usage of these 3 parameters in each the 4 above listed fields (or in in the ones among them in which it can be used according to the answer to Q4-1) fully independent of their usage in the other fields?

5)	[Add more here if needed]

2. Actions:

To RAN1:

ACTION: 	RAN2 respectfully asks RAN1 to answer the above questions, which are necessary for RAN2 to complete the ASN.1 review of TS 38.331.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#110-e		01-12, June, 2020					Online

RAN2#111		24-28, August, 2020					Toulouse, France