3GPP TSG-RAN WG2 Meeting #109bis-e	                               R2-20xxxxx

# Electronic meeting, 20th - 30th April 2020

Source:	vivo

Agenda Item:	6.0.1

Title:	Summary of [AT109bis-e][070][NR RIL] DiscMail7+DiscMail9(vivo)

Document for:	Discussion and Decision

- Introduction & Background

This contribution will summary the following email discussion on ASN.1 RILs:

[AT109bis-e][070][NR RIL] DiscMail7 + DiscMail9 (vivo)

Scope: Discussion and implementation of review issues.

Wanted outcome: a) Agreed RIL Status update in the email discussion report b) Agreed ASN.1/procedure text proposal included in the email discussion report.
After email discussion report is agreed, the TPs will be included in the ASN.1 Review file, for the continued ASN.1 review.

Deadline: Thursday, 30th April 2020 EOM (short extension may be considered, if necessary)

This email discussion addresses the following RILs:

- ServingCellConfig RILs: [S651], [S652]: MIMO

- ServingCellConfig RIL: [Z015]: NR-U

- ServingCellConfigCommon RIL: [Z019]: NR-U

- MCGFailureInformation RIL: [M005]: DCCA/MDT

- RRCReconfiguration RIL: [Z265]: MobEnh

- SIB2 RILs: [Q002], [Q003]: PowSave

- SIB2 RIL: [Q004]: TEI

- SIB4 RIL: [Q005]: TEI

- Discussion

  - ServingCellConfig RILs [S651], [S652], [Z015]

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                   OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                       OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                  OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink            OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                      OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                                                                  OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease { PDCCH-ServingCellConfig }                                OPTIONAL,   -- Need M

pdsch-ServingCellConfig             SetupRelease { PDSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

csi-MeasConfig                      SetupRelease { CSI-MeasConfig }                                         OPTIONAL,   -- Need M

sCellDeactivationTimer              ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}       OPTIONAL,   -- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                    OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

dummy                               ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

pathlossReferenceLinking            ENUMERATED {spCell, sCell}                                       OPTIONAL,   -- Cond SCellOnly

servingCellMO                       MeasObjectId                                                    OPTIONAL,   -- Cond MeasObject

...,

[[

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                                OPTIONAL,   -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern       OPTIONAL,   -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId     OPTIONAL,   -- Need N

downlinkChannelBW-PerSCS-List       SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

supplementaryUplinkRelease          ENUMERATED {true}                                                       OPTIONAL,   -- Need N

tdd-UL-DL-ConfigurationDedicated-iab-mt-v16xy    TDD-UL-DL-ConfigDedicated-IAB-MT-v16xy                     OPTIONAL,   -- Need FFS

firstWithinActiveTimeBWP-Id-r16     BWP-Id                                          OPTIONAL,   -- Cond MultipleNonDormantBWP

firstOutsideActiveTimeBWP-Id-r16    BWP-Id                                          OPTIONAL,   -- Cond MultipleNonDormantBWP-WUS

ca-SlotOffset-r16                   CHOICE {

refSCS15kHz                         INTEGER (-2..2),

refSCS30KHz                         INTEGER (-5..5),

refSCS60KHz                         INTEGER (-10..10),

refSCS120KHz                        INTEGER (-20..20)

}                                                                                   OPTIONAL,   -- Cond AsyncCA

channelAccessConfig-r16            ChannelAccessConfig-r16                         OPTIONAL    -- Need M

]]

}

UplinkConfig ::=                    SEQUENCE {

initialUplinkBWP                    BWP-UplinkDedicated                                         OPTIONAL,   -- Need M

uplinkBWP-ToReleaseList             SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                  OPTIONAL,   -- Need N

uplinkBWP-ToAddModList              SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink              OPTIONAL,   -- Need N

firstActiveUplinkBWP-Id             BWP-Id                                                      OPTIONAL,   -- Cond SyncAndCellAdd

pusch-ServingCellConfig             SetupRelease { PUSCH-ServingCellConfig }                    OPTIONAL,   -- Need M

carrierSwitching                    SetupRelease { SRS-CarrierSwitching }                       OPTIONAL,   -- Need M

...,

[[

powerBoostPi2BPSK                   BOOLEAN                                                     OPTIONAL,   -- Need M

uplinkChannelBW-PerSCS-List         SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier         OPTIONAL    -- Need S

]],

[[

bdFactorR-r16                       ENUMERATED {n1}                                             OPTIONAL,   -- Need R

lte-CRS-PatternList-r16             SetupRelease { LTE-CRS-PatternList-r16 }                    OPTIONAL,   -- Cond LTE-CRS

lte-CRS-PatternListSecond-r16       SetupRelease { LTE-CRS-PatternList-r16 }                    OPTIONAL,   -- Cond CORESETPool

enablePLRS-UpdateForPUSCH-SRS       ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUSCH0       ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUCCH        ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

enableDefaultBeamPL-ForSRS          ENUMERATED {enabled}                                        OPTIONAL    -- Need R

]]

}

ChannelAccessConfig-r16 ::=            SEQUENCE {

maxEnergyDetectionThreshold-r16         INTEGER(-85..-52),

energyDetectionThresholdOffset-r16      INTEGER (-20..-13),

ul-toDL-COT-SharingED-Threshold-r16     INTEGER (-85..-52)    OPTIONAL,   -- Need R

absenceOfAnyOtherTechnology-r16         ENUMERATED {true}     OPTIONAL    -- Need R

}

ul-toDL-COT-SharingED-Threshold

Maximum energy detection threshold that the UE should use to share channel occupancy with gNB for DL transmission with length no longer than 2, 4, and 8 OFDM symbols for 15Khz, 30Khz, 60KHz SCS respectively, as specified in TS 37.213 [48].

    - RIL [S651]-MIMO

As shown, in current 38.331 ASN.1, lte-CRS-ToMatchAround is placed directly under ServingCellConfig, As, lte-CRS-ToMatchAround, lte-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 are all for CRS rate matching pattern configuration. RIL [S651] thinks that, both lte-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 should be aligned with lte-CRS-ToMatchAround.

As way of aligning lte-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 with lte-CRS-ToMatchAround in ASN.1, RIL [S651] proposes to move lte-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 from UplinkConfig and place them under ServingCellConfig.

Q1: Do companies agree to align lte-CRS-PatternList-r16 and lte-CRS-PatternListSecond-r16 with lte-CRS-ToMatchAround by moving them from UplinkConfig to ServingCellConfig?

| Company | Yes/No | Comments |
| --- | --- | --- |

    - RIL [S652]-MIMO

According to [1], BDFactorR is an optional UE-specific per DL serving cell parameter for determining and distributing the maximum numbers of BD/CCE for mPDCCH based mPDSCH transmission. But, as shown in 38.331 ASN.1, bdFactorR is captured under UplinkConfig. So, the RIL[S652] proposes to move bdFactorR from UplinkConfig to ServingCellConfig or PDCCH-servingCellConfig.

Q2: To align with RAN1 parameter list for Rel-16 [1] description, bdFactorR should be move from UplinkConfig to:

- ServingCellConfig

- PDCCH-servingCellConfig

| Company | Option: a or b | Comments |
| --- | --- | --- |

    - RIL[Z015]-NR-U

As shown in 38.331 ASN.1, ul-toDL-COT-SharingED-Threshold (the Maximum energy detection threshold that the UE should use to share channel occupancy with gNB for DL transmission with length no longer than 2, 4, and 8 OFDM symbols for 15Khz, 30Khz, 60KHz SCS respectively) is captured with only a need code R.

At RAN1#98bis, RAN1 has made the following agreement:

Agreement: - The ED threshold that the UE applies when initiating a channel occupancy to be shared with the gNB is configured by gNB (RRC signaling) - if ED threshold that the UE applies when initiating a channel occupancy to be shared with the gNB is not configured, the transmission of the gNB in UE initiated COT may include only control/broadcast signals/channels transmissions of up to 2/4/8 OFDM symbols in duration for 15/30/60 kHz SCS.

According to the above RAN1 agreement RIL [Z015] thinks that there is a default behavior that needs to be specified when the field is not signaled. So, the need code needs to reflect that behavior and we need to specify the default behavior according to this agreement.

Q3: Do companies agree to update the need code of ul-toDL-COT-SharingED-Threshold and define the default behavior? If Yes what may be the expected UE behavior?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | The current field description says “, as specified in TS 37.213”, then I guess we can refer to the 213 which is supposed to capture the default behavior when the parameter is not configured |
| MediaTek | No | It seems not necessary. The default behavior while not configured is already clear in RAN1 specificaiton. |
| vivo | No | When this field is not configured, the corresponding UE behavior has been explicitly specified in TS 37.213 subclause 4.1.3. Thus we think it is okay to keep “Need R”. |

  - ServingCellConfigCommon RIL [Z019]-NR-U

ServingCellConfigCommon ::=         SEQUENCE {

physCellId                          PhysCellId                                                          OPTIONAL,   -- Cond HOAndServCellAdd,

downlinkConfigCommon                DownlinkConfigCommon                                                OPTIONAL,   -- Cond HOAndServCellAdd

uplinkConfigCommon                  UplinkConfigCommon                                                  OPTIONAL,   -- Need M

supplementaryUplinkConfig           UplinkConfigCommon                                                  OPTIONAL,   -- Need S

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                                   OPTIONAL,   -- Need S

ssb-PositionsInBurst                CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}                                                                                                       OPTIONAL, -- Cond AbsFreqSSB

ssb-periodicityServingCell          ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }   OPTIONAL, -- Need S

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                            OPTIONAL, -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL, -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL, -- Need N

ssbSubcarrierSpacing                SubcarrierSpacing                                                   OPTIONAL, -- Cond HOAndServCellWithSSB

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                              OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...,

[[

channelAccessMode-r16               CHOICE {

dynamic                             NULL,

semistatic                          SemiStaticChannelAccessConfig

}                                                                                                       OPTIONAL, -- Need M

discoveryBurst-WindowLength-r16         ENUMERATED {s0dot5, s1, s2, s3, s4, s5}                         OPTIONAL, -- Need M

ssb-PositionQCL-r16                     SSB-PositionQCL-Relationship-r16                                OPTIONAL, -- Need M

intraCellGuardBandUL-r16                IntraCellGuardBand-r16                                          OPTIONAL, -- Need M

intraCellGuardBandDL-r16                IntraCellGuardBand-r16                                          OPTIONAL  -- Need M

]]

}

IntraCellGuardBand-r16 ::=          SEQUENCE (SIZE (1..ffsValue)) OF GuardBand-r16 -- FFS upper size 4, assuming 100Mhz cell

GuardBand-r16       ::=   SEQUENCE {

startCRB-r16             INTEGER (0..ffsValue), --FFS upper range 275

nrofCRBs-r16             INTEGER (1..ffsValue)

}

RAN1 has made the following agreement:

Agreement: For FBE operation - FFP configuration is included in SIB-1 - FFP configuration can be signaled for a UE with UE-specific RRC signaling

As shown in 38.331 ASN.1, the semiStaticChannelAccessConfig is captured under ServingCellConfigCommon. Based on above RAN1 agreement the RIL[Z019] thinks as the semiStaticChannelAccessConfig field can be UE specific, then it is proposed to add the FFP semiStaticChannelAccessConfig to servingCellConfig instead of servingCellConfigCommon.

Q4: Do companies agree to move the FFP semiStaticChannelAccessConfig from servingCellConfigCommon to servingCellConfig instead?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Maybe not | It seems better to keep this in common configuration (servingCellConfigCommon) and add also this parameter to dedicate configuration (servingCellConfig). It would be up to NW that whether it want to put the value in common field (so all UE use the same value) or it want to have UE specific configuration. |
| vivo | No | In our understanding, the above-mentioned configuration is a cell-specific parameter rather than a UE-specific parameter. We are not sure whether the NW would configure the UE-specific configuration. Thus, we prefer to keep the current text. |

  - MCGFailureInformation RIL [M005]-DCCA/MDT

For measResultFreqListEUTRA, the procedural text is described as follows:

1> for each EUTRA frequency the UE is configured to measure by measConfig for which measurement results are available: 2> set the measResultFreqListEUTRA to include the best measured cells, ordered such that the best cell is listed first using RSRP to order if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

RIL[M005] pointed out that it included several neighbor cells per frequency. However, ASN.1 allows only one neighbor cell per frequency [2].

-- ASN1START

-- TAG-MCGFAILUREINFORMATION-START

MCGFailureInformation-r16 ::=    SEQUENCE {

criticalExtensions               CHOICE {

mcgFailureInformation-r16        MCGFailureInformation-r16-IEs,

criticalExtensionsFuture         SEQUENCE {}

}

}

MCGFailureInformation-r16-IEs ::= SEQUENCE {

failureReportMCG-r16              FailureReportMCG-r16                             OPTIONAL,

nonCriticalExtension              SEQUENCE {}                                      OPTIONAL

}

FailureReportMCG-r16 ::=          SEQUENCE {

failureType-r16                   ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, spare},

measResultFreqList-r16            MeasResultList2NR                                OPTIONAL,

measResultFreqListEUTRA-r16       MeasResultList2EUTRA                             OPTIONAL,

measResultSCG-r16                 OCTET STRING (CONTAINING MeasResultSCG-Failure)  OPTIONAL,

measResultSCG-EUTRA-r16           OCTET STRING                                     OPTIONAL,

...

}

MeasResultList2EUTRA ::=          SEQUENCE (SIZE (1..maxNrofServingCellsEUTRA)) OF MeasResult2EUTRA

-- TAG-MCGFAILUREINFORMATION-STOP

-- ASN1STOP

measResultFreqListEUTRA

The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by measConfig associated with the MCG.

MeasResult2EUTRA information element

-- ASN1START

-- TAG-MEASRESULT2EUTRA-START

MeasResult2EUTRA ::=       SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

measResultServingCell               MeasResultEUTRA                 OPTIONAL,

measResultBestNeighCell             MeasResultEUTRA                 OPTIONAL,

...

}

-- TAG-MEASRESULT2EUTRA-STOP

-- ASN1STOP

Q5-1: Do companies agree that there is discrepancy between measResultFreqListEUTRA procedural text and ASN.1 for measResultFreqListEUTRA in MCGFailureInformation in term of neighbor cell number per frequency that UE can report?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | Regarding the EUTRA measurements results in MCGFailureInformation it has been applied the same principle of the SCGFailureInformation. Therefore, we think that nothing is needed here. |
| Huawei, HiSilicon | No | We do not see a strong need of these changes. In addition, in SCGFailureInformationEUTRA message, there are similar field and procedural text, and that definition applies the same principle of MCGFailureInformation. |
| ZTE | Yes | First, during email discussion [Post109e#37][DCCA], it is already agreed to change maxNrofServingCellsEURA into maxFreq, because UE should be allowed to include measurement results of multiple EUTRAN frequencies. MeasResultList2EUTRA ::= SEQUENCE (SIZE (1..maxNrofServingCellsEUTRAmaxFreq)) OF MeasResult2EUTRA So each entry of the list can include measurement results of multiple neighbour cells on the same ETURA frequency. We agree with this RIL, and proposed change. |
| MediaTek | Yes (Proponent) | We understand similar procedure text and ASN.1 code is used for SCGFailureInformationEUTRA, which means it is already problematic from Rel-15. Anyway, the use of SCGFailureInformationEUTRA is to report the measurement result of LTE serving cells and we are discussing here is the LTE neighbor cell measurement result configured by MN. We think make sense to allow multiple cells reported per frequency as normal measurement reporting. The change is quite simple as MDT WI already introduce a new IE for this. |
| CATT | Yes, but | I agree with that the UE should include more than one neighbor cells measurement results per frequency according to the procedure text, but the ASN.1 only support one neighbor cell per frequency. If it is a problem needed to be solve, the SCG failure information report should also need to be modify. The same ASN.1 structure and text procedure are used for SCGFailureInformationEutra message. > set the measResultFreqListMRDC to include the best measured cells, ordered such that the best cell is listed first using RSRP to order if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available; FailureReportSCG-EUTRA ::= SEQUENCE { failureType ENUMERATED { t313-Expiry, randomAccessProblem, rlc-MaxNumRetx, scg-ChangeFailure, spare4, spare3, spare2, spare1}, measResultFreqListMRDC MeasResultFreqListFailMRDC OPTIONAL, measResultSCG-FailureMRDC OCTET STRING OPTIONAL, ..., [[ locationInfo-r16 LocationInfo-r16 OPTIONAL ]] } MeasResultFreqListFailMRDC ::= SEQUENCE (SIZE (1.. maxFreq)) OF MeasResult2EUTRA |
| OPPO | Tend to NO | Agree with Huawei, we can keep the same principle as SCGFailureInformation. |
| vivo | Yes, but | We agree that the is some inconsistency between the procedural text and ASN.1 But we agree with CATT that is also an issue for. SCGFailureInformation. |

MeasResultList2NR-r16 ::=            SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2NR-r16

MeasResultList2EUTRA-r16 ::=         SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2EUTRA-r16

MeasResult2NR-r16 ::=                SEQUENCE {

ssbFrequency-r16                     ARFCN-ValueNR                       OPTIONAL,

refFreqCSI-RS-r16                    ARFCN-ValueNR                       OPTIONAL,

measResultList-r16                   MeasResultListNR

}

MeasResultListLogging2NR-r16 ::=     SEQUENCE(SIZE (1..maxFreq)) OF MeasResultListLoggingNR-r16

MeasResultListLoggingNR-r16 ::=      SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultLoggingNR-r16

MeasResultLoggingNR-r16 ::=          SEQUENCE {

physCellId-r16                       PhysCellId,

resultsSSB-Cell-r16                  MeasQuantityResults,

numberOfGoodSSB-r16                  INTEGER (1..maxNrofSSBs-r16) OPTIONAL

}

MeasResult2EUTRA-r16 ::=             SEQUENCE {

carrierFreq-r16                      ARFCN-ValueEUTRA,

measResultList-r16                   MeasResultListEUTRA

}

As shown in ASN.1, in R16 MDT WI, the IE MeasResult2EUTRA-r16 is added which could report multiple neighbor cells per frequency. To resolve the above issue discussed in [M005], it is suggested that MCG failure information also use this MeasResult2EUTRA-r16 IE

Q5-2: If the ANS to Q5-1 is YES, do companies agree to use MeasResult2EUTRA-r16 IE instead of MeasResult2EUTRA IE for measResultFreqListEUTRA in MCGFailureInformation?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | See comment to Q5-1. |
| ZTE | Yes | See comment to Q5-1. |
| MediaTek | Yes | See comment to Q5-1. |
| CATT |  | See comment to Q5-1. |
| vivo | Yes |  |

  - RRCReconfiguration RIL [Z265]-MobEnh

RIL [Z265] pointed out that, it would better to clarify that “the cell” is “the PCell” to restrict the scenario of target CHO configuration in the legacy HO command since the change of SCell in masterCellGroup is allowed.

Q6-1: Do companies agree that it is necessary to clarify that “the cell” is “the PCell” to restrict the scenario of target CHO configuration in the legacy HO command since the change of SCell in masterCellGroup is allowed?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| MediaTek | Yes | We agree the intention |
| CATT | yes | Agree to make it clear. |
| vivo | Yes |  |

For solution to clarify “the cell” is “the PCell”, RIL [Z265] further proposes to change “cell” and “serving cell” to “PCell” and “serving PCell” respectively in the following field description sentence “The field is absent if dapsConfig is configured for any DRB or the cell indicated in masterCellGroup is different from the serving cell”. Based on rapporteur comment to this RIL, as an alternative solution, it was also proposed to consider to build the condition on “SpCell change”. E.g. “This field is absent upon SpCell change and when dapsConfig is configured for any DRB”.

Q6-2: If the ANS to Q6-1 is YES, do companies agree to reflect the restriction that the scenario of target CHO configuration in target CHO command is not supported in Rel-16 by:

- Update the sentence “The field is absent if dapsConfig is configured for any DRB or the cell indicated in masterCellGroup is different from the serving cell” by changing “Cell” and “serving cell” to “PCell” and “serving PCell” respectively

- Build condition on “SpCell change” e.g. “This field is absent upon SpCell change and when dapsConfig is configured for any DRB”

- “This field is absent upon PCell handover, PSCell change and when dapsConfig is configured for any DRB”

| Company | Option: a) or b) | Comments |
| --- | --- | --- |
| OPPO | Option c) | Not sure SpCell change really covers PCell handover, especially for the case of intra-cell handover. So we propose a modified option, i.e. option c) |
| Huawei, HiSilicon | Option b) | In the 38.331 ASN1 excel file, there is a rapporteur view as below: Rapp1: Could rather consider a Cond. Also consider to biuld the condition on “Spcell change”. E.f. ““This field is absent upon SpCell change and when dapsConfig is configured for any DRB”. We share the same view with Rapporteur’s comment, i.e the whole field description can be reworded following the restriction pattern suggested by Rapporteur. Option b) is the same as proposed by Rapp1. |
| MediaTek | Option b) | To build a conditional code is a formal way to have this kind of configuration restriction. No strong view, we slightly prefer the suggestion from rapporteur. |
| CATT | No strong view | Both options can make the present restriction for the ConditionalReconfiguration. But we wonder whether the “SpCell change” or “PCell indicated in masterCellGroup is different from the serving PCell” is accurate to cover all the legacy cases, considering the case of intra-cell change to update the security key is also a kind of legacy reconfiguration with sync which is ambiguous whether it belongs to SpCell change. So We suggest making the restriction based on the presence of the filed “ReconfigurationWithSync” in the cell group. |
| vivo | Option b) | Building condition is clear and formal. |

  - SIB2 RILs [Q002], [Q003], [Q004], SIB4 RIL [Q005]

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=                            SEQUENCE {

cellReselectionInfoCommon           SEQUENCE {

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)          OPTIONAL,       -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                     OPTIONAL,       -- Need S

rangeToBestCell                     RangeToBestCell                                 OPTIONAL,       -- Need R

q-Hyst                              ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars           SEQUENCE {

mobilityStateParameters             MobilityStateParameters,

q-HystSF                        SEQUENCE {

sf-Medium                       ENUMERATED {dB-6, dB-4, dB-2, dB0},

sf-High                         ENUMERATED {dB-6, dB-4, dB-2, dB0}

}

}                                                                                   OPTIONAL,       -- Need R

...,

[[

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvalutation-r16          SEQUENCE {

s-SearchDeltaP-r16                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1}                 OPTIONAL,       -- Need S

t-SearchDeltaP-r16                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}         OPTIONAL        -- Need S

}                                                                                   OPTIONAL,       -- Cond OptMandatory

cellEdgeEvalutation-r16             SEQUENCE {

s-SearchThresholdP-r16              ReselectionThreshold                        OPTIONAL,       -- Need R

s-SearchThresholdQ-r16              ReselectionThresholdQ                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Cond OptMandatory

relaxedMeasCondition-r16            ENUMERATED {

lowMobilityOrNotAtCellEdge,

lowMobilityAndNotAtCellEdge}                OPTIONAL,       -- Cond MultRelaxCriteria

highPriorityMeasRelax-r16           ENUMERATED {true}                               OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]]

},

cellReselectionServingFreqInfo      SEQUENCE {

s-NonIntraSearchP                   ReselectionThreshold                            OPTIONAL,       -- Need S

s-NonIntraSearchQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need S

threshServingLowP                   ReselectionThreshold,

threshServingLowQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need R

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                      OPTIONAL,       -- Need R

...

},

intraFreqCellReselectionInfo        SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                      OPTIONAL,       -- Need R

q-QualMin                           Q-QualMin                                       OPTIONAL,       -- Need S

s-IntraSearchP                      ReselectionThreshold,

s-IntraSearchQ                      ReselectionThresholdQ                           OPTIONAL,       -- Need S

t-ReselectionNR                     T-Reselection,

frequencyBandList                   MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need S

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need R

p-Max                               P-Max                                           OPTIONAL,       -- Need S

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

ss-RSSI-Measurement                 SS-RSSI-Measurement                             OPTIONAL,       -- Need R

ssb-ToMeasure                       SSB-ToMeasure                                   OPTIONAL,       -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

...,

[[

t-ReselectionNR-SF                  SpeedStateScaleFactors                          OPTIONAL        -- Need N

]],

[[

smtc2-LP-r16                        SSB-MTC2-LP-r16                                 OPTIONAL,        -- Need R

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relationship-r16                OPTIONAL         -- Need R

]]

},    ...

}

RangeToBestCell    ::= Q-OffsetRange

-- TAG-SIB2-STOP

-- ASN1STOP

SIB2 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

cellEdgeEvalutation Indicates the criteria for a UE to detect that it is not at cell edge, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.X.2).

cellReselectionInfoCommon Cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection.

cellReselectionServingFreqInfo Information common for non-intra-frequency cell re-selection i.e. cell re-selection to inter-frequency and inter-RAT cells.

deriveSSB-IndexFromCell This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the serving frequency as specified in TS 38.133 [14].

frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in subclause 5.2.2.4.3.

highPriorityMeasRelax Indicates whether measurements can be relaxed on high priority frequencies (see TS 38.304 [20], clause 5.2.4.X.0). If the field is absent, the UE shall not relax measurements on high priority frequencies

intraFreqCellReselectionInfo Cell re-selection information common for intra-frequency cells.

lowMobilityEvalutation Indicates the criteria for a UE to detect low mobility, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.X.1).

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent the UE uses the measurement quantity as specified in TS 38.304 [20].

p-Max Value in dBm applicable for the intra-frequency neighbouring NR cells. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39].

q-Hyst Parameter "Qhyst" in TS 38.304 [20], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on.

q-HystSF Parameter "Speed dependent ScalingFactor for Qhyst" in TS 38.304 [20]. The sf-Medium and sf-High concern the additional hysteresis to be applied, in Medium and High Mobility state respectively, to Qhyst as defined in TS 38.304 [20]. In dB. Value dB-6 corresponds to -6dB, dB-4 corresponds to -4dB and so on.

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells.

rangeToBestCell Parameter "rangeToBestCell" in TS 38.304 [20]. The network configures only non-negative (in dB) values.

relaxedMeasCondition When both lowMobilityEvalutation and cellEdgeEvalutation are present in SIB2, this parameter configures the condition for the UE to relax measurements (see TS 38.304 [20], clause 5.2.4.X.0).

relaxedMeasurement Configuration to allow relaxation of RRM measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.X).

s-IntraSearchP Parameter "SIntraSearchP" in TS 38.304 [20].

s-IntraSearchQ Parameter "SIntraSearchQ2 in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SIntraSearchQ.

s-NonIntraSearchP Parameter "SnonIntraSearchP" in TS 38.304 [20]. If this field is absent, the UE applies the (default) value of infinity for SnonIntraSearchP.

s-NonIntraSearchQ Parameter "SnonIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SnonIntraSearchQ.

s-SearchDeltaP Parameter "SSearchDeltaP" in TS 38.304 [20]. Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on. If the field is absent, the UE applies the (default) value of 6 dB for s-SearchDeltaP.

s-SearchThresholdP Parameter "SSearchThresholdP" in TS 38.304 [20].

s-SearchThresholdQ Parameter "SSearchThresholdQ" in TS 38.304 [20].

Smtc Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequnecy cells.

smtc2-LP-r16 Measurement timing configuration for intra-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP-r16. The timing offset and duration are equal to the offset and duration indicated in smtc in intraFreqCellReselectionInfo. The periodicity in smtc2-LP-r16 can only be set to a value strictly larger than the periodicity in smtc in intraFreqCellReselectionInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP-r16 cannot be configured). The pci-List, if present, includes the physical cell identities of the intra-frequency neighbour cells with Long Periodicity. If smtc2-LP-r16 is absent, the UE assumes that there are no intra-frequency neighbour cells with a Long Periodicity.

ssb-PositionQCL-Common Indicates the QCL relationship between SS/PBCH blocks for intra-frequency neighbor cells as specified in TS 38.213 [13], clause 4.1.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent the UE measures on all SS-blocks.

t-ReselectionNR Parameter "TreselectionNR" in TS 38.304 [20].

t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 38.304 [20]. If the field is absent, the UE behaviour is specified in TS 38.304 [20].

threshServingLowP Parameter "ThreshServing, LowP" in TS 38.304 [20].

threshServingLowQ Parameter "ThreshServing, LowQ" in TS 38.304 [20].

t-SearchDeltaP Parameter "TSearchDeltaP" in TS 38.304 [20]. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on. If the field is absent, the UE applies the (default) value of 60 seconds for t-SearchDeltaP.

| Conditional Presence | Explanation |
| --- | --- |
| MultRelaxCriteria | The field is mandatory present if lowMobilityEvalutation and cellEdgeEvalutation are present in SIB2; otherwise it is absent. |
| OptMandatory | Either lowMobilityEvalutation or cellEdgeEvalutation field is mandatory present if relaxedMeasurement is configured. The field is optionally present, Need R, otherwise. |

    - RIL [Q002]-PowSave

As highlighted in yellow in ASN.1, RIL[Q002] pointed out that relaxedMeasurement-r16 field is implemented before release-15 fields. And further suggests to move this field at the end of release-15 fields.

Q7: Do companies agree to move the relaxedMeasurement-r16 extension field at the end after Release-15 fields?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Extension has to be at the end otherwise R16 isn’t backwards compatible. |
| OPPO | Yes | Agree with Huawei. |
| Intel | Yes | Note that the impacted sections are also updated in the drafted PWS CR to 38.331 discussed on email discussion #504 (details provided in RIL [Q003] below), therefore we wonder whether this change should be implemented there. |
| MediaTek | Yes |  |
| CATT | No strong view | The current ASN.1 is considered as an extension within cellReselectionInfoCommon, which means cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection. The current extension is ok. But if companies want to define a separate and clear IE for relaxed measurement, it’s ok to extend at the end. No strong view. |
| vivo | Yes | The latest extension has to be at the end. |
| ZTE | Yes |  |

    - RIL [Q003]-PowSave

RIL [Q003] think that the use of need codes is not sufficiently clear and may be confusing. RIL [Q003] understands that:

- In case of low mobility based relaxation, s-SearchDeltaP-r16 is mandatory present and t-SearchDeltaP-r16 is optional.

- In case of not-at-cell-edge based relaxation, at least one of s-SearchThresholdP-r16 and s-SearchThresholdQ-r16 shall be configured.

RIL [Q003] suggests to sufficiently capture 38.304 logics such as the relaxed measurement requires either low mobility based or not-at-cell-edge based condition to be configured. RIL [Q003] further proposes the following changes:

- Make s-SearchDeltaP-r16 as mandatory present.

- Change t-SearchDeltaP-r16 need code to Need R

- Remove the condition of lowMobilityEvalutation-r16 and the need code Need R

- Remove the condition of cellEdgeEvalutation-r16 and the need code Need R

Q8: Do companies agree to update relaxedMeasurement-r16 with the following changes:

- Make s-SearchDeltaP-r16 as mandatory present.

- Change t-SearchDeltaP-r16 need code to Need R

- Remove the condition of lowMobilityEvalutation-r16 and the need code Need R

- Remove the condition of cellEdgeEvalutation-r16 and the need code Need R

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Partially Yes | s-SearchDeltaP-r16, t-SearchDeltaP-r16: - We think both IEs should be MP as it saves 1 bit per IE in SIBs. It was previously agreed to be optional with default but there is no good reason for that. s-SearchThresholdP-r16: - should also be MP as it is required for the cell edge evaluation. s-SearchThresholdQ-r16: - should remain as optional, this is really optional. |
| OPPO |  | This issue has been discussed in the power saving session, and the following agreements were made. 1. IEs s-SearchDeltaP and t-searchDeltaP are mandatory fields [CB if an issue is identified]. 2. Leave it to NW implementation to ensure that at least lowMobilityEvalutation or cellEdgeEvalutation IEs are present when relaxedMeasurement is configured. Based on the agreement, both s-SearchDeltaP and t-searchDeltaP are mandatory present. And we agree with the proposed change for lowMobilityEvalutation-r16 and cellEdgeEvalutation-r16. |
| Intel | - | No need to re-discuss this issue as it was already discussed on Monday CB session as part of email discussion #504 (with report on R2-2003957). Related agreements copied below for your reference: - Update relaxedMeasCondition IE to a Boolean flag ‘combineRelaxedMeasConditions’ - IEs s-SearchDeltaP and t-searchDeltaP are mandatory fields [CB if an issue is identified] - Leave it to NW implementation to ensure that at least lowMobilityEvalutation or cellEdgeEvalutation IEs are present when relaxedMeasurement is configured. |
| CATT | - | This issue has already been discussed in power saving WI. |
| vivo |  | We shall follow the latest Agreements[2020/4/28] - Update relaxedMeasCondition IE to a Boolean flag ‘combineRelaxedMeasConditions’ - IEs s-SearchDeltaP and t-searchDeltaP are mandatory fields [CB if an issue is identified] Leave it to NW implementation to ensure that at least lowMobilityEvalutation or cellEdgeEvalutation IEs are present when relaxedMeasurement is configured. |
| ZTE |  | It has been discussed under power saving WI with the following agreements made; 1.Update relaxedMeasCondition IE to a Boolean flag ‘combineRelaxedMeasConditions’ 2.IEs s-SearchDeltaP and t-searchDeltaP are mandatory fields [CB if an issue is identified] 3.Leave it to NW implementation to ensure that at least lowMobilityEvalutation or cellEdgeEvalutation IEs are present when relaxedMeasurement is configured. For agreement 2, the original agreement was to have both IEs optional with a default value, which means we should either use OTPIONAL Need R or OPTIONAL Need S. Since we have agreed the general principle to reduce the use of Need S, we suggest to use Need R for s-SearchDeltaP and t-searchDeltaP. If companies would like to make them mandatory, we may have to revise the agreement we made about the default value. |

    - RIL [Q004], [Q005]-TEI

RIL [Q004] and RIL [Q005] are common issues for SIB2 and SIB4. They expressed concern that, as the (first) smtc field can be absent and the UE assumes 5ms for all cells, UE behaviour is not entirely clear in case of absence of pci-List. So, RIL [Q004] proposes to clarify UE behavior and consider correcting the need code for pci-List in SSB-MTC2-LP-r16, depending on the outcome.

SSB-MTC2-LP-r16 ::=                 SEQUENCE {

pci-List                            SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,   -- Need R

periodicity                         ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

Q9: Do companies agree that more clarification is needed on UE behaviour in case of absence of pci-List, and further correcting the need code for pci-List? If yes, what may be the expected behavior?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | For both Q004 and Q0005: Not needed because the pci-List in SSB-MTC2-LP is “need R” instead of “need S”. Besides, there’s no reason for the network to configure SSB-MTC2-LP while not including the pci-List. For the smtc2 in measObjectNR, there’s no similar default behavior either. |
| ZTE | Yes | We think for system information, when providing SSB-MTC2-LP-r16, pci-list must be included. So we are ok to remove the “OPTIONAL –Need R”, to make pci-list mandatory within SSB-MTC2-LP-r16 structure. |
| MediaTek | Yes | We think it is better to make the pci-list mandatory as it would be pointless to have SMTC2 without the cell(s) that use SMTC2. In addition, it is better to say the NW does not include SMTC2 if SMTC1 is absent. The UE does not really know what would be the offset value used for SMTC2 if SMTC1 is absent. |
| CATT | No | We think it needs to be mandatory present if SSB-MTC2-LP-r16 is present. |
| OPPO | Yes | I think the PCI list is essential for the SMTC2-LP purpose, so we can make it mandatory. |
| vivo | Yes | We thing PCI-list should be mandatory for SMTC2-LP. |

- Conclusion

Outcome

- Reference

[1] R1-2001478, “Updated consolidated parameter list for Rel-16 NR”

[2] R2-2003654 38.331 CR Discussion on MeasResult2EUTRA, MediaTek Inc.