3GPP TSG RAN WG2 Meeting #109bis-e                                      R2-200xxxx

# Electronic, 20th – 30th April 2020

Agenda item:		6.0.1

Source:			Lenovo

Title:		Report from email discussion [AT109bis-e][071][NR RIL] DiscMail10

Document for:	 	Discussion and decision

# Introduction

This contribution summarizes the discussion and result of the email discussion below that took place during RAN2#109bis-e:

[AT109bis-e][071][NR RIL] DiscMail10 (Lenovo)

Scope: Discussion and implementation of review issues.

Wanted outcome: a) Agreed RIL Status update in the email discussion report b) Agreed ASN.1/procedure text proposal included in the email discussion report.
After email discussion report is agreed, the TPs will be included in the ASN.1 Review file, for the continued ASN.1 review.

Deadline: Email discussion Stop at EOM, April 30 (short extension 1 week could be considered if needed).

In detail, the following issues from [1] and [2] were discussed in the email discussion:

| RIL # | Issue | Feature |
| --- | --- | --- |
| Q001 | SIB validity check for non-NPN-only / NPN-only cell | NPN |
| Z101 | Description of SIB1 reception | NPN |
| Z102 | Definition for selected NPN | NPN |
| H233 | Changes for MRDC-SecondaryCellGroupConfig in RRCReconfiguration and mrdc-SecondaryCellGroup-r16 in RRCResume | DCCA |
| I675 | Issues with availableCombToAddModList-r16, availableCombToReleaseList-r16 and iabDuCellId-AI-r16 | IAB |
| H199 | Description of the condition AsyncCA | DCCA |
| I644 | Need code for field monitoringCapabilityConfig-r16 | URLLC |
| B002 | IE DL-AM-RLC-v16xy in RLC-Config | URLLC |
| I654 | IE DL-AM-RLC-v16xy in RLC-Config | URLLC |
| I653 | IE DL-AM-RLC-v16xy in RLC-Config | URLLC |
| M004 | Need code for field t316-r16 | DCCA |
| S206 | Unused IE SchedulingRequestResourceConfig-v16xy | IIOT |
| Z016 | Missing field description for enableConfiguredUL-r16 | NR-U |

# Discussion

## RIL #Q001

Issue #Q001 is related to the description of SIB validity check for non-NPN-only / NPN-only cell, see highlighted part in the procedure text of 5.2.2.2.1 below. The description of the issue is shown below.

| RIL # | Description | Proposed Change |
| --- | --- | --- |
| Q001 | These bullets are related to legacy behaviour, but addtion of this text somehow implies that the UE not supporting NPN needs to check if the cell is NPN-only cell. It is clear from the following defition in section 3.1 that only NPN capable UE can identify NPN-only cell. NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo. | The following structure could be used. > if the UE is NPN capable and the cell is an NPN-only cell […new text…] > else […legacy text…] |

##### 5.2.2.2.1	SIB validity

<Text omitted>

The UE shall:

1>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the cell is non-NPN-only cell and the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

3>	if the cell is an NPN-only cell and the first NPN-Identity included in the NPN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the cell is non-NPN-only cell and the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

3>	if the cell is an NPN-only cell and the first NPN-Identity in the NPN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

Question 1: Do you agree with the intention for restructuring the description of SIB validity check for non-NPN-only / NPN-only cell as proposed by #Q001?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Qualcomm | Agree | Proponent |

## RIL #Z101, Z102

Issues #Z101, Z102 are related to the description of SIB1 reception and definition for selected NPN, see highlighted part in the procedure text of 5.2.2.4.2 below. The description of the issues is shown below.

| RIL # | Description | Proposed Change |
| --- | --- | --- |
| Z101 | Since the upper layer will provide either a selected NPN or a selected PLMN to AS layer, there is no need for UE to differentiate between a NPN-only cell and a non NPN-only cell in this case. | The following changes in the procedural text are proposed for simplicity: Upon receiving the SIB1 the UE shall: 1> store the acquired SIB1; 1> if the cell is not an NPN-only cell and the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN: 2> in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN; 1> else if the cellAccessRelatedInfo contains an entry with the NPN-Identity of the selected NPN: 2> in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding NPN-IdentityInfo containing the selected NPN; |
| Z102 | There has been clear definition for selected PLMN in TS38.304 (see below) but there is no definition for selected NPN, we suggest to add one. Selected PLMN: This is the PLMN that has been selected by the NAS, either manually or automatically. | Add definition for selected NPN as follows: Selected NPN: This is the SNPN or PNI-NPN that has been selected by the NAS, either manually or automatically. The selected SNPN is identified by a NID in combination with a PLMN ID. The selected PNI-NPN is identified by a CAG-ID in combination with a PLMN ID. |

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the cell is not an NPN-only cell and the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN;

1>	if the cellAccessRelatedInfo contains an entry with the NPN-Identity of the selected NPN:

2>	in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding NPN-IdentityInfo containing the selected NPN;

<Text omitted>

Question 2: Do you agree with the changes for the description of SIB1 reception and to add a definition for selected NPN as proposed by #Z101, Z102?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |

## RIL #H233

Issue #H233 is related to the mrdc-SecondaryCellGroup-r16 in RRCResume that was introduced in the context of DCCA. The description of the issue is shown below.

| RIL # | Description | Proposed Change |
| --- | --- | --- |
| H233 | In RRCReconfiguration there is: mrdc-SecondaryCellGroupConfig SetupRelease { MRDC-SecondaryCellGroupConfig } OPTIONAL, -- Need M and MRDC-SecondaryCellGroupConfig ::= SEQUENCE { mrdc-ReleaseAndAdd ENUMERATED {true} OPTIONAL, -- Need N mrdc-SecondaryCellGroup CHOICE { nr-SCG OCTET STRING (CONTAINING RRCReconfiguration), eutra-SCG OCTET STRING } } These two "nr-SCG" and "nr-SCG-r16" should be considered as the same parameter, so that it is possible to use the RRCReconfiguration message to modify what was configured with RRCResume. In order to achieve this RRCResume should have a SetupRelese of MRDC-SecondaryCellGroup, which needs to be made an IE. | Move the MRDC-SecondaryCellGroupConfig in RRCReconfiguration to an IE and use this IE for mrdc-SecondaryCellGroup-r16. |

RRCResume-v16xy-IEs ::=             SEQUENCE {

idleModeMeasurementReq-r16          ENUMERATED {ffs}                              OPTIONAL, -- Need N

restoreMCG-SCells-r16               ENUMERATED {true}                             OPTIONAL, -- Need N

restoreSCG-r16                      ENUMERATED {true}                             OPTIONAL, -- Need N

mrdc-SecondaryCellGroup-r16         CHOICE {

nr-SCG-r16                          OCTET STRING (CONTAINING RRCReconfiguration),

eutra-SCG-r16                       OCTET STRING

}                                                                                 OPTIONAL, -- Need M

nonCriticalExtension                SEQUENCE{}                                    OPTIONAL

}

Question 3: Do you agree with the changes for MRDC-SecondaryCellGroupConfig in RRCReconfiguration and mrdc-SecondaryCellGroup-r16 in RRCResume as proposed by #H233?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Ericsson | Disagree | What is proposed is more an enhancement than a fix for something that is broken. Further, we are not sure that we can port MRDC-SecondaryCellGroup as a separate IE in a easy way. For instance, is not clear how the mrdc-ReleaseAndAdd will be handled in this case (as this does not apply to the resume case). According to this, our preference is to stick with the current signalling. |

## RIL #I675

Issue #I675 is related to the availableCombToAddModList-r16, availableCombToReleaseList-r16 and iabDuCellId-AI-r16 which were introduced in the context of IAB. The description of the issue is shown below.

| RIL # | Description | Proposed Change |
| --- | --- | --- |
| I675 | There seems to be few issues with this addMod and Release list. 1) We normally have an index in the list and use it in the release list. CellID doesn’t look like an index 2) the list size for the addMod and Release are different – should be the same normally 3) In the IE definition of IAB-DU-CellID-AI-r16, abDuCellId-AI-r16 can directly refet to CellIdentity (i.e., there is no need to define IAB-DU-CellID-AI-r16) | Add an index to the list AvailabilityCombinationsPerCell-r16 and refer to it here. Change the size of the addMod and Release list to the same size. In the IE definition of IAB-DU-CellID-AI-r16, update as follows: iabDuCellId-AI-r16 CellIdentity |

AvailabilityIndicator-r16 ::=    SEQUENCE {

ai-RNTI-r16                      AI-RNTI-r16,

dci-PayloadSize-AI-r16           INTEGER (1..maxAI-DCI-PayloadSize-r16),

availableCombToAddModList-r16    SEQUENCE (SIZE(1..maxNrofAssociatedDUCellsPerMT-r16)) OF AvailabilityCombinationsPerCell-r16 												OPTIONAL, -- Need FFS

availableCombToReleaseList-r16   SEQUENCE (SIZE(1..maxNrofDUCells-r16)) OF CellIdentity           OPTIONAL, -- Need FFS

...

}

AI-RNTI-r16 ::=                      RNTI-Value

AvailabilityCombinationsPerCell-r16 ::= SEQUENCE {

iabDuCellId-AI-r16                      IAB-DU-CellID-AI-r16,

positionInDCI-AI-r16                    INTEGER(0..maxAI-DCI-PayloadSize-r16-1)                  OPTIONAL, -- Need FFS (M)

availabilityCombinations-r16            SEQUENCE (SIZE (1..maxNrofAvailabilityCombinationsPerSet-r16)) OF AvailabilityCombination-r16,

...

}

AvailabilityCombination-r16 ::=         SEQUENCE {

availabilityCombinationId-r16           AvailabilityCombinationId-r16,

resourceAvailability-r16                SEQUENCE (SIZE (1..maxNrofResourceAvailabilityPerCombination-r16)) OF INTEGER (0..7)

}

IAB-DU-CellID-AI-r16 ::=                CellIdentity

AvailabilityCombinationId-r16 ::=       INTEGER (0..maxNrofAvailabilityCombinationsPerSet-r16-1)

Question 4: Do you agree with the changes for availableCombToAddModList-r16, availableCombToReleaseList-r16 and iabDuCellId-AI-r16 as proposed by #I675?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Qualcomm | Agree | Agree Intel to use an index for the list. There may be some ambiguity to use CellID for addMod and Release structure. |

## RIL #H199

Issue #H199 is related to the description of the condition AsyncCA in MeasGapConfig that was introduced in the context of DCCA. The description of the issue is shown below.

| RIL # | Description | Proposed Change |
| --- | --- | --- |
| H199 | The statement it is unclear what the following statement exactly refers to "when configuring FR2 gap pattern to UE in (NG)EN-DC / NR SA with asynchronous CA involving FR2 carrier(s) , and NE-DC / NR-DC with asynchronous CA involving FR2 carrier(s))". In our understanding,"if the field" (not IE) "refServCellIndicator is set to mcg-FR2" is sufficient. Also, need code is missing in case of absence. | 1) Remove " when configuring FR2 gap pattern to UE in (NG)EN-DC / NR SA with asynchronous CA involving FR2 carrier(s), and NE-DC / NR-DC with asynchronous CA involving FR2 carrier(s)" 2) change "IE" to "the field" 3) add ",Need R" after "absent". |

MeasGapConfig ::=                   SEQUENCE {

gapFR2                              SetupRelease { GapConfig }                  OPTIONAL,   -- Need M

...,

[[

gapFR1                              SetupRelease { GapConfig }                  OPTIONAL,   -- Need M

gapUE                               SetupRelease { GapConfig }                  OPTIONAL    -- Need M

]]

}

GapConfig ::=                       SEQUENCE {

gapOffset                           INTEGER (0..159),

mgl                                 ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp                                ENUMERATED {ms20, ms40, ms80, ms160},

mgta                                ENUMERATED {ms0, ms0dot25, ms0dot5},

...,

[[

refServCellIndicator                ENUMERATED {pCell, pSCell, mcg-FR2}  OPTIONAL   -- Cond NEDCorNRDC

]],

[[

refFR2ServCellAsyncCA-r16       ServCellIndex                            OPTIONAL   -- Cond AsyncCA

]]

}

MeasGapConfig field descriptions

refFR2ServCellIAsyncCA Indicates the FR2 serving cell identifier whose SFN and subframe is used for FR2 gap calculation for this gap pattern with asynchronous CA involving FR2 carrier(s).

| Conditional Presence | Explanation |
| --- | --- |
| AsyncCA | This field is mandatory present when configuring FR2 gap pattern to UE in (NG)EN-DC / NR SA with asynchronous CA involving FR2 carrier(s), and NE-DC / NR-DC with asynchronous CA involving FR2 carrier(s) if IE refServCellIndicator is set to mcg-FR2. Otherwise, it is absent. |

Question 5: Do you agree with the changes for the description of the condition AsyncCA in MeasGapConfig as proposed by #H199?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Ericsson | Tend to agree | We agree that the explanation is not crystal clear. However, we may need some iteration to come up with a good text. What is proposed by Huawei can be taken as baseline. We also agree that the need code for when the field is absent is missing (Need R). |
| Qualcomm | Disagree suggested change 1) Agree suggested change 2) and 3) | It is not sufficient if only “refServCellIndicator” is set to mcg-FR2, which is only present in NE-DC and NR-DC: =============Copy from 38.331===================== refServCellIndicator ENUMERATED {pCell, pSCell, mcg-FR2} OPTIONAL -- Cond NEDCorNRDC ======================================= In case of (NG)EN-DC / NR SA, the field “refServCellIndicator” is absent, but a serving cell in FR2 is still required to be indicated by RRC for FR2 gap timing reference. Note that in RAN2#109-e, we have made the agreement: In (NG)EN-DC and NR SA with async CA involving FR2 carrier(s), NW indicates which FR2 serving cell as FR2 gap timing reference via a newly introduced RRC IE refFR2ServCellAsyncCA. Same as NR rel-15, SpCell in FR1 can’t be used as FR2 gap timing reference. If Huawei has better wording how to capture this agreement, we are glad to consider it. |

## RIL #I644

Issue #I644 is related to the field monitoringCapabilityConfig-r16 in PDCCH-Config that was introduced in the context of URLLC. The description of the issue is shown below.

| RIL # | Description | Proposed Change |
| --- | --- | --- |
| I644 | Need code missing | Add need code considering possibility to release the configuration. |

PDCCH-Config ::=                    SEQUENCE {

controlResourceSetToAddModList      SEQUENCE(SIZE (1..3)) OF ControlResourceSet                 OPTIONAL,   -- Need N

controlResourceSetToReleaseList     SEQUENCE(SIZE (1..3)) OF ControlResourceSetId               OPTIONAL,   -- Need N

searchSpacesToAddModList            SEQUENCE(SIZE (1..10)) OF SearchSpace                       OPTIONAL,   -- Need N

searchSpacesToReleaseList           SEQUENCE(SIZE (1..10)) OF SearchSpaceId                     OPTIONAL,   -- Need N

downlinkPreemption                  SetupRelease { DownlinkPreemption }                         OPTIONAL,   -- Need M

tpc-PUSCH                           SetupRelease { PUSCH-TPC-CommandConfig }                    OPTIONAL,   -- Need M

tpc-PUCCH                           SetupRelease { PUCCH-TPC-CommandConfig }                    OPTIONAL,   -- Need M

tpc-SRS                             SetupRelease { SRS-TPC-CommandConfig}                       OPTIONAL,   -- Need M

...,

[[

controlResourceSetToAddModList-r16  SEQUENCE (SIZE (1..5)) OF ControlResourceSet                 OPTIONAL,   -- Need N

controlResourceSetToReleaseList-r16 SEQUENCE (SIZE (1..5)) OF ControlResourceSetId-r16           OPTIONAL,   -- Need N

searchSpacesToAddModList-r16        SEQUENCE(SIZE (1..10)) OF SearchSpace-v16xy                 OPTIONAL,   -- Need N

searchSpaceSwitchingTimer-r16       INTEGER (1..ffsValue)                                       OPTIONAL,    -- Need R

searchSpaceSwitchingGroupList-r16   SEQUENCE(SIZE (1..ffsValue)) OF SearchSpaceSwitchingGroup-r16 OPTIONAL, -- Need R

uplinkCancellation-r16              SetupRelease { UplinkCancellation-r16 }                     OPTIONAL,    -- Need M

monitoringCapabilityConfig-r16      ENUMERATED { r15monitoringcapability,r16monitoringcapability } OPTIONAL

]]

}

Question 6: Which need code should be defined for field monitoringCapabilityConfig-r16 in PDCCH-Config?

| Company | Need code (N/R/M) | Additional comments |
| --- | --- | --- |
| Ericsson | M | Need R is also fine. The field value r15monitoringcapability is equivalent to that the Rel-16 feature is not configured, and so it seems okay to have need M. |
| Qualcomm | Need M | Considering the UE may not dynamically change PDCCH monitoring capability, the UE can store the field. Thus, “Need M” seems to be more appropriate. |

## RIL #B002, I654, I653

Issues #B002, I654, I653 are related to the IE DL-AM-RLC-v16xy in RLC-Config that was introduced in the context of IIOT. The description of the issues is shown below.

| RIL # | Description | Proposed Change |
| --- | --- | --- |
| B002 | New values are introduced by t-StatusProhibitExt-r16 which are not overlapping with the legacy ones in t-StatusProhibit. In such case there is no need to introduce them by t-StatusProhibitExt-r16. Instead they can be introduced as Rel-16 NCE of t-StatusProhibit. Furthermore, there is no stringent need to have extension marker in DL-AM-RLC-v16xy. | Replace “Ext-r16” by “-v16xy” and remove extension marker as shown below. DL-AM-RLC-v16xy ::= SEQUENCE { t-StatusProhibit-v16xy T-StatusProhibit-v16xy OPTIONAL -- Need N } |
| I654 | DL-AM-RLC-v16xy is not used anywhere and is an orphan. | Update where it is to be used. |
| I653 | Looks like a configuration parameter that is stored. Hence should be Need M/R. Difficult to say which one without understanding how the IE is used but the extension marker and field description implies it should be possible to release this field independently and hence Need R might be more appropriate. See also comment I654. | Change Need code to M/R. |

RLC-Config ::=                      CHOICE {

am                                  SEQUENCE {

ul-AM-RLC                           UL-AM-RLC,

dl-AM-RLC                           DL-AM-RLC

},

um-Bi-Directional                   SEQUENCE {

ul-UM-RLC                           UL-UM-RLC,

dl-UM-RLC                           DL-UM-RLC

},

um-Uni-Directional-UL               SEQUENCE {

ul-UM-RLC                           UL-UM-RLC

},

um-Uni-Directional-DL               SEQUENCE {

dl-UM-RLC                           DL-UM-RLC

},

...

}

DL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                      OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly,

t-StatusProhibit                    T-StatusProhibit

}

T-StatusProhibit ::=                ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms1200, ms1600, ms2000, ms2400, spare2, spare1}

DL-AM-RLC-v16xy ::=                 SEQUENCE {

t-StatusProhibitExt-r16             T-StatusProhibitExt-r16                     OPTIONAL,   -- Need N

...

}

T-StatusProhibitExt-r16 ::=         ENUMERATED { ms1, ms2, ms3, ms4, spare4, spare3, spare2, spare1}

RLC-Config field descriptions

t-StatusProhibit Timer for status reporting in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on.

t-StatusProhibitExt Timer for status reporting in TS 38.322 [4], in milliseconds. Value ms1 means 1 ms, value ms2 means 2 ms and so on. If this field is present, the field t-StatusProhibit is ignored and t-StatusProhibitExt is used instead.

Question 7: Do you agree with the changes for IE DL-AM-RLC-v16xy in RLC-Config as proposed by #B002, I654, I653?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Ericsson | B002: Agree I654, I653: Agree | On B002: A critical extension should be avoided. Here the comment is more on the naming such as the use of “Ext” and “-r16” versus “-v16”. We prefer an aligned approach throughout the RRC spec on how to deal with these extensions. In that regard, we agree with the suggestion. On I653: It seems it should be Need R. The Rel-15 version of t-StatusProhibit is mandatory and so if the rel-16 version is present, it would override the rel-15 version. If we use need “M”, then it seems no way to configure back to a Rel-15 version value. |
| Qualcomm | B002: Agree I654: disagree (not clear what needs to update) I653: prefer “need R” | For B002, we have the same understanding on Lenovo, and think it is aligned with the below agreement related to S352 which was made in last Monday: => We consider to Remove Ext (at least) For I654, our understanding is that it is used in URLLC to configure short t-StatusProhibit (1/2/3/4ms). And it seems nothing more needs to be updated. Maybe Intel can make clear what is suggested change. For I653, we think “need R” is more appropriate |

## RIL #M004

Issue #M004 is related to the timer t316-r16 in RLF-TimersAndConstants that was introduced in the context of DCCA. The description of the issue is shown below.

| RIL # | Description | Proposed Change |
| --- | --- | --- |
| M004 | For the timer T316 configuration, need code Need N is used. Need N means one-shot configuration field that is not stored and whose presence causes a one-time action by the UE. Value for timer T316 is not such parameter. | Change to “Need M” |

-- ASN1START

-- TAG-RLF-TIMERSANDCONSTANTS-START

RLF-TimersAndConstants ::=          SEQUENCE {

t310                                ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000, ms4000, ms6000},

n310                                ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

n311                                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

...,

[[

t311                                ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000}

]],

[[

t316-r16                        	SetupRelease {T316-r16 } 		OPTIONAL     -- Cond MCG-Only

]]

}

T316-r16 ::=         ENUMERATED {ms50, ms100, ms200, ms300, ms400, ms500, m600, ms1000, ms1500, ms2000}

-- TAG-RLF-TIMERSANDCONSTANTS-STOP

-- ASN1STOP

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need N, in the RLF-TimersAndConstants of the MCG, if the UE is configured with split SRB1 or SRB3. It is absent otherwise. |

Question 8: Should the need code for field t316-r16 in RLF-TimersAndConstants be changed to “Need M” as proposed by #M004?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Ericsson | Agree but… | We agree with the intention of this RIL. However, most likely the configuration of timer T316 will be moved to the RRCReconfiguration message. Therefore, this is something to remind the new implementation of T316 will be done. |
| Qualcomm | Agree | We understand the UE will store T316 |

## RIL #S206

Issue #S206 is related to the IE SchedulingRequestResourceConfig-v16xy in SchedulingRequestResourceConfig that was introduced in the context of IIOT. The description of the issue is shown below.

| RIL # | Description | Proposed Change |
| --- | --- | --- |
| S206 | SchedulingRequestResourceConfig-v16xy is defined but not used anywhere. | schedulingRequestResourceToAddModList-v16xy should be added in PUCCH-Config |

SchedulingRequestResourceConfig ::=     SEQUENCE {

schedulingRequestResourceId             SchedulingRequestResourceId,

schedulingRequestID                     SchedulingRequestId,

periodicityAndOffset                    CHOICE {

sym2                                    NULL,

sym6or7                                 NULL,

sl1                                     NULL,                       -- Recurs in every slot

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639)

}                                                                              OPTIONAL,   -- Need M

resource                                PUCCH-ResourceId                       OPTIONAL    -- Need M

}

SchedulingRequestResourceConfig-v16xy ::=   SEQUENCE {

phy-PriorityIndex-r16                       ENUMERATED {p0, p1}                OPTIONAL,   -- Need M

...

}

Question 9: Should IE SchedulingRequestResourceConfig-v16xy in SchedulingRequestResourceConfig be added in IE PUCCH-Config as proposed by #S206?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Ericsson | Agree |  |
| Qualcomm | Disagree | phy-PriorityIndex-r16 is used to indicate the priority of each SR resource which is identified by SchedulingRequestResourceId. It is more logical to be configured within SchedulingRequestResourceConfig |

## RIL #Z016

Issue #Z016 is related to the field enableConfiguredUL-r16 in SlotFormatCombinationsPerCell that was introduced in the context of NR-U. The description of the issue is shown below.

| RIL # | Description | Proposed Change |
| --- | --- | --- |
| Z016 | field description is missing for the field enableConfiguredUL | Add field description. E.g: If configured, UE is allowed to transmit UL signals(SRS, PUCCH, CG-PUSCH) in the set of symbols of the slot which is configured for transmission of UL signals. |

SlotFormatCombinationsPerCell ::=   SEQUENCE {

servingCellId                       ServCellIndex,

subcarrierSpacing                   SubcarrierSpacing,

subcarrierSpacing2                  SubcarrierSpacing                           OPTIONAL, -- Need R

slotFormatCombinations              SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination                                                               OPTIONAL, -- Need M

positionInDCI                       INTEGER(0..maxSFI-DCI-PayloadSize-1)        OPTIONAL, -- Need M

...,

[[

enableConfiguredUL-r16          ENUMERATED {enabled}                            OPTIONAL  -- Need N

]]

}

Question 10: Should a field description for enableConfiguredUL-r16 in SlotFormatCombinationsPerCell be added as proposed by #Z016?

| Company | Agree/Disagree | Additional comments |
| --- | --- | --- |
| Qualcomm | Agree | The added field description looks fine to us |

# Conclusion

xxx

# Reference

[1]	R2-2003309, TS 38.331 Rel-16 ASN.1 review file, phase 1, Ericsson

[2]	R2-2003310, RIL list TS 38.331 Rel-16 ASN.1 review file, phase 1, Ericsson