3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003895

Electronic, 20 – 30 April 2020

Agenda item:	6.18.2

Source:	Nokia (Rapporteur)

Title:	Offline discussion 105: PRN open issues - first round

WID/SID:	NG_RAN_PRN-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

This document is the report about the first round of the following email discussion

- [AT109bis-e][105][PRN] Open issues (Nokia)

Scope: Continue the discussion on PRN open issues, based on R2-2002659

Initial intended outcome: Set of proposals with full consensus agreeable via email, based on the list in Section 4.1 of R2-2002659 (final list to be reflected in R2-2003895)

Initial intermediate deadline (for companies' feedback): Tuesday 2020-04-21 09:00 UTC

Proposals in Section 4.1 of R2-2002659 not challenged until Tuesday 2020-04-21 09:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

# 2	Proposals from Section 4.1 of R2-2002659

Proposal 1 (Proposal 4 from R2-2002659): Remove the Editor’s Note: “It is FFS whether the above needs to capture the condition that the cell is “not reserved for operator use for UEs not belonging to AC 11 or 15” from Table 4.2-1 of 38.304.

Proposal 2 (Proposal 7 from R2-2002659): The PRN rapporteur of 38.304 (Qualcomm) will create a documentation proposal for the following agreement: “For unlicensed spectrum and for a UE with non-empty allowed CAG list, if the highest ranked cell or best cell according to absolute priority reselection rules is a cell which is not suitable due to not broadcasting the selected/registered/equivalent PLMN, the UE with no empty allowed CAG list shall behave according to NR-U agreement.” as a part of the running CR.

Proposal 3 (Proposal 8 from R2-2002659): In unlicensed band the case when the highest ranked cell or best cell is not suitable due to belonging to the correct operator, but it is not a CAG member cell is handled in the same way as the cell does not belong to the correct operator. The relevant changes are to be captured in 38.304.

Proposal 4 (Proposal 13 from R2-2002659): Follow the CT4 agreement on NID size in RRC specification. To be captured into ASN.1 review file as RIL comment (by the rapporteur) as shown below.

******** Start of Change ********

NPN-Identity information element

-- ASN1START

-- TAG-NPN-IDENTITY-START

NPN-Identity-r16 ::=             CHOICE {

pni-npn-r16                      SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

cag-IdentityList-r16             SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-Identity-r16

},

snpn-r16                         SEQUENCE {

plmn-Identity                    PLMN-Identity,

nid-List-r16                     SEQUENCE (SIZE (1..maxNPN-r16)) OF NID-r16

}

}

CAG-Identity-r16 ::=             BIT STRING (SIZE (32))

NID-r16 ::=                      BIT STRING (SIZE (44))

-- TAG-NPN-IDENTITY-STOP

-- ASN1STOP

NPN-Identity field descriptions

CAG-Identity A CAG-ID as specified in TS 23.003 [21]. The PLMN ID and a CAG ID in the NPN-Identity identifies a PNI-NPN.

cag-IdentityList The cag-IdentityList contains one or more CAG-Identity. All CAG IDs associated to the same PLMN ID are listed in the same cag-IdentityList entry.

NID A NID as specified in TS 23.003 [21]. The PLMN ID and a NID in the NPN-Identity identifies a SNPN.

nid-List The nid-List contains one or more NID.

******** End of Change ********

Proposal 5 (Proposal 14 from R2-2002659): TAC is “mandatory” within NPN-IdentityInfoList. To be captured into ASN.1 review file as RIL comment (by the rapporteur) as shown below.

******** Start of Change ********

NPN-IdentityInfoList information element

-- ASN1START

-- TAG-NPN-IDENTITYINFOLIST-START

NPN-IdentityInfoList-r16 ::=     SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-IdentityInfo-r16

NPN-IdentityInfo-r16 ::=         SEQUENCE {

npn-IdentityList-r16             SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r16,

trackingAreaCode-r16             TrackingAreaCode,

ranac-r16                        RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity-r16                 CellIdentity,

cellReservedForOperatorUse-r16   ENUMERATED {reserved, notReserved},

...

}

-- TAG-NPN-IDENTITYINFOLIST-STOP

-- ASN1STOP

NPN-IdentityInfoList field descriptions

NPN-IdentityInfo The NPN-IdentityInfo contains one or more NPN identities and additional information associated with those NPNs. Only the same type of NPNs (either SNPNs or PNI-NPNs) can be listed in a NPN-IdentityInfo element.

npn-IdentityList The npn-IdentityList contains one or more NPN Identity elements.

trackingAreaCode Indicates the Tracking Area Code to which the cell indicated by cellIdentity field belongs.

ranac Indicates the RAN Area Code to which the cell indicated by cellIdentity field belongs.

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs.

cellReservedForOperatorUse Indicates whether the cell is reserved for operator use (for the NPN(s) identified in the npn-IdentyList) as defined in TS 38.304 [20].

******** End of Change ********

Proposal 6 (Proposal 15 from R2-2002659): Use 48 octets (Option C) as the maximum size of HRNNs. To be captured into ASN.1 review file as RIL comment (by the rapporteur) as shown below.

******** Start of Change ********

maxHRNN-Len-r16                         INTEGER ::= 48 -- Maximum length of HRNNs

******** End of Change ********

Proposal 7 (Proposal 18 from R2-2002659): Follow the NR-U agreement in unlicensed on the use of the IFRI flag:

- For the SNPN case, UE only follows the IFRI in MIB of a barred cell if the cell belongs to a SNPN which matches the registered SNPN of the UE. Otherwise the UE may select other cell in the same frequency

- For the CAG (PNI-NPN) case, there is no change to the existing NR-U behaviour: UE only follows the IFRI in MIB of a barred cell if the cell belongs to a registered/selected (e)PLMN. Otherwise the UE may select other cell in the same frequency.

The agreement is to be captured in TS 38.304.

Question: Which proposal(s) from the above proposals do you disagree? (Adding your company’s view is only needed if you disagree with one or more of the proposals. Please indicate the proposal number and the reason of the disagreement in separate lines in the table below.)

| Company | Prop # | Reasoning |
| --- | --- | --- |

Summary:

# 2	Conclusions