3GPP TSG-RAN WG2 Meeting #109bis-e	Draft_R2-2004044

20-30 April 2020, online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1746 | rev | 1 | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates for Rel-16 additional enhancements NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BlackBerry |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB-IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2020-04-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR captures the agreements for NB-IoT Rel-16 at RAN2-109bis-e. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following agreements have been captured in this CR: RAN2-109-e: Scheduling Multiple DL/UL Transport Blocks (common with MTC): - For NB-IoT, support of multiTB-UL-r16 and multiTB-DL-r16 is conditional on support of two HARQ processes (NB-IoT only). - For LTE-M and NB-IoT, multiple TBs scheduling in multicast is optional without capability reporting. In addition, in the RRC NB-IoT CR, the following additional capabilties have been captured: multiTB-DL-Interleaving, multiTB-UL-Interleaving Defines whether the UE supports interleaved transmission when multiple TBs is scheduled in RRC_CONNECTED for DL and UL respectively. If multiTB-DL-Interleaving or multiTB-UL-Interleaving is included, the UE shall also indicate support for multiTB-DL or multiTB-UL respectively multiTB-HARQ-ACK-Bundling Defines whether the UE supports HARQ ACK bundling for interleaved transmission for DL. If multiTB-HARQ-ACK-Bundling is included, the UE shall also indicate support for multiTB-DL-Interleaving. It is suggested to align for now TS 36.306 with the above. This can be revisited if necessary when the UE Features list is received from RAN1. Connection to 5GC (common with MTC): - RRC Connection re-establishment for the control plane for NB-IoT UEs connected to 5GC is optional, without capability reporting (NB-IoT only). (From RAN2-108). - The existing capability multipleDRB-r13 is also applicable to 5GC (NB-IoT only). - DL channel quality report can be supported for both NB-IoT and eMTC connected to 5GC. [No impact as CN type not mentioned means that in general the capability applies to both]. - PUR is supported in EPC and 5GC. - Introduce separate UE capabilities pur-UP-5GC-r16 and pur-CP-5GC-r16. UE-group wake-up signal (WUS) (common with MTC): - Support of Release 16 WUS is independent to support of Release 15 WUS. Network management tool enhancement: - RACH report is not applicable to 5GC. - RLF report is not applicable to 5GC. - Support of RACH report is optional with capability reporting. - Support of RLF report is optional at the UE without capability reporting. Coexistence with NR: - Working assumption: Introduce two UE capabilities for handling resources reservation on DL or UL in PhyLayerParameters-NB-v16xy. - Working assumption: Two UE capabilities for handling resources reservation on DL or UL in PhyLayerParameters-NB-v16xy can be applied to both FDD and TDD, e.g., with separate values for FDD or TDD. - Working assumption: Two UE capabilities for handling resources reservation on DL or UL can be introduced into TS 36.306, e.g., section 4.3.4. RAN2-109bis-e: GWUS: Confirm the working assumption: “Support of Release 16 WUS is independent to support of Release 15 WUS”. 1-1: For NB-IoT and eMTC, the existing capability wakeUpSignalMinGap-eDRX-r15 also applies to Rel-16 WUS. 1-2: For NB-IoT, Rel-16 GWUS is only applicable to FDD. 1-3: For eMTC, separate capability indications are introduced for FDD and TDD. 1-4: For NB-IoT and eMTC, Rel-16 GWUS is applicable to both EPC and 5GC, and there is no need for capability differentiation. Multi-TB scheduling 2-1: For NB-IoT, multiple TB scheduling in unicast and in multicast is only applicable to FDD. 2-2: For NB-IoT and eMTC, multiple TB scheduling in unicast is applicable to both EPC and 5GC without differentiation. 2-3: For NB-IoT and eMTC, multiple TB scheduling in multicast is only applicable to EPC. SON 3-1: For NB-IoT, support of ANR, RACH report and RLF report are applicable to both FDD and TDD and there is no need for FDD/TDD differentiation. DL channel quality reporting in MSG3 4-1: For NB-IoT, move the feature DL channel quality reporting in MSG3 for non-anchor carrier to section 6.17. 4-2’: DL channel quality reporting in Msg3 for NB-IoT anchor carrier and DL channel quality reporting in Msg3 for eMTC are two separate optional features. 4-3’: For NB-IoT, update the description of the legacy feature DL channel quality reporting in MSG3 (6.17.2) to reflect that it applies to the anchor carrier. 4-4: For NB-IoT, DL channel quality reporting in MSG3 for non-anchor carrier is only applicable to FDD. 4-5: For NB-IoT and eMTC, DL channel quality reporting in MSG3 is applicable to both EPC and 5GC without capability differentiation. DL channel quality reporting in connected mode 5-1: Keep a common capability for NB-IoT and eMTC for DL channel quality reporting in connected mode and clarify in the description that reporting of the serving cell applies to E-UTRAN and reporting of the configured carrier applies to NB-IoT. 5-2: For NB-IoT, DL channel quality reporting in connected mode is only applicable to FDD. For eMTC, it is applicable to both FDD and TDD. 5-3: For NB-IoT and eMTC, DL channel quality reporting in connected mode is applicable to both EPC and 5GC without capability differentiation. NRS presence on non-anchor carrier 6-1: For NB-IoT, Idle mode RRM measurements on non–anchor paging carrier is only applicable to FDD. 6-2: For NB-IoT, Idle mode RRM measurement on non–anchor carrier is applicable to EPC and 5GC without capability differentiation. Co-existence with NR 8-1: For NB-IoT and eMTC, UL and DL resource reservation for coexistence with NR are applicable to EPC and 5GC without capability differentiation. Connection to 5GC 9-1: For NB-IoT, introduce a new optional feature, NB-IoT/5GC, in section 6.18. 9-2: For NB-IoT and eMTC, remove the capabilities introduced in 6.18.1 (User Plane CIoT 5GS optimisations) and 6.18.2 (Control Plane CIoT 5GS optimisations). 9-3: For NB-IoT and eMTC, introduce a new optional feature, MO-EDT for Control Plane CIoT 5GS Optimisation, in section 6.18 and remove the editor’s note in 6.8.4. 9-4’: For NB-IoT, all pre-Rel15 capabilities not CIoT EPS optimisations related and other than rai-Support-r14 are applicable to 5GC without capability differentiation. FFS rai-Support-r14. BB: no change to the description for this (green line to be removed later). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4, 4.3.4.182, 4.3.4.183, 4.3.4.a1 (new), 4.3.4.a2 (new), 4.3.4.a3 (new), 4.3.4.a4 (new), 4.3.4.a5 (new), 4.3.4.a6 (new), 4.3.4.114, 4.3.8.5, 4.3.8.b1 (new), 4.3.8.b2 (new), 4.3.12.c (new), 4.3.19.22, 6.7.6, 6.8.4, 6.10.d (new), 6.16.e (new), 6.17.2, 6.17.f (new), 6.18.1, 6.18.2, 6.18.g (new), 6.18.h (new), 6.18.i (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.331 CR xxxx |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

1xRTT	CDMA2000 1x Radio Transmission Technology

ACK	Acknowledgement

ACDC	Application specific Congestion control for Data Communication

ANDSF	Access Network Discovery and Selection Function

ANR	Automatic Neighbour Relation

BCCH	Broadcast Control Channel

CAS	Cell Acquisition Subframes

CFI	Control Format Indicator

CG	Cell Group

CRS	Cell-specific Rerefence Signal

CSG	Closed Subscriber Group

CSI	Channel State Information

DC	Dual Connectivity

DCI	Downlink Control Information

DL-SCH	Downlink Shared Channel

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

GERAN	GSM/EDGE Radio Access Network

HARQ	Hybrid Automatic Repeat Request

HRPD	High Rate Packet Data

IRC	Interference Rejection Combining

MAC	Medium Access Control

MMSE	Minimum Mean Squared Error

MO-EDT	Mobile Originated Early Data Transmission

MRO	Mobility Robustness Optimisation

MT-EDT	Mobile Terminated Early Data Transmission

MTSI	Multimedia Telephony Service for IMS

MUST	MultiUser Superposition Transmission

NAICS	Network Assisted Interference Cancellation/Suppression

NB-IoT	Narrow Band Internet of Things

OS	OFDM Symbol

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDCP	Packet Data Convergence Protocol

PDSCH	Physical Downlink Shared Channel

PHR	Power Headroom Reporting

ProSe	Proximity-based Services

PUCCH	Physical Uplink Control Channel

PUR	Preconfigured Uplink Resource

PUSCH	Physical Uplink Shared Channel

QoE	Quality of Experience

RACH	Random Access CHannel

RAI	Release Assistance Indication

RAT	Radio Access Technology

RLC	Radio Link Control

RLF	Radio Link Failure

ROHC	RObust Header Compression

RRC	Radio Resource Control

SC-PTM	Single Cell Point to Multipoint

SCC	Secondary Component Carrier

SCell	Secondary Cell

SI	System Information

SL	Sidelink

SL-DCH	Sidelink Discovery CHannel

SL-SCH	Sidelink Shared CHannel

SON	Self Organizing Networks

SPT	Short Processing Time

SR	Scheduling Request

SSAC	Service Specific Access Control

SSTD	SFN and Subframe Timing Difference

STTI	Short TTI

TDD	Time Division Duplex

TTI	Transmission Time Interval

UCI	Uplink Control Information

UDC	Uplink Data Compression

UE	User Equipment

UL-SCH	Uplink Shared Channel

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

V2X	Vehicle-to-Everything

WLAN	Wireless Local Area Network

Next changes

# 4	UE radio access capability parameters

The following clauses define the UE radio access capability parameters and minimum capabilities for MBMS capable UE. Only parameters for which there is the possibility for UEs to signal different values are considered as UE radio access capability parameters. Therefore, mandatory features without capability parameters that are the same for all UEs are not listed here. Also capabilities which are optional or conditionally mandatory for UEs to implement but do not have UE radio access capability parameter are listed in this specification.

E-UTRAN needs to respect the signalled UE radio access capability parameters when configuring the UE and when scheduling the UE.

All parameters shown in italics are signalled and correspond to a field defined in TS 36.331 [5].

For optional features, the UE radio access capability parameter indicates whether the feature has been implemented and successfully tested. For mandatory features with the UE radio access capability parameter, the parameter indicates whether the feature has been successfully tested.

The mandatory features required to be supported by a UE are the same for all UE categories unless explicitly specified elsewhere in the specifications.

Unless otherwise stated, the requirements on the maximum number of transport block bits are applicable for a TTI length of 1 ms. For other TTI lengths, the requirements shall be scaled according to clause 7.1.7 in TS 36.213 [22] in order to get the corresponding requirement.

The following UE radio access capability parameters specified in Chapter 4 are applicable in NB-IoT:

-	ue-Category-NB in NB-IoT (clause 4.1C)

-	supportedROHC-Profiles-r13 (clause 4.3.1.1A)

-	maxNumberROHC-ContextSessions-r13 (clause 4.3.1.2A)

-	rlc-UM-r15 (clause 4.3.2.5)

-	multiTone-r13 (clause 4.3.4.55)

-	multiCarrier-r13 (clause 4.3.4.56)

-	twoHARQ-Processes-r14 (clause 4.3.4.62)

-	multiCarrier-NPRACH-r14 (clause 4.3.4.75)

-	multiCarrierPaging-r14 (clause 4.3.4.76)

-	interferenceRandomisation-r14 (clause 4.3.4.80)

-	wakeUpSignal-r15 (clause 4.3.4.113)

-	wakeUpSignalMinGap-eDRX-r15 (clause 4.3.4.114)

-	mixedOperationMode-r15 (clause 4.3.4.115)

-	sr-WithHARQ-ACK-r15 (clause 4.3.4.117)

-	sr-WithoutHARQ-ACK-r15 (clause 4.3.4.118)

-	nprach-Format2-r15 (clause 4.3.4.119)

-	multiCarrierPagingTDD-r15 (clause 4.3.4.134)

-	additionalTransmissionSIB1-r15 (clause 4.3.4.137)

-	npusch-3dot75kHz-SCS-TDD-r15 (clause 4.3.4.177)

-	multiTB-UL-r16 (clause 4.3.4.182)

-	multiTB-DL-r16 (clause 4.3.4.183)

-	multiTB-UL-Interleaving-r16 (clause 4.3.4.a1)

-	multiTB-DL-Interleaving-r16 (clause 4.3.4.a2)

-	multiTB-HARQ-ACK-Bundling-r16 (clause 4.3.4.a3)

-	groupWakeUpSignal-r16 (clause 4.3.4.a4)

-	ul-NR-ResourceReservation-r16 (clause 4.3.4.a5)

-	ul-NR-ResourceReservation-r16 (clause 4.3.4.a6)

-	supportedBandList-r13 (clause 4.3.5.1A)

-	multiNS-Pmax-r13 (clause 4.3.5.16A)

-	powerClassNB-20dBm-r13 (clause 4.3.5.1A.1)

-	powerClassNB-14dBm-r14 (clause 4.3.5.1A.2)

-	dl-ChannelQualityReporting-r16 (clause 4.3.6.37)

-	accessStratumRelease-r13 (clause 4.3.8.1A)

-	multipleDRB-r13 (clause 4.3.8.5)

-	earlyData-UP-r15 (clause 4.3.8.7)

-	earlySecurityReactivation-r16 (clause 4.3.8.11)

-	pur-CP-r16 (clause 4.3.8.12)

-	pur-UP-r16 (clause 4.3.8.13)

-	pur-CP-5GC-r16 (clause 4.3.8.b1)

-	pur-UP-5GC-r16 (clause 4.3.8.b2)

-	anr-Report-r16 (clause 4.3.12.2)

-	rach-Report-r16 (clause 4.3.12.c)

-	logicalChannelSR-ProhibitTimer (clause 4.3.19.2)

-	dataInactMon-r14 (clause 4.3.19.9)

-	rai-Support-r14 (clause 4.3.19.10)

-	earlyContentionResolution-r14 (clause 4.3.19.14)

-	sr-SPS-BSR-r15 (clause 4.3.19.15)

-	rai-SupportEnh-r16 (clause 4.3.19.22)

-	earlyData-UP-5GC-r16 (clause 4.3.36.9)

The UE radio access capabilities specified in Chapter 4 are not applicable in NB-IoT, unless they are listed above.

The following optional features without UE radio access capability parameters specified in Chapter 6 are applicable in NB-IoT:

-	RRC Connection Re-establishment for the Control Plane CIoT EPS Optimization (clause 6.7.5)

-	System Information Block Type 16 (clause 6.8.1)

-	Enhanced random access power control (clause 6.8.3)

-	MT-EDT for Control Plane CIoT EPS Optimisation (clause 6.8.10)

-	MT-EDT for User Plane CIoT EPS Optimisation (clause 6.8.11)

-	EDT for Control Plane CIoT EPS Optimization (clause 6.8.4)

-	Enhanced PHR (clause 6.8.6)

-	Radio Link Failure Report for NB-IoT (clause 6.10.d)

-	SC-PTM in Idle mode (clause 6.16.1)

-	Multiple TB scheduling for multicast (clause 6.16.e)

-	Relaxed monitoring (clause 6.17.1)

-	DL channel quality reporting in Msg3 for the anchor carrier (clause 6.17.2)

-	Serving cell idle mode measurements reporting (clause 6.17.3)

-	NSSS-Based RRM measurements (clause 6.17.4)

-	NPBCH-Based RRM measurements (clause 6.17.5)

-	RRM measurements on non-anchor paging carriers (clause 6.17.6)

-	DL channel quality reporting in MSG3 for non-anchor carrier (clause 6.17.f)

-	User Plane CIoT 5GS optimisations (clause 6.18.1)

-	RRC Connection Re-establishment for the Control Plane CIoT 5GS Optimisation (clause 6.18.g)

-	NB-IoT 5GC (clause 6.18.h)

-	 MO-EDT for Control Plane CIoT 5GS Optimisation (clause 6.18.i)

The optional features without UE radio access capability parameters specified in Chapter 6 are not applicable in NB-IoT, unless they are listed above.

Next changes

#### 4.3.4.114	wakeUpSignalMinGap-eDRX-r15

This field indicates the minimum gap required between end of WUS and start of PO by a UE indicating support of extended idle mode DRX for FDD, as specified in TS 24.301 [28]. A UE indicating support of wakeUpSignalMinGap-eDRX-r15 shall also indicate support of either wakeUpSignal-r15 or groupWakeUpSignal-r16. This feature is only applicable if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB.

Next changes

#### 4.3.4.182	multiTB-UL-r16

This field indicates whether the UE supports multiple TB scheduling in the uplink for FDD as specified in TS 36.213 [22]. A UE indicating support of multiTB-UL-r16 shall also indicate support of twoHARQ-Processes-r14. This feature is only applicable if the UE supports category NB2.

#### 4.3.4.183	multiTB-DL-r16

This field indicates whether the UE supports multiple TB scheduling in the downlink for FDD as specified in TS 36.213 [22]. A UE indicating support of multiTB-DL-r16 shall also indicate support of twoHARQ-Processes-r14. This feature is only applicable if the UE supports category NB2.

Next changes

#### 4.3.4.a1	multiTB-UL-Interleaving-r16

This field indicates whether the UE supports interleaved transmissions when multiple TB scheduling is scheduled in the uplink for FDD as specified in TS 36.213 [22]. A UE indicating support of multiTB-UL-Interleaving-r16 shall also indicate support of multiTB-UL-r16. This feature is only applicable if the UE supports category NB2.

#### 4.3.4.a2	multiTB-DL-Interleaving-r16

This field indicates whether the UE supports interleaved transmissions when multiple TB scheduling is scheduled in the downlink for FDD as specified in TS 36.213 [22]. A UE indicating support of multiTB-DL-Interleaving-r16 shall also indicate support of multiTB-DL-r16. This feature is only applicable if the UE supports category NB2.

#### 4.3.4.a3	multiTB-HARQ-ACK-Bundling-r16

This field indicates whether the UE supports HARQ ACK bundling for interleaved transmission in the downlink for FDD as specified in TS 36.213 [22]. A UE indicating support of multiTB-HARQ-ACK-Bundling-r16 shall also indicate support of multiTB-DL-Interleaving-r16. This feature is only applicable if the UE supports category NB2.

#### 4.3.4.a4	groupWakeUpSignal-r16

This field indicates whether the UE supports Group WUS for FDD as specified in TS 36.211 [17], TS 36.213 [22] and TS 36.304 [14]. This feature is only applicable if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB.

#### 4.3.4.a5	ul-NR-ResourceReservation-r16

This field defines whether the UE supports UL resource reservation for NB-IoT coexistence with NR, as specified in TS 36.211 [17]. This feature is only applicable if the UE supports any ue-Category-NB.

#### 4.3.4.a6	dl-NR-ResourceReservation-r16

This field defines whether the UE supports DL resource reservation for NB-IoT coexistence with NR, as specified in TS 36.211 [17]. This feature is only applicable if the UE supports any ue-Category-NB.

Next changes

#### 4.3.6.37	dl-ChannelQualityReporting-r16

This field defines whether the UE supports DL channel quality reporting of the serving cell when the UE is operating in coverage enhancement mode A or B, or of the configured carrier for NB-IoT, in RRC_CONNECTED as specified in TS 36.331 [5]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

Next changes

#### 4.3.8.5	multipleDRB-r13

This field defines whether the UE supports multiple DRBs. This field is only applicable if the UE supports S1-U data transfer or User plane CIoT EPS Optimisation as defined in TS[28] or NG-U data transfer or User plane CIoT 5GS Optimisation as defined in TS 24.501 [39], and any ue-Category-NB. If a UE of this release supports multiple DRBs, the UE shall support two simultaneous DRBs.

Next changes

#### 4.3.8.b1	pur-CP-5GC-r16

This field indicates whether the UE supports Transmission using PUR for Control Plane CIoT 5GS optimisation as specified TS 36.300 [30]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

#### 4.3.8.b2	pur-UP-5GC-r16

This field indicates whether the UE supports Transmission using PUR for User Plane CIoT 5GS EPS optimisation as specified TS 36.300 [30]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

Editor's note: In RRC the 4 PUR capabilities are part of MAC parameters for eMTC, but are part of general parameters for NB-IoT. Need to align one way or another.

Next changes

#### 4.3.12.2	anr-Report-r16

This field defines whether the UE supports ANR measurement configuration and reporting in RRC_IDLE as specified in TS 36.304 [14] and TS 36.331 [5]. This feature is only applicable if the UE supports any ue-Category-NB.

Next changes

#### 4.3.12.c	rach-Report-r16

This field defines whether the UE supports supports delivery of rachReport upon request from the network as specified in TS 36.331 [5] when connected to EPC. This feature is only applicable if the UE supports any ue-Category-NB.

Next changes

#### 4.3.19.22	rai-SupportEnh-r16

This field defines whether the UE supports AS Release Assistance Indication (AS RAI) MAC CE as specified in TS 36.321 [4] when connected to EPC. This feature is only applicable if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB.

Next changes

### 6.7.6	Void

Next changes

### 6.8.4	MO-EDT for Control Plane CIoT EPS Optimization

It is optional for UE to support MO-EDT for Control Plane CIoT EPS optimizations, as defined in TS 24.301 [28]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

Next changes

## 6.10	SON features

### 6.10.1	Radio Link Failure Report for inter-RAT MRO

It is optional for UE to include previousUTRA-CellId and selectedUTRA-CellId in RLF-Report upon request from the network as specified in TS 36.331 [5].

### 6.10.d	Radio Link Failure Report for NB-IoT

It is optional for UE to support the storage of RLF-Report and the reporting in UEInformationResponse message as specified in TS 36.331 [5] when connected to EPC. This feature is only applicable if the UE supports any ue-Category-NB.

Next changes

## 6.16	SC-PTM features

### 6.16.1	SC-PTM in Idle mode

It is optional for UE to support the SC-PTM reception in RRC_IDLE as specified in TS 36.331 [5]. This feature is only applicable if the UE supports UE category M1 or UE category M2 or if the UE supports coverage enhancements (ce-ModeB-r13 and/or ce-ModeA-r13) or for FDD, if the UE supports any ue-Category-NB.

### 6.16.e	Multiple TB scheduling for SC-PTM in Idle mode

It is optional for UE to support multiple TB scheduling for multicast as specified in TS 36.331 [5] when connected to EPC. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

Next changes

### 6.17.2	DL channel quality reporting in Msg3 for the anchor carrier

It is optional for UE to support DL channel quality reporting in Msg3 for the anchor carrier for FDD , as specified in TS 36.331 [5]. This feature is only applicable if the UE supports any ue-Category-NB.

Next changes

### 6.17.6	RRM measurements on non-anchor paging carriers

It is optional for UE to support idle mode RRM measurements on non-anchor paging carriers for FDD, as defined in TS 36.133 [6]. This feature is only applicable if the UE supports any ue-Category-NB.

Next changes

### 6.17.f	DL channel quality reporting in MSG3 for non-anchor carrier

It is optional for UE to support DL channel quality reporting for a non-anchor carrier for FDD in MSG3, as defined in TS 36.331 [5]. This feature is only applicable if the UE supports any ue-Category-NB.

Last changes

## 6.18	E-UTRA/5GC features

### 6.18.1	Void

### 6.18.2	Void

### 6.18.g	RRC Connection Re-establishment for the Control Plane CIoT 5GS Optimisation

It is optional for UE to support RRCConnectionReestablishment for the Control Plane CIoT 5GS Optimisation as specified in TS 36.331 [5]. This feature is only applicable if the UE supports any ue-Category-NB.

### 6.18.h	NB-IoT 5GC

It is optional for UE to support NB-IoT 5GC. This feature is only applicable if the UE supports any ue-Category-NB.

### 6.18.i	MO-EDT for Control Plane CIoT 5GS Optimisation

It is optional for UE to support MO-EDT for Control Plane CIoT 5GS optimisations, as defined in TS 24.501 [39]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.