3GPP TSG-RAN WG2 #109bise	R2- 200XXXX

# Online meeting, 20th - 30th April 2020

Agenda Item:	7.2.2.

Source:	Ericsson (Rapporteur)

Title:	Report - Email discussion [AT109bis-e][310][NBIOT eMTC] WUS open issues

Document for:	Discussion, Decision

1	Introduction

RAN2 has made the following agreement in RAN2#109bis-e regarding UE group wake up signal (WUS):

“The following codepoints are used to indicate a paging probability threshold value: {p20,p30,p40,p50,p60,p70,p80,p90}”

The discussion is ongoing and in order to finalize the remaining open issues for UE group WUS, it was agreed to continue the discussion with the following offline discussion:

[AT109bis-e][310][NBIOT/eMTC] WUS open issues (Ericsson)

Scope: Remaining open issues on WUS

Intended outcome: Finalise the open issues, report in R2-2004045

Deadline: 22-04-2020, 16:00 UTC

In this document, companies are invited to provide their views regarding the remaining open issues on UE groups WUS based on the agreements made so far.

2	Discussion

2.1 Configuration of paging probability thresholds

A UE that supports Rel-16 group WUS is configured by MME with a paging probability class via NAS layer, e.g. during attach or TA update. It is up to the CN how to determine the paging probability class as it can be based on e.g., information provided by the UE, subscription information, data collected by the CN etc. In CT1 it was agreed to introduce the following values for such configuration: ““p00, p05, p10, p15, p20, p25, p30, p35, p40, p45, p50, p55, p60, p65, p70, p75, p80, p85, p90, p95, p100”.

When the network needs to reach the UE, MME sends a paging request to the eNB including the configured paging probability class so that the eNB knows which WUS group the UE is supposed to monitor prior to its paging occasion. RAN3 has been discussing what value range and resolution are sufficient to be provided from the MME to the eNB along with the paging request. There are 2 options under discussion:

- Option 1 [5-6]: introduce a WUS Assistance Information IE in S1AP PAGING Message which contains the following codepoints “p00, p05, p10, p15, p20, p25, p30, p35, p40, p45, p50, p55, p60, p65, p70, p75, p80, p85, p90, p95, p100”. (same as CT1)

-	Option 2 [7-8]: similar to option 1 but with a different set of codepoints such as “p10, p20, p30, p40, p50, p60, p80, p100”. The exact value range is FFS.

RAN2 has agreed to have a configuration where maximum number of probability thresholds is 3 giving 4 groups in a working assumption. Those thresholds are provided in probabilityThresholdList, as part of broadcast signalling in the serving cell, so that it would be possible for the UE to know which WUS group set and thus the WUS group, based on the formula agreed in RAN2#109e, it should use when monitoring for WUS.

In the paging request message from the MME, the eNB receives the configured paging probability class for the UE and pages the UE with WUS using the corresponding WUS group set and thus the WUS group by mapping it according to the probability thresholds eNB broadcasts in the serving cell.

Considering that RAN2 assumes maximum number of probability thresholds is 3 giving 4 groups, the codepoints, i.e., {p20,p30,p40,p50,p60,p70,p80,p90}, agreed during the online session on Monday, April 20th, to indicate a paging probability threshold value is associated with the configuration provided by MME via NAS layer and the information provided from the MME to the eNB along with the paging request via S1AP. Therefore, those codepoins are a recommendation to CN working groups and RAN3 from RAN2.

Discussion point 1: Do you confirm the understanding provided above? If no, please elaborate on why and provide your interpretation.

| Company | Yes / No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Lenovo | Yes |  |
| ZTE | No? | We understand the {p20,p30,p40,p50,p60,p70,p80,p90} (the value range for paging probability threshold) in SIB is mainly for segmentation of different WUS group sets in RAN2. It only needs to kind of align with the value range of paging probability in NAS, e.g., the minimum value for paging probability threshold should be larger than the minimum value for paging probability, and the maximum value for paging probability threshold should be less than the maximum value for paging probability. That’s enough. We don’t think setting of paging probability threshold in SIB is a recommendation to CN working groups and RAN3, also don’t think it needs to be associated with the configuration provided by MME via NAS layer or the information provided from the MME to the eNB along with the paging request via S1AP. And we think the above option 1 (same as CT1) is more suitable for RAN3 paging request signalling. |

2.2 Support for 4 WUS group sets

Discussion point 2: Do you think there is a need to support 4 WUS group sets considering that 1 WUS group set is assigned for UEs with no paging probability class? If no, please elaborate on why.

| Company | Yes / No | Comments |
| --- | --- | --- |
| Ericsson | No | We think the agreed number of WUS group sets would be enough considering that Rel-16 group WUS is introduced mainly to reduce the impact on UEs with low paging probability due to false wake ups. |
| Lenovo | Yes or FFS. | In last RAN2#109e, there is a WA: Maximum number probability thresholds is 3 giving 4 group. Thus, we think the agreed number of WUS group sets for service-based grouping is 4. The WUS group set with highest paging probability class has been agreed to be applied by the UE without paging probability if no other UE-ID based group set is configured based on current agreement. This is our understanding to the WUS progress. If there is any issue on this point, we can further discuss it. |
| ZTE | Yes | Maybe I don’t clearly understand the question. Based on the current configuration in 36.331, we understand we can have at most 4 WUS group sets. And all the UEs without paging probability can belong to the “last” group set which is also for the UEs with “highest” paging probability. |

2.3 Assistance information for paging probability classes

Discussion point 3: Do you think it would be beneficial for the eNB if the MME provides assistance information regarding a particular paging probability class.? Please elaborate on why.

| Company | Yes / No | Comments |
| --- | --- | --- |
| Ericsson | Yes | MME can provide a relative measure to the eNB so that the eNB can configure the number of WUS groups in a particular WUS group set accordingly. This does not have to be an exact number, a relative measure will do, e.g., let’s assume MME uses the following codepoints to inform the eNB about the configured class as part of the paging request: {p20,p30,p40,p50,p60,p70,p80,p90}. If MME indicates to the eNB that the number of UEs configured with a particular codepoint is represented with x units, which is a normalized absolute value so no actual value is needed, eNB can configure the number of WUS groups in a particular WUS group more realistically. Note that this does not need to be done with every paging request, once will be enough unless there has been in update in the CN. For example, for p40 let’s assume 1 unit is indicated whereas for p20 2 units are indicated. This would mean that there are roughly twice the number of UEs configured with a paging probability up to %20 compared to ones with a probability between %30 - %40. The eNB may use that information as a reference to configure twice the number of WUS groups in the corresponding WUS group sets. |
| Lenovo | Yes | We are positive to above point that MME could give some UE information, such as the number of UEs to the specific codepoint to help eNB configure the reasonable WUS resource to each WUS group set. Otherwise, eNB may not know how much the WUS resource could be allocated to the WUS group set in which a large number of UEs or a small number of UEs is applied, that will increase the wrong paging alarm of UE. |
| ZTE | Yes | We can understand such assistance information, e.g, a roughly statistics on UE distribution on different paging probabilities, would be helpful for eNB to set more suitable paging probability thresholds or configure the number of groups for a certain WUS group set more realistically. We are fine to have such information. We also agree with Ericsson that once or few delivery of such information is enough. So we think it’s more suitable to send such information in an eNB-specific signalling, not UE-specific signalling (e.g., paging). Anyway, the details should be discussed and decided in RAN3. |

2.4 Mechanism to minimize false wake-up

In SA2 WG a mechanism has been proposed in Rel-15 to reduce the impact on UE power consumption due to false wake-up signalling caused by paging “mobile” UEs. In RAN2#109e, it was discussed whether a mechanism is needed for Rel-16 and companies stated their preference regarding whether there is a need and if yes how the mechanism should work [4].

Based on the feedback provided to the email discussion prior to the meeting; 4 + 1 companies think that the mechanism proposed in SA2 for Release 15 to reduce false wake-up be should be used in Rel-16. The support from one company, i.e., representing + 1 above, depends on whether their interpretation on how the mechanism is supposed work is correct. The rapporteur assumes that this is the case and counted their support. 2 companies think that it would be better to utilize the Rel-16 WUS mechanism to address the issue for various reasons.

The following proposal was made by the rapporteur: “The mechanism proposed in SA2 for Release 15 to reduce false wake-up is used in Rel-16.”

Discussion point 4: Considering the outcome of the email discussion above, please comment only if you think the proposal above is not agreeable and elaborate on why.

| Company | Comments |
| --- | --- |
| ZTE | We have a little strong view that we’d better not to use the R15 SA2 scheme in R16. With R16 GWUS, the (bad) impact of using WUS in all the cells is far less compared to Release 15 WUS. In previous discussion, I can see more companies can agree with such thinking. But as no enough time to discuss, they are still ok to take R15 scheme as baseline. Per our understanding, to give such restriction of stopping using R16 GWUS in non-serving cell would cause worse result of greatly reducing the benefits of R16 GWUS. As we provide our comments a little late in last email discussion, our comments may not be completely understood by everyone, here we want to emphasize the reasons again: For the R15 scheme itself, it has the following two issues: - First issue, per our roughly evaluation, with a general (not so high) paging density or paging possibility, to stop using WUS in the non-serving cell would cause more power consumption for the target mobile UE. This is the obvious shortcoming for the R15 scheme. - Second issue, the bad impacts in R15 cause by mobile UE is, when a target mobile UE moves to a non-serving cell and if WUS is still used, this cell-specific WUS for this target UE would false wake up all the other UEs in the non-serving cell. But one thing we want to indicate is, when we say the bad impacts is big, we may have a very “ideal” assumption that there has no real paging for any other UEs, then we can say those other UEs are false waken up. But if there have real paging for some of those other UEs at the same time when the target UE is paged, the wake up of these UEs is not “false” but “intentional”. In summary, how big the bad impacts of false wake up may be very depend on the possibility of multiple UEs being paged at the same time. For R16, different from in R15 all the other UEs in the non-serving cell would be false waken up, only the UEs (maybe 1/16 of all the UEs) belong to same service or UE-ID group as the target UE will be possibly false waken up. Furthermore, if this target UE also belongs to a low paging probability group and the paging for this target UE is anyway sparse, the bad impacts will further reduce. On the other hand, if this target UE belongs to a high paging probability group, e.g., the paging for this target UE may be very dense, as we can assume the possibility of paging the other UEs in the same service group is also high, with reference to the second issue mentioned above, we think the false wake up for other UEs can be seen very low, e.g., among all the wake up of other UEs in this high paging probability group, only a small part is “false” wake up caused by target UE and the other wake up are normal or intentional. With all the above analysis, we think the disadvantage of re-using R15 scheme in R16 is obvious (more power consumption for the mobile UE) while the benefit may be unclear (even how big the false wake up issue in R16 is also unclear). We don't think it's a good idea to introduce R15 scheme in R16 or set restriction on usage of R16 GWUS in a hurry. |

2.5 Support of Rel-16 WUS vs. Rel-15 WUS

Discussion point 5: Do you think RAN2 should confirm the following working assumption: “Support of Release 16 WUS is independent to support of Release 15 WUS”? Please elaborate on why.

| Company | Yes / No | Comments |
| --- | --- | --- |
| Lenovo | Yes | In last RAN2#109e meeting, some other company said maybe they would apply only the R16 network and UE, it is the reason leading to this work assumption. We think it is reasonable. |
| ZTE | No | Such working assumption may cause confusion on whether R15 common WUS can be sent when multiple R16 UE groups need to be waken up (if R15 common WUS has been configured). Therefore, we are ok to revert the working assumption or at least to clarify a Rel-16 group WUS capable UE shall be able to monitor R15 WUS sequence for common WUS. |

3	Summary

TBD

4	Conclusion

Based on the discussion and summary, the following proposals are made:

???

4	References

RP-192875, “Additional MTC enhancements for LTE”, Ericsson, RAN#86, Sitges, Spain, 9th – 12th December 2019.

RP-193224, “Additional enhancements for NB-IoT”, Futurewei, RAN#86, Sitges, Spain, 9th – 12th December 2019.

R2-2001886, “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”, Blackberry, Rel-16, LTE_eMTC5-Core, NB_IOTenh3-Core

R2-2001789, “Report of WUS: Progress the FFS from Email Discussion 108#94 and Summary”, Qualcomm, Rel-16, LTE_eMTC5-Core, NB_IOTenh3-Core.

R3-201676, Consideration on UE group wake up signal (WUS), Huawei, Vodafone

R3-201677 Support of WUS grouping, S1AP CR#1762, Huawei, Vodafone

R3-202190, Introduction of WUS grouping, Ericsson, ZTE

R3-202191, Introduction of WUS grouping, S1AP CR#1772, Ericsson, ZTE