3GPP TSG-RAN WG2 Meeting #109bis-e	draftR2-2004044

Online, 20th – 30th  April 2020

Agenda item:	7.2.5

Source:	Huawei (offline email discussion rapporteur)

Title:	Report of [AT109bis-e][313][NBIOT] UE capabilities, TDD/FDD differentiation and 5GC applicability for NB-IoT and eMTC (Huawei  (Huawei)

Document for:	Report

# 1	Introduction

This document is the report of the following e-mail discussion:

[AT109bis-e][313][NBIOT] UE capabilities, TDD/FDD differentiation and 5GC applicability for NB-IoT and eMTC (Huawei)

Scope: Discuss the open issues on UE capabilities

Intended outcome: Finalise the issues, report in R2-2004048

Deadline: 27-04-2020, 10:00 UTC

The discussion is based on the proposals in [2].

# 2	Discussion

## 2.1	GWUS

Proposal S1-1: For NB-IoT and eMTC, the existing capability wakeUpSignalMinGap-eDRX-r15 also applies to Rel-16 WUS.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | This is rather a device feature. |

Conclusion:

Proposal:

Proposal S1-2: For NB-IoT, Rel-16 GWUS is only applicable to FDD.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Same as the Rel-15. |

Conclusion:

Proposal:

Proposal S1-3: For eMTC, separate capability indications are introduced for FDD and TDD.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Same as the Rel-15. |

Conclusion:

Proposal:

Proposal S1-4: For NB-IoT and eMTC, Rel-16 GWUS is applicable to both EPC and 5GC, and there is no need for capability differentiation.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Probably better for simplicity if we can keep no difference as seen from the AS level. |

Conclusion:

Proposal:

## 2.2	Multiple TB scheduling

Proposal S2-1: For NB-IoT, multiple TB scheduling in unicast and in multicast is only applicable to FDD.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Seems to align with RAN1 view. |

Conclusion:

Proposal:

Proposal S2-2: For NB-IoT and eMTC, multiple TB scheduling in unicast is applicable to both EPC and 5GC without differentiation.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Probably better for simplicity if we can keep no difference as seen from the AS level. |

Conclusion:

Proposal:

Proposal S2-3: For NB-IoT and eMTC, multiple TB scheduling in multicast is only applicable to EPC

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | SC-PTM is not supported in 5GC. |

Conclusion:

Proposal:

## 2.3	SON

Proposal S3-1: For NB-IoT, support of ANR, RACH report and RLF report are applicable to both FDD and TDD and there is no need for FDD/TDD differentiation.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | RRM measurements are defined for FDD and TDD. |

Conclusion:

Proposal:

## 2.4	DL channel quality reporting in MSG3

Proposal S4-1: For NB-IoT, move the feature DL channel quality reporting in MSG3 for non-anchor carrier to section 6.17.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | This should go under 6.17 idle mode measurements (sorry my mistake). Note that it is already moved there in the latest version of the running CR. |

Conclusion:

Proposal:

Proposal S4-2: For eMTC, introduce a separate capability for DL channel reporting in MSG3

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | The scope of the feature is different between eMTC and NB-IoT (cell versus carrier). |

Conclusion:

Proposal:

Proposal S4-3: For NB-IoT, update the description of the legacy feature DL channel quality reporting to avoid conflicting description with the Rel-16 capabilities.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Much better for clarity. |

Conclusion:

Proposal:

Proposal S4-4: For NB-IoT, DL channel quality reporting in MSG3 for non-anchor carrier is only applicable to FDD.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Same as legacy. |

Conclusion:

Proposal:

Proposal S4-5: For NB-IoT and eMTC, DL channel quality reporting in MSG3 is applicable to both EPC and 5GC without capability differentiation.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Better for simplicity if we can keep no difference as seen from the AS level. |

Conclusion:

Proposal:

## 2.5	DL channel quality reporting in connected mode

Proposal S5-1: Keep a common capability for NB-IoT and eMTC for DL channel quality reporting in connected mdoe and clarify in the description that reporting of the serving cell applies to E-UTRAN and reporting of the configured carrier applies to NB-IoT.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry |  | No strong view, the scope of the feature is different (cell/carrier) but the reporting mechanism is the same. |

Conclusion:

Proposal:

Proposal S5-2: For NB-IoT, DL channel quality reporting in MSG3 in connected mode is only applicable to FDD. For eMTC, it is applicable to both FDD and TDD.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Same as legacy. |

Conclusion:

Proposal:

Proposal S5-3: For NB-IoT and eMTC, DL channel quality reporting in connected mode is applicable to both EPC and 5GC without capability differentiation.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Better for simplicity if we can keep no difference as seen from the AS level. |

Conclusion:

Proposal:

## 2.6 NRS presence on non-anchor carrier

Proposal S6-1: For NB-IoT, Idle mode RRM measurements on non–anchor paging carrier is only applicable to FDD.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Seems to align with RAN1 view. |

Conclusion:

Proposal:

Proposal S6-2: For NB-IoT, Idle mode RRM measurement on non–anchor carrier is applicable to EPC and 5GC without capability differentiation.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Better for simplicity if we can keep no difference as seen from the AS level. |

Conclusion:

Proposal:

## 2.7	Inter-RAT cell selection

Proposal S7-1: For NB-IoT and eMTC, there is no need to define a optional feature for support of assistance information for inter-RAT cell selection to/from NB-IoT.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry |  | No strong view. May need to be re-discussed later if some optional UE behaviours are added in other specs. |

Conclusion:

Proposal:

## 2.8	Co-existence with NR

Proposal S8-1: For NB-IoT and eMTC, UL and DL resource reservation for coexistence with NR are applicable to EPC and 5GC without capability differentiation.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Better for simplicity if we can keep no difference as seen from the AS level. |

Conclusion:

Proposal:

## 2.9	Connection to 5GC

Proposal S9-1: For NB-IoT, introduce a new optional feature, NB-IoT/5GC, in section 6.18.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | eNB does not need to know, so no need to have a capability reporting (clause 4). |

Conclusion:

Proposal:

Proposal S9-2: For NB-IoT and eMTC, remove the capabilities introduced in 6.18.1 (User Plane CIoT 5GS optimisations) and 6.18.2 (Control Plane CIoT 5GS optimisations).

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | We don’t want to replicate the NAS options in 306. |

Conclusion:

Proposal:

Proposal S9-3: For NB-IoT and eMTC, introduce a new optional feature, MO-EDT for Control Plane CIoT 5GS Optimisation, in section 6.18 and remove the editor’s note in 6.8.4.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | We need the 5GS equivalent. |

Conclusion:

Proposal:

Proposal S9-4: For NB-IoT, all pre-Rel15 capabilities not CIoT EPS optimisations related and other than rai-Support-r14 are applicable to 5GC without capability differentiation.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes | Question: This will mean that when the EPC/5GC applicability is not stated, this applies to both by default (same as what is being done in Rel-16 NB-IoT/eMTC). Do the other parallel groups follow the same approach ? It would be useful to have a coherent use in 36.306. |

Conclusion:

Proposal:

Proposal S9-5: For NB-IoT and eMTC connected to 5GC, support of AS RAI enhancement is optional at the UE, a new optional feature RAI Enhancement is introduced in section 6.18.

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry | Yes |  |

Conclusion:

Proposal:

Proposal S9-6: For eMTC, introduce the following capabilities for support of connection to 5GC:

- ce-eutra-5GC

- ce-eutra-5GC-HO-ToNR-FDD-FR1

- ce-eutra-5GC-HO-ToNR-TDD-FR1

- ce-eutra-5GC-HO-ToNR-FDD-FR2

- ce-eutra-5GC-HO-ToNR-TDD-FR2

Company’s views

| Company | do you agree (yes/no) | Comments |
| --- | --- | --- |
| BlackBerry |  | No strong view, normally we use FGI bits instead if the issue may come from testing. |

Conclusion:

Proposal:

# 3	Summary

Conclusions:

Potential easy agreements

To be completed

For further discussion

To be completed

# 4	List of referenced documents

R2-2002588	Updates for Rel-16 additional enhancements NB-IoT	BlackBerry UK Limited	CR	Rel-16	36.306	16.0.0	1746	-	C	NB_IOTenh3-Core	Late

R2-2003248	UE capabilities, TDD/FDD differentiation and 5GC applicability for NB-IoT and eMTC	Huawei, HiSilicon	discussion	Rel-16	NB_IOTenh3-Core, LTE_eMTC5-Core