3GPP TSG-RAN WG2 Meeting #109bis-e	draft-R2-2003920

# Online, April 20th  – April 30  2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0784 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to WUS group for eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  |  |  |  | 2020-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture the remaining RAN2 agreements related to GWUS monitoring |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section for WUS Resource identification for BL UE and UE in enhanced coverage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-16 eMTC enhancements for GWUS will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.1, 7.5.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 4239 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.300 CR 1277 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

First Change

## 7.5	Paging with Group Wake Up Signal

### 7.5.1	General

When the UE supports GWUS and GWUS configuration (gwus-Config)  is provided in system information, the UE shall monitor GWUS using the GWUS parameters provided in System Information.

A UE supporting GWUS can be configured to monitor a WUS Group and a common WUS. Upon detecting either of them UE shall monitor POs as defined in clause 7.4.

For NB-IoT, E-UTRAN may configure up to 2 WUS resources (numbered 0 and 1). The time offset, g0, from the end of WUS resource 0 to the start of corresponding PO is determined as defined in subclause 7.4. When both wus-Config and gwus-Config are present, WUS resource 0 shares radio resources with wus-Config.The time offset from the end of WUS resource 1 to the start of corresponding PO is sum of the time offset g0 and the maximum WUS duration.

For BL UEs and UEs in enhanced coverage, E-UTRAN may configure up to 4 WUS resources. The resource number, time and frequency location of these resources is determined as specified in subclause 7.5.x

Next change

### 7.5.x		WUS Resource Location for BL UEs and UEs in Enhanced coverage

A BL UE or UE in enhanced coverage determines the time/frequency location of WUS resources based on freqLocation parameter which indicates the frequency location of WUS Resource 0 (NIDresource=0). The frequency location of other WUS Resources (Resource ID 1,2,3) based on FreqLocation given in Table 7.5.x-1. The resource pattern which indicates the WUS Resources applicable for WUS Groups is signalled either by resourceMappingPattern or resourceMappingPatternWithoutLegacy.

Table 7.5.x-1: WUS Resource frequency location

| WUS Resource (NIDresource) | Frequency location of WUS Resource ID 0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | n0 | n2 |  | n4 (Note 1) |
|  |  | NB below centre frequency | NB above centre frequency |  |
| WUS Resource 1 | n2 | n0 | n4 | n2 |
| WUS Resource 2 | n0 | n2 | n2 | n4 |
| WUS Resource 2 (Note 2) | n4 | n4 | n0 | n0 |
| WUS Resource 3 | n2 | n0 | n4 | n2 |
| Note 1: This column is applicable if wus-Config is present. Note 2: This row is applicable for the resource pattern ID 7 |  |  |  |  |

The time offset, g0, from the end of WUS resource 0 and WUS resource 1 to the start of corresponding PO is determined as defined in subclause 7.4. The time offset from the end of WUS resource 2 and WUS resource 3 to the start of corresponding PO is sum of the time offset g0 and the maximum WUS duration for all value of resource patterns except resource-Pattern-ID-7.  The time offset g0 for WUS resource 2 is same as WUS resource 0 and 1 for resource pattern ID 7.

The WUS Resource IDs are selected based on as given in Table 7.5.x-2.

Table 7.5.x-2: WUS Resources applicable for Resource Pattern

|  |  |  | Resource Pattern_ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  |
| WUS Resource (NIDresource) | 0 | X |  |  |  | X |  |  |  | X |  |  |  | X |  | X |  |
|  | 1 |  |  | X |  | X |  | X |  | X |  | X |  | X |  | X |  |
|  | 2 |  |  |  |  |  |  | X |  | X |  | X |  | X |  | X |  |
|  | 3 |  |  |  |  |  |  |  |  |  |  | X |  | X |  |  |  |

If NIDresource = 0 is not used, the first entry in the numGroupsList corresponds to NIDresource = 1. Otherwise, NIDresource is the index of the WUS resources in numGroupsList.

End of Changes