3GPP TSG-RAN WG2 Meeting #109bis-e	draft R2-2003921

# Online, 20th - 30th April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1752 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of UE capabilities for eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update of UE capabilities related to additional enhancements for eMTC in Rel-16 according to agreements in RAN2#109-e |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following capabilities and agreements have been captured in this CR: WUS: Working assumption: - Support of Release 16 WUS is independent to support of Release 15 WUS PUR: - Introduce separate UE capabilities pur-UP-5GC-r16 and pur-CP-5GC-r16. Multiple TB scheduling - For LTE-M and NB-IoT, multiple TBs scheduling in multicast is optional without capability reporting. NR coexistence - Working assumption: Introduce four UE capabilities for handling resources reservation on UL and DL, and for CE mode A and CE mode B separately, in PhyLayerParameters-v16xy. - Working assumption: Introduce two UE capabilities for handling DL subcarrier puncturing for CE mode A and CE mode B separately, in PhyLayerParameters-v16xy. - Working assumption: Six UE capabilities mentioned in Proposal 2-7 and Proposal 2-8 for handling resources reservation or DL subcarrier puncturing can be applied to both FDD and TDD, e.g., with separate values for FDD or TDD. - Working assumption: Six UE capabilities for handling resources reservation or DL subcarrier puncturing can be introduced into TS 36.306. Miscellaneous small corrections added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.331 CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.3.4.181	srs-DCI7-TriggeringFS2-r15

This field indicates whether the UE supports SRS triggering via DCI format 7 for FS2.

#### 4.3.4.182	multiTB-UL-r16

This field indicates whether the UE supports multiple TB scheduling in the uplink as specified in TS 36.213 [22]. This feature is only applicable if the UE supports any ue-Category-NB.

#### 4.3.4.183	multiTB-DL-r16

This field indicates whether the UE supports multiple TB scheduling in the downlink as specified in TS 36.213 [22]. This feature is only applicable if the UE supports any ue-Category-NB.

#### 4.3.4.184	ce-ModeA-PUSCH-MultiTB-r16

This field indicates whether the UE supports multiple TB scheduling in the uplink when the UE is operating in coverage enhancement mode A as specified in TS 36.213 [22]. This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.4.185	ce-ModeA-PDSCH-MultiTB-r16

This field indicates whether the UE supports multiple TB scheduling in the downlink when the UE is operating in coverage enhancement mode A as specified in TS 36.213 [22]. This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.4.186	ce-ModeB-PUSCH-MultiTB-r16

This field indicates whether the UE supports multiple TB scheduling in the uplink when the UE is operating in coverage enhancement mode B as specified in TS 36.213 [22]. This feature is only applicable if the UE supports ce-ModeB-r13.

#### 4.3.4.187	ce-ModeB-PDSCH-MultiTB-r16

This field indicates whether the UE supports multiple TB scheduling in the downlink when the UE is operating in coverage enhancement mode B as specified in TS 36.213 [22]. This feature is only applicable if the UE supports ce-ModeB-r13.

#### 4.3.4.188	ce-ModeA-CSI-RS-Feedback-r16

This field indicates whether the UE supports CSI-RS based feedback when the UE is operating in coverage enhancement mode A, as specified in TS 36.213 [22]. This feature is only applicable if the UE supports ce-ModeA-r13 and a UE Category other than Category M1 and M2.

#### 4.3.4.189	ce-RxInLTE-ControlRegion-r16

This field indicates whether the UE supports PDSCH or MPDCCH reception in the LTE control channel region when the UE is operating in coverage enhancement mode A or B as specified in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.4.190	ce-CRS-ChannelEstMPDCCH-r16

This field defines whether the UE supports CRS for improving MPDCCH channel estimation, as specified in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.4.191	widebandPRG-Slot-r16, widebandPRG-Subslot-r16, widebandPRG-Subframe-r16

This field indicates whether the UE supports wideband precoding resource block group size for slot/subslot/subframe PDSCH operation as specified in TS 36.213 [22].

#### 4.3.4.xa	groupWakeUpSignal-r16

This field indicates whether the UE supports Group WUS for FDD as specified in TS 36.211 [17], TS 36.213 [22] and TS 36.304 [14]. This feature is only applicable if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB.

Editor’s note: FFS: Dependency on support of R15 WUS, currently RAN1 agreement and RAN2 working assumption are in conflict.

Editor’s note: FFS: For ce-ModeA-r13, FFS whether a separate capability is required for TDD or not.

#### 4.3.4.xb	ce-ModeA-NR-ResourceResvUL-r16

This field defines whether the UE supports UL resource reservation for coexistence with NR when the UE is operating in coverage enhancement mode A, as specified in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.4.xc	ce-ModeB-NR-ResourceResvUL-r16

This field defines whether the UE supports UL resource reservation for coexistence with NR when the UE is operating in coverage enhancement mode B, as specified in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeB-r13.

#### 4.3.4.xd	ce-ModeA-NR-ResourceResvDL-r16

This field defines whether the UE supports DL resource reservation  for coexistence with NR when the UE is operating in coverage enhancement mode A, as specified in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.4.xe	ce-ModeB-NR-ResourceResvDL-r16

This field defines whether the UE supports DL resource reservation for coexistence with NR when the UE is operating in coverage enhancement mode B, as specified in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeB-r13.

#### 4.3.4.xf	ce-ModeA-NR-SubcarrierPuncturing-r16

This field defines whether the UE supports DL subcarrier puncturing for coexistence with NR when the UE is operating in coverage enhancement mode A, as specified in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeA-r13.

#### 4.3.4.xg	ce-ModeB-NR-SubcarrierPuncturing-r16

This field defines whether the UE supports DL subcarrier puncturing for coexistence with NR when the UE is operating in coverage enhancement mode A, as specified in TS 36.211 [17]. This feature is only applicable if the UE supports ce-ModeB-r13.

NEXT CHANGE

#### 4.3.8.7	earlyData-UP-r15

This field defines whether the UE supports MO-EDT for User Plane CIoT EPS optimizations, as defined in TS 24.301 [28]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

#### 4.3.8.8	void

#### 4.3.8.9	extendedNumberOfDRBs-r15

This field defines whether the UE supports up to 15 DRBs. The UE shall support any combination of RLC AM and RLC UM entities for the configured DRBs. A UE that supports extendedNumberOfDRBs-r15 shall also support the extended LCID as specified in TS 36.321 [4].

#### 4.3.8.10	reducedCP-Latency-r15

This field defines whether the UE supports reduced control plane latency as defined in TS 36.213 [22] and TS 36.331 [5].

#### 4.3.8.11	earlySecurityReactivation-r16

This field defines whether the UE supports early security reactivation when resuming a suspended RRC connection as specified in TS 36.331 [5].

#### 4.3.8.12	pur-CP-EPC-r16

This field indicates whether the UE supports Transmission using PUR for Control Plane CIoT EPS optimisation, as defined in TS 36.300 [30]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

#### 4.3.8.13	pur-UP-EPC-r16

This field indicates whether the UE supports Transmission using PUR for User Plane CIoT EPS optimisation, as defined in TS 36.300 [30]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

#### 4.3.8.14	dl-DedicatedMessageSegmentation-r16

Indicates whether the UE supports reception of segmented DL RRC messages.

#### 4.3.8.xa	pur-CP-5GC-r16

This field indicates whether the UE supports Transmission using PUR for Control Plane CIoT 5GS optimisation as specified TS 36.300 [30]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

#### 4.3.8.xb	pur-UP-5GC-r16

This field indicates whether the UE supports Transmission using PUR for User Plane CIoT 5GS EPS optimisation as specified TS 36.300 [30]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

Editor’s note: In RRC the 4 PUR capabilities are part of MAC parameters for eMTC, but are part of general parameters for NB-IoT. Need to align one way or another.

NEXT CHANGE

#### 4.3.19.20	extendedLCID-Duplication-r15

This field indicates whether the UE supports use of extended LCIDs 32-38  for PDCP duplication. A UE that supports extendedLCID-Duplication-r15 shall also support the extended LCID as specified in TS 36.321 [4].

#### 4.3.19.21	eLCID-Support-r15

This field indicates whether the UE supports LCID "10000" and MAC PDU subheader containing the eLCID field as specified in TS 36.321 [4].

#### 4.3.19.22	rai-SupportEnh-r16

This field defines whether the UE supports AS Release Assistance Indication (AS RAI) MAC CE when connected to EPC as specified in TS 36.321 [4]. This feature is only applicable if the UE supports ce-ModeA-r13 or if the UE supports any ue-Category-NB.

NEXT CHANGE

#### 4.3.29.13	ce-ModeA-ETWS-CMAS-RxInConn-r16

This field indicates whether the UE supports ETWS/CMAS indication reception in RRC_CONNECTED state when the UE is operating in coverage enhancement mode A as specified in TS 36.331 [5]. This feature is only applicable if the UE supports ce-ModeA-r13 and a UE Category other than Category M1 and M2.

#### 4.3.29.14	ce-ModeB-ETWS-CMAS-RxInConn-r16

This field indicates whether the UE supporting CE Mode B supports ETWS/CMAS indication reception in RRC_CONNECTED state when the UE is operating in coverage enhancement mode B as specified in TS 36.331 [5]. This feature is only applicable if the UE supports ce-ModeB-r13 and a UE Category other than Category M1 and M2.

NEXT CHANGE

## 6.16	SC-PTM features

### 6.16.1	SC-PTM in Idle mode

It is optional for UE to support the SC-PTM reception in RRC_IDLE as specified in TS 36.331 [5]. This feature is only applicable if the UE supports UE category M1 or UE category M2 or if the UE supports coverage enhancements (ce-ModeB-r13 and/or ce-ModeA-r13) or for FDD, if the UE supports any ue-Category-NB.

### 6.16.x	Multiple TB scheduling for SC-PTM in Idle mode

It is optional for UE to support multiple TB scheduling for multicast as specified in TS 36.331 [5]. This feature is only applicable if the UE supports ce-ModeA-r13, or for FDD if the UE supports any ue-Category-NB.

END OF CHANGES