3GPP TSG-RAN WG2 Meeting #109bis-e	R2-20xxxxx

# Online, April 20 – 30, 2020

Agenda item:	6.8.2.4

Source: 	Ericsson

Title: 	Email discussion report: [AT109bis-e][610][POS] LPP proposals (Ericsson)

Document for:		Discussion and Decision

1. 	Introduction

This document summarizes the following email discussion:

- [AT109bis-e][610][POS] LPP proposals (Ericsson)

Scope: Discuss proposals 2, 4, 5, 6, 7, 8 from R2-2003783

Intended outcome: Summary of agreements in R2-2003997

Deadline:  Wednesday 2020-04-29 1000 UTC

  - 2	Discussion

The following rappoorted proposals from [1] are discussed in the below subsections:

Rapporteur’s Proposal 2: RAN2 should discuss the interpretation of additional paths measurements (nr-AdditionalPathList) in case of additional timing measurements are reported.

Rapporteur’s Proposal 4: RAN2 should discuss the use cases for a new LocationInformationType ‘locationEstimateAndMeasurementsRequired’ in IE CommonIEsProvideLocationInformation first, before introducing the feature in LPP.

Rapporteur’s Proposal 5: RAN2 should discuss whether PSCell/Scell information should be provided by a target device in CommonIEsRequestAssistanceData.

Rapporteur’s Proposal 6: RAN2 should inform RAN1 of the RAN2 discussion and concerns related to the SMTC information in the SSB assistance data, and ask RAN1 for any status update of the working assumption in RAN1.

Rapporteur’s Proposal 7: RAN2 should inform RAN1 of the RAN2 discussion and ask whether PRS-PRS QCL Type D indication is still needed (this may be a combined LS to RAN1 incl. Rapporteur’s Proposal 6).

Rapporteur’s Proposal 8: RAN2 should discuss whether

(a)	to change the name of the IE TRP-ID (e.g., to distinguish from the RAN3 TRP-ID), or

(b)	 to remove IE TRP-ID from LPP and add the relevant TRP-ID fields to the individual parent IEs.

  - 2.1	Interpretation of additional paths measurements

The current definition in LPP for the additional path reporting is ambiguous/unclear; in particular together with the additional measurement reporting capability. The figure below is provided in [2] which illustrates the different path timing possibilities (for different resources of two exemplary TRPs):

Reference Path

Reference TRP

Path#1

Neighbouring TRP

Path#2

Path#2

Path#1

Path#1

Path#2

Path#2

Path#1

Path#1

Path#2

Path#2

Resource#0

Resource#1

Resource#2

Resource#0

Resource#1

Resource#2

RSTD

The black line illustrates the "main RSTD", nr-RSTD measurement.

The blue line illustrates the nr-RSTD-ResultDiff (provided in IE NR-DL-TDOA-AdditionalMeasurementElement).

The red line illustrates the nr-AdditionalPathList for the "main RSTD" (outside the NR-DL-TDOA-AdditionalMeasurementElement).

The green and orange dashed curves are the candidates for the nr-AdditionalPathList for NR-DL-TDOA-AdditionalMeasurementElement according to [2].

The proposal in [2] is to clarify the definition of the time reference of the additional paths, with the following two options:

Option 1.1.  The additional path time reference is the first path of the resource (the reference path) illustrated in “orange” in the figure

Option 1.2.  The additional path time reference is the first path of the resource used to determine RSTD  illustrated in “green” in the figure.

Companies are asked to provide comments and a preference of option in the table below

| 1 Additional path time reference of the additional measurements (RSTD and UE RxTx) |  |
| --- | --- |
| Company | Comments |
| Huawei, HiSIicon | Option 1.2 We prefer not to break the differential RSTD into two parts. |

  - 2.2	Use cases for reporting both measurements and location

[3] argus that due to the introduction of UE-based DL-TDOA and DL-AoD positioning modes in Rel-16, "the concept of information type needs to be updated" [3] such that the UE reports both, a location estimate and location measurements.

With UE-assisted positioning, the operator obtains information via the reported measurements and quality from the devices as well as the UE-assisted positioning and estimated uncertainty that can be used to optimize configurations, procedures, deployments costs etc with respect to the performance of UE-asisted positioning.

With UE-based positioning the UE can benefit from regular measurements over time and can obtain a better position estimate compared to UE-assisted position estimated in the network based on reported UE measurements. The operator can currently request measurements as part of UE-assisted positioning or estimated UE-based positions but not both. Therefore, the operator cannot correlate the signal configurations and measurements on the one hand and the UE-based position estimates on the other hand, in order to to optimize configurations, procedures, deployments costs etc with respect to the performance of UE-based positioning.

The discussion can be summarized in two options:

Option 2.1. Introduce the location information type locationEstimateAndMeasurementsRequired, enabling the operator to request for both a UE-based location estimate and measurements to allow positioning configuration optimization.

Option 2.2. Do not introduce the location information type locationEstimateAndMeasurementsRequired, meaning that the operator has to rely on a network-based UE-assisted location estimate to represent the UE-based location estimate in the positioning configuration optimization.

Companies are asked to provide comments and a preference of option in the table below

| 2 Introduction of a new location information type for compbined location estimate and measurements |  |
| --- | --- |
| Company | Comments |
| Huawei, HiSilicon | We don’t have a strong opinion on this. |

  - 2.3	PSCell/Scell location information provisioning

In a request for assistance data, according to [2], the UE should not only provide the UE current Pcell identity, but also PSCell/Scell information. It was commented in [2] that PSCell/Scell information could be helpful for the LMF e.g., in case a target device does not support inter-frequency DL RSTD.

According to Summary Rapporteur’s understanding, this issue was proposed in previous Releases. One of the arguments against this feature was that the activation/deactivation of Scell’s can be a rather dynamic process, and the information provided by a UE may be outdated/less useful by a location server.

Option 3.1.  The UE only provides the Pcell identity in the request for assistance data

Option 3.2.  The UE provides the identity of the PCell as well as of any PSCells/SCells in the request for assistance data.

Companies are asked to provide comments and a preference of option in the table below

| 3 PSCell/Scell location information provisioning |  |
| --- | --- |
| Company | Comments |
| Huawei, HiSilicon | Option 3.2. If more serving cells are repored, LMF can more easily estimate the locaiton of the UE. If serving cells of the UE are non-co-located. Inter-frequency measuremnet is one UE capability and some of the UEs do not support. If all the serving cells are reported, the LMF can know for a certain PRS, whether it is inter-/intra-frequency measurement for the UE. In the (NG-)EN-DC scenario, UE can report LTE and NR at the same time. It would be better to report all the cells than to only report LTE PCell, if the UE is using NR positioning, while the UE only report LTE Pcell. In the legacy release, the reason why reporting of Scell is not supported was Scell may experienece frequent activation/deactivation. If this is the reason, we are ok not to support for Scell. But for the PSCell in the MR-DC scenario, this still can be supported. |

  - 2.4	SMTC window information as part of the SSB assistance data

It was argued in [2] that the use case for the SMTC included in the SSB assistance data is not quite clear. It was pointed out in that the SMTC information in the SSB assistance data is indeed still a working assumption in RAN1. It is proposed to remove the SMTC information in the SSB assistance data in [2].

Since the SMTC information is part of the RAN1 parameter list, it should not be removed without RAN1 consultation. The situation can be summarized in the following two options

Option 4.1.  Leave the SSB configuration as is, including the SMTC window parametsr

Option 4.2.  Send an LS to RAN1 informing that from a RAN2 perspective, SMTC parameters do not seem the be necessary in the SSB assistance data over LPP, and ask about RAN1s view concerning these parameters

Companies are asked to provide comments and a preference of option in the table below

| 4 SMTC window information as part of the SSB assistance data |  |
| --- | --- |
| Company | Comments |
| Huawei, HiSilicon | Option4.2 Actually, this was not an agreement, but included as a working assumption in the LS from RAN1 to RAN2, waiting RAN2 to confirm. RAN1’s working assumption is that the SMTC is per SSB frequency rather than per cell. Whil, for the current spec, the SMTC window is configured under NR-SSB-Config, which is not per frequency layer. The question is who determins the SMTC. We think that LMF cannot determine the SMTC. If this can be enabled, it requires complex signaling between gNB and LMF in NRPPa. RAN4 is also discussing this issue and is likely to conclude that the UE is not required to additionaly measure SSB for purpose of receiving PRS. In this way, SMTC is useless. Also, when UE receives this SMTC configuration, the UE does not know what is the reference timing and the reference timing is based on which cell. |

  - 2.5	QCL information in the DL-PRS assistance data

Accoring to [2], QCL Type D information between two DL-PRS Resources was only introduced to support DL-AoD positioning ; i.e., to force a target device to use the same RX beam for receiving multiple DL-PRS Resources from the same TRP. Since the assistance data and/or location request can be customized for DL-AoD, there appears to be no need for QCL Type D information between two DL-PRS Resources according to [2].

Option 5.1.  Leave the QCL Type D information as is in the assistance data

Option 5.2.  Send an LS to RAN1 to inform about the RAN2 discussion and ask whether PRS-PRS QCL Type D indication is still needed

Companies are asked to provide comments and a preference of option in the table below

| 5 QCL information in the DL-PRS assistance data |  |
| --- | --- |
| Company | Comments |
| Huawei, HiSilicon | Option 5.2 We know that it may be hard to accept 5.2 for most of the comapnies. It is OK for us to compromise to 5.1 but optimization in signalling is needed. Now there is a large overhead. A rough estimation for the overhead: To configured PRS source, we need (2bit resorurce set id + 6bit resource id )*64 resource. Hence, if we want to configure the source RS for a PRS resouce in a TRP, it will requires around ~500 bits for one TRP. |

  - 2.6	TRP ID

It is argued in [2] and [4] that the IE TRP-ID in RAN2 need to be better defined to avoid confusion with RAN3, its use needs to be clarified and in what IEs it is needed	and how the identifiers associated to a TRP shall be represented.

According to [2], it is enough with the PRS ID to uniquely identify a TRP within an LPP session between LMF and a UE. Furthermore, [4] provides a summary of TRP-ID issues. Ultimately, the following Table of required TRP-ID elements for various IEs is derived in [4]:

| IE name | Required fields |
| --- | --- |
| NR-Multi-RTT-MeasElement | dl-PRS-ID |
| NR-DL-AoD-MeasElement | dl-PRS-ID |
| NR-DL-TDOA-MeasElement | dl-PRS-ID |
| NR-MeasuredResultsElement | pci, CGI and ARFCN |
| NR-TimeStamp | None |
| DL-PRS-IdInfo | dl-PRS-ID |
| NR-DL-PRS-AssistanceDataPerTRP | dl-PRS-ID and ARFCN |
| NR-SSB-Config | PCI and ARFCN |
| ReferenceTRP-RTD-Info | dl-PRS-ID |
| RTD-InfoElement | dl-PRS-ID |
| NR-DL-PRS-BeamInfo | dl-PRS-ID |
| TRP-LocationInfoElement | dl-PRS-ID |

The above Table summarizes the IEs which currently make use of the IE TRP-ID, and which fields of the IE TRP-ID is/are required for the functionality in the corresponding parent IE.

In addition, the summary Rapporteur’s comment [1] is that a Cell-ID may be required in NR-TimeStamp to indicate the cell/TRP from which the SFN has been derived.

Companies are asked to provide comments about what IEs and procedures that make use of the different TRP identifiers in the table below

| 6.1 IEs and procedures that make use of different TRP identifiers within an LPP session |  |
| --- | --- |
| Company | Comments |
| Huawei, HiSilicon | We can see from the above table that most of the cases only use PRS id. It is fine with us that we only use individual fields of the current TRP-ID, which means that to remove this IE and ues the fields within individually. Also, we would like to point out that the following fields does not need ARFCN NR-DL-PRS-AssistanceDataPerTRP; dl-PRS-ID and ARFCN. |

Another set of issues raised are

- appropriate name for the TRP identifier that identifies one of the up to 256 TRPs that a UE can handle (currently the name PRS ID is used)

- if the identifiers associated to TRPs shall be gathered in an IE, or if they should be present as individual fields

- the appropriate name of  an IE gathering the TRP identifiers if agreed (currently, the name TRP-ID is used)

Companies are asked to provide comments about how different TRP identifiers are represented and named within an LPP session, in the table below

| 6.2 TRP identifier representation and naming within an LPP session |  |
| --- | --- |
| Company | Comments |
| Huawei, HiSilicon | If the individual fields are used in the current IE TRP-ID and the TRP-ID is removed, there is no need for the above discussion on the naming and this is our prefered option. If the fields are still grouped under one IE, we think the proper name for the IE can be ”positioning node id” |

4. 	Summary

5. 	References

- R2-2003783, “Summary of LPP agenda item 6.8.2.3”, Qualcomm Incorporated (Summary Rapporteur)

- R2-2003061, "Remaining issues with LPP", Huawei, HiSilicon.

- R2-2003130, "Measurement Reporting for UE based positioning", Ericsson

- R2-2003318, "Handling on TRP-ID", Intel Corporation