3GPP TSG-RAN WG2 Meeting #109bis electronic	R2-2003012

20 April – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0153 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous correction to 38.304 for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2020-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The agreement “IAB-MTs are not under UAC control” should be implemented in 38.304. When the IAB-MT is barred by the network, IAB-MT “shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds” in the currect specification. However, it is not necessary/madatory for MT to not consider the cell for 300 seconds, since IAB is the network node. IAB-MT implementation should be allowed to re-check the cell frequently to reduce the access latency. In addition, whether “intraFreqReselection” in MIB should be considered by IAB-MT is not clear. Since IAB-MT should ignore the “cellBarred” in MIB, it is straight forward to also ingnore the “intraFreqReselection” in MIB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.3.0, add IAB-MT does not apply the unified access control. In section 5.3.1, clarify that “If operating as an IAB-MT and the cell is to be treated as if the cell status is "barred" due to iab-Support being absent in SIB1, the IAB-MT may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds; the IAB-MT may select another cell on the same frequency if the selection criteria are fulfilled.”; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB-MT is not allowed to consider the barred cell as a candidate for cell selection/reselection for 300 seconds. It is not clear whether IAB-MT follow the UAC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.0, 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------- [Start of 1st change] ---------------------------------

5.3.0	Introduction

There are two mechanisms which allow an operator to impose cell reservations or access restrictions. The first mechanism uses indication of cell status and special reservations for control of cell selection and reselection procedures. The second mechanism, referred to as Unified Access Control as specified in TS 38.331 [3], shall allow preventing selected access categories or access identities from sending initial access messages for load control reasons.

IAB-MT does not apply the unified access control.

--------------------- [End of 1st change] ---------------------------------

--------------------- [Start of 2nd change] ---------------------------------

5.3.1	Cell status and cell reservations

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of following fields:

-	cellBarred (IE type: "barred" or "not barred") 
Indicated in MIB message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN.

-	cellReservedForOtherUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs.

-	cellReservedForFutureUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs.

NOTE:	For IAB node, it ignores the cellBarred, cellReservedForOperatorUse and cellReservedForOtherUse as defined in TS 38.331 [3].

-	iab-Support (IE type: "true")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN.

When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use and cellReservedForFutureUse IE is not indicated as "true",

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell broadcasts any CAG IDs or NIDs and the cell status is indicated as "not barred" and "not reserved" for operator use and "true" for other use, and cellReservedForFutureUse IE is not indicated as "true":

-	All UEs in SNPN AM or with non-empty Allowed CAG list shall treat this cell as candidate during the cell selection and cell reselection procedures.

Editor's note: The applicability of above behaviour for non-NPN capable UE is FFS.

When cell status is indicated as "true" for other use, and either cell does not broadcast any CAG-IDs or NIDs or does not broadcast any CAG-IDs and the UE is not operating in SNPN Access Mode,

-	The UE shall treat this cell as if cell status is "barred".

When cellReservedForFutureUse IE is indicated as "true",

-	The UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN/SNPN and not "true" for other use and cellReservedForFutureUse IE is not indicated as "true",

-	UEs assigned to Access Identity 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved".

-	UEs assigned to Access Identity 11 or 15 shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for selected/registered SNPN is set to "reserved".

-	UEs assigned to an Access Identity 0, 1, 2 and 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN/SNPN or the selected PLMN/SNPN.

NOTE 1:	Access Identities 11, 15 are only valid for use in the HPLMN/ EHPLMN; Access Identities 12, 13, 14 are only valid for use in the home country as specified in TS 22.261 [12].

Editor's note: It is FFS whether above NOTE needs to be updated to consider SNPNs

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	If operating as an IAB-MT and the cell is to be treated as if the cell status is "barred" due to iab-Support being absent in SIB1:

-	The IAB-MT may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	The IAB-MT may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1 or due to trackingAreaCode being absent in SIB1 as specified in TS 38.331 [3]:

-	The UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled;

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "not allowed":

-	If the cell operates in licensed spectrum or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN:

-	the UE shall not re-select a cell on the same frequency as the barred cell;

-	else:

-	the UE may select to another cell on the same frequency if reselection criteria are fulfilled.

-	The UE shall exclude the barred cell and, if the cell operates in licensed spectrum or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN, also the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

5.3.2	Unified access control

The information on cell access restrictions associated with Access Categories and Identities is broadcast in SIB1 as part of Unified Access Control as specified in TS 38.331 [3].

The UE shall ignore Access Category and Identity related cell access restrictions for cell reselection. A change of the indicated access restriction shall not trigger cell reselection by the UE.

The UE shall consider Access Category and Identity related cell access restrictions for NAS initiated access attempts and RNAU as specified in TS 38.331 [3].

--------------------------[End of change] ------------------------------