3GPP TSG-RAN WG2 #109bis-e	R2-2003039

Electronic meeting, 20th – 30th April 2020

Agenda Item:	6.9.4 Conditional PSCell addition/change

Source: 	Ericsson

Title:  	UE capabilities for conditional PSCell change

Document for:	Discussion and Decision

# Introduction

In the work item for NR Mobility Enhancements [1], one objective is to improve the robustness at handover. In RAN2#105bis was agreed to support Conditional handover (CHO) in NR and in RAN2#107 it was agreed to support conditional NR PSCell addition/change. Later in RAN86 it was decided to limit the scope in rel-16 to intra SN change without MN involvement:

	Conditional handover based NR PSCell addition/change for any architecture option with NR PSCell- limit to intra SN change without MN involvement;

RAN2 has proceeded with the work and aims at finalizing CPC-intra SN change in rel-16. In this contribution UE capability for CPC is discussed.

# Discussion

In rel-16 only intra-SN conditional PSCell change is supported. The remaining work to add the inter-SN case is mainly RAN3 work. From a UE point of view the procedure would be the same, to apply a reconfiguration message when a condition is fulfilled. However, more events may be supported in rel-17 or other changes related to the conditions. A rel-16 capability for CPC could be distinguished from a rel-17 capability for CPC based on the release though.

There does not seem to be any reason to further split of the UE capability for CPC. One capability for rel-16 should be sufficient and it could be optional as it is not a critical feature.

Proposal 1: Introduce one optional UE capability bit for Conditional PSCell Change in release 16.

# Summary

Based on the discussions above, the following is proposed:

Proposal 1: Introduce one optional UE capability bit for Conditional PSCell Change in release 16.

# References

- RP-181433, Work Item on NR Mobility Enhancements, Intel, RAN#80, La Jolla, USA, May 21 – May 25 2018 September 2018.

- R2-1911344, Support of Conditional PSCell addition/change	NTT DOCOMO INC., Nokia, Ericsson, Fujitsu, KT Corp, Media Tek Inc., NEC, Qualcomm Inc., SoftBank Corp, 3GPP TSG-RAN WG2 Meeting #107, Prague, Czech, August 26-30, 2019.