3GPP TSG RAN WG2#109bis-e                                                	R2-2003119

Electronic meeting, 20th April– 30th April

Source:	CATT, CMCC

Title:	Consideration on Adding the Re-connection Attempt Cell Identity

Agenda Item:	6.12.4

Document for:	Discussion and Decision

Introduction

At the RAN2 109-e meeting, some conclusions related to UE RLF Report have been reached and a reply LS [1] was sent back to RAN3. But the decisions made in RAN2 109-e meeting seem there would be some problem to support inter/intra-system MRO which is also agreed in RAN3. In this contribution, we would like to make some further analysis and hope to reach an agreement to support inter/intra-system MRO.

Discussion

In the BLCR for TS38.300, the description related to inter-system MRO is as below:

These problems are defined as follows: - [Inter-system/ Too Late Handover] An RLF occurs after the UE has stayed in a cell belong to NG-RAN node which connects with 5GC for a long period of time; the UE attempts to re-connect to an E-UTRAN cell which connects with EPC. - [Inter-system/ Too Early Handover] An RLF occurs shortly after a successful handover from a E-UTRAN cell which connects with EPC to a target cell in a NG-RAN node which connects with 5GC; the UE attempts to re-connect to the source cell or to another E-UTRAN cell which connects with EPC.

From the description in the BLCR for TS38.300, UE needs to record the ECGI of the re-connection attempt cell after RLF in order to support the inter-system MRO detection. It is quite similar with the inter-RAT MRO mechanism in LTE, i.e. UE would report the PCI/CGI of the selected UTRAN cell after RLF happen in LTE. Only with the necessary information i.e. cell id of source cell, target cell and reconnection attempt cell, available in UE RLF Report, network could know whether it is an inter-system HO too early/late or not.

Observation 1: CGI of re-connection attempt E-UTRAN cell is essential information for inter-system MRO detection.

In last meeting RAN2 concluded not to include CGI of re-connection attempt E-UTRAN cell for inter-system MRO in Rel-16 since lack of time, and then the inter-system MRO function could not be supported, so we propose a second time to introduce the CGI of re-connection attempt E-UTRAN cell which is a fundamental information:

It is, if the UE cannot re-establish to a prepared cell and attempts a RRC connection setup procedure, the re-connection attempt E-UTRAN cell should be recorded. Especially for the inter-RAT case, there is not any prepared inter-RAT cell, and the re-connection attempt cell CGI would help the network to detect the root cause of the failure.

For inter-RAT case, UE could not perform re-establishment after failure, so the “selectedUTRA-CellId-r11” is added in the LTE RLF Report if the UE selects a UTRA cell and the UE could support RLF Report for Inter-RAT MRO:

selectedUTRA-CellId-r11			SEQUENCE {

carrierFreq-r11					ARFCN-ValueUTRA,

physCellId-r11					CHOICE {

fdd-r11							PhysCellIdUTRA-FDD,

tdd-r11							PhysCellIdUTRA-TDD

}

}																	OPTIONAL

Therefore for the similar purpose, the “Re-connection attempt cell CGI” requested by RAN3 could also be supported in NR RLF Report following LTE for inter-RAT case.

Proposal 1: Add “Re-connection attempt cell CGI” of E-UTRAN cell to the NR RLF Report.

Besides the CGI of re-connection attempt E-UTRAN cell, TAC of reconnection attempt cell is also helpful for inter-system MRO detection. The reason is that it is possible that eNB may connect with both EPC and 5GC, it could be regarded as inter-system MRO only when UE attempts to access to E-UTRAN cell connected with EPC after RLF happen.

Proposal 2: Also include the TAC of re-connection attempt E-UTRAN cell.

Similarly, for intra-system MRO, since NG-RAN node could be either gNB or eNB connected with 5GC, CGI of re-connection attempt NR cell is also needed. For example, RLF happens in ng-eNB1, and then UE attempts to re-connect to gNB2. In this case, it may be a ng-eNB1 to gNB2 handover too late.

Observation 2: Considering the scenario that eNB connects with 5GC, it is also necessary to include the CGI of re-connection attempt NR cell.

Proposal 3: Add “Re-connection attempt cell CGI” of NR cell to the NR RLF Report.

For RRC connection attempt, the re-connection attempt cell should be the first accessed cell after handover failure or RLF. If the time between handover failure or RLF and the first connection attempt is relatively long, the re-connection attempt cell could not be used to detection the root cause of failure event. For example, UE is moving from cell 1 to cell 2 and after successfully connected to cell 2, RLF happened and UE re-connects to cell 3 after one hour. In this case, it should not be regarded as wrong handover from cell1 to cell 3.

Observation 3: The time interval between connection failure and re-connection attempt is beneficial for failure cause detection.

Therefore besides cell identity, the time between the RLF and the first re-connection attempt should also be recorded for accurate MRO detection.

Proposal 4: Add “reconnectionTimeSinceFailure” besides NR/E-UTRAN attempt cell ID to the NR RLF Report.

Conclusion

According to the analysis in section 2, we propose:

Observation 1: CGI of re-connection attempt E-UTRAN cell is essential information for inter-system MRO detection.

Proposal 1: Add “Re-connection attempt cell CGI” of E-UTRAN cell to the NR RLF Report.

Proposal 2: Also include the TAC of re-connection attempt E-UTRAN cell.

Observation 2: Considering the scenario that eNB connects with 5GC, it is also necessary to include the CGI of re-connection attempt NR cell.

Proposal 3: Add “Re-connection attempt cell CGI” of NR cell to the NR RLF Report.

Observation 3: The time interval between connection failure and re-connection attempt is beneficial for failure cause detection.

Proposal 4: Add “reconnectionTimeSinceFailure” besides NR/E-UTRAN attempt cell ID to the NR RLF Report.

Reference

- R2-2002004 " Reply LS on Information Needed for MRO in UE RLF Report "

TP for TS38.331

This TP is based on “38331-g00” and “36331-g00”.

## ***Change on TS38.331***

5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE and it has acquired essential system information as described in 5.2.2.1.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T300;

1>	initiate transmission of the RRCSetupRequest message in accordance with 5.3.3.3;

1>	if the UE has radio link failure or handover failure information available and if the noSuitableCellFound is set and if the reconnectionCell is empty in VarRLF-Report:

2>	set the reconnectionCellIdentity in VarRLF-Report to include the cell information:

3>	set the nr in reconnectionCellIdentity to indicate global cell identity of current cell;

2>	set the reconnectionTimeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure;

- 6.2.2	Message definitions

–	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

UEInformationResponse message

-- ASN1START

-- TAG-UEINFORMATIONRESPONSE-START

UEInformationResponse-r16 ::=        SEQUENCE {

rrc-TransactionIdentifier            RRC-TransactionIdentifier,

criticalExtensions                   CHOICE {

ueInformationResponse-r16            UEInformationResponse-r16-IEs,

criticalExtensionsFuture             SEQUENCE {}

}

}

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16    OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16        OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL

}

ConnEstFailReport-r16 ::=            SEQUENCE {

measResultFailedCell-r16             MeasResultFailedCell-r16,

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultList2NR-r16           OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16        OPTIONAL

},

numberOfConnFail-r16                 INTEGER (0..7),

perRAInfoList-r16                            PerRAInfoList-r16                   OPTIONAL,

timeSinceFailure-r16                 TimeSinceFailure-r16,

...

}

MeasResultServingCell-r16 ::=        SEQUENCE {

physCellId                           PhysCellId                          OPTIONAL,

resultsSSB-Cell                      MeasQuantityResults                 OPTIONAL,

resultsSSB                           SEQUENCE{

best-ssb-Index                       SSB-Index,

best-ssb-Results                     MeasQuantityResults             OPTIONAL,

numberOfGoodSSB                      INTEGER (1..maxNrofSSBs-r16)    OPTIONAL

}                                                                        OPTIONAL,

...

}

MeasResultFailedCell-r16 ::=         SEQUENCE {

cgi-Info                             CGI-Info-Logging-r16,

physCellId-r16                       PhysCellId                          OPTIONAL,

measResult-r16                       SEQUENCE {

cellResults-r16                      SEQUENCE{

resultsSSB-Cell-r16                  MeasQuantityResults         OPTIONAL

},

rsIndexResults-r16                   SEQUENCE{

resultsSSB-Indexes-r16               ResultsPerSSB-IndexList     OPTIONAL

}                                                                    OPTIONAL

}

}

RA-ReportList-r16 ::= SEQUENCE (SIZE (1..maxRAReport-r16)) OF RA-Report-r16

RA-Report-r16 ::=                    SEQUENCE {

cellId-r16                           CGI-Info-LoggingDetailed-r16,

absoluteFrequencyPointA-r16          ARFCN-ValueNR,

locationAndBandwidth-r16             INTEGER (0..37949),

subcarrierSpacing-r16                SubcarrierSpacing,

msg1-FrequencyStart-r16              INTEGER (0..maxNrofPhysicalResourceBlocks-1),

msg1-SubcarrierSpacing-r16           SubcarrierSpacing,

msg1-FDM-r16                         ENUMERATED {one, two, four, eight},

raPurpose-r16                        ENUMERATED {accessRelated, beamFailureRecovery, reconfigurationWithSync, ulUnSynchronized,

schedulingRequestFailure, noPUCCHResourceAvailable, sCellAdditionTAAdjestment,

requestForOtherSI, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

perRAInfoList-r16                    PerRAInfoList-r16

}

PerRAInfoList-r16 ::= SEQUENCE (SIZE (1..200)) OF PerRAInfo-r16

PerRAInfo-r16 ::=                    CHOICE {

perRASSBInfoList-r16                 PerRASSBInfo-r16,

perRACSI-RSInfoList-r16              PerRACSI-RSInfo-r16

}

PerRASSBInfo-r16 ::=                 SEQUENCE {

ssb-Index-r16                        SSB-Index,

numberOfPreamblesSentOnSSB-r16       INTEGER (1..200),

perRAAttemptInfoList-r16             PerRAAttemptInfoList-r16

}

PerRACSI-RSInfo-r16 ::=              SEQUENCE {

csi-RS-Index-r16                     CSI-RS-Index,

numberOfPreamblesSentOnCSI-RS-r16    INTEGER (1..200),

perRAAttemptInfoList-r16             PerRAAttemptInfoList-r16

}

PerRAAttemptInfoList-r16 ::=         SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16

PerRAAttemptInfo-r16 ::=             SEQUENCE {

contentionDetected-r16               BOOLEAN,

dlRSRPAboveThreshold-r16             BOOLEAN,

...

}

RLF-Report-r16 ::=                   CHOICE {

nr-RLF-Report-r16                    SEQUENCE {

measResultLastServCell-r16           MeasResultRLFNR-r16,

measResultNeighCells-r16             SEQUENCE {

measResultListNR-r16                 MeasResultList2NR-r16       OPTIONAL,

measResultListEUTRA-r16              MeasResultList2EUTRA-r16    OPTIONAL

}                                                OPTIONAL,

c-RNTI-r16                           RNTI-Value,

previousPCellId-r16                  CGI-Info-LoggingDetailed-r16    OPTIONAL,

failedPCellId-r16                    CHOICE {

cellGlobalId-r16                     CGI-Info-LoggingDetailed-r16,

pci-arfcn-r16                        SEQUENCE {

physCellId-r16                       PhysCellId,

carrierFreq-r16                      ARFCN-ValueNR

}

}                                                                    OPTIONAL,

reestablishmentCellId-r16            CGI-Info-Logging-r16            OPTIONAL,

timeConnFailure-r16                  INTEGER (0..1023)               OPTIONAL,

timeSinceFailure-r16                 TimeSinceFailure-r16,

connectionFailureType-r16            ENUMERATED {rlf, hof}           OPTIONAL,

rlf-Cause-r16                        ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx,

beamFailureRecoveryFailure, spare4, spare3, spare2, spare1},

locationInfo-r16                     LocationInfo-r16                OPTIONAL,

absoluteFrequencyPointA-r16          ARFCN-ValueNR                   OPTIONAL,

locationAndBandwidth-r16             INTEGER (0..37949)              OPTIONAL,

subcarrierSpacing-r16                SubcarrierSpacing               OPTIONAL,

msg1-FrequencyStart-r16              INTEGER (0..maxNrofPhysicalResourceBlocks-1)  OPTIONAL,

msg1-SubcarrierSpacing-r16           SubcarrierSpacing    OPTIONAL,

msg1-FDM-r16                         ENUMERATED {one, two, four, eight}  OPTIONAL,

perRAInfoList-r16                    PerRAInfoList-r16               OPTIONAL,

noSuitableCellFound-r16              ENUMERATED {true}               OPTIONAL,

reconnectionCell-r16             	 SEQUENCE {

reconnectionCellIdentity-r16        	CHOICE {

nr-r16									CGI-Info-Logging-r16,

e-utran-r16								CGI-InfoEUTRALogging

},

reconnectionTimeSinceFailure-r16     INTEGER (0..1023)

}                                                                    OPTIONAL

},

eutra-RLF-Report-r16                 SEQUENCE {

failedPCellId-EUTRA                  CGI-InfoEUTRALogging,

measResult-RLF-Report-EUTRA-r16      OCTET STRING

}

}

MeasResultList2NR-r16 ::=            SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2NR-r16

MeasResultList2EUTRA-r16 ::=         SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2EUTRA-r16

MeasResult2NR-r16 ::=                SEQUENCE {

ssbFrequency-r16                     ARFCN-ValueNR                       OPTIONAL,

refFreqCSI-RS-r16                    ARFCN-ValueNR                       OPTIONAL,

measResultList-r16                   MeasResultListNR

}

MeasResultListLogging2NR-r16 ::=     SEQUENCE(SIZE (1..maxFreq)) OF MeasResultListLoggingNR-r16

MeasResultListLoggingNR-r16 ::=      SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultLoggingNR-r16

MeasResultLoggingNR-r16 ::=          SEQUENCE {

physCellId-r16                       PhysCellId,

resultsSSB-Cell-r16                  MeasQuantityResults,

numberOfGoodSSB-r16                  INTEGER (1..maxNrofSSBs-r16) OPTIONAL

}

MeasResult2EUTRA-r16 ::=             SEQUENCE {

carrierFreq-r16                      ARFCN-ValueEUTRA,

measResultList-r16                   MeasResultListEUTRA

}

MeasResultRLFNR-r16 ::=              SEQUENCE {

measResult-r16                       SEQUENCE {

cellResults-r16                      SEQUENCE{

resultsSSB-Cell-r16                  MeasQuantityResults         OPTIONAL,

resultsCSI-RS-Cell-r16               MeasQuantityResults         OPTIONAL

},

rsIndexResults-r16                   SEQUENCE{

resultsSSB-Indexes-r16               ResultsPerSSB-IndexList     OPTIONAL,

ssbRLMConfigBitmap-r16               BIT STRING (SIZE (64))      OPTIONAL,

resultsCSI-RS-Indexes-r16            ResultsPerCSI-RS-IndexList  OPTIONAL,

csi-rsRLMConfigBitmap-r16            BIT STRING (SIZE (96))      OPTIONAL

}                                                                    OPTIONAL

}

}

TimeSinceFailure-r16 ::= INTEGER (0..172800)

MobilityHistoryReport-r16 ::= VisitedCellInfoList-r16

-- TAG-UEINFORMATIONRESPONSE-STOP

-- ASN1STOP

UEInformationResponse-IEs field descriptions

logMeasReport This field is used to provide the measurement results stored by the UE associated to logged MDT.

measResultIdleEUTRA EUTRA measurement results performed during RRC_INACTIVE or RRC_IDLE.

measResultIdleNR NR measurement results performed during RRC_INACTIVE or RRC_IDLE.

ra-Report This field is used to provide the list of RA reports that is stored by the UE for the past upto maxRAReport-r16 number of successful random access procedues.

rlf-Report This field is used to indicated the RLF report related contents.

LogMeasReport field descriptions

absoluteTimeStamp Indicates the absolute time when the logged measurement configuration logging is provided, as indicated by E-UTRAN within absoluteTimeInfo.

logMeasResultListBT This field refers to the Bluetooth measurement results.

logMeasResultListWLAN This field refers to the WLAN measurement results.

measResultServCell This field refers to the log measurement results taken in the Serving cell.

relativeTimeStamp Indicates the time of logging measurement results, measured relative to the absoluteTimeStamp. Value in seconds.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

timeStamp Includes time stamps for the waypoints that describe planned locations for the UE.

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

ConnEstFailReport field descriptions

measResultFailedCell This field refers to the last measurement results taken in the cell, where connection establishment failure happened.

measResultNeighCells This field refers to the neighbour cell measurements when connection establishment failure happened.

numberOfConnFail This field is used to indicate the number of failed connection setup attempts after radio link failure.

numberOfPreamblesSent This field is used to indicate the number of random access preambles that were transmitted.

maxTxPowerReached This field is used to indicate whether or not the maximum power level was used for the last transmitted preamble.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (establishment) failure. Value in seconds. The maximum value 172800 means 172800s or longer.

RA-Report field descriptions

absoluteFrequencyPointA This field indicates the absolute frequency position of the reference resource block (Common RB 0).

cellID This field indicates the CGI of the cell in which the associated random access procedure was performed.

contentionDetected This field is used to indicate that contention was detected for the transmitted preamble in the given random access attempt or not.

csi-RS-Index This field is used to indicate the CSI-RS index corresponding to the random access attempt.

dlRSRPAboveThreshold This field is used to indicate whether the DL beam (SSB or CSI-RS) qualtiy associated to the random access attempt was above or below the threshold (rsrp-ThresholdSSB when NUL is used and rsrp-ThresholdSSB-SUL when SUL is used).

locationAndBandwidth Frequency domain location and bandwidth of the bandwidth part associated to the random-access resources used by the UE.

msg1-FrequencyStart Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0 of the UL BWP.

msg1-SubcarrierSpacing Subcarrier spacing of PRACH resources.

numberOfPreamblesSentOnCSI-RS This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding CSI-RS.

numberOfPreamblesSentOnSSB This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding SSB/PBCH block.

perRAAttemptInfoList This field provides detailed information about a random access attempt.

perRAInfoList This field provides detailed information about each of the random access attempts in the chronological order of the random access attempts.

perRACSI-RSInfoList This field provides detailed information about the successive random acess attempts associated to the same CSI-RS.

perRASSBInfoList This field provides detailed information about the successive random access attempts associated to the same SS/PBCH block.

raPurpose This field is used to indicate the RA scenario for which the RA report entry is triggered. The RA accesses associated to Initial access from RRC_IDLE, transition from RRC-INACTIVE and the MSG3 based SI request are indicated using the indicator 'accessRelated'.

ssb-Index This field is used to indicate the SS/PBCH index of the SS/PBCH block corresponding to the random access attempt.

ssbRSRPQualityIndicator This field is used to indicate the SS/PBCH RSRP of the SS/PBCH block corresponding to the random access attempt is above rsrp-ThresholdSSB or not.

subcarrierSpacing Subcarrier spacing used in the BWP associated to the random-access resources used by the UE.

RLF-Report field descriptions

connectionFailureType This field is used to indicate whether the connection failure is due to radio link failure or handover failure.

csi-rsRLMConfigBitmap This field is used to indicate the CSI-RS indexes that are also part of the RLM configurations.

c-RNTI This field indicates the C-RNTI used in the PCell upon detecting radio link failure or the C-RNTI used in the source PCell upon handover failure.

failedCellId This field is used to indicate the cell in which connection establishment failed.

failedPCellId This field is used to indicate the PCell in which RLF is detected or the target PCell of the failed handover. The UE sets the ARFCN according to the frequency band used for transmission/ reception when the failure occurred.

failedPCellId-EUTRA This field is used to indicate the PCell in which RLF is detected or the target PCell of the failed handover in an E-UTRA RLF report.

measResultLastServCell This field refers to the last measurement results taken in the PCell, where radio link failure or handover failure happened.

measResultListEUTRA This field refers to the last measurement results taken in the neighboring EUTRA Cells, when the radio link failure or handover failure happened.

measResultListNR This field refers to the last measurement results taken in the neighboring NR Cells, when the radio link failure or handover failure happened. UE does not include the resultsSSB-Indexes IE, if the measResultListNR IE is included in the LogMeasInfo-r16 IE.

measResultServCell This field refers to the log measurement results taken in the Serving cell.

measResult-RLF-Report-EUTRA Includes the E-UTRA RLF-Report-r9 IE as specified in TS 36.331 [10].

noSuitableCellFound This field is set by the UE when the T311 expires.

previousPCellId This field is used to indicate the source PCell of the last handover (source PCell when the last RRCReconfiguration message including reconfigurationWithSync was received).

reestablishmentCellId This field is used to indicate the cell in which the re-establishment attempt was made after connection failure.

rlf-Cause This field is used to indicate the cause of the last radio link failure that was detected. In case of handover failure information reporting (i.e., the connectionFailureType is set to 'hof'), the UE is allowed to set this field to any value.

reconnectionCell This field is used to indicate the first cell UE attempted to establish if the re-establishment after the last HOF/RLF is not successful.

reconnectionTimeSinceFailure This field is used to indicate the time elapsed since the last HOF/RLF until the first establishment if the re-establishment after the last HOF/RLF is not successful. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

ssbRLMConfigBitmap This field is used to indicate the SS/PBCH block indexes that are also part of the RLM configurations.

timeConnFailure This field is used to indicate the time elapsed since the last HO initialization until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (establishment) failure. Value in seconds. The maximum value 172800 means 172800s or longer.

## ***Change on TS36.331***

5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment or resume of an RRC connection while the UE is in RRC_IDLE or when upper layers request resume of an RRC connection or RRC layer requests resume of an RRC connection for, e.g. RNAU or reception of RAN paging while the UE is in RRC_INACTIVE.

Except for NB-IoT, upon initiation of the procedure, if the UE is connected to EPC, the UE shall:

1>	if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

1>	else

2>	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SystemInformationBlockType2;

1>	if SystemInformationBlockType2 contains acdc-BarringPerPLMN-List and the acdc-BarringPerPLMN-List contains an ACDC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the ACDC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected ACDC-BarringPerPLMN entry for ACDC barring check (i.e. presence or absence of access barring parameters in this entry) irrespective of the acdc-BarringForCommon parameters included in SystemInformationBlockType2;

1>	else:

2>	in the remainder of this procedure use the acdc-BarringForCommon (i.e. presence or absence of these parameters) included in SystemInformationBlockType2 for ACDC barring check;

1>	if upper layers indicate that the RRC connection is subject to EAB (see TS 24.301 [35]):

2>	if the result of the EAB check, as specified in 5.3.3.12, is that access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that EAB is applicable, upon which the procedure ends;

1>	if upper layers indicate that the RRC connection is subject to ACDC (see TS 24.301 [35]), SystemInformationBlockType2 contains BarringPerACDC-CategoryList, and acdc-HPLMNonly indicates that ACDC is applicable for the UE:

2>	if the BarringPerACDC-CategoryList contains a BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers:

3>	select the BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers;

2>	else:

3>	select the last BarringPerACDC-Category entry in the BarringPerACDC-CategoryList;

2>	stop timer T308, if running;

2>	perform access barring check as specified in 5.3.3.13, using T308 as "Tbarring" and acdc-BarringConfig in the BarringPerACDC-Category as "ACDC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring is applicable due to ACDC, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile terminating calls:

2>	if timer T302 is running:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile terminating calls is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for emergency calls:

2>	if SystemInformationBlockType2 includes the ac-BarringInfo:

3>	if the ac-BarringForEmergency is set to TRUE:

4>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11]:

NOTE 1:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

5>	if the ac-BarringInfo includes ac-BarringForMO-Data, and for all of these valid Access Classes for the UE, the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to one:

6>	consider access to the cell as barred;

4>	else:

5>	consider access to the cell as barred;

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

2>	if access to the cell is barred:

3>	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls is applicable, upon which the procedure ends;

3>	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

4>	if timer T306 is not running, start T306 with the timer value of T303;

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating signalling:

2>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating CS fallback:

2>	if SystemInformationBlockType2 includes ac-BarringForCSFB:

3>	perform access barring check as specified in 5.3.3.11, using T306 as "Tbarring" and ac-BarringForCSFB as "AC barring parameter";

3>	if access to the cell is barred:

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating CS fallback is applicable, due to ac-BarringForCSFB, upon which the procedure ends;

2>	else:

3>	perform access barring check as specified in 5.3.3.11, using T306 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

3>	if access to the cell is barred:

4>	if timer T303 is not running, start T303 with the timer value of T306;

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating CS fallback and mobile originating calls is applicable, due to ac-BarringForMO-Data, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating MMTEL voice, mobile originating MMTEL video, mobile originating SMSoIP or mobile originating SMS; or

1>	if the UE is establishing the RRC connection after EPS fallback for IMS voice (see TS 23.502 [102]) was triggered in NR via RRCRelease with voiceFallbackIndication (see TS 38.331 [82]):

2>	if the UE is establishing the RRC connection for mobile originating MMTEL voice and SystemInformationBlockType2 includes ac-BarringSkipForMMTELVoice; or

2>	if the UE is establishing the RRC connection for mobile originating MMTEL video and SystemInformationBlockType2 includes ac-BarringSkipForMMTELVideo; or

2>	if the UE is establishing the RRC connection for mobile originating SMSoIP or SMS and SystemInformationBlockType2 includes ac-BarringSkipForSMS:

3>	consider access to the cell as not barred;

2>	else:

3>	if establishmentCause received from higher layers is set to mo-Signalling (including the case that mo-Signalling is replaced by highPriorityAccess according to TS 24.301 [35] or by mo-VoiceCall according to the clause 5.3.3.3):

4>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

4>	if access to the cell is barred:

5>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

3>	if establishmentCause received from higher layers is set to mo-Data (including the case that mo-Data is replaced by highPriorityAccess according to TS 24.301 [35] or by mo-VoiceCall according to the clause 5.3.3.3):

4>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

4>	if access to the cell is barred:

5>	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

6>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls is applicable, upon which the procedure ends;

5>	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

6>	if timer T306 is not running, start T306 with the timer value of T303;

6>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

Upon initiation of the procedure, if the UE is connected to 5GC, the UE shall:

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.16 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.16 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.16 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNAU:

2>	if an emergency service is ongoing:

3>	select '2' as the Access Category;

3>	set the resumeCause to emergency;

2>	else:

3>	select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.16 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [95];

3>	if the access attempt is barred:

4>	set the variable pendingRnaUpdate to 'TRUE';

4>	the procedure ends;

Except for NB-IoT, upon initiating the procedure, if connected to EPC or 5GC, the UE shall:

1>	if the UE is resuming an RRC connection from a suspended RRC connection or from RRC_INACTIVE:

2>	if the UE was configured with (NG)EN-DC:

3>	if the UE does not support maintaining SCG configuration upon connection resumption:

4>	perform MR-DC release, as specified in TS 38.331 [82], clause 5.3.5.10;

3>	release p-MaxEUTRA, if configured;

3>	release p-MaxUE-FR1, if configured;

3>	release tdm-PatternConfig, if configured;

2>	if the UE does not support maintaining the MCG SCell configurations upon connection resumption:

3>	release the MCG SCell(s), if configured, in accordance with 5.3.10.3a;

2>	release powerPrefIndicationConfig, if configured and stop timer T340, if running;

2>	release reportProximityConfig and clear any associated proximity status reporting timer;

2>	release obtainLocationConfig, if configured;

2>	release idc-Config, if configured;

2>	release sps-AssistanceInfoReport, if configured;

2>	release measSubframePatternPCell, if configured;

2>	if the UE was configured with DC:

3>	release the entire SCG configuration, if configured, except for the DRB configuration (as configured by drb-ToAddModListSCG);

2>	release naics-Info for the PCell, if configured;

2>	release the LWA configuration, if configured, as described in 5.6.14.3;

2>	release the LWIP configuration, if configured, as described in 5.6.17.3;

2>	release bw-PreferenceIndicationTimer, if configured and stop timer T341, if running;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release ailc-BitConfig, if configured;

2>	release uplinkDataCompression, if configured;

NOTE 1a:	The parameters and configurations are released from the UE Inactive AS context if the UE is resuming an RRC connection from RRC_INACTIVE.

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	start timer T300;

1>	if the UE is resuming an RRC connection from a suspended RRC connection:

2>	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

1>	else if the UE is resuming an RRC connection from RRC_INACTIVE:

2>	set the variable pendingRnaUpdate to 'FALSE';

2>	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

1>	else:

2>	if stored, discard the UE AS context, UE Inactive AS context and resumeIdentity;

2>	release rrc-InactiveConfig, if configured;

2>	if the UE is initiating CP-EDT in accordance with conditions in 5.3.3.1b; or

2>	if the UE is initiating CP transmission using PUR in accordance with conditions in 5.3.3.1c:

3>	initiate transmission of the RRCEarlyDataRequest message in accordance with 5.3.3.3b;

2>	else:

3>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state or UEs in RRC_INACTIVE. However, the UE needs to perform system information acquisition upon cell re-selection.

1>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 38.331 [82] and if the reconnectionCell in VarRLF-Report of TS 38.331 [82] is empty:

2>	set the reconnectionCellIdentity in VarRLF-Report of TS 38.331 [82] to include the cell information:

3>	set the e-utran in reconnectionCellIdentity to indicate global cell identity and the tracking area code of current cell;

2>	set the reconnectionTimeSinceFailure in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the last radio link failure or handover failure;

For NB-IoT, upon initiation of the procedure, the UE shall:

1>	if the UE is connected to EPC:

2>	if the UE is establishing or resuming the RRC connection for mobile originating exception data; or

2>	if the UE is establishing or resuming the RRC connection for mobile originating data; or

2>	if the UE is establishing or resuming the RRC connection for delay tolerant access; or

2>	if the UE is establishing or resuming the RRC connection for mobile originating signalling;

3>	perform access barring check as specified in 5.3.3.14;

3>	if access to the cell is barred:

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring is applicable, upon which the procedure ends;

1>	if the UE is connected to 5GC:

2>	if the Access Category provided by the upper layers is different from '0':

3>	perform access barring check for per-NRSRP barring as specified in 5.3.3.14;

3>	if access to the cell is barred:

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication, upon which the procedure ends;

3>	else:

4>	perform the unified access control procedure as specified in 5.3.16 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	start timer T300;

1>	if the UE is establishing an RRC connection:

2>	if stored, discard the UE AS context and resumeIdentity;

2>	if the UE is initiating CP-EDT in accordance with conditions in 5.3.3.1b; or

2>	if the UE is initiating CP transmission using PUR in accordance with conditions in 5.3.3.1c:

3>	initiate transmission of the RRCEarlyDataRequest message in accordance with 5.3.3.3b;

2>	else:

3>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

1>	else if the UE is resuming an RRC connection:

2>	release schedulingRequestConfig, if configured;

2>	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

NOTE 3:	Upon initiating the connection establishment or resumption procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

NOTE 4:	For EDT and transmission using PUR, upon initiating the connection establishment or resumption procedure, it is up to UE implementation whether to continue cell re-selection related measurements as well as cell re-selection evaluation and, if the conditions for cell re-selection are fulfilled, whether to perform cell re-selection as specified in 5.3.3.5.

Editor's Note: Where to capture PUR release due to RACH initiation on a new cell.